| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0024_r2 |
| --- | --- |

| Title: | CE4 summary report: Inter prediction and motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang Shan Liu Kai Zhang | Email: | haitao.yang@huawei.com shanl@tencent.com zhangkai.video@bytedance.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

This contribution provides a summary report of Core Experiment 4 on inter prediction and motion vector coding. CE4 comprises seven categories, 1) affine motion compensation, 2) merge mode enhancement, 3) motion vector coding, 4) generalized bi-prediction, 5) reference picture boundary padding, 6) local illumination compensation, and 7) interpolation filter improvement. Test results against VTM anchor are provided to show the coding efficiency and complexity trade-off of each tool. Test results against BMS anchor are also provided to show the interaction with BMS coding tools. Crosschecking results are integrated in this contribution.

CE4.1: Affine motion compensation

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.1.1.a | BMS Affine with 4x4 sub-block only | JVET-K0184 (Huawei) |
| 4.1.1.b | BMS Affine with 8x8 sub-block only |  |
| 4.1.2.a | Affine flexing using sub-block size of 4x4 | JVET-K0047 (Nokia) |
| 4.1.2.b | Affine flexing using sub-block size of 8x8 |  |
| 4.1.3.a | Affine MVP construction | JVET-K0337 (Qualcomm) |
| 4.1.3.b | 4.1.3.a + MVD prediction |  |
| 4.1.3.c | 4.1.3.b + CU level 4-para/6-para switching |  |
| 4.1.3.d | 4.1.3.c + slice level 4-para/6-para switching |  |
| 4.1.4.a | Affine model seed storage | JVET-K0218 (Technicolor) |
| 4.1.4.b | Affine MVP construction with added spatial candidates |  |
| 4.1.4.c | Withdraw |  |
| 4.1.4.d | 4.1.4.a + 4.1.4.b |  |
| 4.1.5.a | Affine MVP construction | JVET-K0185 (Huawei) |
| 4.1.5.b | 4.1.5.a + MVD coding |  |
| 4.1.5.c | 4.1.5.b + 4 para/6-para switching |  |
| 4.1.5.d | EIF on top of BMS Affine |  |
| 4.1.5.e | EIF on top 4.1.5.c |  |
| 4.1.6 | Affine MVP construction | JVET-K0244 (MediaTek) |
| 4.1.7.a | Up to two affine candidate for affine inter | JVET-K0094 (LGE) |
| 4.1.7.b | Four and six parameter model |  |
| 4.1.7.c | Four and six parameter model with the conditional affine parameter signaling |  |
| 4.1.8.a | Alternative 3-para (scaling) motion model | JVET-K0124 (Sony) |
| 4.1.8.b | Alternative 3-para (rotation) motion model |  |
| 4.1.8.c | 4.1.8.a + 4.1.8.b |  |

Test results

Random access

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| BMS AFFINE | -2.99% | -2.19% | -2.21% | 137% | 112% | -1.92% | -1.34% | -1.36% | 108% | 102% |
| 4.1.1.a | -3.00% | -2.22% | -2.21% | 138% | 113% | -1.92% | -1.36% | -1.40% | 109% | 103% |
| 4.1.1.b | -2.43% | -1.73% | -1.71% | 130% | 106% | -1.40% | -1.01% | -0.99% | 109% | 100% |
| 4.1.2.a | -3.31% | -2.38% | -2.37% | 131% | 113% | -2.13% | -1.46% | -1.45% | 105% | 100% |
| 4.1.2.b | -2.90% | -1.97% | -1.93% | 126% | 107% | -1.81% | -1.10% | -1.09% | 105% | 98% |
| 4.1.3.a | -3.22% | -2.40% | -2.43% | 134% | 110% | -2.12% | -1.54% | -1.56% | 112% | 107% |
| 4.1.3.b | -3.42% | -2.55% | -2.61% | 136% | 112% | -2.24% | -1.61% | -1.67% | 111% | 108% |
| 4.1.3.c | -3.90% | -2.94% | -2.92% | 160% | 113% | -2.72% | -1.96% | -1.97% | 118% | 107% |
| 4.1.3.d | -3.88% | -2.95% | -2.92% | 146% | 113% | -2.69% | -1.98% | -2.00% | 112% | 106% |
| 4.1.4.a | -3.90% | -2.95% | -2.98% | 148% | 115% | -2.72% | -2.01% | -2.02% | 113% | 107% |
| 4.1.4.b | -3.89% | -2.93% | -2.95% | 148% | 115% | -2.71% | -2.02% | -2.05% | 113% | 106% |
| 4.1.4.c | / | / | / | / | / | / | / | / | / | / |
| 4.1.4.d | -3.91% | -2.96% | -3.00% | 148% | 115% | -2.72% | -2.01% | -2.01% | 113% | 106% |
| 4.1.5.a | -3.24% | -2.41% | -2.47% | 138% | 110% | -2.10% | -1.54% | -1.53% | 110% | 101% |
| 4.1.5.b | -3.23% | -2.41% | -2.43% | 139% | 110% | -2.10% | -1.52% | -1.53% | 110% | 101% |
| 4.1.5.c | -3.83% | -2.86% | -2.89% | 144% | 111% | -2.65% | -1.91% | -1.94% | 111% | 102% |
| 4.1.5.d | -3.23% | -2.41% | -2.52% | 132% | 102% | -1.85% | -1.42% | -1.44% | 109% | 99% |
| 4.1.5.e | -4.20% | -3.08% | -3.30% | 137% | 103% | -2.67% | -2.00% | -2.07% | 109% | 99% |
| 4.1.6 | -3.21% | -2.38% | -2.40% | 132% | 107% | -2.10% | -1.50% | -1.56% | 111% | 88% |
| 4.1.7.a | -3.23% | -2.37% | -2.43% | 142% | 114% | -2.10% | -1.52% | -1.56% | 111% | 102% |
| 4.1.7.b | -3.53% | -2.56% | -2.62% | 200% | 116% | -2.40% | -1.69% | -1.70% | 128% | 102% |
| 4.1.7.c | -3.48% | -2.54% | -2.57% | 159% | 116% | -2.31% | -1.67% | -1.65% | 114% | 102% |
| 4.1.8.a | -3.10% | -2.27% | -2.29% | 181% | 114% | -1.99% | -1.40% | -1.42% | 118% | 101% |
| 4.1.8.b | -3.07% | -2.26% | -2.28% | 181% | 112% | -1.97% | -1.41% | -1.44% | 118% | 101% |
| 4.1.8.c | -3.19% | -2.32% | -2.31% | 221% | 114% | -2.06% | -1.44% | -1.50% | 127% | 102% |

Low delay B

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| BMS AFFINE | -2.06% | -1.33% | -1.52% | 168% | 108% | -1.98% | -1.52% | -1.70% | 125% | 104% |
| 4.1.1.a | -2.09% | -1.22% | -1.49% | 168% | 109% | -2.06% | -1.59% | -1.49% | 125% | 105% |
| 4.1.1.b | -1.60% | -0.99% | -1.03% | 156% | 103% | -1.64% | -1.10% | -1.33% | 122% | 101% |
| 4.1.2.a | -2.27% | -1.48% | -1.48% | 161% | 111% | -2.18% | -1.79% | -1.61% | 122% | 102% |
| 4.1.2.b | -1.94% | -1.08% | -1.17% | 152% | 111% | -1.94% | -1.40% | -1.65% | 119% | 102% |
| 4.1.3.a | -2.08% | -1.46% | -1.62% | 162% | 92% | -2.08% | -1.62% | -1.92% | 127% | 108% |
| 4.1.3.b | -2.23% | -1.50% | -1.60% | 164% | 99% | -2.21% | -1.80% | -1.84% | 127% | 109% |
| 4.1.3.c | -2.35% | -1.53% | -1.82% | 210% | 103% | -2.37% | -1.91% | -1.89% | 145% | 107% |
| 4.1.3.d | -2.37% | -1.53% | -1.53% | 199% | 105% | -2.36% | -1.92% | -1.74% | 139% | 109% |
| 4.1.4.a | -2.38% | -1.61% | -1.59% | 213% | 107% | -2.39% | -1.89% | -2.07% | 142% | 109% |
| 4.1.4.b | -2.37% | -1.56% | -1.65% | 213% | 107% | -2.39% | -1.98% | -2.03% | 143% | 110% |
| 4.1.4.c | / | / | / | / | / | / | / | / | / | / |
| 4.1.4.d | -2.39% | -1.59% | -1.75% | 213% | 108% | -2.42% | -1.93% | -1.85% | 142% | 108% |
| 4.1.5.a | -2.13% | -1.26% | -1.59% | 169% | 107% | -2.07% | -1.69% | -1.57% | 126% | 103% |
| 4.1.5.b | -2.11% | -1.40% | -1.51% | 170% | 107% | -2.07% | -1.72% | -1.86% | 126% | 104% |
| 4.1.5.c | -2.36% | -1.54% | -1.47% | 174% | 108% | -2.31% | -1.69% | -2.13% | 128% | 104% |
| 4.1.5.d | -2.47% | -1.61% | -1.94% | 158% | 102% | -1.89% | -1.58% | -1.64% | 122% | 100% |
| 4.1.5.e | -2.95% | -1.84% | -2.02% | 165% | 102% | -2.21% | -1.60% | -1.73% | 124% | 99% |
| 4.1.6 | -2.10% | -1.43% | -1.48% | 159% | 104% | -2.04% | -1.57% | -1.74% | 128% | 108% |
| 4.1.7.a | -2.12% | -1.46% | -1.47% | 181% | 115% | -2.09% | -1.87% | -1.89% | 129% | 104% |
| 4.1.7.b | -2.33% | -1.49% | -1.60% | 297% | 116% | -2.35% | -1.85% | -1.90% | 170% | 105% |
| 4.1.7.c | -2.28% | -1.43% | -1.42% | 218% | 117% | -2.27% | -1.61% | -1.75% | 141% | 106% |
| 4.1.8.a | -2.12% | -1.31% | -1.39% | 252% | 113% | -2.10% | -1.56% | -1.68% | 152% | 104% |
| 4.1.8.b | -2.12% | -1.28% | -1.34% | 252% | 113% | -2.08% | -1.72% | -1.64% | 152% | 104% |
| 4.1.8.c | -2.15% | -1.35% | -1.55% | 324% | 114% | -2.14% | -1.71% | -1.77% | 176% | 104% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.1.1.a | Peter Chuang(MediaTek) | 2 | F | Y | M |
| 4.1.1.b | Peter Chuang(MediaTek) | 2 | P | Y | M |
| 4.1.2.a | Timofey Solovyev (Huawei) | 2 | F | Y | G |
| 4.1.2.b | Timofey Solovyev (Huawei) | 2 | F | Y | M |
| 4.1.3.a | Huanbang Chen (Huawei) | 2 | F | Y | G |
| 4.1.3.b | Huanbang Chen (Huawei) | 2 | F | Y | G |
| 4.1.3.c | Huanbang Chen (Huawei) | 2 | F | Y | G |
| 4.1.3.d | Huanbang Chen (Huawei) | 2 | F | Y | G |
| 4.1.4.a | Huanbang Chen (Huawei) | 2 | P | N | M |
| 4.1.4.b | Huanbang Chen (Huawei) | 2 | P | N | M |
| 4.1.4.c | / | / | / | / |  |
| 4.1.4.d | Huanbang Chen (Huawei) | 2 | P | N | M |
| 4.1.5.a | Fangdong Chen (Hikvision) | 2 | F | Y | M |
| 4.1.5.b | Fangdong Chen (Hikvision) | 2 | F | Y | M |
| 4.1.5.c | Fangdong Chen (Hikvision) | 2 | F | Y | M |
| 4.1.5.d | Fangdong Chen (Hikvision) | 2 | F | Y | M |
| 4.1.5.e | Fangdong Chen (Hikvision) | 2 | F | Y | M |
| 4.1.6 | Guichun Li (Tencent) | 2 | F | Y | M |
| 4.1.7.a | Kenji Kondo (Sony) | 2 | F | Y | M |
| 4.1.7.b | Kenji Kondo (Sony) | 2 | F | Y | M |
| 4.1.7.c | Kenji Kondo (Sony) | 2 | F | Y | M |
| 4.1.8.a | Anish Tamse (Samsung) | 2 | F | Y | G |
| 4.1.8.b | Anish Tamse (Samsung) | 2 | F | Y | G |
| 4.1.8.c | Anish Tamse (Samsung) | 2 | F | Y | G |

Crosschecker comments on CE4.1.2

Higher (+8-9%) encoder complexity was observed for VTM, but for "clean affine" +6% encoder complexity was also noticed.  Decoder complexity for VTM, enc/dec for BMS generally match.

Crosschecker comments on CE4.1.3

The running time especially the decoding time provided by the tester fluctuates too much. The tester agrees that encoding/decoding times from crosschecker are more reasonable.

Crosschecker comments on CE4.1.4

Different results are observed when using different compiler.

Enc/Dec mismatch was found in Test a (RitualDance, qp37, LB, BMS cfg).

Crosschecker comments on CE4.1.7

Encoder of test c is faster than test b. The reason is expected that it may skip the affine search process by checking neighbouring blocks.

Crosschecker comments on CE4.1.8

Lower encoder and decoder complexity observed for VTM (both RA and LD).

Higher decoder complexity observed for BMS RA low resolutions (class C and D).

Detailed description of tests

CE4.1.1 in JVET-K0184 (Huawei)

This contribution reports the results of affine motion compensation fixed the sub-block size 4x4 (test a) and 8x8 (test b).

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.1 | BMS a) BMS Affine with 4x4 sub-block only b) BMS Affine with 8x8 sub-block only | Huanbang Chen (Huawei) | Tzu-Der Chuang (MediaTek) |

CE4.1.2 in JVET-K0047 (Nokia)

The approach performs block-based motion compensation followed by a resampling or "flexing" filtering of the prediction block. The original proposal is based on 4x4 luma block motion compensation as in JEM 7.0. In addition, a version with 8x8 luma block compensation was tested. The test was using the 16-position JEM motion compensation filter for the flexing operation.

The affine flexing tool calculates the horizontal and vertical motion vector differences on the affine sub-PU boundaries. Based on whether the nature of motion is more rotational or zooming, the method decides to apply either lateral compensation (moving the sub-PU boundary samples laterally with respect to the neighboring sub-PU samples) or perpendicular compensation (moving the sub-PU boundary samples away or closer to the samples of the neighboring sub-PU), respectively. The remapping offsets applied to different lines are calculated based on motion vector difference and distance from the sub-PU border. A look-up table implementation is provided in the CE software. An example of lateral processing is illustrated in the figure below.

Further details:

- Flexing is applied to both horizontally and vertically, and the result of the flexing in first dimension is used as an input to the second dimension

- Flexing is applied only for the luma channel in the case of 4:2:0 video

- Line ends are padded with the last available samples on the line

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.2 | a) Affine flexing using sub-block size of 4x4 b) Affine flexing using sub-block size of 8x8 | Jani Lainema (Nokia) | Solovyev Timofey (Huawei) |

CE4.1.3 in JVET-K0337 (Qualcomm)

Test 4.1.3.a: Affine mvp construction

Three types of affine mv predictor set

- Inherited affine mv predictor set from neighboring blocks in affine mode, if the same reference picture is referred to.

Up to two different affine MV predictor sets are derived from affine motion of the neighboring blocks.  Neighboring blocks A0, A1, B0, B1, and B2 as shown in Figure 2 are checked. If the neighboring block is coded using affine motion model and its reference frame is same as the reference frame of the current block, MVs at two (for 4-parameter affine model) or three (for 6-parameter affine model) control points of the current block are derived from the affine model of this neighbor.

- Constructed virtual affine mv predictor set from neighboring mv, if the same reference picture is referred to.

The neighboring MVs are divided into three groups: S0=mvA,mvB,mvC , S1=mvD,mvE and S2=mvF,mvG. mv0  is the first MV in S0 that refers to the same reference picture as the current block; mv1 is the first MV in S1 that refers to the same reference picture of the current block; and mv2 is the first in S2 that refers to the same reference picture of the current block.

- If only mv0 and mv1 can be found, mv2 is derived as:

mv2x=mv0x-hmv1y-mv0yw, mv2y=mv0y+hmv1x-mv0xw,

where the current block size is W×H.

- If only mv0 and mv2 can be found, mv1 is derived as:

mv1x=mv0x+hmv2y-mv0yw, mv1y=mv0y-hmv2x-mv0xw.

- HEVC affine mv predictor

Test 4.1.3.b: Affine mv prediction

Two candidate sets with two (three) candidates mv0, mv1  mv0, mv1, mv2 are used to predict two (three) control points of the affine motion model. Given motion vector difference vectors, mvd0, mvd1, mvd2, the control points are calculated:

mv0=mv0+mvd0

mv1=mv1+mvd1+mvd0

mv2=mv2+mvd2+mvd0

Test 4.1.3.c&d: 4-param/6-param affine model switching at CU/slice level

The affine models are adaptively selected at both slice level and block level. Slice header flags affine4_flag and affine6_flag indicate whether the 4-parameter affine and 6-parameter affine model are applied. If one of them is equal to 1, affine_flag signaled at block level indicates whether the allowed affine model is used for this block. If both are equal to 1, there is an affine_type flag signaled at block level. If affine_type is equal to 1, the 6-parameter affine model is used, otherwise the 4-parameter affine model is applied. To determine the affine4_flag and affine6_flag for the current slice the statistics from the previous coded slice are used.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.3 | a) Affine MVP construction b) a + MVD prediction c) b + CU level 4-para/6-para switching d) c + slice level 4-para/6-para switching | Yu Han (Qualcomm) | Huanbang Chen (Huawei) |

CE4.1.4 in JVET-K0218 (Technicolor)

Test 4.1.4.a: Affine motion seed storage

In the JEM, the affine model seeds are stored in the top-left, top-right and bottom-left 4x4 sub-blocks in the considered CU. In the proposed solution, the affine model seeds are stored separately as a motion information associated to the whole CU. The motion model is thus decoupled from the motion vectors used for actual motion compensation at the 4x4 block level. This storage allows preserving the complete motion vector field at the 4x4 sub-block level. It also allows using affine motion compensation for block of size 4 in width or height.

Test 4.1.4.b: Affine motion vector prediction

|  | JEM | Proposed |
| --- | --- | --- |
| Motion information predictor composition | inherited MV triplet | inherited MV triplet |
| Candidate list | 2 candidates, among: - constructed affine models from spatial neighbors {a1, b1, a0, b0, b2} - classical AMVP candidates | 2 candidates, among: - spatial Affine positions {b1, a1, a0, b0, b2, a2, b3} - improved constructed affine models from spatial neighbors - classical AMVP candidates - zero MVs |
| Signaling | For each reference picture: - reference picture idx - predictor idx - 2 MVDs Affine flag Residual is coded | For each reference picture: - reference picture idx - predictor idx - 2 MVDs Affine flag Residual is coded |

The number of affine merge candidates is up to 8.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.4 | a) Affine model seed storage b) Affine MVP construction with added spatial and temporal c) Template affine derivation for MVP construction d) a + b + c | A. Robert (Technicolor) | Huanbang Chen (Huawei) |

CE4.1.5 in JVET-K0185 (Huawei)

This contribution reports the results of integrating affine inter prediction in JVET-J0024 [1] to BMS1.0. In this contribution, affine inter prediction is improved by 4 aspects:

1) Model based affine candidates are inserted into affine AMVP candidates with the same scan order of AMVP in HEVC; the candidate reorder operation in BMS affine is removed.

2) Affine MVD zero flag is signaled to indicate whether affine MVDs is zero or not;

3) Additional 6-parameter affine model is added and different models are adaptively selected in CU level;

4) Enhanced bi-linear Interpolation Filter (EIF) is used for affine motion compensation, when the width or height of affine sub-block is less than 8.

For affine AMVP improvement, experimental results reportedly show on average 0.26%/0.06% luma BD-rate gain in RA/LB configurations over to VTM with affine on. For EIF, experimental results reportedly show on average 0.25%/0.41% luma BD-rate gain with 9%/6% decoding time reduction in RA/LB configurations over VTM with affine on.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.5 | a) Affine MVP construction b) a) + MVD coding c) b) + 4 para/6-para switching d) EIF on top of BMS Affine e) EIF on top c) | Huanbang Chen (Huawei) | Fangdong Chen (Hikvision) |

CE4.1.6 in JVET-K0244 (MediaTek)

There are two kinds of MVP pair candidates. One kind is SMVP-pair candidate, and the other kind is CMVP-pair candidate. If the neighboring CUs are coded with affine, SMVP-pair candidates can be generated.

For SMVP-pair candidates, the searching order is A1B1B0A0B2, as shown in Figure 1. If the neighboring CU is coded with affine but the reference picture is different from the target reference picture, the control points’ MVs are scaled to current target picture to derive current CU’s affine model.

If the number of MVP pair candidates is smaller than two after searching SMVP-pair candidates, CMVP-pair candidates will be searched. The neighboring MVs at A0, A1, A2, B0, B1, C0, and C1, as shown in Figure 2, are used to derive CMVP-pair candidates. The first available MV in set A (A0, A1, and A2) and the first available MV in set B (B0 and B1) are used to calculate the first CMVP-pair candidate. The first available MV in set A and the first available MV in set C (C0 and C1) are used to calculate the second CMVP-pair candidate.

Key points

- Number of candidates is 2

- Two kinds of candidates : SMVP-pair candidate, and CMVP-pair candidate

- MV scaling is supported in the derivation of both type of candidates

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.6 | Affine MVP construction | Tzu-Der Chuang (MediaTek) | Guichun Li (Tencent) |

CE4.1.7 in JVET-K0094 (LGE)

Test 4.1.7.a: Affine motion vector prediction

Affine motion vector prediction constructs up to two candidates derived from the affine motion models of neighboring affine coded blocks. Neighbouring blocks, A, B, C, D, and E are checked whether it is coded as the affine prediction and its reference frame is same as the reference frame of the current block. Neighboring blocks with affine prediction and same reference frame are considered firstly, and then neighboring blocks with affine prediction and different reference frame are considered to generate the affine candidate. If the number of generated affine candidates is less than two, JEM affine process is performed until the number of candidate is two.

- Checking neighbor blocks in the same order as HEVC AMVP, candidate number is two

- Firstly derive affine MVP from the affine model of neighboring blocks

- Secondly construct affine MVP from motion vector of neighboring blocks as BMS affine

Test 4.1.7.b: Four and six parameter motion model

In the proposed method, four parameter affine model with two control point motion vectors (CPMVs) and six parameter affine model with three CPMVs are used. The affine models are selected at the CU level by using an additional affine parameter flag if neighboring blocks are coded as affine prediction. If no neighboring block is coded as the affine prediction, four parameter model is only used for the affine prediction without affine parameter flag.

- Alternative 6-param affine model is used and the indication flag is by-pass coded

Test 4.1.7.c: Adaptive four and six parameter motion model

Difference compared with test 4.1.7.b

- Alternative 6-param affine model is allowed only when a neighboring block is in affine mode

- When both 4-param and 6-param is allowed, an indication flag is by-pass coded

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.7 | a) Up to two affine candidate for affine inter b) Four and six parameter model c) Four and six parameter model with the conditional affine parameter signaling | Jaeho Lee (LGE) | K. Kondo (Sony) |

CE4.1.8 in JVET-K0124 (Sony)

An encoder can choose four modes which are a conventional translate, scaling, rotation and Affine. A number of parameters are 2 (one motion vector), 3, 3 and 4 (two MVs) for translate mode, scaling mode, rotation mode and Affine mode.

As shown in the below figure, the motion model index is defined and it is sent to the decoder. The number in parenthesizes shows the number of parameters of differential motion vector that are sent to a decoder.

When the scaling mode is chosen, three parameters are used to predict as shown in the below figure. It is one motion vector (v0x, v0y) and one element (v1x). For the prediction process, the frame work is used the same as for Affine transform in JEM software. The parameter (v1y) is implicitly assumed as the same as (v0y).

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.8 | Affine model extension (additional 2 3-param models) a) 3-para (scaling) / 4-para switching b) 3-para (rotation) / 4-para switching c) a + b | K. Kondo (Sony) | P. Bordes (Technicolor) Anish Tamse (Samsung) |

Categorization & comparative study

For easier comparison, only VTM tool test results in RA configuration is listed here. We can refer to more results when necessary. The description is also for VTM implementation.

Motion compensation modifications

| Test# | Description | BD-Rate (Y) RA | EncT/ | DecT |
| --- | --- | --- | --- | --- |
| AFFINE | BMS AFFINE | -2.99% | 137% | 112% |
| 4.1.2.a | Affine flexing for 4x4 sub-block size | -3.31% | 131% | 113% |
| 4.1.2.b | Affine flexing for 8x8 sub-block size | -2.90% | 126% | 107% |
| 4.1.5.d | EIF when sub-block is smaller than 8x8 | -3.23% | 132% | 102% |

Current affine inter prediction in BMS firstly divides a coding unit to sub-blocks, and then calculates motion vector for each sub-block based on affine motion model. When doing MC for each sub-block, translational motion is assumed. This was a reasonable trade when affine MC was proposed. Because “true” affine MC requires per-pixel operation and existing MC operation is too expensive.

The two techniques here try to perform “true” affine MC by low-complexity MC operation.

Flexing in test 4.1.2 performs additional motion compensation horizontally for each line of a sub-block and vertically for each columns of a sub-block, separately, trying to approximate per-pixel affine MC. Whether horizontal or vertical MC is performed first depends on the whether the motion of a CU is more rotational featured and more zooming featured. Existing 8-tap DCTIF is used for interpolation. A look-up table implementation is provided in the implementation.

EIF in test 4.1.5.b performs per-pixel affine MC using a two-stage interpolation, bi-linear + 3-tap filter. It is asserted the new interpolation mechanism is much simpler than 8-tap DCTIF and thus could do per-pixel MC with low cost.

Motion vector prediction

| Test# | Description | BD-Rate (Y) RA | EncT | DecT |
| --- | --- | --- | --- | --- |
| AFFINE | BMS AFFINE | -2.99% | 137% | 112% |
| 4.1.3.a | Affine MVP construction | -3.22% | 134% | 110% |
| 4.1.4.b | Affine MVP construction with added spatial and temporal candidates | -3.89% | 148% | 115% |
| 4.1.5.a | Affine MVP construction | -3.24% | 138% | 110% |
| 4.1.6 | Affine MVP construction | -3.21% | 132% | 107% |
| 4.1.7.a | Up to two affine candidate for affine inter | -3.23% | 142% | 114% |

In BMS affine inter mode, two motion vectors at the top-left and top-right corners of a coding unit is encoded. The predictor for the motion vector pair is constructed using motion vectors of neighboring blocks.

In the five tests here, a new types of candidate predictor is proposed. This candidate inherits the affine model from neighboring blocks and use that model to derive the motion vector at the control points of the coding block. The derived motion vector pair is then used as a predictor for CPMVs of the coding block.

Tests here seems converged to constructing the MVP list with two types of predictors, inherited predictor and constructed predictor.

The differences among the tests are,

- The position from where a candidate is derived

- The order in which the MVP list is constructed

- The number of candidates of a particular type

- Whether MV scaling is allow when deriving a candidate

- Whether a top-right MV derived from top-left and bottom-left MV could be used as the predictor for the top-right CPMV

MVD coding

| Test# | Description | BD-Rate (Y) RA | EncT | DecT |
| --- | --- | --- | --- | --- |
| 4.1.3.b | 4.1.3.a + MVD prediction | -3.42% | 136% | 112% |
| 4.1.5.b | 4.1.5.a + MVD coding | -3.23% | 139% | 110% |

Test 4.1.3.b tries to exploit the similarity of CPMVs of a coding block, refining the MV predictor of a CPMV by the MVD of the other one.

Test 4.1.5.b tries to save the bits for coding two MVDs by explicitly signaling whether the pair of MVDs in a prediction direction is skipped or not.

Alternative motion models

| Test# | Description | BD-Rate (Y) RA | EncT | DecT |
| --- | --- | --- | --- | --- |
| AFFINE | BMS AFFINE | -2.99% | 137% | 112% |
| 4.1.3.c | 4.1.3.b + CU level 4-para/6-para switching | -3.90% | 160% | 113% |
| 4.1.3.d | 4.1.3.c + slice level 4-para/6-para switching | -3.88% | 146% | 113% |
| 4.1.5.c | 4.1.5.b + 4 para/6-para switching | -3.83% | 144% | 111% |
| 4.1.7.b | Four and six parameter model | -3.53% | 200% | 116% |
| 4.1.7.c | Four and six parameter model with the conditional affine parameter signaling | -3.48% | 159% | 116% |
| 4.1.8.a | Alternative 3-para (scaling) motion model | -3.10% | 181% | 114% |
| 4.1.8.b | Alternative 3-para (rotation) motion model | -3.07% | 181% | 112% |
| 4.1.8.c | 4.1.8.a + 4.1.8.b | -3.19% | 221% | 114% |

Most of tests here add an additional 6-param affine model. 6-param affine model requires coding 3 CPMVs, compared with coding 2CPMVs for 4-param affine model in BMS affine. Adaptive selection at CU level or slice level are proposed.

Test 4.1.8 propose two types of 3-param model models, scaling model and rotation model. The 3-param model is represented by 1.5 motion vector, top-left motion vector and x or y component of the top-right motion vector.

Generally, additional motion model introduces operations handling the additional motion parameters, e.g. motion estimation, motion vector prediction, motion vector difference coding, motion parameter storage, etc.

On affine merge mode

Though affine merge mode is put into CE4.2 for study, together with other merge modification, joint study of affine inter mode and affine merge mode is also important and encouraged.

Related contributions

| JVET # | Title | Source |
| --- | --- | --- |
| JVET-K0052 | Non-CE4: A study on the affine merge mode | M. Zhou (Broadcom) |
| JVET-K0095 | CE4 related: Harmonization of CE4.1.7 and CE4.1.3 | J. Lee, J. Nam, N. Park, H. Jang, J. Lim, S. Kim (LGE) |
| JVET-K0101 | CE4-related: Affine MVD Coding | S. Paluri, M. Salehifar, S. Kim (LGE) |
| JVET-K0102 | CE4-related: Interweaved Prediction for Affine Motion Compensation | K. Zhang, L. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance) |
| JVET-K0103 | CE4-related: Simplified Affine Prediction | K. Zhang, L. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance) |
| JVET-K0193 | CE4-related: On performance improvements of Enhanced Interpolation Filter (EIF) | M. Sychev, G. Zhulikov, T. Solovyev, H. Chen, J. Chen (Huawei) |
| JVET-K0194 | CE4-related: On further complexity reduction of Enhanced Interpolation Filter (EIF) | M. Sychev, G. Zhulikov, T. Solovyev, J. Chen (Huawei) |
| JVET-K0268 | CE4-related: Affine tools from J0022 | F. Galpin, A. Robert, T. Poirier, F. LeLeannec (Technicolor) |
| JVET-K0335 | CE4-related: Shape dependent control point selection for affine mode | Y. He, X. Xiu, Y. Ye (InterDigital) |
| JVET-K0367 | Non-CE4: BMS affine improvements | H. Chen, H. Yang, J. Chen (Huawei) |

CE4.2: Merge mode enhancement

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.2.1 | Non-adjacent spatial merge candidates | JVET-K0228 (Ericsson) |
| 4.2.2 | Restricted merge | JVET-K0279 (HHI) |
| 4.2.3.a | Non-adjacent spatial merge candidate candidates | JVET-K0339 (Qualcomm) |
| 4.2.3.b | 4.2.3.a + Enlarge merge candidate list size to 10 |  |
| 4.2.3.c | 4.2.3.a + Enlarge merge candidate list size to 15 |  |
| 4.2.3.d | 4.2.3.c + Merge candidate list reordering |  |
| 4.2.3.e | 4.2.3.d + Unified merge candidate list |  |
| 4.2.4.a | Combined test 4.2.3 with model storage at CU level | JVET-K0219 (Technicolor) |
| 4.2.4.b | Combined test 4.2.3 with added affine candidates |  |
| 4.2.4.c | 4.2.4.a + 4.2.4.b |  |
| 4.2.5.a | ATMVP with one fixed collocated picture | JVET-K0341 (InterDigital) |
| 4.2.5.b | Picture/slice-level adaptation of ATMVP sub-block-size |  |
| 4.2.5.c | 4.2.5.a + 4.2.5.b |  |
| 4.2.6.a | Simplified ATMVP with one fixed collocated picture | JVET-K0079 (LGE) |
| 4.2.6.b | 4.2.6.a + 4.2.5.b |  |
| 4.2.7 | Affine merge with up to two affine merge candidate | JVET-K0094 (LGE) |
| 4.2.8.a | Adding affine merge candidates | JVET-K0245 (MediaTek) |
| 4.2.8.b | Adding middle spatial and multiple temporal candidates |  |
| 4.2.8.c | Adding pairwise-average candidates and removing HEVC combined candidates |  |
| 4.2.8.d | Double ATMVP candidates with uniformity check and decoder-side speedup |  |
| 4.2.9.a | UMVE candidates as additional merge candidates with additional information signaled | JVET-K0115 (Samsung) |
| 4.2.9.b1 | UMVE candidates as independent merge candidates with additional information signaled, number of candidates is 120 |  |
| 4.2.9.b2 | UMVE candidates as independent merge candidates with additional information signaled, number of candidates is 384 |  |
| 4.2.9.b3 | UMVE candidates as independent merge candidates with additional information signaled, number of candidates is 32 |  |
| 4.2.10.a | Model based affine candidates (separate test for different number) | JVET-K0186 (Huawei) |
| 4.2.10.b | Additional control point based affine candidates (separate test for different number) |  |
| 4.2.11 | MV Planar | JVET-K0349 (NHK) |
| 4.2.12.a | Use affine model of the neighbor coding unit with largest size as the merge candidate | JVET-K0355 (HIT/USTC) |
| 4.2.12.b | Add a separate merge list with 8 candidates using different motion models |  |
| 4.2.12.c | 4.2.12.a + 4.2.12.b |  |
| 4.2.13.a | Additional merge candidates | JVET-K0286 (Tencent) |
| 4.2.13.b | Further reduce the additional merge candidate number |  |
| 4.2.14 | MV Planar | JVET-K0135 (Hisilicon) |
| 4.2.15.a | Extended spatial positions from 6 to 27 | JVET-K0198 (Hisilicon) |
| 4.2.15.b | Merge index 0 with MV offsets |  |
| 4.2.15.c | Combined average merge candidates |  |
| 4.2.16.a | ATMVP in BMS | JVET-K0338 (Qualcomm) |
| 4.2.16.b | STMVP in JEM |  |
| 4.2.16.c | 4.2.16.a + 4.2.16.b |  |

Test results

Random access

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| BMS AFFINE | -2.99% | -2.19% | -2.21% | 137% | 112% | -1.92% | -1.34% | -1.36% | 108% | 102% |
| BMS ATMVP | -0.87% | -0.81% | -0.83% | 104% | 120% | -0.74% | -0.62% | -0.61% | 100% | 105% |
| BMS ATMVP+ HPMV | -1.10% | -0.88% | -0.85% | 102% | 122% | / | / | / | / | / |
| 4.2.1 | -0.45% | -0.45% | -0.44% | 100% | 100% | -0.22% | -0.21% | -0.20% | 98% | 97% |
| 4.2.2 | -0.21% | -0.32% | -0.32% | 139% | 102% | -0.20% | -0.28% | -0.29% | 120% | 100% |
| 4.2.3.a | -0.66% | -0.55% | -0.49% | 100% | 101% | -0.35% | -0.27% | -0.31% | 100% | 101% |
| 4.2.3.b | -0.88% | -0.81% | -0.77% | 105% | 101% | -0.62% | -0.54% | -0.57% | 102% | 101% |
| 4.2.3.c | -0.89% | -0.86% | -0.85% | 108% | 102% | -0.73% | -0.66% | -0.68% | 105% | 102% |
| 4.2.3.d | -0.88% | -0.85% | -0.85% | 109% | 103% | -0.76% | -0.71% | -0.78% | 105% | 103% |
| 4.2.3.e | / | / | / | / | / | -1.05% | -0.97% | -1.04% | 104% | 104% |
| 4.2.4.a | -4.03% | -3.28% | -3.26% | 146% | 117% | -2.85% | -2.22% | -2.28% | 119% | 99% |
| 4.2.4.b | -4.03% | -3.24% | -3.27% | 148% | 119% | -2.80% | -2.15% | -2.24% | 117% | 101% |
| 4.2.4.c | -4.08% | -3.35% | -3.35% | 150% | 129% | -2.86% | -2.22% | -2.29% | 119% | 107% |
| 4.2.5.a | -1.09% | -0.87% | -0.87% | 105% | 122% | -0.73% | -0.63% | -0.63% | 100% | 105% |
| 4.2.5.b | -1.06% | -0.82% | -0.80% | 103% | 106% | -0.69% | -0.57% | -0.57% | 100% | 101% |
| 4.2.5.c | -1.04% | -0.81% | -0.80% | 102% | 105% | -0.68% | -0.56% | -0.59% | 99% | 101% |
| 4.2.6.a | -1.09% | -0.86% | -0.84% | 106% | 114% | -0.73% | -0.66% | -0.65% | 101% | 106% |
| 4.2.6.b | -1.03% | -0.79% | -0.77% | 102% | 105% | -0.69% | -0.62% | -0.59% | 110% | 100% |
| 4.2.7 | -3.10% | -2.26% | -2.32% | 143% | 115% | -1.97% | -1.36% | -1.39% | 111% | 102% |
| 4.2.8.a | -3.36% | -2.58% | -2.62% | 132% | 122% | -2.12% | -1.53% | -1.53% | 110% | 89% |
| 4.2.8.b | -0.29% | -0.29% | -0.23% | 104% | 99% | -0.09% | -0.10% | -0.10% | 103% | 100% |
| 4.2.8.c | -0.41% | -0.37% | -0.38% | 103% | 100% | -0.51% | -0.37% | -0.36% | 103% | 101% |
| 4.2.8.d | -1.00% | -0.92% | -0.90% | 105% | 118% | -0.77% | -0.67% | -0.62% | 102% | 106% |
| 4.2.9.a | -1.06% | -1.13% | -1.13% | 142% | 118% | -0.36% | -0.34% | -0.31% | 124% | 108% |
| 4.2.9.b1 | -1.93% | -2.05% | -2.10% | 127% | 110% | -0.56% | -0.59% | -0.66% | 115% | 105% |
| 4.2.9.b2 | -1.53% | -1.61% | -1.63% | 144% | 113% | -0.63% | -0.60% | -0.63% | 126% | 107% |
| 4.2.9.b3 | -1.67% | -1.78% | -1.83% | 121% | 102% | -0.48% | -0.50% | -0.57% | 114% | 102% |
| 4.2.10.a | -3.09% | -2.27% | -2.28% | 137% | 110% | -1.97% | -1.39% | -1.42% | 110% | 101% |
| 4.2.10.b | -4.08% | -3.22% | -3.29% | 155% | 117% | -2.75% | -2.02% | -2.13% | 120% | 103% |
| 4.2.11 | -0.32% | -0.26% | -0.23% | 107% | 110% | -0.33% | -0.19% | -0.20% | 103% | 104% |
| 4.2.12.a | -3.05% | -2.21% | -2.22% | 138% | 112% | -1.94% | -1.40% | -1.41% | 111% | 102% |
| 4.2.12.b | -3.97% | -2.75% | -2.72% | 167% | 129% | -2.69% | -1.52% | -1.55% | 120% | 108% |
| 4.2.12.c | -3.99% | -2.74% | -2.72% | 167% | 129% | -2.69% | -1.56% | -1.59% | 121% | 108% |
| 4.2.13.a | -0.88% | -0.89% | -0.88% | 108% | 104% | -0.68% | -0.73% | -0.71% | 100% | 100% |
| 4.2.13.b | -0.88% | -0.86% | -0.85% | 107% | 104% | -0.61% | -0.59% | -0.61% | 100% | 101% |
| 4.2.14 | -0.91% | -0.94% | -0.94% | 108% | 119% | -0.49 % | -0.46% | -0.43 % | 104% | 107% |
| 4.2.15.a | -1.14% | -1.12% | -1.17% | 109% | 101% | -1.22% | -1.21% | -1.30% | 106% | 102% |
| 4.2.15.b | -1.04% | -1.09% | -1.15% | 126% | 101% | -0.58% | -0.68% | -0.80% | 116% | 101% |
| 4.2.15.c | -0.31% | -0.22% | -0.24% | 102% | 100% | -0.25% | -0.12% | -0.14% | 102% | 100% |
| 4.2.16.a | -0.87% | -0.81% | -0.83% | 103% | 115% | 0.75% | 0.63% | 0.62% | 100% | 95% |
| 4.2.16.b | -0.76% | -1.01% | -0.95% | 105% | 121% | 0.06% | -0.19% | -0.20% | 100% | 103% |
| 4.2.16.c | -1.30% | -1.36% | -1.38% | 109% | 128% | -0.37% | -0.39% | -0.41% | 101% | 105% |

Low delay B

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| BMS AFFINE | -2.06% | -1.33% | -1.52% | 168% | 108% | -1.98% | -1.52% | -1.70% | 125% | 104% |
| BMS ATMVP | -1.08% | -1.27% | -1.23% | 105% | 114% | -1.02% | -1.08% | -1.25% | 99% | 106% |
| BMS ATMVP+ HPMV | -1.31% | -1.29% | -1.25% | 104% | 116% | / | / | / | / | / |
| 4.2.1 | -0.12% | 0.07% | 0.11% | 100% | 99% | -0.09% | -0.06% | 0.07% | 99% | 96% |
| 4.2.2 | 0.01% | 0.02% | -0.03% | 145% | 104% | -0.10% | -0.19% | -0.03% | 131% | 99% |
| 4.2.3.a | -0.36% | 0.06% | 0.18% | 102% | 105% | -0.25% | 0.08% | 0.25% | 99% | 99% |
| 4.2.3.b | -0.49% | -0.18% | -0.25% | 107% | 103% | -0.37% | -0.48% | -0.28% | 102% | 102% |
| 4.2.3.c | -0.49% | -0.24% | -0.14% | 112% | 103% | -0.44% | -0.24% | -0.08% | 103% | 102% |
| 4.2.3.d | -0.49% | -0.24% | -0.14% | 111% | 104% | -0.44% | -0.24% | -0.08% | 103% | 100% |
| 4.2.3.e | / | / | / | / | / | -0.52% | -0.35% | -0.31% | 102% | 104% |
| 4.2.4.a | -2.50% | -1.53% | -1.57% | 181% | 100% | -2.47% | -1.83% | -1.83% | 134% | 104% |
| 4.2.4.b | -2.60% | -1.72% | -1.64% | 182% | 105% | -2.52% | -2.05% | -2.10% | 136% | 105% |
| 4.2.4.c | -2.67% | -1.86% | -1.82% | 187% | 117% | -2.57% | -2.11% | -1.94% | 136% | 113% |
| 4.2.5.a | -1.31% | -1.28% | -1.14% | 106% | 115% | -1.10% | -1.03% | -1.43% | 99% | 108% |
| 4.2.5.b | -1.29% | -1.29% | -1.17% | 103% | 109% | -1.02% | -1.11% | -1.28% | 99% | 104% |
| 4.2.5.c | -1.30% | -1.26% | -1.28% | 103% | 108% | -1.07% | -1.13% | -1.38% | 99% | 104% |
| 4.2.6.a | -1.30% | -1.10% | -1.10% | 105% | 113% | -1.05% | -1.14% | -1.35% | 99% | 106% |
| 4.2.6.b | -1.27% | -1.12% | -1.10% | 102% | 106% | -1.08% | -0.98% | -1.29% | 100% | 104% |
| 4.2.7 | -2.09% | -1.37% | -1.49% | 181% | 115% | -2.05% | -1.81% | -1.97% | 130% | 104% |
| 4.2.8.a | -2.23% | -1.53% | -1.67% | 159% | 110% | -2.21% | -1.81% | -1.75% | 126% | 113% |
| 4.2.8.b | -0.39% | -0.35% | -0.30% | 104% | 99% | -0.10% | -0.02% | 0.29% | 103% | 99% |
| 4.2.8.c | -0.13% | 0.03% | 0.11% | 102% | 100% | -0.27% | -0.21% | -0.19% | 102% | 101% |
| 4.2.8.d | -1.19% | -1.38% | -1.32% | 104% | 109% | -1.15% | -1.35% | -1.48% | 101% | 105% |
| 4.2.9.a | -0.21% | -0.08% | -0.26% | 155% | 118% | 0.02% | 0.06% | 0.01% | 125% | 109% |
| 4.2.9.b1 | -0.36% | -0.52% | -0.49% | 129% | 108% | -0.09% | 0.03% | 0.17% | 112% | 106% |
| 4.2.9.b2 | -0.22% | -0.36% | -0.38% | 153% | 117% | -0.02% | -0.13% | 0.06% | 124% | 108% |
| 4.2.9.b3 | -0.24% | -0.31% | -0.44% | 122% | 103% | 0.01% | 0.06% | 0.06% | 111% | 101% |
| 4.2.10.a | -2.11% | -1.37% | -1.39% | 168% | 107% | -2.05% | -1.73% | -1.73% | 125% | 102% |
| 4.2.10.b | -2.67% | -1.99% | -1.96% | 194% | 113% | -2.51% | -1.86% | -2.25% | 146% | 105% |
| 4.2.11 | -0.09% | 0.00% | -0.02% | 111% | 108% | -0.22% | -0.10% | 0.05% | 106% | 103% |
| 4.2.12.a | -2.11% | -1.36% | -1.48% | 169% | 109% | -2.11% | -1.58% | -1.63% | 127% | 105% |
| 4.2.12.b | -2.63% | -1.48% | -1.46% | 203% | 122% | -2.61% | -1.58% | -1.69% | 141% | 112% |
| 4.2.12.c | -2.66% | -1.42% | -1.25% | 203% | 122% | -2.63% | -1.63% | -1.63% | 142% | 111% |
| 4.2.13.a | -0.51% | -0.27% | -0.22% | 108% | 103% | -0.41% | -0.22% | -0.09% | 100% | 97% |
| 4.2.13.b | -0.52% | -0.23% | -0.37% | 107% | 105% | -0.39% | -0.32% | -0.13% | 100% | 99% |
| 4.2.14 | -0.70% | -0.64% | -0.75% | 109% | 112% | -0.51% | -0.50% | -0.42% | 105% | 104% |
| 4.2.15.a | -0.84% | -0.65% | -0.70% | 111% | 104% | -0.89% | -0.83% | -0.67% | 103% | 100% |
| 4.2.15.b | -0.42% | -0.48% | -0.70% | 125% | 102% | -0.38% | -0.60% | -0.48% | 110% | 101% |
| 4.2.15.c | 0.00% | 0.00% | 0.00% | 102% | 100% | 0.00% | 0.00% | 0.00% | 102% | 96% |
| 4.2.16.a | -1.08% | -1.27% | -1.23% | 104% | 112% | 1.03% | 1.11% | 1.28% | 101% | 94% |
| 4.2.16.b | -0.68% | -0.54% | -0.58% | 106% | 115% | 0.11% | 0.13% | 0.23% | 101% | 104% |
| 4.2.16.c | -1.31% | -1.37% | -1.39% | 111% | 120% | -0.37% | -0.14% | -0.09% | 101% | 101% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.2.1 | Saverio Blasi (BBC) | 2 | F | Y | M |
| 4.2.2 | Yuling Hsiao (MediaTek) | 2 | F | Y | M |
| 4.2.3.a | Jing Ye (Tencent) | 2 | F | Y | M |
| 4.2.3.b | Jing Ye (Tencent) | 2 | F | Y | M |
| 4.2.3.c | Jing Ye (Tencent) | 2 | F | Y | M |
| 4.2.3.d | Byeongdoo Choi (Sharp) | 2 | F | Y | M |
| 4.2.3.e | Byeongdoo Choi (Sharp) | 2 | F | Y | M |
| 4.2.4.a | Jonathan Taquet (Canon) | 2 | P | Y | / |
| 4.2.4.b | Jonathan Taquet (Canon) | 2 | P | Y | / |
| 4.2.4.c | Jonathan Taquet (Canon) | 2 | P | Y | / |
| 4.2.5.a | Yue Yu (Arris) | 2 | F | Y | M |
| 4.2.5.b | hyeongmun Jang(LGE) | 2 | F | Y | M |
| 4.2.5.c | hyeongmun Jang(LGE) | 2 | F | Y | M |
| 4.2.6.a | Xiaoyu Xiu(InterDigital) | 2 | F | Y | M |
| 4.2.6.b | Xiaoyu Xiu(InterDigital) | 2 | F | Y | M |
| 4.2.7 | Xiaoyu Xiu(InterDigital) | 3 | F | Y | M |
| 4.2.8.a | Huanbang Chen (Huawei) | 2 | F | Y | M |
| 4.2.8.b | Huanbang Chen (Huawei) | 2 | P | Y | M |
| 4.2.8.c | Huanbang Chen (Huawei) | 2 | P | Y | M |
| 4.2.8.d | Huanbang Chen (Huawei) | 2 | P | Y | M |
| 4.2.9.a | Ruoyang Yu (Ericsson) | 2 | F | Y | G |
| 4.2.9.b1 | Byeongdoo Choi (Sharp) | 2 | F | Y | M |
| 4.2.9.b2 | Shih-Ta Hsiang (MediaTek) | 2 | F | Y | M |
| 4.2.9.b3 | Shih-Ta Hsiang (MediaTek) | 2 | F | Y | M |
| 4.2.10.a | Fangdong Chen (Hikvision) | 2 | F | Y | M |
| 4.2.10.b | Fangdong Chen (Hikvision) | 2 | F | Y | M |
| 4.2.11 | Xiaozhong Xu (Tencent) | 2 | P | Y | M |
| 4.2.12.a | Jaeho LEE (LGE) | 2 | F | Y | M |
| 4.2.12.b | Jaeho LEE (LGE) | 2 | P | Y | M |
| 4.2.12.c | Jaeho LEE (LGE) | 2 | F | Y | M |
| 4.2.13.a | Jungwon Kang (ETRI) Timofey Solovyev (Huawei) | 22 | PF | YY | MM |
| 4.2.13.b | Byeongdoo Choi (Sharp) Jungwon Kang (ETRI) Timofey Solovyev (Huawei) | 222 | FPF | YYY | MMM |
| 4.2.14 | Xiaozhong Xu (Tencent) | 2 | P | Y | M |
| 4.2.15.a | Jing Ye (Tencent) | 2 | F | Y | M |
| 4.2.15.b | Jing Ye(Tencent) | 2 | P | N | M |
| 4.2.15.c | Peter Chuang(MediaTek) | 2 | P | Y | M |
| 4.2.16.a | Hahyun Lee (ETRI) | 2 | P | Y | M |
| 4.2.16.b | Hahyun Lee (ETRI) | 2 | P | Y | M |
| 4.2.16.c | Hahyun Lee (ETRI) | 2 | P | Y | M |

Crosschecker comments on CE4.2.9

The crosschecker's enc/dec numbers are not reliable.

Crosschecker comments on CE4.2.12

Different results have been observed in Linux and Windows.

It was reported by the proponent that a bug had been found and fixed.

It was later reported by the crosschecker that the problem was solved and the cross-check result is updated.

Crosschecker comments on CE4.2.13

Test a: (Solovyev) small complexity increase was observed in BMS (enc/dec): 103/102 for RA, 102/101 for LDB. In comparison with the CE description one feature was added: adaptive grid size. The source code is fully in line with the new description: JVET-K0286.

Test b: (Solovyev) decoder complexity for VTM low delay B is less in my tests (102%). In comparison with the CE description one feature was added: adaptive grid size. The source code is fully in line with the new description: JVET-K0286.

Crosschecker comments on CE4.2.15.b

Mismatch of the crosschecker's and proponent's results are observed.

Crosschecker comments on CE4.2.16

1. cross-check encoding time is not reliable for some groups (A1,A2,B) but other groups(C,D,E) are reliable.

2. Unmodified BMS-1.0 was used.

Detailed description of tests

CE4.2.1 in JVET-K0228 (Ericsson)

Key points

- Firstly check vertical positions and then horizontal positions, duplication checking is performed

- Checking in each direction stop if one candidate is found or max distance is reached

- Max 2 new candidates in total

- New candidates are added before the TMVP candidate in the merge candidate list when the number of already added spatial candidates is less than a threshold (3 in currently implementation)

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.1 | Adding non-adjacent spatial merge candidates, on/off test | Ruoyang Yu (Ericsson) | Saverio Blasi (BBC) |

CE4.2.2 in JVET-K0279 (HHI)

In merge mode (but not SKIP mode) in B slices, three types of merge list is constructed

- Regular merge list

- Merge list with uni-prediction candidate in L0 direction

- Merge list with uni-prediction candidate in L1 direction

The syntax element merge_ref_pic_list_idc can take three different values for the indication.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.2 | Restricted merge | Martin Winken (HHI) | Yuling Hsiao (MediaTek) |

CE4.2.3 in JVET-K0339 (Qualcomm)

Candidates are added to the list in a pre-defined order:

- Spatial candidates for blocks 1-4.

- Extrapolated affine candidates for blocks 1-4.

- Re-ordering, the bi-prediction ones are inserted before the ones with uni-prediction.

- ATMVP (BMS test).

- Virtual affine candidate.

- Spatial candidate (block 5) (used only when the number of the available candidates is smaller than 6 (or 4 for VTM test)).

- Extrapolated affine candidate (block 5).

- Temporal candidate (derived as in HEVC).

- Non-adjacent spatial candidate followed by extrapolated affine candidate, and extrapolated affine candidate (blocks 6 to 49).

- Combined candidates.

- Zero candidates.

Moreover, for the first four spatial candidates (and extrapolated affine candidates in Test4.2.3.e), the bi-prediction ones are inserted before the ones with uni-prediction.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.3 | a) Non-adjacent spatial merge candidate candidates b) a + Enlarge merge candidate list size to 10 c) a + Enlarge merge candidate list size to 15 d) c + Merge candidate list reordering e) d + Unified merge candidate list | Yi-Wen Chen (Qualcomm) | Jing Ye (Tencent) d),e): Byeongdoo Choi (Sharp) |

CE4.2.4 in JVET-K0219 (Technicolor)

Test 4.2.4.a: Affine motion seed storage

In the JEM, the affine model seeds are stored in the top-left, top-right and bottom-left 4x4 sub-blocks in the considered CU. In the proposed solution, the affine model seeds are stored separately as a motion information associated to the whole CU. The motion model is thus decoupled from the motion vectors used for actual motion compensation at the 4x4 block level. This storage allows preserving the complete motion vector field at the 4x4 sub-block level. It also allows using affine motion compensation for block of size 4 in width or height.

Test 4.2.4.b: Affine merge candidate derivation

|  | JEM | Proposed |
| --- | --- | --- |
| Motion information predictor composition | - inherited reference picture index - inherited associated MV - inherited associated IC flag | - inherited reference picture index - inherited associated MV (triplet) - inherited associated IC flag - inferred affine flag |
| Candidate list | 7 candidates, among: - spatial positions in {a1, b1, b0, a0} - ATMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - STMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - spatial positions in {b2} - a temporal position {c} - bi-directional combinations - zero MV for each reference picture | 17 candidates with up to 8 Affine ones, among: - spatial positions in {a1, b1, b0, a0} - spatial affine positions in {a1, b1, b0, a0} - ATMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - STMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - constructed affine candidate based on spatial non-affine positions - spatial positions in {b2} - spatial affine positions in {b2, a2, b3} - a temporal position {c} - temporal affine positions in {curr, c} - non-adjacent spatial positions - non-adjacent spatial affine positions - bi-directional combinations - zero MV for each reference picture |
| Signaling | For each reference picture: - predictor idx Residual is coded if mode != skip | For each reference picture: - predictor idx Residual is coded if mode != skip |

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.4 | a) Combined test 4.2.3 with model storage at CU level b) Combined test 4.2.3 with added affine candidates c) a+b | A. Robert (Technicolor) |  |

CE4.2.5 in JVET-K0341 (InterDigital)

CE4.2.5.a Simplified collocated block derivation with one fixed collocated picture

- Using the same collocated picture as in HEVC for ATMVP derivation is signaled at the slice header

- The scaled MV is used in ATMVP if the original MV from a neighboring block points to a reference picture other than the collocated picture.

CE4.2.5.b Adaptive ATMVP sub-block size

Slice-level adaptation of the sub-block size for the ATMVP motion derivation

- One default sub-block size is signaled at sequence level

- One flag is signaled at slice-level to indicate if the default sub-block size is used for the current slice

- If the flag is false, the corresponding ATMVP sub-block size is further signaled in the slice header for the slice.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.5 | a) ATMVP with one fixed collocated picture b) Picture/slice-level adaptation of ATMVP sub-block-size c) a+b | Xiaoyu Xiu (InterDigital) | a) Yue Yu (Arris) b) Hyeongmun Jang (LGE) c) Hyeongmun Jang (LGE) |

CE4.2.6 in JVET-K0079 (LGE)

The proposed simplified ATMVP is basically similar to ATMVP as place holder for the temporal motion vector derivation. However, in the proposed method, the usage of memory is effectively reduced because the reference picture to find the corresponding block is restricted to the collocated picture (designated in the slice segment header), while 4 reference pictures are required in the place holder method. To find the corresponding block, a temporal vector is derived from one of spatial candidates in a scanning order. If the current candidate has the reference picture same with the collocated picture, the search is finished. If all of candidates do not refer the collocated picture then zero motion is applied.

In CE4-2.6(a), it is applied that Simplified ATMVP refers only one fixed collocated picture to derive temporal motion information, and the sub-block size is decided between 4x4 and 8x8 based on slice level syntax. This sub-block size decision algorithm is identical with CE4-2.5(b). The sub-block size decision algorithm decides sub-block size for ATMVP based on statistic of the prediction unit size and pre-defined threshold value for each slice and each temporal layer in encoder side. In decoder side, adaptive sub-block decision on/off flag is signaled and sub-block size is also signaled when the on/off flag is set as true in slice header.

- CE4-2.6(a): one fixed collocated picture is used to derive temporal motion information

- CE4-2.6(b): CE4-2.6(a) plus the slice level adaptive sub-block decision in CE4-2.5(b).

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.6 | a) Simplified ATMVP with one fixed collocated picture b) a+4.2.5.b | Hyeongmun Jang (LGE) | Xiaoyu Xiu (InterDigital) |

CE4.2.7 in JVET-K0094 (LGE)

In the proposed algorithm, affine merge candidate is derived from the affine motion model of neighboring affine-coded block and up to two candidates can be considered in the affine merge list. Affine merge mode is skipped when there is no neighboring affine coded block. Affine merge index is signaled only when the available neighboring affine coded block is more than one.

- Max candidate number is 2

- Affine merge candidate signaling depends on whether neighboring block is in affine mode

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.7 | Affine merge with up to two affine merge candidate. | Jaeho LEE (LGE) | Xiaoyu Xiu (InterDigital) |

CE4.2.8 in JVET-K0245 (MediaTek)

CE4.2.8.A: Affine merge candidates

- Two kinds of affine candidates: spatial inherited one, corner derived.

- Spatial inherited affine candidate

  - Only use 6-parameter affine model

  - If available, regular merge candidate in the same position will not be used

- Corner derived affine candidates

  - Only use 6-parameter affine model

  - At most four candidates

  - Inserted in regular merge list after spatial candidate at location B2.

- Number of all candidates is 6/7 for VTM/BMS

CE4.2.8.B: Middle spatial and multiple temporal candidates

- Two middle spatial candidates located at ML and MT are added after temporal candidates

- Four temporal candidates are added in the order {RB, CT, LB, RT}

- Number of all candidates is 10

CE4.2.8.C: Pairwise-average candidates as replacement of HEVC combined candidates

- Predefined pairs are defined as {(0, 1), (0, 2), (1, 2), (0, 3), (1, 3), (2, 3)}

- Rule for MV averaging

  - If both MVs are available in one list, the MV with the larger merge index is scaled to the reference picture of the merge candidate with the smaller merge index

  - If only one MV is available, use the MV directly

  - If no MV is available, keep this list invalid

- Number of all candidates is 10

CE4.2.8.D: Double ATMVP candidates with uniformity check and decoder-side speed-up

- Two MVs from merge candidate list instead of one are used as temporal MVs to generate two ATMVP candidates

- Uniformity check at four corners

  - If the same for the two ATMVP candidates, the second one is removed from the list

  - If the same as the center motion, ATMVP is treated as TMVP

- Number of all candidates is 7

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.8 | a) Adding affine merge candidates b) Adding middle spatial and multiple temporal candidates c) Adding pairwise-average candidates and removing HEVC combined candidates d) Double ATMVP candidates with uniformity check and decoder-side speedup | Tzu-Der Chuang (MediaTek) | Huanbang Chen (Huawei) |

CE4.2.9 in JVET-K0115 (Samsung)

UMVE is used for either skip or merge modes with a proposed motion vector expression method.

UMVE re-uses merge candidate as same as using in VVC. Among the merge candidates, a candidate can be selected, and is further expanded by the proposed motion vector expression method.

UMVE provides a new motion vector expression with simplified signaling. The expression method includes prediction direction information, starting point, motion magnitude, and motion direction.

This proposed technique uses a merge candidate list as it is. But only candidates which are default merge type (MRG_TYPE_DEFAULT_N) are considered for UMVE’s expansion.

Prediction direction information indicates a prediction direction among L0, L1 and L0 and L1 predictions. In B slice, the proposed method can generate bi-prediction candidates from merge candidates with uni-prediction by using mirroring technique. For example, if a merge candidate is uni-prediction with L1, a reference index of L0 is decided by searching a reference picture in list 0, which is mirrored with the reference picture for list 1. If there is no corresponding picture, the nearest reference picture to current picture is used. L0’ MV is derived by scaling L1’s MV. The scaling factor is calculated by POC distance.

If the prediction direction of the UMVE candidate is the same with one of the original merge candidate, the index with value 0 is signaled as an UMVE prediction direction. But, if not the same, same with one of the original merge candidate, the index with value 1 is signaled. After sending first bit, remaining prediction direction is signaled based on the pre-defined priority order of UMVE prediction direction. Priority order is L0/L1 prediction, L0 prediction and L1 prediction.

If the prediction direction of merge candidate is L1, signaling ‘0’ is for UMVE’ prediction direction L1. Signaling ‘10’ is for UMVE’ prediction direction L0 and L1. Signaling ‘11’ is for UMVE’ prediction direction L0.

If L0 and L1 prediction lists are same, UMVE’s prediction direction information is not signaled.

Base candidate index defines the starting point. Base candidate index indicates the best candidate among candidates in the list as follows.

| Base candidate IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Nth MVP | 1st MVP | 2nd MVP | 3rd MVP | 4th MVP |

If the number of base candidate is equal to 1, Base candidate IDX is not signaled.

Distance index is motion magnitude information. Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

| Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-axis | + | – | N/A | N/A |
| y-axis | N/A | N/A | + | – |

In terms of Test a, one of merge candidate indexes is used as an UMVE flag. MRG_MAX_NUM_CANS is increased by 1. In this test, 3rd index of merge candidate is used as an UMVE flag. In decoder side, if skip/merge index is 3, decoder starts to parse UMVE syntaxs for UMVE information. If skip/merge index is over 3, Actual skip/merge index is going to be skip/merge index minus 1. If received index is 4, 3rd candidate is selected as a candidate of original skip/merge mode.

In terms of Test b, UMVE flag is singnaled after sending a skip flag and merge flag. If skip and merge flag is true, UMVE flage is parsed. If UMVE flage is equal to 1, UMVE syntaxes are parsed. But, if not 1, skip/merge index is parsed for VTM/BMS’s skip/merge mode.

Additional line buffer due to UMVE candidates is not needed. Because a skip/merge candidate of software is directly used as a base candidate. Using input UMVE index, the supplement of MV is decided right before motion compensation. There is no need to hold long line buffer for this.

The NUM_MRG_SATD_CAND is changed according to test option. It will be described with the performance at next chapter.

To reduce the encoder complexity, block restriction is applied. If either width or height of a CU is less than 4, UMVE is not performed.

- Test 4.2.9 (a) UMVE candidates as additional merge candidates with additional information signaled,

- Test 4.2.9 (b) UMVE candidates as independent merge candidates with additional information signaled.

- Test 4.2.9 (b) is consist of 3 sub-tests with changing parameters.

  - The number of NUM_MRG_SATD_CAND is set to 120 at Sub-CE 4.2.9 b-1.

  - The number of NUM_MRG_SATD_CAND is set to 384 at Sub-CE 4.2.9 b-2.

  - The number of NUM_MRG_SATD_CAND is set to 32 at Sub-CE 4.2.9 b-3.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.9 | a) UMVE candidates as additional merge candidates with additional information signaled b) UMVE candidates as independent merge candidates (using new merge candidate list) with additional information signaled (including conventional merge candidates, “best“ set of candidate from this subCE, etc.) | Seungsoo Jeong (Samsung) | a) Ruoyang Yu (Ericsson) b)Byeongdoo Choi (Sharp) |

CE4.2.10 in JVET-K0186 (Huawei)

The affine merge candidate list is constructed as following steps:

- Insert model based affine candidates

Model based affine candidate means that the candidate is derived from the valid neighbor reconstructed block coded with affine mode. As shown in the figure, the scan order for the candidate block is from left, above, above right, left bottom to above left. The same derived method in BMS is used [2].

- Insert control point based affine candidates

Control points based candidate means the candidate is constructed by combining the neighbor motion information of each control point.

The motion information for the control points is derived firstly from the specified spatial neighbors and temporal neighbor shown in the figure. CPk (k=1, 2, 3, 4) represents the k-th control point. A, B, C, D, E, F and G are spatial positions for predicting CPk (k=1, 2, 3); H is temporal position for predicting CP4.

The motion information of each control point is obtained according to the following priority order:

- For CP1, the checking priority is ABC, A is used if it is available. Otherwise, if B is available, B is used. If both A and B are unavailable, C is used. If all the three candidates are unavailable, the motion information of CP1 cannot be obtained.

- For CP2, the checking priority is ED;

- For CP3, the checking priority is GF;

- For CP4, H is used.

Secondly, the combinations of controls points are used to construct the motion model.

Motion vectors of two control points are needed to compute the transform parameters in 4-parameter affine model. The two control points can be selected from one of the following six combinations ({CP1, CP4}, {CP2, CP3}, {CP1, CP2}, {CP2, CP4}, {CP1, CP3}, {CP3, CP4}). For example, use the CP1 and CP2 control points to construct 4-parameter affine motion model, denoted as Affine (CP1, CP2).

Motion vectors of three control points are needed to compute the transform parameters in 6-parameter affine model. The three control points can be selected from one of the following four combinations ({CP1, CP2, CP4}, {CP1, CP2, CP3}, {CP2, CP3, CP4}, {CP1, CP3, CP4}). For example, use CP1, CP2 and CP3 control points to construct 6-parameter affine motion model, denoted as Affine (CP1, CP2, CP3).

All of these models will be converted to 6-parameter affine model represented by top-left, top-right, and bottom-left control point. During MC and motion vector derivation for sub-block, unified 6-parameter affine model is used.

Key points

- Two type of affine merge candidates: 1) 4-param affine merge candidates represented by 2 CPMV, 2) 6-param affine merge candidates represented by 3 CPMV.

- Two ways of candidate derivation: 1) derive 3 CPMV using the affine model of neighboring affine coded block, 2) derive 3 CPMV from the motion vector of neighboring blocks.

- A separate merge list is used.

- Max candidate number is 5.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.10 | a) Model based affine candidates (separate test for different number) b) Additional control point based affine candidates (separate test for different number) | Huanbang Chen (Huawei) | Fangdong Chen (Hikvision) |

CE4.2.11 in JVET-K0349 (NHK)

Key points

- The interpolation is carried out in a similar way as intra planar prediction.

- In the case that neighboring CU is intra coded, the closest neighboring MV is substituted in a similar way as intra reference sample substitution.

- If the mv_palanr_flag is equal to 1, inter prediction index “inter_pred_idc” syntax and reference index “ref_idx” syntaxes for both L0 and L1 lists are additionally signaled.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.11 | MV Planar | Shunsuke Iwamura (NHK) | Xiaozhong Xu (Tencent) |

CE4.2.12 in JVET-K0355 (HIT&USTC)

CE4.2.12.a BMS affine merge modification

Instead of finding the first neighboring block with affine mode, the affine model from the neighboring coding unit with largest size is used.

CE4.2.12.b Complex merge mode

Three types of motion model, all are constructed from MV of neighboring blocks

- Bilinear model

- 6-param affine model

- 4-param affine model

Candidates are added to the list in the order

- Affine (CP2, CP3)

- Affine (CP1, CP3)

- Affine (CP1, CP2, CP3)

- Affine (CP1, CP2)

- Affine (CP2, CP4);

- Affine (CP3, CP4);

- Affine (CP1, CP4);

- Bilinear

- Affine (CP1, CP2, CP4);

- Affine (CP2, CP3, CP4);

- Affine (CP1, CP3, CP4);

The reference index with the highest utilization rate among all the reference indices is selected as the final reference index. The control points with different reference indices are scaled to the final reference index.

Redundancy checking is performed.

All candidates are put in a separate list.

Number of candidates are 8, and FL 3 bins are used for index coding.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.12 | a) Modify the simple affine merge mode in BMS (use the largest coding unit size in affine mode as the affine merge candidate) b) Add a complex merge mode with 8 candidates, using different models. Separate test of different kinds of candidates will be provided. c) a) + b). | Yang Wang (HIT) | Jaeho LEE (LGE) |

CE4.2.13 in JVET-K0286 (Tencent)

Non-adjacent spatial candidates are added

- Derivation of new candidates

  - The search grid is based on block width and block height, with maximum search range 96

  - Total (96 / max (width, height)) search points are checked. The detailed search pattern for each round is described in the figure above.

  - When max (width, height) is greater than the threshold (64), the search grid is 32x32

- Redundancy checking (in a different way for other merge candidates pruning) is performed for the added merge candidates.

- New candidates are added after TMVP candidates in the merge candidate list.

- Maximum merge candidate number is 10 or 8 in VTM 1.0 and 11 or 9 in BMS 1.0.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.13 | a) Additional merge candidates (10 candidates for VTM, 11 candidates for BMS) b) Further reduce the additional merge candidate number (8 candidates for VTM, 9 candidates for BMS) | Jing Ye (Tencent) | Jungwon Kang (ETRI) Solovyev Timofey (Huawei) b) Byeongdoo Choi (Sharp) |

CE4.2.14 in JVET-K0135 (Hisilicon)

Planar motion vector prediction is achieved by averaging a horizontal and vertical linear interpolation on 4x4 block basis as follows.

W and H denote the width and the height of the block. (x,y) is the coordinates of current sub-block relative to the above left corner sub-block. All the distances are denoted by the pixel distances divided by 4.is the motion vector of current sub-block.

The horizontal prediction  and the vertical prediction  for location (x,y) are calculated as follows:

where  and  are the motion vectors of the 4x4 blocks to the left and right of the current block.  and  are the motion vectors of the 4x4 blocks to the above and bottom of the current block.

Key points

- Separately signaled (not in the current merge list)

- Motion in the L-shaped area is padded in the same manner as intra reference pixel padding, separately for L0 and L1.

- Bi-directional or uni-directional: decided based on the prediction direction of the motion in the L-shaped area. Once there is a L0 or L1 MV, L0 or L1 direction is activated.

- Ref_idx in L0 and L1 is set to 0.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.14 | MV Planar | Na Zhang (HiSilicon) | Xiaozhong Xu (Tencent) |

CE4.2.15 in JVET-K0198 (Hisilicon)

Test A : More spatial candidates

More spatial positions are checked as shown in Figure. The extended spatial positions from 6 to 27 are checked according to their numerical order after the temporal candidate. In order to save the MV line buffer, all the spatial candidates are restricted within two CTU lines. That is, the spatial candidates beyond the CTU line above the current CTU line are excluded.

Key points

- Additional 6 merge candidates are added

- New candidates are appended to the end of the current merge list

- Redundancy checking is performed for the added merge candidates

Test B: Merge offset extension

In Merge offset extension, more candidates are checked based on existed candidate. New candidates has mv offset to the mv of existed candidate (except ATMVP candidate). New extended mv offset candidates are constructed only based on the first candidate of merge candidate list.

Key points

- Additional 8 merge candidates are added

- Ordering of the candidates : cross direction offset is checked firstly, and then X direction offsets

- New candidates are appended to the end of the current merge list

Test C: Combined Average Merge Candidates

- The first 4 candidates in the merge list is used for deriving the new candidates

- The reference picture in L0 and L1 is decided by voting, separately

- Combination of the 4 candidates is performed to derive 6 pair candidates

- For each pair of candidates, L0 and L1 motion vectors are averaged separately

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.15 | a) Extended spatial positions from 6 to 27 b) Merge index 0 with MV offsets c) Combined average merge candidates | Jicheng An Xu Chen Na Zhang (Hisilicon) | Tzu-Der Chuang (MediaTek) Jing Ye (Tencent) |

CE4.2.16 in JVET-K0338 (Qualcomm)

BMS ATMVP and JEM STMVP are tested.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.16 | Sub-block merge candidates in BMS and JEM a) ATMVP b) STMVP c) Combination of a) and b) | Yi-Wen Chen (Qualcomm) | Hauyun Lee (ETRI) |

Categorization & comparative study

For easier comparison, only VTM tool test results in RA configuration is listed here. We can refer to more results when necessary. The description is also for VTM implementation.

CU based candidate – Long distance spatial candidates

| Test# | Description | BD-Rate (Y) RA | EncT | DecT |
| --- | --- | --- | --- | --- |
| 4.2.1 | Non-adjacent spatial merge candidates (number of candidates is 5) | -0.45% | 100% | 100% |
| 4.2.3.a | Non-adjacent spatial merge candidate candidates (number of candidates is 5) | -0.66% | 100% | 101% |
| 4.2.3.b | 4.2.3.a + Enlarge merge candidate list size to 10 | -0.88% | 105% | 101% |
| 4.2.3.c | 4.2.3.a + Enlarge merge candidate list size to 15 | -0.89% | 108% | 102% |
| 4.2.3.d | 4.2.3.c + Merge candidate list reordering | -0.88% | 109% | 103% |
| 4.2.8.b | Adding middle spatial and multiple temporal candidates (number of candidates is 10) | -0.29% | 104% | 99% |
| 4.2.13.a | Additional merge candidates (number of candidates is 10) | -0.88% | 108% | 104% |
| 4.2.13.b | Further reduce candidate number (number of candidates is 8) | -0.88% | 107% | 104% |
| 4.2.15.a | Extended spatial positions from 6 to 27 (number of candidates is 11) | -1.14% | 109% | 101% |

Tests here add additional spatial merge candidates. Differences are, 1) the position from where candidates are derived, 2) the order of searching more candidates, 3) the number of candidates.

Possible explanations for decoding time increase are, 1) constructing longer list, 2) searching for valid motion info within a search range.

From test 4.2.3 and 4.2.13, it seems the coding gain saturate as the number of candidates increase. 5 looks like the magic number.

Test 4.2.15a shows -1.14% coding gain which is the highest coding gain in this category. On the other hand, the number of RD checks should be taken into account when making a comparison since it may have an impact on the coding gain.

Generally speaking, techniques in this category requires accessing more motion data in the coded area, additional buffer including the line number may be required.

CU based candidate – Extended spatial candidates

| Test# | Description | BD-Rate (Y) RA | EncT | DecT |
| --- | --- | --- | --- | --- |
| 4.2.9.a | UMVE candidates as additional merge candidates with additional information signaled | -1.06% | 142% | 118% |
| 4.2.9.b1 | UMVE candidates as independent merge candidates with additional information signaled, number of candidates is 120 | -1.93% | 127% | 110% |
| 4.2.9.b2 | UMVE candidates as independent merge candidates with additional information signaled, number of candidates is 384 | -1.53% | 144% | 113% |
| 4.2.9.b3 | UMVE candidates as independent merge candidates with additional information signaled, number of candidates is 32 | -1.67% | 121% | 102% |
| 4.2.15.b | Merge index 0 with MV offsets (number of candidates is 13) | -1.04% | 126% | 101% |

Test 4.2.9 constructs a list of four candidates by reusing existing merge candidates, and then extend the merge candidates in two aspects, 1) prediction direction specification, including constructing mirrored motion vector, 2) MV offsets around each starting point, in cross pattern and non-linearly spaced. Additional syntax for indicating the prediction direction, starting point selection, and MV offset are signaled. MV offset is represented by distance IDX and Direction IDX, instead of the regular x/y component representation. The number of candidates for b1, b2 and b3 is 120, 384 and 32, separately.

All the new candidates in test 4.2.9 is encapsulated as UMVE. It is signaled by using an index of a regular merge list (4.2.9.a) separately by a flag after skip/merge flag. Note that the coding gain gap of different signaling method is big.

Test 4.2.15.b derives 8 neighboring motion vectors around the first merge candidates. All the 8 candidates are horizontally and vertically 1-pel away from the position the first merge candidate points to. The 8 candidates are appended to the end of the current merge list.

CU based candidate – Combined/Split merge candidates

| Test# | Description | BD-Rate (Y) RA | EncT | DecT |
| --- | --- | --- | --- | --- |
| 4.2.2 | Restricted merge | -0.21% | 139% | 102% |
| 4.2.8.c | Adding pairwise-average candidates and removing HEVC combined candidates (number of candidates is 10) | -0.41% | 103% | 100% |
| 4.2.15.c | Combined average merge candidates | -0.31% | 102% | 100% |

Test 4.2.2 construct two additional merge list, one with uni-prediction candidate in L0 direction, the other with uni-prediction candidate in L1 direction. The new uni-directional candidates are constructed by splitting regular merge candidates.

Test 4.2.8.a and test 4.2.15.c are quite similar. Combination of the first 4 regular merge candidates is performed to derive 6 pair candidates, and then each pair of candidates are averaged to get one combined candidate. The difference is in the reference picture selection when the two MV to be averaged points to different reference pictures.

Sub-block based candidate – Affine candidates

| Test# | Description | BD-Rate (Y) RA | Enc | DecT |
| --- | --- | --- | --- | --- |
| AFFINE | BMS AFFINE as is | -2.99% | 137% | 112% |
| 4.2.3.e | 4.2.3.d + Unified merge candidate list (number of all candidates is 15) | / | / | / |
| 4.2.4.a | Combined test 4.2.3.e with model storage at CU level | -4.03% | 146% | 117% |
| 4.2.4.b | Combined test 4.2.3.e with added affine candidates (17 candidates with up to 8 Affine ones) | -4.03% | 148% | 119% |
| 4.2.4.c | 4.2.4.a + 4.2.4.b | -4.08% | 150% | 129% |
| 4.2.7 | Affine merge with up to two affine merge candidate | -3.10% | 143% | 115% |
| 4.2.8.a | Adding affine merge candidates (number of all candidates is 6) | -3.36% | 132% | 122% |
| 4.2.10.a | Model based affine candidates (number of affine candidates is 2) | -3.09% | 137% | 110% |
| 4.2.10.b | Additional control point based affine candidates (number of affine candidates is 5) | -4.08% | 155% | 117% |
| 4.2.12.a | Use affine model of the neighbor coding unit with largest size as the merge candidate | -3.05% | 138% | 112% |
| 4.2.12.b | Add a separate merge list with 8 candidates using different motion models | -3.97% | 167% | 129% |
| 4.2.12.c | 4.2.12.a + 4.2.12.b | -3.99% | 167% | 129% |

Since tests here inherently requires affine motion compensation, BMS affine is suggested to be turned on for both anchor the test. And therefore, the RD performance of BMS affine is listed for comparison.

In BMS affine merge mode, only one affine merge candidate is allowed. It is inherited from the first available neighboring block with affine mode. Using the inherited affine model, two motion vectors at the top-left and top-right corners of the current CU is derived, and then the motion vector for each sub-block is computed using the two derived CPMVs.

The commonality among all tests are

- BMS affine merge mode is extended by adding more inherited affine merge candidates.

- Various constructed affine merge candidates are added, including 4-param / 6-param affine model based candidates.

Differences are

- The position from where a candidate is derived, including added neighboring and long distance spatial positions, and temporal positions.

- The number of inherited affine candidates and constructed affine candidates.

- Reference index derivation when constructing an affine merge candidates.

- Whether affine candidates are put in a separate list or in the existing merge list.

- In case of separate list, the order in which affine candidates are organized.

- In case of no separate list, the way of inserting affine candidates in the existing candidate list.

Note that the motion of four neighboring blocks are used to construct a bilinear motion model in test 4.2.12.

Highest coding gain from adding more affine merge candidates shown here is about 1% in VTM tool test.

Sub-block based candidate – Planar candidates

| Test# | Description | BD-Rate (Y) RA | Enc | DecT |
| --- | --- | --- | --- | --- |
| 4.2.11 | MV Planar | -0.32% | 107% | 110% |
| 4.2.14 | MV Planar | -0.91% | 108% | 119% |

Basic idea of the two MV planar mechanisms here is extrapolating motion vectors of each sub-block inside a coding block similarly as intra prediction planar mode.

In case of no motion info in the L-shaped neighboring area, the motion vector is derived in the same manner as intra reference pixel padding, separately for L0 and L1.

In test 4.2.11, inter prediction direction “inter_pred_idc” and reference index “ref_idx” for both L0 and L1 lists are signalled. In contrast, prediction direction is derived at the decoder side, and the ref_idx is set to 0 for both L0 and L1.

Another major difference from test 4.2.11 is using the bottom-right corner temporal motion vectors for planar prediction.

Sub-block based candidate – ATMVP modification

| Test# | Description | BD-Rate (Y) RA | Enc | DecT |
| --- | --- | --- | --- | --- |
| ATMVP | BMS ATMVP as is | -0.87% | 104% | 120% |
| ATMVP +HPMV | BMS ATMVP + HPMV | -1.10% | 102% | 122% |
| 4.2.5.a | ATMVP with one fixed collocated picture | -1.09% | 105% | 122% |
| 4.2.5.b | Picture/slice-level adaptation of ATMVP sub-block-size | -1.06% | 103% | 106% |
| 4.2.5.c | 4.2.5.a + 4.2.5.b | -1.04% | 102% | 105% |
| 4.2.6.a | Simplified ATMVP with one fixed collocated picture | -1.09% | 106% | 114% |
| 4.2.6.b | 4.2.6.a + 4.2.5.b | -1.03% | 102% | 105% |
| 4.2.8.d | Double ATMVP candidates with uniformity check and decoder-side speedup | -1.00% | 105% | 118% |
| 4.2.16.a | ATMVP in BMS | -0.87% | 103% | 115% |

Test 4.2.5.a and test 4.2.6.a propose using one fixed collocated picture for fetching motion for ATMVP mode. The difference is in handling the situation when the motion vector of a neighboring block points to a reference picture other than the fixed collocated picture. Test 4.2.5.a performs MV scaling while test 4.2.6.a simply use the collocated blocks.

The same slice level adaptation of sub-block size is applied in 4.2.5.b and 4.2.6.b.

In test 4.2.8.d, two MVs from merge candidate list instead of one are used as temporal MVs to generate two ATMVP candidates. Two types of uniformity check are then performed to the two candidates

- If the motion of the four corners are the same for the two ATMVP candidates, the second candidate is removed from the list.

- If the motion of the four corners the same as the center motion for an ATMVP candidate, it is treated as TMVP.

Sub-block based candidate – STMVP

| Test# | Description | BD-Rate (Y) RA | Enc | DecT |
| --- | --- | --- | --- | --- |
| ATMVP | BMS ATMVP as is | -0.87% | 104% | 120% |
| 4.2.16.b | STMVP in JEM | -0.76% | 105% | 121% |
| 4.2.16.c | 4.2.16.a + 4.2.16.b | -1.30% | 109% | 128% |

The tests provide results of JEM STMVP and the combination with ATMVP.

Related contributions

| JVET # | Title | Source |
| --- | --- | --- |
| JVET-K0056 | Non-CE4: Merge mode modification | T. Solovyev, J. Chen, S. Ikonin (Huawei) |
| JVET-K0065 | CE4 related: Candidate list reordering | L. Xu, F. Chen, S. Ye, L. Wang (Hikvision) |
| JVET-K0104 | CE4-related: History-based Motion Vector Prediction | L.Zhang, K.Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance) |
| JVET-K0189 | Non-CE4: ATMVP simplification | H. Chen, H. Yang, J. Chen (Huawei) |
| JVET-K0246 | CE4.2-related: MV buffer reduction for non-adjacent spatial merge candidates | Y.-L. Hsiao, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-K0267 | Non CE4 : Virtual Temporal Affine | F. Galpin, T. Poirier, F. LeLeannec, A. Robert (Technicolor) |
| JVET-K0297 | CE4-related: Reduce line buffer for additional merge candidates | J. Ye, X. Li, S. Liu, X. Xu, G. Li(Tencent) |
| JVET-K0301 | CE4-related: extension of merge and AMVP candidates for inter prediction | G. Li, X. Xu, X. Li, S. Liu (Tencent) |
| JVET-K0304 | CE4-related: ranking based spatial merge candidate list for inter prediction | G. Li, X. Xu, X. Li, S. Liu (Tencent) |
| JVET-K0346 | CE4-related: One simplified design of advanced temporal motion vector prediction (ATMVP) | X. Xiu, Y. He, Y. Ye (InterDigital) |
| JVET-K0350 | CE4-related: Improvement on Merge/Skip mode with line buffer restriction | Y. Han, Y.-H. Chao, W.-J. Chien, M. Karczewicz (Qualcomm) |
| JVET-K0352 | CE4-related: Encoder optimization on top of CE 4.2.13 | J. Ye, X. Li, S. Liu (Tencent) |
| JVET-K0364 | Non-CE4: separate merge candidate list for sub-block modes | T. Fu, H. Chen, H. Yang, J. Chen (huawei) |

CE4.3: Motion vector coding

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.3.1 | MV prediction between two directions for AMVP mode | JVET-K0234 (Ericsson) |
| 4.3.2 | Symmetrical mode for bi-prediction | JVET-K0188 (Huawei) |
| 4.3.3.a | Adaptive MV resolution (1/4 or 1 or 4) | JVET-K0357 (Qualcomm) |
| 4.3.3.b | 4.3.3.a + Efficient MVD coding |  |
| 4.3.4 | Adaptive motion vector resolution in J0018 (1/4 or 1 or 4) | JVET-K0247 (MediaTek) |
| 4.3.5.a | AMVR in J0024 with three MV resolutions ( ¼ or 1 or 4) | JVET-K0116 (Samsung) |
| 4.3.5.b | AMVR in J0024 with five MV resolutions (1/4 or ½ or 1 or 2 or 4) |  |
| 4.3.5.c | 4.3.5.a + combined signaling of MVR index and MVP index |  |
| 4.3.5.d | 4.3.5.b + combined signaling of MVR index and MVP index |  |
| 4.3.6 | Unicity Check for AMVP candidate list | JVET-K0208 (Technicolor) |

Test results

Random access

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| BMS AMVR | -1.75% | -2.36% | -2.39% | 116% | 100% | -1.10% | -1.51% | -1.49% | 103% | 98% |
| 4.3.1 | -0.29% | -0.27% | -0.24% | 102% | 100% | -0.13% | -0.11% | -0.14% | 101% | 99% |
| 4.3.2 | -0.84% | -0.62% | -0.56% | 105% | 100% | -0.15% | -0.14% | -0.18% | 102% | 100% |
| 4.3.3.a | -1.75% | -2.36% | -2.39% | 118% | 101% | -1.10% | -1.51% | -1.49% | 103% | 98% |
| 4.3.3.b | -1.79% | -2.40% | -2.43% | 117% | 101% | -1.10% | -1.56% | -1.49% | 103% | 98% |
| 4.3.4 | -1.79% | -2.41% | -2.52% | 121% | 100% | -1.12% | -1.57% | -1.58% | 105% | 98% |
| 4.3.5.a | -1.46% | -1.91% | -1.98% | 91% | 98% | -0.74% | -1.35% | -1.38% | 91% | 89% |
| 4.3.5.b | -1.60% | -2.17% | -2.35% | 111% | 99% | -0.81% | -1.49% | -1.56% | 97% | 89% |
| 4.3.5.c | -1.04% | -1.48% | -1.57% | 93% | 99% | -0.52% | -1.08% | -1.11% | 92% | 89% |
| 4.3.5.d | -1.16% | -1.75% | -1.87% | 114% | 99% | -0.59% | -1.24% | -1.33% | 98% | 89% |
| 4.3.6 | -0.07% | -0.08% | -0.03% | 101% | 102% | -0.05% | -0.12% | -0.06% | 101% | 103% |

Low delay B

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| BMS AMVR | -0.68% | -1.27% | -1.39% | 131% | 102% | -0.57% | -0.99% | -1.10% | 107% | 97% |
| 4.3.1 | / | / | / | / | / | / | / | / | / | / |
| 4.3.2 | / | / | / | / | / | / | / | / | / | / |
| 4.3.3.a | -0.68% | -1.27% | -1.39% | 129% | 100% | -0.57% | -0.99% | -1.10% | 108% | 98% |
| 4.3.3.b | -0.76% | -1.20% | -1.35% | 129% | 100% | -0.67% | -1.21% | -1.13% | 111% | 98% |
| 4.3.4 | -0.72% | -1.39% | -1.56% | 142% | 99% | -0.70% | -1.16% | -1.31% | 115% | 99% |
| 4.3.5.a | -0.65% | -1.25% | -1.33% | 130% | 98% | -0.65% | -1.25% | -1.33% | 130% | 98% |
| 4.3.5.b | -0.67% | -1.55% | -1.66% | 167% | 99% | -0.67% | -1.55% | -1.66% | 167% | 99% |
| 4.3.5.c | -0.53% | -1.21% | -1.34% | 132% | 98% | -0.53% | -1.21% | -1.34% | 132% | 98% |
| 4.3.5.d | -0.56% | -1.34% | -1.59% | 166% | 78% | -0.47% | -1.02% | -1.21% | 116% | 91% |
| 4.3.6 | -0.01% | -0.09% | 0.04% | 101% | 96% | -0.19% | -0.20% | 0.00% | 100% | 104% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.3.1 | Huanbang Chen (Huawei) | 2 | F | Y | G |
| 4.3.2 | Krit Panusopone (Arris) | 2 | F | Y | M |
| 4.3.3.a | Xiaoyu Xiu (InterDigital) | 2 | F | Y | M |
| 4.3.3.b | Xiaoyu Xiu (InterDigital) | 2 | F | Y | M |
| 4.3.4 | Seungsoo Jeong (Samsung) | 2 | F | Y | M |
| 4.3.5.a | Guichun Li (Tencent) | 2 | F | Y | M |
| 4.3.5.b | Guichun Li (Tencent) | 2 | F | Y | M |
| 4.3.5.c | Guichun Li (Tencent) | 2 | F | Y | M |
| 4.3.5.d | Guichun Li (Tencent) | 2 | F | N | M |
| 4.3.6 | Huanbang Chen (Huawei) | 2 | P | Y | M |

Crosschecker comments on CE4.3.1

The proponent ran the simulation with "SIMD=AVX2", while crosschecker ran with "SIMD=SSE42".

Crosschecker comments on CE4.3.5

Test results matched on all configurations except for VTM LDB, which is caused by an issue in the tester's simulation

Detailed description of tests

CE4.3.1 in JVET- K0234 (Ericsson)

As described in JVET-J0012 section 2.1.7.4, the proposed tool exploits correlation between the two motion vectors for bi-prediction when the current block is coded using the AMVP mode. The tool allows a motion vector in a bi-prediction pair to be scaled and used as a predictor for the second vector. Furthermore, the tool makes the reconstruction order of the bi-prediction pair adaptively depending on how promising the L1 motion vector prediction candidate list is. A promising candidate list here means a list containing a non-scaled prediction candidate.

No change is made to the parsing of the motion vector differences. The reconstruction process of the bi-prediction pair starts with deriving the two candidate lists for L0 and L1 as normal. If the L1 candidate list does not contain a scaled candidate, the L1 vector is determined to be promising and therefore get reconstructed first. After that, if the L0 prediction list contains a scaled candidate, the first scaled candidate in the L0 candidate list is replaced by a scaled version of the L1 vector. Otherwise, if the L1 candidate list does contain a scaled candidate, the L0 vector is reconstructed first. After that, the first scaled candidate in the L1 candidate list is replaced with a scaled version of the L0 vector.

The tool is not enabled when the two reference picture lists contains the same reference pictures with mvd_l1_zero_flag being enabled.

Key points

- MV in one direction is used as MVP in the other direction

- The prediction direction (L0->L1 or L1->L0) depends on whether an MVP is a scaled one or not

- Enabled only when mvd_l1_zero_flag being is disabled

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.3.1 | J0012 MV prediction between two directions for AMVP mode, on/off test. | Ruoyang Yu (Ericsson) | Huanbang Chen (Huawei) |

CE4.3.2 in JVET- K0188 (Huawei)

In this contribution, a new mode is proposed for motion information coding in bi-prediction. A symmetrical mode flag indicating whether symmetrical mode is used or not is explicitly signaled if the prediction direction is bi-prediction. When the flag is true, the reference index and MVD for list 0 are derived from motion information of list 1. More ever, the reference index of list 1 is inferred as 0 for this mode.

Key points

- A symmetrical mode flag is explicitly signaled for each CU in inter bi-prediction mode

- Ref_idx in L1 is enforced to be 0, Ref_idx in L1 is inferred to be symmetrical of L0 reference

- MVD in L0 is mirrored from MVD in L1

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.3.2 | Symmetrical mode for bi-prediction in J0063. | Huanbang Chen (Huawei) | Krit Panusopone (Arris) |

CE4.3.3 in JVET- K0357 (Qualcomm)

Motion vector differences (MVDs) can be signaled in different precision to allow flexible MVD coding for video sequences with different resolution. In JEM and BMS, MVD could be signaled either in units of quarter luma sample, integer luma sample or four luma samples.

A variable length MVD resolution flag (0 to 2 bits) is conditionally signaled in CU level for those that have at least one non-zero MVD components with the first bit identifying whether quarter luma sample MVD precision is used. When the first bit (equal to 1) indicates that quarter luma sample MVD precision is not used, a second bit is signaled to indicate if integer luma sample MVD precision or four luma samples MVD precision is used.

When a zero is signaled for the first bit of the MVD resolution flag, quarter luma sample MVD resolution is used. When the MVD resolution flag is not signaled (which means both MVDs for reference list 0 and reference list 1 are zero), quarter luma sample MVD resolution is inferred. In the other cases when integer-luma sample MVD precision or four luma samples MVD precision is adopted, the MVP candidates in the AMVP candidate list is rounded to the corresponding precision.

The scheme of MVD coding context is modified in the proposed MVD coding method so that the binarization and context modeling are dependent on the MVD precision and the POC distance between the current frame and the reference frame.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.3.3 | LAMVR and MVD coding a) Adaptive MV resolution (1/4 or 1 or 4). b) a) + MVD coding. | Yan Zhang (Qualcomm) | Xiaoyu Xiu (InterDigital) |

CE4.3.4 in JVET- K0247 (MediaTek)

Key points

- The AMVR flag is always coded in AMVP mode to indicate the selection of MVD and MVP resolution, even if the coded MVD is zero.

- The AMVR flag is coded before MVD and after affine flag.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.3.4 | Adaptive motion vector resolution in J0018 (1/4 or 1 or 4). | Tzu-Der Chuang (MediaTek) | Seungsoo Jeong (Samsung) |

CE4.3.5 in JVET- K0116 (Samsung)

Supported motion vector resolutions in proposed method range from 1/4–pel to 4–pel (1/4–pel, 1/2–pel, 1–pel, 2–pel and 4–pel). Information about the motion vector resolution is signaled at the CU level when MVD information is signaled.

Depending on the resolution of CU, both motion vector (MV) and motion vector predictor (MVP) of the CU are adjusted. If applied motion vector resolution is denoted as R, MV (MVx, MVy) and MVP (MVPx, MVPy) are represented as follows:

(MVx, MVy) = (Round(MVx / (R * 4)) * (R * 4), Round(MVy / (R * 4)) * (R * 4))

(MVPx, MVPy) = (Round(MVPx / (R * 4)) * (R * 4), Round(MVPy / (R * 4)) * (R * 4))

Since the motion vector predictor and MV are both adjusted by an adaptive resolution, MVD (MVDx, MVDy) is also aligned to the resolution and, is signaled according to the resolution as follows:

(MVDx, MVDy) = ((MVx – MVPx) / (R * 4), (MVy – MVPy) / R * 4))

In HEVC, AMVP mode uses MVP index for selecting the motion vector predictor among those in the MVP candidate list. In this proposal, motion vector resolution index (MVR index) indicates MVP index as well as the motion vector resolution. As result, the proposed method has no MVP index signaling. Table 1 shows what each value of MVR index represents.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.3.5 | J0024 a) AMVR in J0024 with three MV resolutions ( ¼ or 1 or 4) b) AMVR in J0024 with five MV resolutions (1/4 or ½ or 1 or 2 or 4) c) Combined signaling of MVR index and MVP index with three MV resolutions ( ¼ or 1 or 4) in J0024 d) Combined signaling of MVR index and MVP index with five MV resolutions (1/4 or ½ or 1 or 2 or 4) in J0024 | Anish Tamse (Samsung) | Guichun Li (Tencent) |

CE4.3.6 in JVET- K0208 (Technicolor)

All the inter coding modes need to apply a process for generating the lists of candidate motion information. In the current response, these processes are homogenized and designed to ensure the full diversity and unicity of candidates.

The first generic aspect consists in unicity checking before adding any new candidate to the list. If the new candidate is already present in the list, then it is not added. This check is performed for all inter coding modes: AMVP, affine AMVP, merge/skip (and template matching merge modes).

In some inter coding modes, other processes also improve the diversity of the lists, as explained below.

- In AMVP, the unicity check is also ported into the spatial candidate collection which constructs temporary candidate list. Furthermore, in AMVP and affine AMVP, when rounding and/or clipping operations are performed, these operations are achieved as soon as a new candidate is reached. The rounded and/or clipped candidate is checked before being added or not into the list.

- In Merge, whereas in the JEM only the spatial candidate redundancy was checked, the unicity check is applied to all candidate types except the ATMVP (and STMVP) ones.

- (Candidates used to build the Template Matching and Bilateral Matching lists are the same as the ones used to build the merge list. The main difference with the merge list derivation is that, during the unicity check, the IC flag of those candidates is not considered, because it is not part of the derived/predicted motion information in these modes.)

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.3.6 | J0022 Unicity Check for AMVP candidate list | A. Robert (Technicolor) | H. Chen (Huawei) |

Categorization & comparative study

For easier comparison, only VTM tool test results in RA configuration is listed here. We can refer to more results when necessary. The description is also for VTM implementation.

Adaptive motion vector resolution

| Test# | Description | BD-Rate (Y) RA | EncT | DecT |
| --- | --- | --- | --- | --- |
| AMVR | BMS AMVR as is | -1.75% | 116% | 100% |
| 4.3.3.a | Adaptive MV resolution (1/4 or 1 or 4) | -1.75% | 118% | 101% |
| 4.3.3.b | 4.3.3.a + Efficient MVD coding | -1.79% | 117% | 101% |
| 4.3.4 | Adaptive motion vector resolution in J0018 (1/4 or 1 or 4) | -1.79% | 121% | 100% |
| 4.3.5.a | AMVR in J0024 with three MV resolutions ( ¼ or 1 or 4) | -1.46% | 91% | 98% |
| 4.3.5.b | AMVR in J0024 with five MV resolutions (1/4 or ½ or 1 or 2 or 4) | -1.60% | 111% | 99% |
| 4.3.5.c | 4.3.5.a + combined signaling of MVR index and MVP index | -1.04% | 93% | 99% |
| 4.3.5.d | 4.3.5.b + combined signaling of MVR index and MVP index | -1.16% | 114% | 99% |

Test 4.3.3.b modifies MVD coding in a way that the binarization and context modeling are dependent on MVD precision and POC distance between the current frame and the reference frame.

Test 4.3.4 is different from BMS AMVR in that AMVR flag is always signaled. Results shown removing the conditional checking does not cause performance drop. A possible reason is effectively more MVP candidates are provided in addition to the two AMVP candidates.

Similarly, AMVR flag signaling in test 4.3.5.a does not depend on the existence of MVD. With more additional two resolutions (1/2 and 2) the coding gain increase about 0.16%.

Test 4.3.5.c/d use the MVR index to indicate both the motion vector resolution and the motion vector predictor at the same time. In other words, each AMVP candidate and the motion vector difference from the predictor has a dedicated resolution. This effectively reduces the flexibility of the combination of motion vector predictor and motion vector resolution, which brings performance drop as results shows.

AMVP list construction

| Test# | Description | BD-Rate (Y) RA | EncT | DecT |
| --- | --- | --- | --- | --- |
| 4.3.6 | Unicity Check for AMVP candidate list | -0.07% | 101% | 102% |

This unicity check is performed for various inter coding modes including AMVP, affine AMVP, merge / skip, in order to remove duplications in a candidate list.

Motion vector prediction

| Test# | Description | BD-Rate (Y) RA | EncT | DecT |
| --- | --- | --- | --- | --- |
| 4.3.1 | MV prediction between two directions for AMVP mode | -0.29% | 102% | 100% |
| 4.3.2 | Symmetrical mode for bi-prediction | -0.84% | 105% | 100% |

In test 4.3.1, MV in one direction is used as MVP in the other direction. The prediction direction (L0->L1 or L1->L0) depends on whether an MVP is a scaled one or not. And therefore the order of reconstructing MV in L0 and L1 depends on the way of constructing the MVP. Note that no syntax is signaled for the proposed prediction.

In test 4.3.2, a symmetrical mode flag is explicitly signaled for each CU in inter bi-prediction mode. L1 ref_idx is enforced to be 0, and L1 ref_idx is inferred to be symmetrical of L0’s. L0 MVD is mirrored from L1 MVD.

Both methods are not applied to LDB configuration.

Related contributions

| JVET # | Title | Source |
| --- | --- | --- |
| JVET-K0302 | CE4-related: Bilateral Motion Vector Prediction | B. Choi, F. Bossen, A. Segall (Sharp) |

CE4.4: Generalized bi-direction

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.4.1 | Generalized bi-prediction | JVET-K0248 (MediaTek) |

Test results

Random access

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.4.1 | -0.86% | -0.96% | -0.95% | 114% | 102% | -0.56% | -0.74% | -0.80% | 108% | 102% |

Low delay B

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.4.1 | -1.07% | -0.37% | -0.26% | 114% | 102% | -0.40% | -0.54% | -0.20% | 112% | 100% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.4.1 | Saverio Blasi (BBC) | 2 | F | Y | M |

Detailed description of tests

CE4.4.1 in JVET- K0248 (MediaTek)

In this contribution, GBi is presented to allow applying different weights to predictors from L0 and L1. The predictor generation is shown in Equ. (2).

| - PGBi = ( (1-w1)* PL0 + w1 * PL1 + RoundingOffsetGBi) >> shiftNumGBi, | (2) |
| --- | --- |

Weights for true bi-prediction cases in random access (RA) condition.

| GBi Index | Weight value of w1 | Binarization of GBi Index |
| --- | --- | --- |
| 0 | 3/8 | 00 |
| 1 | 1/2 | 1 |
| 2 | 5/8 | 01 |

Weights in generalized bi-prediction

| GBi Index | Weight value of w1 | Binarization of GBi Index |
| --- | --- | --- |
| 0 | -1/4 | 0000 |
| 1 | 3/8 | 001 |
| 2 | 1/2 | 1 |
| 3 | 5/8 | 01 |
| 4 | 5/4 | 0001 |

For advanced motion vector prediction (AMVP) mode, the weight selection in GBi is explicitly signaled at CU-level if this CU is coded by bi-prediction. For merge mode, the weight selection is inherited from the merge candidate. In this proposal, GBi supports DMVR to generate the weighted average of template as well as the final predictor for BMS-1.0.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.4.1 | Generalized bi-prediction in J0018 | Ching-Yeh Chen (MediaTek) | Saverio Blasi (BBC) |

CE4.5: Reference picture boundary padding

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.5.1 | Multi-directional boundary padding (MDBP) | JVET-Kxxxx (HHI) |
| 4.5.2 | MC Padding | JVET-K0363 (Qualcomm) |
| 4.5.3 | Boundary pixel padding using motion compensation | JVET-K0117 (Samsung) |

Test results

Random access

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.5.1 | -0.07% | -0.05% | -0.07% | 102% | 101% | -0.04% | -0.10% | -0.08% | 102% | 102% |
| 4.5.2 | -0.22% | -0.20% | -0.17% | 100% | 104% | -0.22% | -0.22% | -0.19% | 107% | 108% |
| 4.5.3 | -0.18% | -0.19% | -0.16% | 100% | 107% | -0.18% | -0.17% | -0.17% | 100% | 103% |

Low delay B

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.5.1 | -0.07% | -0.07% | -0.11% | 104% | 103% | -0.13% | -0.08% | -0.07% | 103% | 100% |
| 4.5.2 | -0.02% | 0.04% | 0.10% | 102% | 105% | -0.08% | -0.11% | 0.11% | 104% | 105% |
| 4.5.3 | -0.01% | 0.12% | 0.01% | 101% | 109% | -0.07% | -0.15% | 0.03% | 99% | 105% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.5.1 | Yuling Hsiao (MediaTek) | 2 | F | Y | M |
| 4.5.2 | Minsoo Park (Samsung) | 2 | F | Y | M |
| 4.5.3 | Robert Skupin (HHI) | 2 | F | Y | M |

Detailed description of tests

CE4.5.1 in JVET-Kxxxx (HHI)

Note that there is no separate input document for this test.

This contribution proposes an intra prediction based padding method for referenced areas outside the reference picture.

Key points:

- Intra prediction based boundary padding uses one out of 33 angular intra prediction modes (-45° to +45°) to pad the outside area of a referenced block.

- For each outside area of a referenced block the intra prediction mode is estimated in encoder and decoder.

- Estimation uses a template based approach, with a template size of two lines and a template line width equal to the width of the referenced block.

- During motion vector estimation, the estimation of the angular intra prediction mode is performed only once, after final motion vector refinement has been performed.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.5.1 | Multi-directional boundary padding (MDBP) in J0014 | Jens Brandenburg (HHI) | Yuling Hsiao (MediaTek) |

CE4.5.2 in JVET-K0363 (Qualcomm)

Key points

- Uni-directional MC is performed for boundary pixel padding

  - If the boundary 4x4 block is predicted using uni-directional inter prediction. The only motion vector within the block will be used for motion compensated boundary pixel padding.

  - In case of bi-directional inter prediction, only the motion vector, which points to the pixel position farther away from the frame boundary in the reference slice in terms of the padding direction, is used in motion compensated boundary pixel padding.

- If the boundary 4x4 block is intra coded or the motion vector is not available, repetitive padding will be used.

- Residual compensation
The difference between the DC values of the boundary 4x4 block in the current slice and its corresponding reference 4x4 block in the reference slice is used as the offset to filter the fetched motion compensated image slip before it is copied to the padding area beyond the image boundary.

- Padding area width/height: smaller than 64.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.5.2 | MC Padding | Yan Zhang (Qualcomm) | Minsoo Park (Samsung) |

CE4.5.3 in JVET-K0117 (Samsung)

This contribution proposes a padding method for padding outside areas of a picture with motion compensation with motion information of edge pixel of the picture.

Key points

- If the area padded by MC is shorter than the size of the padding area, the rest of the area is filled with extrapolated edge pixels of the external picture.

- A conventional padding method is performed when an adjacent area is unavailable or there is no information about a motion in a boundary block.

- In case of bi-directional motion in a boundary 4x4 block, bi-directional MC is used for padding.

- The last pixel of each position is extrapolated to induce a left upper portion, a right upper portion, a left lower portion, and a right padding area.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.5.3 | Boundary pixel padding using motion compensation in J0025 | Minsoo Park (Samsung) | Robert Skupin (HHI) |

Related contributions

| JVET # | Title | Source |
| --- | --- | --- |
| JVET-K0195 | CE4-5 related: Inter/Intra Boundary Padding | J. Brandenburg, R. Skupin, H. Schwarz, D. Marpe, T. Schierl, T. Wiegand (HHI) |

CE4.6: Local illumination compensation

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.6.1 | Bi-directional illumination compensation (BIC) | Withdraw |
| 4.6.2 | Combination of BIC, generalized OBMC and Simplified BIC parameters derivation | Withdraw |
| 4.6.3 | Inter prediction refinement | JVET-K0118 (Samsung) |

Test results

Random access

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.6.1 | / | / | / | / | / | / | / | / | / | / |
| 4.6.2 | / | / | / | / | / | / | / | / | / | / |
| 4.6.3 | -0.37% | -0.32% | -0.29% | 127% | 112% | -0.31% | -0.10% | -0.17% | 113% | 106% |

Low delay B

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.6.1 | / | / | / | / | / | / | / | / | / | / |
| 4.6.2 | / | / | / | / | / | / | / | / | / | / |
| 4.6.3 | -0.31% | -0.09% | -0.16% | 146% | 111% | -0.30% | -0.24% | -0.11% | 125% | 107% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.6.1 | / | / | / | / | / |
| 4.6.2 | / | / | / | / | / |
| 4.6.3 | Technicolor | 2 | P | Y | M |

Crosschecker comments on CE4.6.3

Code use LIC parameters from above PUs only.

Detailed description of tests

CE4.6.1 (withdraw)

CE4.6.2 (withdraw)

CE4.6.3 in JVET- JVET-K0118 (Samsung)

At the end of processing of each CU, linear models (Ax + B = y) are computed by comparing the prediction pixels (x) and the reconstructed pixels (y). The models are computed using linear regression. The lower edge pixels are used for model computation. Two sets of models are computed, one for each colour component. Each set contains models for the two directions (if available). These parameters (sets of A and B) are then stored in the motion information buffer for later access.

The conditions under which IPR is applicable is kept same as that of JEM LIC. These conditions depend on the size of the current CU, type of prediction mode and whether the CU uses merge mode or not. If applicable, a flag is signaled to indicate whether IPR is to be applied to a given inter coded CU or not. If the tool is to be applied, the motion compensation pixels (x) are refined using the application of the linear model (Ax + B) to obtain the new set of prediction pixels. The model used is derived from the current CU’s neighbor. Top and top left neighbour are considered for borrowing the parameter. Since both the neighbors may be available, pruning is performed to obtain a single model. Intra coded neighbours are discarded in the pruning process. Next, the neighbours which have trivial model [x = y] are discarded in the pruning process. If no model is remaining, a trivial model is used. If model(s) still remain, the neighbours with the same reference index as the current CU are given higher priority. If multiple neighbours have the same reference index, the top neighbour is given higher priority compared to the top-left neighbour.

Additionally, when performing RDO check for determining whether IPR will be used or not, mean-removed SAD (Sum of Absolute Difference) is used instead of the regular SAD for the ME (Motion Estimation) process.

The linear regression method used is same as that in JEM. Also, using two sides for parameter derivation improves the gain.

Key points

- LIC model pre-computation and storage after processing a CU

- LIC model pruning to get the only one model applied to a CU

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.6.3 | J0024 Inter prediction refinement | Anish Tamse (Samsung) | P. Bordes (Technicolor) |

CE4.7: Chroma Interpolation filters

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.7.1 | Modified 4-tap chroma interpolation filter | JVET-K0207 (Huawei) |

Test results

Random access

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.7.1 | -0.01% | -0.57% | -0.62% | 102% | 99% | 0.02% | -0.33% | -0.41% | 100% | 99% |

Low delay B

|  | VTM_tool_test |  |  |  |  | BMS_tool_test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.7.1 | 0.03% | 0.31% | 0.31% | 100% | 98% | -0.03% | -0.22% | -0.08% | 100% | 103% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.7.1 | Anastasia Henkel (HHI) | 2 | F | Y | M |

Detailed description of tests

CE4.7.1 in JVET- JVET-K0207 (Huawei)

Chroma filter coefficients

| 0 | 0 | 64 | 0 | 0 |  | 16 | -7 | 39 | 39 | -7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 62 | 2 | 0 |  | 17 | -6 | 36 | 40 | -6 |
| 2 | -1 | 61 | 5 | -1 |  | 18 | -6 | 34 | 42 | -6 |
| 3 | -1 | 59 | 7 | -1 |  | 19 | -5 | 31 | 43 | -5 |
| 4 | -2 | 58 | 10 | -2 |  | 20 | -5 | 29 | 45 | -5 |
| 5 | -2 | 56 | 12 | -2 |  | 21 | -4 | 26 | 46 | -4 |
| 6 | -2 | 54 | 14 | -2 |  | 22 | -4 | 24 | 48 | -4 |
| 7 | -3 | 53 | 17 | -3 |  | 23 | -4 | 22 | 50 | -4 |
| 8 | -3 | 51 | 19 | -3 |  | 24 | -3 | 19 | 51 | -3 |
| 9 | -4 | 50 | 22 | -4 |  | 25 | -3 | 17 | 53 | -3 |
| 10 | -4 | 48 | 24 | -4 |  | 26 | -2 | 14 | 54 | -2 |
| 11 | -4 | 46 | 26 | -4 |  | 27 | -2 | 12 | 56 | -2 |
| 12 | -5 | 45 | 29 | -5 |  | 28 | -2 | 10 | 58 | -2 |
| 13 | -5 | 43 | 31 | -5 |  | 29 | -1 | 7 | 59 | -1 |
| 14 | -6 | 42 | 34 | -6 |  | 30 | -1 | 5 | 61 | -1 |
| 15 | -6 | 40 | 36 | -6 |  | 31 | 0 | 2 | 62 | 0 |

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.7.1 | J0024 Chroma interpolation filter | Sychev Maxim (Huawei) | Anastasia Henkel (HHI) |

Recommendations

Review all CE document if needed. Review all CE-related documents.