| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0025-v1 |
| --- | --- |

| Title: | CE5: Summary report of CE on Arithmetic Coding Engine |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tung Nguyen Fraunhofer HHI Amir Said Qualcomm Inc. | Email: | tung.nguyen@hhi.fraunhofer.de asaid@qti.qualcomm.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This report summarizes the experimental results and the findings for the Core Experiment 5 on Arithmetic Coding Engine. Twelve experiments have been conducted: two experiments in the main category (experiments 5.1.1 and 5.1.2), four experiments in Subset A (experiments 5.2.1 – 5.2.4), four experiments in Subset B (experiments 5.3.1 – 5.3.4), and two experiments in Subset C (experiments 5.4.1 and 5.4.2). The experimental results indicate that further analysis is necessary on the topic of probability estimators and its memory requirements. Furthermore, the results show that a final rLPS design that has a maximum size equal to or less than 2048 bit is sufficient to achieve the compression efficiency.

# Introduction

The experiments in this Core Experiment (CE) are divided into subsets addressing different aspects of the Arithmetic Coding Engine. The main category covers proposals that address all aspects of the design (experiments 5.1.1 and 5.1.2). The Subset A (experiments 5.2.1 – 5.2.4) addresses modifications to the probability estimator (PE), the Subset B (5.3.1 – 5.3.4) addresses modifications to the sub-interval range derivation for LPS (rLPS table), and the Subset C (experiments 5.4.1 and 5.4.2) addresses modifications to context model initialization and parameterization. It was mandatory to provide results without changes to the context initialization values when the proposal introduces new initialization values.

# Summary of the Results

## Main Category

The proposals in the main category indicate the best performance in compression efficiency that can be achieved for the time of this document, considering the increase in complexity. There were only two experiments in this sub-category, and they share the same basic design: (a) high-precision probability estimation; (b) double window probability estimation using different window pairs per context; (c) small multiplication tables; and (d) Context probability initialization from previous frames.

The results below summarize the performance of Experiment 5.1.1 (JVET-K0381).

| 5.1.1 | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -0.97% | -0.45% | -0.41% | 103% | 103% | -1.04% | -0.30% | -0.36% | 103% | 99% |
| RA | -1.02% | -0.14% | -0.28% | 102% | 101% | -1.17% | -0.21% | -0.44% | 103% | 100% |
| LB | -0.87% | 1.06% | 1.11% | 102% | 103% | -1.23% | 0.93% | 1.47% | 101% | 100% |

and for experiment 5.1.2 (JVET-K0283)

| 5.1.2 | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -0.98% | -0.66% | -0.68% | 106% | 103% | -0.84% | -0.47% | -0.58% | 105% | 107% |
| RA | -1.00% | -0.51% | -0.48% | 103% | 101% | -0.98% | -0.39% | -0.64% | 104% | 104% |
| LB | -1.17% | 0.44% | 0.55% | 103% | 102% | -1.32% | 0.57% | 0.82% | 102% | 100% |

The results for experiment 5.1.2 with a new array of context probability initializers are

| 5.1.2 (new init.) | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -1.03% | -0.64% | -0.74% | 106% | 104% | -0.91% | -0.58% | -0.67% | 105% | 101% |
| RA | -1.03% | -0.46% | -0.60% | 103% | 102% | -1.04% | -0.48% | -0.73% | 103% | 100% |
| LB | -1.14% | 0.59% | 0.56% | 103% | 102% | -1.34% | 0.96% | 0.39% | 102% | 99% |

Performance: Both experiments yield an improvement in coding efficiency of about 1% in BD-rate in the AI and RA configurations. Slight differences can be observed, but with deltas   between the two proposals smaller than 0.2%. Both the encoding and the decoding run times are similar with an increase of about 3% for the encoder and 1-2% for the decoder. Experiment 5.1.2 introduces new context initialization values. The difference in the performance is not significant, i.e., an improvement of about 0.05% in BD-rate can be observed with new initialization values.

Notes: Both experiments include the usage of context model states from the previously coded frame.

Observation: An improvement in BD-rate of about 0.9 – 1.3% can be achieved by modifying all parts of the arithmetic coding engine. A slight increase in run times can be observed. The performance can be achieved with a rLPS table size similar to the reference (compare experiments 5.1.1 and 5.1.2, and 5.2.1). The new initialization values do not improve the performance significantly (see detailed results for experiment 5.1.2).

Memory: As the reference, CABAC, as specified in HEVC, has a 64×4×8 (2048 bit) rLPS table, and each context model requires 7 bits. Moreover, a 64×6 (384 bit) transition table is necessary. Experiment 5.1.1 employs a 16×16×8 (2048 bits) rLPS table and custom window sizes for each context model. Each context model requires 2×15+4 (34 bits). The experiment 5.1.2 employs a 32×8×8 (2048 bits) rLPS table and custom window sizes for each context model, resulting in 2×15+4 (34 bits). For both experiments, each context model requires about 5.14 times the memory of the reference. The following table summarizes the memory requirements with all values in number of bit. The numbers are derived as follows:

- Number of context models: 359, by analyzing the BMS-1.0 software implementation

- Initialization values: 8×3×359=8616 (8-bit initialization values, three slice types, 359 context models)

- Total number of context memory: number of context models multiplied by memory per context

- CABAC in HEVC: Transition table for the states 64×6 (384 bit)

- Experiment 5.1.1: 4 bit for each context model specifying custom window size (1436 bit in total)

- Experiment 5.1.2: same as 5.1.1, but separately for each slice type (4308 bit in total)

| Configuration | rLPS | stateTable | cwInit | ROM | perCtx | RAM |
| --- | --- | --- | --- | --- | --- | --- |
| HEVC | 2048 | 384 | 0 | 11048 | 7 | 2513 |
| 5.1.1 | 2048 | 0 | 1436 | 12100 | 34 | 12206 |
| 5.1.2 | 2048 | 0 | 4308 | 14972 | 34 | 12206 |

## Subset A and B: Probability Estimation and Derivation of Sub-Interval Range for LPS

The idea of these two subsets is the evaluation of the proposed probability estimators (PE) and the modified rLPS tables separately. It is however not always possible to decouple the derivation of the sub-interval range for LPS from the probability estimation. The table below, therefore, summarizes the results of the experiments of both subsets.

| Experiment | Document / Proponent | Description | Results (AI/RA/LB-VTM/BMS) |
| --- | --- | --- | --- |
| CE5-2.1 | JVET-K0380 Qualcomm | PE fix, rLPS variation PE: 2×15+4 (34) Observation: Smaller rLPS table sizes can be realized, but requires more operations for the access. Memory: 1. Configuration: 10564 ROM / 12206 RAM 2. Configuration: 12100 ROM / 12206 RAM | rLPS: 8×8×8 (512) -0,95% -0,86% -0,61% -1,02% -X.XX% -0.92% rLPS: 16×16×8 (2048) -0.97% -0.88% -0.64% -1.04% -1.00% -0.95% |
| CE5-2.2 | JVET-K0283 HHI | rLPS fix, PE variation rLPS: 32×8×8 (2048) Observation: The memory requirement of multi-parameter PE with custom window sizes can be further reduced (see the second set of results). Memory: 1. Configuration: 10664 ROM / 8616 RAM 2. Configuration: 14972 ROM / 10052 RAM 3. Configuration: 10664 ROM / 10770 RAM 4. Configuration: 14972 ROM / 12206 RAM | PE: 10+14 (24) -0.74% -0.64% -0.50% -0.86% -0.81% -0.89% PE: 10+14+4 (28) -1.02% -1.00% -0.81% -0.94% -1.03% -1.10% PE: 15+15 (30) -0.76% -0.66% -0.55% -0.86% -0.80% -0.86% PE: 15+15+4 (34) -1.03% -0.99% -0.80% -0.91% -1.00% -1.11% |
| CE5-2.3 | JVET-K0282 DJI and Peking University | rLPS: 512×64×9 (294912) PE: the introduction of a counter, initial values of the MP are kept when the counter is below a threshold, equal to 2×15+10 (40) per context model. Observation: Results worse when compared to 5.1.1, especially for RA and LB. Memory: 14360 RAM | -0.66% 0.13% 0.22% -0.53% 0.05% 0.04% |
| CE5-2.4 | JVET-K0170 Samsung | rLPS: 512×64×9 (294912) PE: Different update strategy for the MP. Introduction of a counter, only short window is updated when the counter is below a threshold, equal to per 2×15+5 (35) context model. Observation: Performs similar to the configuration without custom window sizes but requires 1 bit less memory per context model. Memory: 12565 RAM | -0.81% -0.56% -0.39% -0.89% -0.64% -0.54% |
| CE5-3.1 | JVET-K0383 Qualcomm | PE fix, rLPS variation PE: 2×15 (CEM1) Observation: In comparison to 5.2.1, the “virtual” performance of custom window sizes can be derived. The improvement is about 0.5% in BD-rate for RA and of about 0.4% in LB. Not clear whether the drop in performance is due to initialization. For AI, the delta is 0.3% in BD-rate. Memory: 1. Configuration: 12100 ROM / 10770 RAM 2. Configuration: 10564 ROM / 10770 RAM | Relative to CABAC engine 1 of BMS-1.0 rLPS: 16×16×8 (2048) 0.00% 0.03% 0.03% 0.02% 0.0x% 0.0x% rLPS: 8×8×8 (512) 0.03% 0.06% 0.07% 0.04% 0.0x% 0.0x% |
| CE5-3.2 | JVET-K0283 HHI | PE fix, smaller rLPS table tested. PE: 10+14+2x3 (30 bit) Observation: Similar to experiments 5.1.1 and 5.3.1, an even smaller rLPS table performs close to larger rLPS table sizes. Memory: 13948 ROM / 10770 RAM | Relative to BMS-1.0 rLPS: 32×8×8 (2048) -0.99 -0.96 -0.77 -0.90 -0.98 -1.05 rLPS: 16×8×8 (1024) -0.99 -0.96 -0.77 -0.90 -0.98 -1.05 |
| CE5-3.3 | JVET-K0249 MediaTek | PE fix, smaller rLPS table tested. PE: 2×15 (CEM1) Observation: Performs similar to experiment 5.3.1 with a slightly lower performance for RA and LB. Memory: 1. Configuration: 9640 ROM / 10770 RAM 2. Configuration: 10664 ROM / 10770 RAM | Relative to CABAC engine 3 of BMS-1.0 rLPS: 16×8×8 (1024) 0.04% 0.03% 0.09% 0.05% 0.03% 0.06% rLPS: 32×8×8 (2048) 0.01% 0.00% 0.04% 0.02% -0.01% 0.02% |
| CE5-3.4 | JVET-K0249 MediaTek | PE fix, smaller rLPS tables, derivation requires multiplications PE: 2×15 (CEM1) Observation: Performs similar to experiment 5.3.3 with even smaller rLPS table sizes. Memory: 1. Configuration: 8816 ROM / 10770 RAM 2. Configuration: 8904 ROM / 10770 RAM | Relative to CABAC engine 3 of BMS-1.0 rLPS: 5×5×8 (200) 0.04% 0.03% 0.09% 0.05% 0.03% 0.06% rLPS: 6×6×8 (288) 0.01% 0.00% 0.04% 0.02% -0.01% 0.02% |

Summary: The main observation is that the size of the rLPS table can be similar to the HEVC design to achieve the higher compression efficiency provided by higher-resolution probability estimation (PE), and  better PE contributes highly to the compression efficiency improvement. The savings in ROM when using smaller rLPS table sizes is relatively small whereas the additional amount of RAM necessary for the multi-parameter estimators increases relatively to CABAC in HEVC.

## Subset C: Context Initialization and Parameterization

The final subset evaluates two experiments employing techniques that allow the inheritance of context model states from previously coded frames. The difference between the two proposals is the granularity level of the state inheritance, either slice level (7.4.1) or CTU-line level (7.4.2). The latter requires more memory but has benefit for parallel processing applications.

Summary: The improvement in compression efficiency is relatively small for RA configuration, whereas the improvement for LB-BMS is almost 0.7% for both schemes. It seems that the LB configuration is generally prone to initialization values.

## Related contributions

- JVET-K0273, “CE5-related: Implementation considerations for entropy coding engine,” F. Bossen (Sharp)

- JVET-K0385, “CE5-related: Context state memory reduction,” A. Said, H. Egilmez, Y-H. Chao, M. Karczewicz, and V. Seregin (Qualcomm)

- JVET-K0430, “CE5-related: State-based probability estimator,” H. Kirchhoffer, J. Stegemann, D. Marpe, H. Schwarz, and T. Wiegand (HHI) [late]

# Detailed Results: Main Category

### Experiment CE5.1.1 (Qualcomm, from JVET-J0021)

Cross-checker (Hikvision) does not provide any data. Cross-checker (HHI) confirms the provided (not completed) results.

### Experiment CE5.1.2 (HHI, from JVET-J0014)

Cross-checker (Qualcomm) confirms the results partially since the simulations are still running. The results shown below are first with and then without new initialization values.

# Detailed Results Subset A: Probability Estimation

### Experiment CE5.2.1 (Qualcomm, from JVET-J0021)

Cross-checker (Samsung) confirms the results. “EncT and DecT measured with AVX. Coding gains and en/decoding time are same with those provided by proponent. Provided code was verified. Two sets of testing were provided by proponent, but CE description was not described about which set will be tested.” The first set of results were generated using an 8×8×8 (512 bit) rLPS table and the second set of results were generated using a 16×16×8 (2048 bit) rLPS table.

### Experiment CE5.2.2 (HHI, from JVET-J0014)

Cross-checker (DJI) does not provide any data. The first set of results show the configuration using 10 and 14-bit counters for the window sizes and the second set of results show the same setup without new initialization values.

The following results were generated using the same configuration for the window sizes but with custom window sizes for each context model.

The following results were generated using two 15-bit counters for the window sizes.

The same window sizes configuration with and custom window sizes.

### Experiment CE5.2.3 (DJI and Peking University, from JVET-J0011)

Cross-checker (Qualcomm) confirms the results partially since the simulations are still running.

### Experiment CE5.2.4 (Samsung, from JVET-J0024)

Cross-checker (HHI) confirms the results.

# Detailed Results Subset B: Derivation of Sub-Interval Range for LPS

### Experiment CE5.3.1 (Qualcomm, from JVET-E0119)

Cross-checker (HHI) confirms the results provided by the proponent (not completed). First set of results show an 8×8×8 configuration and the second set of results shows a 16×16×8 configuration.

### Experiment CE5.3.2 (HHI, from JVET-J0014)

Cross-checker (MediaTek) confirms the results by the proponent. A rLPS of dimension 16×8×8 is used, and the derivation of input values for the look-up employs simple calculations (rounding and exclusive nor).  “The results match those provided by the proponent. The design is matched what is described in the CE description, except the table size is reduced to half (from 32x8 to 16x8) by default.” The first set of results show the performance with new initialization values whereas the second set of results show the performance without new initialization values.

### Experiment CE5.3.3 (MediaTek, from JVET-J0018)

Cross-checker (Qualcomm) confirms the results partially since the simulations are still running. The first set of results show the performance with a rLPS configuration having a dimension of 16×8 whereas the second set of results employs a 32×8 configuration for the rLPS. Proponent reports that the run times are not accurate since different computer types were used for anchor and test.

### Experiment CE5.3.4 (MediaTek, from JVET-J0018)

Cross-checker (Qualcomm) confirms the results partially since the simulations are still running. The first set of results show the performance for 5×5 configuration whereas the second set of results show a 6×5 configuration. Proponent reports that the run times are not accurate since different computer types were used for anchor and test.

# Detailed Results Subset C: Context Initialization and Parameterization

### Experiment CE5.4.1 (Qualcomm, from JVET-J0021)

Cross-checker (HHI) confirms the results.

### Experiment CE5.4.2 (HHI, from JVET-J0014)

Cross-checker (Qualcomm) confirms the results partially since the simulations are still running. The first set of results show the performance with new initialization values, whereas the second set of results show the performance without new initialization values.