| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0026-v2 |
| --- | --- |

| Title: | CE6: Summary Report on Transforms and Transform Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Amir Said Xin Zhao | Email: | asaid@qti.qualcomm.com xinzzhao@tencent.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This contribution summarizes the activities of Core Experiment (CE) on Transforms and Transform Signalling, including list of participants and experiments, summary of test results, brief experiment descriptions, and conclusions. The CE studies were divided into three categories: (1) Primary transforms, with 14 proposals; (2) Secondary transform, with 6 proposals; and (3) and transform combinations and signalling, which has 4 proposals. Unlike other CEs, in this CE all experiments were done using software based on the BMS-1.1 version, and results are reported using the corresponding anchors.

# Introduction

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signaling on top of the Test Model. In this CE, the tradeoff between compression efficiency and complexity of different transformation tools will be investigated.

# Experiment List

Core experiments (CEs) are organized according to the following three categories:

- Primary transforms: CEs 6.1.1-6.1.14 test proposals that introduce changes for primary transforms only.

- Secondary transforms: CEs 6.2.1-6.2.6 test proposals that introduce changes for secondary transforms only.

- Combinations and Signalling: CEs 6.3.1-6.3.4 test proposals that introduce changes for both primary and secondary transforms, and how they can be combined.

The following table lists all the experiments in each category, and the corresponding input document to the Ljubljana meeting.

| CE 6.1 – Primary Transforms |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.1.1 | A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm) | JVET-K0381 | Primary transformation: In addition to transform types of AMT in JEM7, DST-4 and Identity transform are also used for intra prediction residual. | 1. Orange 2. Samsung 3. Brightcove 4. Wilus |
| 6.1.2 | A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm) | JVET-K0272 | AMT complexity reduction: The AMT transforms are implemented using the DCT-2 family of transforms and low-complexity adjustment stages. | 1. Orange 2. Technicolor 3. ETRI 4. Samsung 5. Sony (withdrawn) 6. Tencent 7. Brightcove |
| 6.1.3 | K. Naser, F. Le Léannec, E. François (Technicolor) | JVET-K0264 | Transform Reduction: It is proposed to reduce transform types in AMT to DCT-8, DST-4 and DST-7. DST-7 is implemented based on the DCT-8 with flipping and sign changes. | 1. Brightcove 2. FastVDO (withdrawn) |
| 6.1.4 | S.-C. Lim, J. Kang, H. Lee, J. Lee, S. Cho, H. Y. Kim (ETRI), N.-U. Kim, Y.-L. Lee (Sejong Univ.), D.-Y. Kim, W. J. Jeong (Chips&Media) | JVET-K0167 | Residual flipping: It is proposed to replace the AMT with DCT-II and DST-VII using four types of residual flipping. | 1. Qualcomm 2. 3. |
| 6.1.5 | K. Choi, M. Park, C. Kim (Samsung) | JVET-K0171 | Reduced AMT transform types: The AMT transforms in JEM7 are replaced with a reduced set of transforms using DCT-8 and DST-7. | 1. Qualcomm 2. Brightcove 3. |
| 6.1.6 | K. Choi, M. Park, C. Kim (Samsung) | JVET-K0173 | Primary transformation restriction: When block width or height is over 32 pixels, AMT is not used or signaled. | 1. Qualcomm 2. Tencent 3. |
| 6.1.7 | T. Tsukuba, M. Ikeda, T. Suzuki (Sony) | JVET-K0121 | Adaptive Multiple core Transforms for Luma and Chroma: It is proposed to apply AMT, which includes transform matrix replacement, for both luma and chroma. | 1.Tencent |
| 6.1.8 | T. Tsukuba, M.I keda, T. Suzuki (Sony) | JVET-K0123 | Transform Matrix Replacement: It is proposed to replace transform matrix of DCT-8, DST-1 and DCT-5 used in JEM with that of alternate transforms as below: - Flipped DST-7, which is derived by flipping DST-7 without sign changes, is used instead of DCT8. - DST-6, which is derived by transposing DST-7, is used instead of DST-1. DCT-2 is used instead of DCT-5. | 1.Brightcove 2. Sejong Univ. |
| 6.1.9 | K. Kawamura, Y. Kidani, S. Naito (KDDI) | JVET-K0397 | Shrink Transform: When the transform block size is equal or larger than 64, inverse transform is realized by two steps: inverse transform with half size and up-sampling. | 1. MediaTek 2. Panasonic 3. |
| 6.1.10 | M.-S. Chiang, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | JVET-K0250 | Adaptive multi-transform (AMT) with constraints: AMT transform type is reduced to DCT-II, DST-VII and flipped DST-VII. AMT is only allowed when both CU width and height are smaller than or equal to 64. For some block sizes, the number of AMT candidates is reduced. | 1.Brightcove 2. 3. |
| 6.1.11 | M. Koo, M. Salehifar, J. Lim, S. Kim (LGE) | JVET-K0096 | Primary transform: Only DST7 and DCT8 are used for AMT. AMT is applied up to 32. DST7 and DCT8 are implemented using FFT | - Brightcove - Samsung |
| 6.1.12 | Y. Zhao, H. Yang, J. Chen (Huawei) | JVET-K0139 | Spatial varying transform: For an inter-predicted CU, a TU of half size of the CU may be used, and in this case the TU shape and position information are signaled. | 1. MediaTek 2. bcom |
| 6.1.13 | Y. Lin, M. Mao, S. Song, J. Zheng, J. An, C. Zhu, (HiSilicon) | JVET-K0125 | Prediction dependent transform - Intra prediction mode dependent transform for luma and chroma - Position dependent transform for residual of HEVC merge mode | 1. Brightcove |
| 6.1.14 | X. Zhao, X. Li, S. Liu (Tencent) | JVET-K0083 | Block size dependent zero-out transform: When 64-point, 128-point and 256-point inverse primary transform is applied, only coefficients of the first 32, 16 and 4 are used and the remaining coefficients are not used and considered as 0. | 1. Sony |
| CE 6.2 – Secondary Transforms |  |  |  |  |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.2.1 | Fabrice Urban, Fabrice Le Léannec, Edouard Francois (Technicolor) | JVET-K0271 | Secondary transformation: NSST can be applied only when AMT transform flag or index is equal to 0. NSST transform choice is separately signaled for Cb and Cr chroma components. | 1. Samsung 2. HHI 3. |
| 6.2.2 | A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm) | JVET-K0374 | Hierarchically Structured Matrix-based Transforms: HyGT based NSST is replaced by hierarchically structured matrix-based transforms (HSMTs). | 1. Technicolor 2. Samsung 3. HHI 4. Tencent 5. Brightcove 6. bcom |
| 6.2.3 | K. Choi, M. Park, C. Kim (Samsung) | JVET-K0174 | Secondary transformation: NSST is modified with reduced sets based on grouping intra prediction modes and signaling with on/off flag with 4 modes. | 1. Qualcomm 2. HHI 3. Tencent 4. |
| 6.2.4 | X. Zhao, X. Li, S. Liu (Tencent) | JVET-K0084 | Matrix multiplication based NSST: It is proposed to use matrix multiplication based NSST. | 1. LG Electronics |
| 6.2.5 | M. Koo, M. Salehifar, J. Lim, S. Kim (LGE) | JVET-K0098 | 4x4 Secondary transform: 4x4 Layered Givens Transform (LGT) is tested. | 1. Brightcove |
| 6.2.6 | M. Salehifar, M. Koo, J. Lim, S. Kim (LGE) | JVET-K0099 | 8x8 Secondary transform: 8x8 Reduced Secondary Transform (RST) keeping 16 bases and 32 of the 64-dimensional space (16x64 and 32x64 matrix) with NSST index modification based on RST modification. | 1. Huawei 2. bcom |
| CE 6.3 – Combinations and Signalling |  |  |  |  |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.3.1 | X. Zhao, X. Li, S. Liu (Tencent) | JVET-K0085 | Coupled primary and secondary transform: AMT and NSST is coupled and signaled by only one transform index and applied for both luma and chroma components. | 1. Technicolor 2. Samsung 3. Sony 4. Brightcove |
| 6.3.2 | M. Siekmann, C. Bartnik, H. Schwarz, D. Marpe, T. Wiegand (HHI) | JVET-K0305 | Set of transforms: A set of 5 predefined transform candidates, and each candidate specifies a primary horizontal transform, a primary vertical transform, and a non-separable secondary transform. The transform candidates depend on the block size and the prediction mode. | 1. Qualcomm 2.Brightcove |
| 6.3.3 | K. Abe, T. Toma (Panasonic) | JVET-K0127 | AMT and NSST complexity reduction: (1) NSST is used only when DCT2 is used as primary transform. (2) DST7 is used as the horizontal and vertical transform without signaling when width or height is less than or equal to 4. | 1. KDDI |
| 6.3.4 | M.-S. Chiang, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | JVET-K0250 | AMT and NSST syntax signaling: The NSST index signaling depends on luma/chroma, number of nonzero DC/AC coefficient of upper-left 8x8 or 4x4. | 1. Bcom |

# CE6-1: Primary Transforms

## Summary of results

The following table summarizes the results for CE6-1.

|  | All Intra - Over VTM1.1 |  |  |  |  | Random Access - Over VTM1.1 |  |  |  |  | Low Delay B - Over VTM1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AMT1 | -3.46% | -1.55% | -1.53% | 213% | 121% | -1.53% | -0.50% | -0.63% | 130% | 107% | -0.60% | -0.28% | -0.46% | 117% | 105% |
| AMT3 | -3.46% | -1.55% | -1.53% | 210% | 118% | -2.17% | -0.04% | -0.12% | 169% | 109% | -1.57% | 1.18% | 0.94% | 168% | 109% |
| CE6-1.1.1 | -3.60% | -1.59% | -1.62% | 210% | 127% | -2.20% | 0.03% | -0.02% | 166% | 112% | -1.50% | 0.93% | 0.91% | 159% | 111% |
| CE6-1.2.1 | -3.41% | -1.56% | -1.57% | 208% | 110% | -2.16% | -0.01% | -0.11% | 155% | 104% | -1.53% | 1.02% | 0.81% | 150% | 102% |
| CE6-1.3.1 | -3.60% | -1.63% | -1.69% | 206% | 1253 | -2.21% | -0.02% | -0.07% | 163% | 118% | -1.56% | 0.77% | 1.04% | 163% | 118% |
| CE6-1.4.1 | -3.24% | -1.71% | -1.68% | 198% | 120% | -2.06% | -0.09% | -0.13% | 163% | 109% | -1.49% | 0.95% | 1.01% | 163% | 111% |
| CE6-1.5.1 | -3.30% | -1.60% | -1.60% | 207% | 122% | -2.10% | 0.00% | -0.10% | 167% | 112% | -1.50% | 1.00% | 1.20% | 166% | 115% |
| CE6-1.6.1 | -3.40% | -1.50% | -1.50% | 206% | 121% | -2.00% | 0.00% | -0.10% | 149% | 108% | -1.40% | 1.20% | 1.00% | 142% | 104% |
| CE6-1.7.1 | -3.42% | -2.34% | -2.34% | 214% | 132% | -2.13% | -1.19% | -1.33% | 170% | 116% | -1.54% | -0.16% | 0.01% | 167% | 115% |
| CE6-1.8.1 | -3.24% | -1.59% | -1.58% | 211% | 124% | -2.09% | -0.09% | -0.06% | 167% | 112% | -1.53% | 0.89% | 1.05% | 165% | 114% |
| CE6-1.9.1 | -0.05% | -0.19% | -0.15% | 100% | 101% | -0.09% | -0.18% | -0.14% | 99% | 100% | -0.08% | -0.69% | -0.58% | 100% | 98% |
| CE6-1.10.1 | -3.15% | -1.56% | -1.56% | 191% | 117% | -2.08% | -0.02% | -0.16% | 159% | 108% | -1.51% | 1.03% | 1.15% | 159% | 110% |
| CE6-1.10.2 | -2.67% | -1.87% | -1.85% | 169% | 121% | -1.80% | -0.12% | -0.20% | 152% | 109% | -1.32% | 0.85% | 0.78% | 155% | 109% |
| CE6-1.11.1 | -3.31% | -1.59% | -1.59% | 201% | 104% | -1.98% | 0.01% | -0.06% | 142% | 101% | -1.37% | 0.90% | 1.10% | 135% | 101% |
| CE6-1.12.1 |  |  |  |  |  | -0.62% | 0.19% | 0.16% | 107% | 102% | -1.09% | 0.45% | 0.54% | 115% | 104% |
| CE6-1.13.1a | -1.73% | -1.51% | -1.47% | 108% | 115% |  |  |  |  |  |  |  |  |  |  |
| CE6-1.13.1b | -0.08% | -0.01% | -0.05% | 102% | 100% |  |  |  |  |  |  |  |  |  |  |
| CE6-1.13.1c | -1.88% | -1.71% | -1.77% | 107% | 115% |  |  |  |  |  |  |  |  |  |  |
| CE6-1.13.2 |  |  |  |  |  | -0.05% | 0.10% | 0.16% | 100% | 97% | -0.28% | 0.45% | 0.45% | 100% | 99% |
| CE6-1.13.3 | -1.88% | -1.71% | -1.77% | 107% | 115% | -0.79% | -0.69% | -0.70% | 102% | 102% | -0.57% | 0.15% | 0.04% | 100% | 100% |
| CE6-1.14.1 | 0.00% | 0.00% | 0.00% | 102% | 102% | 0.00% | 0.03% | 0.01% | 101% | 102% | 0.04% | -0.03% | 0.01% | 102% | 103% |
|  | All Intra - Over BMS-1.1 |  |  |  |  | Random Access - Over BMS-1.1 |  |  |  |  | Low Delay B - Over BMS-1.1 |  |  |  |  |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1.1 | -0.31% | -0.17% | -0.26% | 100% | 100% | -0.09% | 0.05% | -0.05% | 99% | 100% | -0.09% | -0.03% | 0.13% | 99% | 99% |
| CE6-1.2.1 | 0.00% | 0.09% | 0.01% | 98% | 91% | 0.02% | 0.12% | -0.26% | 95% | 98% | -0.09% | -0.10% | 0.04% | 94% | 96% |
| CE6-1.3.1 | -0.20% | -0.08% | -0.17% | 101% | 100% | -0.05% | -0.02% | -0.06% | 101% | 100% | -0.08% | -0.06% | 0.18% | 103% | 103% |
| CE6-1.4.1 | 0.25% | 0.13% | 0.06% | 94% | 99% | 0.13% | 0.29% | -0.02% | 96% | 100% | 0.01% | 0.05% | 0.20% | 98% | 100% |
| CE6-1.5.1 | 0.20% | 0.10% | 0.10% | 100% | 99% | 0.10% | 0.30% | 0.10% | 100% | 100% | 0.00% | 0.20% | 0.20% | 100% | 101% |
| CE6-1.6.1 | 0.00% | 0.20% | 0.10% | 98% | 99% | 0.10% | 0.30% | 0.20% | 90% | 99% | 0.10% | -0.10% | 0.10% | 90% | 97% |
| CE6-1.7.1 | 0.05% | -0.43% | -0.38% | 101% | 101% | 0.08% | -0.77% | -0.90% | 101% | 100% | 0.05% | -1.31% | -1.10% | 101% | 101% |
| CE6-1.8.1 | 0.25% | 0.04% | 0.04% | 99% | 96% | 0.11% | 0.11% | 0.03% | 99% | 99% | 0.04% | 0.12% | 0.02% | 100% | 99% |
| CE6-1.9.1 | -0.05% | -0.18% | -0.14% | 101% | 100% | -0.07% | -0.11% | -0.17% | 100% | 100% | -0.16% | -0.48% | -0.70% | 101% | 100% |
| CE6-1.10.1 | 0.21% | 0.07% | 0.05% | 91% | 97% | 0.08% | 0.14% | -0.04% | 93% | 99% | -0.05% | 0.18% | 0.10% | 97% | 99% |
| CE6-1.10.2 | 1.03% | 0.43% | 0.46% | 80% | 98% | 0.56% | 0.48% | 0.32% | 84% | 100% | 0.23% | 0.30% | 0.09% | 91% | 99% |
| CE6-1.11.1 | 0.17% | 0.19% | 0.17% | 97% | 93% | 0.21% | 0.41% | 0.09% | 86% | 98% | 0.14% | 0.03% | -0.61% | 87% | 96% |
| CE6-1.12.1 |  |  |  |  |  | -0.27% | 0.07% | 0.05% | 103% | 101% | -0.56% | -0.22% | -0.08% | 105% | 102% |
| CE6-1.13.1a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.13.1b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.13.1c |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.13.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.13.3 | 1.21% | 2.00% | 2.09% | 47% | 100% | 1.22% | 1.16% | 1.00% | 61% | 99% | 0.73% | -0.21% | 0.21% | 70% | 98% |
| CE6-1.14.1 | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.03% | 0.01% | 101% | 102% | -0.03% | 0.02% | -0.04% | 101% | 103% |

## Crosscheck reports

| Test 6- | Doc. # | Crosscheck | Does crosschecking confirm whether the SW implementation is fully aligned with the algorithm description? | Does crosschecking confirm whether ALL data points coincide with the results provided by proponent? | Does crosschecking inspect other things that are not listed above? If yes, please describe. | Commentary on the potential benefits and disadvantages of the proposed technology (highly encouraged) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.1.1 | JVET-K0381 (Qualcomm) | Orange | Y |  | The software is easily readable It is compliant with the description. Most of the changes are related to the incorporation of the adjustment method of 6.1.2 which are also easily identified. . | The proposal has a minor impact on the storage requirements when operated using the adjustment method of CE6.1.2. Otherwise it introduces additional memory to include the DST4 transform. We cannot comment on the performance currently. |
|  |  | Samsung | Y | Y | N | Additional transform type is needed. Combination between additional kernel type and all possible partitions will be a burden for hardware implementation. |
|  |  | Brightcove |  |  |  |  |
|  |  | Wilus | Y | Y | N | No comments |
| 1.2.1 | JVET-K0272 (Qualcomm) | Orange | Yes the software is aligned with the description. 6taps adjustments are proposed for this version. Some tricks were identified to perform the transforms using only the DCT2 and DCT3 as core transforms. * the forward DST7 computation based on a DCT3 fast algorithm (instead of DST3) after a reversal of the adjusted vector. *the forward DST1 computation is based on a DCT2 fast algorithm (instead of DST2), after a sign change for the odd adjusted coefficients, then reversal. Consequently, only fast DCT2 andDCT3 are required. The DCT8 computation reuses the adjustment coefficients from the DST7. The DCT2 computation has been affected in order to use less rom. | All data points are confirmed in AI/RA/LDB The runtimes are consistent with those announced by the proponent. (The decoder runtimes were not checked for LDB). ) | Does crosschecking inspect other things that are not listed above? If yes, please describe. The software is clear and has been written in order to reveal the complexity reduction offered by the approach w.r.t. the initial matrix multiplications. Commentary on the potential benefits and disadvantages of the proposed technology (highly encouraged) The method is beneficial for reducing encoding/decoding complexity, as a complete set of fast algorithms is provided for the EMT. Consistent speed-up improvements are observed for all conditions and sequences. The ROM storage is significantly reduced as the DCT8, DST1 and DCT5 do not require to be stored as full matrices. There is no impact on the coding performance with the proposed method. | The method is beneficial for reducing encoding/decoding complexity, as a complete set of fast algorithms is provided for the EMT. Consistent speed-up improvements are observed for all conditions and sequences. The ROM storage is significantly reduced as the DCT8, DST1 and DCT5 do not require to be stored as full matrices. There is no impact on the coding performance with the proposed method. |
|  |  | Technicolor |  |  |  |  |
|  |  | ETRI | Y | Only partial results of Class C, D, and E are confirmed | N | No comments |
|  |  | Samsung | Y | Y | N | Additional stage for adjustment is needed. It might cause one cycle delay in hardware implementation. |
|  |  | Tencent | Y | Y | DCT-2 function is also modified which seems to be irrelavant to the propsoal | There are shifts between the sparse matrix multiplication and partial butterfly, which means a two stage operation is needed, and this may cause one cycle delay in hardware implementation |
|  |  | Brightcove |  |  |  |  |
| 1.3.1 | JVET-K0264 (Technicolor) | Brightcove |  |  |  |  |
| 1.4.1 | JVET-K0167 (ETRI, Sejong Univ., Chips& Media) | Qualcomm |  |  |  |  |
| 1.5.1 | JVET-K0171 (Samsung) | Qualcomm |  |  |  |  |
|  |  | Brightcove |  |  |  |  |
| 1.6.1 | JVET-K0173 (Samsung) | Qualcomm |  |  |  |  |
|  |  | Tencent | Y | Y | N | The proposed method provides reasonable compelxity reduction for AMT |
| 1.7.1 | JVET-K0121 (Sony) | Tencent | Y | Y | N | The proposed method may help to unfiy the transform scheme between luma and chroma |
| 1.8.1 | JVET-K0123 (Sony) | Brightcove |  |  |  |  |
|  |  | Sejong Univ. | Y | Y | N | The method is beneficial for reducing the memory sizes of core transforms. |
| 1.9.1 | JVET-K0397 (KDDI) | MediaTek | Y | Y | The lamda value for RDO mode decision is modified |  |
|  |  | Panasonic | Y | Y | N | No comments |
| 1.10.1 | JVET-K0250 (MediaTek) | Brightcove |  |  |  |  |
| 1.10.2 |  |  |  |  |  |  |
| 1.11.1 | JVET-K0096 (LGE) |  |  |  |  |  |
| 12.1 | JVET-K0139 (Huawei) | MediaTek | Y | Y | N |  |
|  |  | b<>com | The software implementation differs from what was originally proposed in JVET-J0024 but is aligned with the description in JVET-K0139. | Since SVT is an inter tool, only RA and LDB configurations were run by the proponents and the cross-checkers. All the required rates and PSNR figures provided by the proponents are confirmed, including the additional tests required for sub-CE 6.1. However, the runtimes could not be confirmed due to a heterogeneous cluster. | Yes, analyzing the software gives information not detailed in JVET-K0139. • The following binarizations are used for SVT syntax elements: o 1 bin for svt_flag, o 1 bins for svt_dir (SVT-V or SVT-H), o 2 bins for svt_pos (position 0, 1 or 2). • Each syntax element uses CABAC contexts: o 1 context for svt_flag, o 3 contexts for svt_dir depending on the relationship between the CU width and height (=,< or >), o 2 contexts for svt_idx, one for each bin. | SVT achieves on average -0.6% and -1.1% Y BD-rate with +7% and +15% encoding runtimes for RA and LDB configurations over VTM 1.1. This is a reasonable trade-off. In the BMS, there is some interaction with InterAMT lowering significantly the gains of SVT. |
| 1.13.1a | JVET-K0125 (HiSilicon) | Brightcove |  |  |  |  |
| 1.13.1b |  | Brightcove |  |  |  |  |
| 1.13.1c |  | Brightcove |  |  |  |  |
| 1.13.2 |  | Brightcove |  |  |  |  |
| 1.13.3 |  | Brightcove |  |  |  |  |
| 1.14.1 | JVET-K0083 (Tencent) | Sony | Y | Y | Yes. Since the maximum transform size is set to 64 in the current VTM/BMS, it is encouraged to check the coding performance with the maximum transform size set to 128 if possible. | It seems beneficial to reduce operation counts of large transform blocks in large coding block, especially for 128x128/128xN/Nx128 case. |

## Complexity metrics

| Test CE6- |  | 1.1.1 | 1.2.1 | 1.3.1 | 1.4.1 |
| --- | --- | --- | --- | --- | --- |
| Operation counts |  |  |  | Same as BMS-1.1 | Same as BMS 1.1 |
| Memory requirements for storing transform cores |  |  |  | Half of the memory used by non-DCT-2 transforms in BMS AMT | Test 1.4.1 saves about 60% of memory for transform matrices compared to AMT and 75% of memory for additional transform matrices in addition to DCT-II compared to AMT. |
| Specify the minimum bit-precision required by the transforms | To represent transform coefficients |  |  | Same as BMS 1.1 | Same as BMS 1.1 |
|  | Precision of arithmetic operations needed during transform computation |  |  | Same as BMS 1.1 | Same as BMS 1.1 |
|  | For storing intermediate data representation |  |  | Same as BMS 1.1 | Same as BMS 1.1 |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. |  |  |  | N/A | Same as BMS 1.1 |
| Other operations and memory requirements |  |  |  | N/A | Residual flipping and its inverse only require reordering the input and output of DST-VII, respectively. |
| Encoder times | AI-VTM | 210% | 208% | 206% | 198% |
|  | AI-BMS | 100% | 98% | 101% | 94% |
|  | RA-VTM | 166% | 155% | 163% | 163% |
|  | RA-BMS | 99% | 95% | 101% | 96% |
|  | LB-VTM | 159% | 150% | 163% | 163% |
|  | LB-BMS | 99% | 94% | 103% | 98% |
| Decoder times | AI-VTM | 127% | 110% | 125% | 120% |
|  | AI-BMS | 100% | 91% | 100% | 99% |
|  | RA-VTM | 112% | 104% | 118% | 109% |
|  | RA-BMS | 100% | 98% | 100% | 100% |
|  | LB-VTM | 111% | 102% | 120% | 111% |
|  | LB-BMS | 99% | 96% | 103% | 100% |
| List of all combinations of transform types |  |  |  | DCT-2, DST-4, DST-7, DCT-8 | DCT-II and DST-VII |
| List of all combinations of transforms block size and transform type used for secondary transformation |  |  |  | N/A | Same as BMS 1.1 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). |  |  |  | Same as BMS-1.1 | Same as BMS 1.1 |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. |  |  |  | N/A | Same as BMS 1.1 |

| Test CE6- |  | 1.5.1 | 1.6.1 | 1.7.1 | 1.8.1 |
| --- | --- | --- | --- | --- | --- |
| Operation counts |  |  |  | Same as BMS-1.1 | DCT-2 & DST-7: Same as BMS-1.1 Flipped DST-7&DST-6: # of Mul: N^2 # of Add: (N-1)*N + 1 # of Shift: N N=4,8,16,32,64 |
| Memory requirements for storing transform cores |  |  |  | Same as BMS-1.1 | At least two core transforms (DCT-2&DST-7) |
| Specify the minimum bit-precision required by the transforms | To represent transform coefficients |  |  | Unchanged |  |
|  | Precision of arithmetic operations needed during transform computation |  |  | Unchanged |  |
|  | For storing intermediate data representation |  |  | Unchanged |  |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. |  |  |  | N/A | N/A |
| Other operations and memory requirements |  |  |  | N/A | N/A |
| Encoder times | AI-VTM | 207% | 206% | 214% | 211% |
|  | AI-BMS | 100% | 98% | 101% | 99% |
|  | RA-VTM | 167% | 149% | 170% | 167% |
|  | RA-BMS | 100% | 99% | 100% | 99% |
|  | LB-VTM | 166% | 142% | 167% | 165% |
|  | LB-BMS | 100% | 90% | 101% | 100% |
| Decoder times | AI-VTM | 122% | 121% | 132% | 124% |
|  | AI-BMS | 99% | 99% | 101% | 96% |
|  | RA-VTM | 112% | 108% | 116% | 112% |
|  | RA-BMS | 100% | 99% | 100% | 99% |
|  | LB-VTM | 115% | 104% | 115% | 114% |
|  | LB-BMS | 101% | 97% | 101% | 99% |
| List of all combinations of transform types |  |  |  | DCT-2 or combination of DCT-5/DCT-8/DST-1/DST-7 | DCT-2 or combination of DCT-2/DST-7/Flipped DST-7/DS-6 |
| List of all combinations of transforms block size and transform type used for secondary transformation |  |  |  | Same as BMS-1.1 Luma: W={4,8,16,32,64} H={4,8,16,32,64} Chroma: W={2,4,8,16,32} H={2,4,8,16,32} | Same as BMS-1.1 Luma: W={4,8,16,32,64} H={4,8,16,32,64} Chroma: W={2,4,8,16,32} H={2,4,8,16,32} |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). |  |  |  | Unchanged | DCT-2 & DST-7: same as BMS-1.1 Flipped DST-7&DST-6: Implemented using matrix multiplication |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. |  |  |  | N/A | N/A |

| Test CE6- |  | 1.9.1 | 1.10.1 | 1.10.2 | 1.11.1 |
| --- | --- | --- | --- | --- | --- |
| Operation counts |  |  | 3 Intra AMT candidates and 4 Inter AMT candidates for those blocks with CU width and height smaller than or equal to 64 | For intra AMT, roughly half of that on top of 1.10.1 and varying with CU size; for inter AMT, varying with CU size | For processing one row or column, 8x8: 64 (Mult.), 64 (Add./Sub.), 8 (Shift) 16x16: 83 (Mult.), 137 (Add./Sub.), 47 (Shift) 32x32: 236 (Mult.), 378 (Add./Sub.), 88 (Shift) |
| Memory requirements for storing transform cores |  |  | Quater of the memory used by non-DCT-2 transforms in BMS AMT | Same as 1.10.1 | Block8x8: 128 bits Block16x16: 440 bits Block32x32: 938 bits |
| Specify the minimum bit-precision required by the transforms | To represent transform coefficients | Unchanged | Unchanged |  | Unchanged |
|  | Precision of arithmetic operations needed during transform computation | Unchanged | Unchanged |  | In forward horizontal transform, 1st DFT: (M) 11bits x 12 bits (A,S) 21 bits 2nd DFT: (M) 12bits (13bits) x 10bits (A,S) 22 bits (23 bits) |
|  | For storing intermediate data representation | Unchanged | Unchanged |  | In forward horizontal transform, 1st DFT: (8x8, 16x16) 22bits (32x32) 23bits 2nd DFT (8x8) N/A, (16x16) 22bits, (32x32) 23bits. |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. |  |  | N/A | N/A | 2 loops (1st DFT and 2nd DFT) |
| Other operations and memory requirements |  |  | N/A | N/A | N/A |
| Encoder times | AI-VTM | 100% | 191% | 169% | 201% |
|  | AI-BMS | 100% | 91% | 80% | 97% |
|  | RA-VTM | 99% | 159% | 152% | 142% |
|  | RA-BMS | 100% | 93% | 84% | 86% |
|  | LB-VTM | 100% | 159% | 155% | 135% |
|  | LB-BMS | 100% | 97% | 91% | 87% |
| Decoder times | AI-VTM | 100% | 117% | 121% | 104% |
|  | AI-BMS | 101% | 97% | 98% | 93% |
|  | RA-VTM | 100% | 108% | 109% | 101% |
|  | RA-BMS | 100% | 99% | 100% | 98% |
|  | LB-VTM | 100% | 110% | 109% | 101% |
|  | LB-BMS | 100% | 99% | 99% | 96% |
| List of all combinations of transform types |  |  | Intra AMT: (DST7, DST7), (DCT2, DST7), (DST7, DCT2) Inter AMT: (FDST7, FDST7), (DST7, FDST7), (FDST7, DST7), (DST7, DST7) | Intra AMT: (DST7, DST7), (DCT2, DST7), (DST7, DCT2) Inter AMT: (FDST7, FDST7), (DST7, FDST7), (FDST7, DST7), (DST7, DST7) | W <= 32 & h < = 32: DST/DCT8 Otherwise: DCT2 |
| List of all combinations of transforms block size and transform type used for secondary transformation |  |  | Unchanged | Unchanged | Unchanged |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). |  |  | Same as BMS-1.1 | Same as BMS-1.1 | The proposed DST-7 is reused in DCT-8 implementation. Forward DST-7 and inverse DST-7 use the same DFT computation except for reordering and partial sign changes of inputs and outputs. Benefited from all the existent symmetries, redundant calculations could be trimmed down, which enabled more simplified formulae that could lead dramatic complexity reduction. |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. |  |  | N/A | N/A |  |

| Test CE6- |  | 1.12.1 | 1.13.1a | 1.13.1b | 1.13.1c |
| --- | --- | --- | --- | --- | --- |
| Operation counts |  | Same as BMS-1.1 | Same as BMS-1.1 | Same as BMS-1.1 | Same as BMS-1.1 |
| Memory requirements for storing transform cores |  | Same as BMS-1.1 | 1/2 of the memory used by non-DCT-2 transforms in BMS AMT | Same as BMS-1.1 | 1/2 of the memory used by non-DCT-2 transforms in BMS AMT |
| Specify the minimum bit-precision required by the transforms | To represent transform coefficients | Unchanged | Unchanged | Unchanged |  |
|  | Precision of arithmetic operations needed during transform computation | Unchanged | Unchanged | Unchanged |  |
|  | For storing intermediate data representation | Unchanged | Unchanged | Unchanged |  |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. |  | N/A | N/A | N/A | N/A |
| Other operations and memory requirements |  |  | N/A | N/A | N/A |
| Encoder times | AI-VTM | 108% | 102% | 107% | 107% |
|  | AI-BMS |  |  | 47% |  |
|  | RA-VTM |  |  |  |  |
|  | RA-BMS |  |  |  |  |
|  | LB-VTM |  |  |  |  |
|  | LB-BMS |  |  |  |  |
| Decoder times | AI-VTM | 115% | 100% | 115% | 115% |
|  | AI-BMS |  |  | 100% |  |
|  | RA-VTM |  |  |  |  |
|  | RA-BMS |  |  |  |  |
|  | LB-VTM |  |  |  |  |
|  | LB-BMS |  |  |  |  |
| List of all combinations of transform types |  | intra CU: same as BMS1.1; Inter CU: DST7, DCT8, DST1 | Intra AMT: (DST7, DST7), (DCT2, DST7), (DST7, DCT2) Inter AMT: unchanged | Intra luma AMT: unchanged Intra chroma AMT: (DST7, DST7), (DCT2, DST7), (DST7, DCT2) Inter AMT: unchanged | Intra AMT: (DST7, DST7), (DCT2, DST7), (DST7, DCT2) Inter AMT: unchanged |
| List of all combinations of transforms block size and transform type used for secondary transformation |  | N/A | Unchanged | Unchanged | Unchanged |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). |  | Same as BMS-1.1 (use transform of IntraAMT) | Same as BMS-1.1 | Same as BMS-1.1 | Same as BMS-1.1 |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. |  | N/A | N/A | N/A | N/A |

| Test CE6- |  | 1.13.2 | 1.3.3 | 1.14.1 |  |
| --- | --- | --- | --- | --- | --- |
| Operation counts |  | Same as BMS-1.1 | Same as BMS-1.1 | Up to 75% reduction of multiplications in a subset of transforms of dimension 64 |  |
| Memory requirements for storing transform cores |  | Same as BMS-1.1 | 1/4 of the memory used by non-DCT-2 transforms in BMS AMT | Unchanged |  |
| Specify the minimum bit-precision required by the transforms | Unchanged | Unchanged |  | Unchanged |  |
|  | Unchanged | Unchanged |  | Unchanged |  |
|  | Unchanged | Unchanged |  | Unchanged |  |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. |  | N/A | N/A | No |  |
| Other operations and memory requirements |  | N/A | N/A | No |  |
| Encoder times |  | 107% | 107% | 102% |  |
|  |  | 47% |  | 101% |  |
|  | 100% | 102% | 102% | 101% |  |
|  |  | 61% |  | 101% |  |
|  | 100% | 100% | 100% | 102% |  |
|  |  | 70% |  | 101% |  |
| Decoder times |  | 115% | 115% | 102% |  |
|  |  | 100% |  | 101% |  |
|  | 97% | 102% | 102% | 102% |  |
|  |  | 99% |  | 102% |  |
|  | 99% | 100% | 100% | 103% |  |
|  |  | 98% |  | 103% |  |
| List of all combinations of transform types |  | Intra AMT: unchanged Inter AMT: (DCT2, DST7), (DST7, DCT2) | Intra AMT: (DST7, DST7), (DCT2, DST7), (DST7, DCT2) Inter AMT: (DCT2, DST7), (DST7, DCT2) | Unchanged |  |
| List of all combinations of transforms block size and transform type used for secondary transformation |  | Unchanged | Unchanged | Unchanged |  |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). |  | Same as BMS-1.1 | Same as BMS-1.1 | N/A |  |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. |  | N/A | N/A | N/A |  |

## Related contributions

# CE6-2: Secondary Transforms

## Summary of results

The following table summarizes the results for CE6-2.

|  |  | All Intra - Over VTM1.1 |  |  |  |  | Random Access - Over VTM1.1 |  |  |  |  | Low Delay B - Over VTM1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | 8x8 NSST | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| NSST |  | -2.99% | -3.58% | -3.68% | 256% | 107% | -1.31% | -3.09% | -3.06% | 129% | 105% | -0.49% | -0.96% | -0.68% | 116% | 105% |
| CE6-2.1.1 |  | -1.60% | -3.41% | -3.52% | 236% | 99% | -0.74% | -3.02% | -3.04% | 128% | 99% |  |  |  |  |  |
| CE6-2.2.1 | Y | -4.61% | -4.59% | -4.78% | 266% | 107% | -2.21% | -3.56% | -3.68% | 129% | 102% | -0.86% | -1.09% | -1.27% | 114% | 101% |
| CE6-2.3.1 | Y | -5.40% | -5.00% | -5.20% | 331% | 110% | -2.60% | -3.80% | -4.00% | 146% | 104% | -1.00% | -1.40% | -1.50% | 121% | 101% |
| CE6-2.4.1 |  | -3.28% | -4.00% | -4.11% | 244% | 102% | -1.44% | -3.41% | -3.48% | 124% | 99% | -0.54% | -1.14% | -1.09% | 112% | 99% |
| CE6-2.4.2 |  | -3.12% | -3.86% | -3.93% | 244% | 101% | -1.39% | -3.36% | -3.39% | 126% | 100% | -0.48% | -1.18% | -1.21% | 113% | 99% |
| CE6-2.5.1 |  | -3.12% | -3.64% | -3.72% | 240% | 104% | -1.36% | -3.27% | -3.19% | 124% | 101% | -0.47% | -1.17% | -1.16% | 112% | 99% |
| CE6-2.6.1 | Y | -5.44% | -5.13% | -5.40% | 250% | 105% | -2.81% | -3.75% | -4.07% | 127% | 102% | -1.11% | -1.72% | -1.73% | 113% | 99% |
| CE6-2.6.2 | Y | -4.97% | -4.54% | -4.80% | 244% | 103% | -2.53% | -3.13% | -3.40% | 125% | 100% | -0.97% | -1.49% | -1.51% | 113% | 100% |
|  |  | All Intra - Over BMS-1.1 |  |  |  |  | Random Access - Over BMS-1.1 |  |  |  |  | Low Delay B - Over BMS-1.1 |  |  |  |  |
| Test # | 8x8 NSST | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.1.1 |  | 0.25% | 0.02% | 0.04% | 84% | 98% | 0.11% | 0.26% | 0.10% | 90% | 100% |  |  |  |  |  |
| CE6-2.2.1 | Y | -1.33% | -0.84% | -0.70% | 102% | 100% | -0.68% | -0.30% | -0.37% | 102% | 101% | -0.27% | -0.16% | -0.25% | 101% | 101% |
| CE6-2.3.1 | Y | -1.20% | -0.40% | -0.20% | 128% | 101% | -0.60% | -0.30% | -0.30% | 119% | 101% | -0.30% | -0.40% | -0.10% | 109% | 101% |
| CE6-2.4.1 |  | -0.17% | -0.21% | -0.25% | 99% | 98% | -0.06% | -0.16% | -0.27% | 100% | 100% | -0.06% | 0.23% | 0.24% | 100% | 97% |
| CE6-2.4.2 |  | -0.04% | -0.14% | -0.11% | 99% | 98% | -0.01% | -0.18% | -0.30% | 100% | 100% | -0.07% | -0.16% | -0.29% | 99% | 97% |
| CE6-2.5.1 |  | 0.01% | 0.10% | 0.03% | 98% | 99% | 0.01% | -0.10% | -0.20% | 99% | 100% | -0.05% | -0.09% | -0.23% | 99% | 99% |
| CE6-2.6.1 | Y | -1.60% | -0.84% | -0.82% | 97% | 100% | -0.95% | -0.53% | -0.72% | 101% | 101% | -0.45% | -0.87% | -0.42% | 100% | 99% |
| CE6-2.6.2 | Y | -1.24% | -0.45% | -0.46% | 93% | 98% | -0.77% | -0.03% | -0.29% | 99% | 100% | -0.34% | -0.49% | -0.41% | 100% | 100% |

## Crosscheck reports

| Test # | Doc. # | Crosscheck | Does crosschecking confirm whether the SW implementation is fully aligned with the algorithm description? | Does crosschecking confirm whether ALL data points coincide with the results provided by proponent? | Does crosschecking inspect other things that are not listed above? If yes, please describe. | Commentary on the potential benefits and disadvantages of the proposed technology (highly encouraged) |
| --- | --- | --- | --- | --- | --- | --- |
| 2.1.1 | JVET-K0271 (Technicolor) | Samsung | No signaling for chroma component. | Y | N | Encoding time reduction is benefit of this method |
|  |  | HHI | The test description also mentions a separate signaling of NSST transform choice for the chorma components, but wasn’t included in the software. The dependent signaling of the NSST index on either AMT flag or index is implemented as described. | Y | N | No comments |
| 2.2.1 | JVET-K0374 (Qualcomm) | Technicolor |  |  |  |  |
|  |  | Samsung | Y | Y | N | No comments |
|  |  | HHI | Y | Y | N | No comments |
|  |  | Tencent |  |  |  |  |
|  |  | Brightcove |  |  |  |  |
|  |  | bcom | Yes, it is aligned, although the tested version in this cross check uses 35 NSST candidates (13 were proposed in the initial CE description). The NSST-IPM LUT is introduced to point on the proposed HSMT for size 8x8. New tables for the HSMT are introduced: g_nsstHier8x8Matrix[35][3][24][8][8] replaces the former 8x8 NSST table g_nsstHyGTPar8x8[35][3][768]. Most of the processing is performed in TrQuant.cpp. | Y ((The decoder runtimes were not checked for LDB) | The changes are localized (TrQuant) and easily readable. | Yes, it is aligned, although the tested version in this cross check uses 35 NSST candidates (13 were proposed in the initial CE description). The NSST-IPM LUT is introduced to point on the proposed HSMT for size 8x8. New tables for the HSMT are introduced: g_nsstHier8x8Matrix[35][3][24][8][8] replaces the former 8x8 NSST table g_nsstHyGTPar8x8[35][3][768]. Most of the processing is performed in TrQuant.cpp. |
| 2.3.1 | JVET-K0174 (Samsung) | Qualcomm |  |  |  |  |
|  |  | HHI | Y | Y | N | No comments |
|  |  | Tencent | Y | Y | N | No comments |
| 2.4.1 | JVET-K0084 (Tencent) | LG Electronics | Y | Y | N | A direct matrix multiplication is proposed for replacing BMS NSST (HyGT), it reduces multilayer complexity, meanwhile improves coding performance by 0.2 % over BMS NSST. Also a map proposed for reducing the memory complexity. |
| 2.4.2 |  | LG Electronics | Y | Y | N | A direct matrix multiplication is proposed for replacing BMS NSST (HyGT), it reduces multilayer complexity, meanwhile improves coding performance by 0.2 % over BMS NSST. Also a map proposed for reducing the memory complexity. |
| 2.5.1 | JVET-K0098 (LG) | Brightcove |  |  |  |  |
| 2.6.1 | JVET-K0099 (LG) | Huawei | Y | Y | N | For RA gain is about -1% and for AI gain is about -1.6% without additional computational complexity in the case of BMS with RST32 instead NSST over BMS |
|  |  | bcom |  |  |  |  |
| 2.6.2 |  | Huawei | Y | Y | N | For RA gain is about -0.77% and for AI gain is about -1.24% without additional computational complexity in the case of BMS with RST16 instead NSST over BMS |
|  |  | bcom |  |  |  |  |

## Complexity metrics

|  |  | 2.1.1 | 2.2.1 | 2.3.1 | 2.4.1 | 2.4.2 | 2.5.1 | 2.6.1 | 2.6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation counts |  |  | NSST4x4: 256 (M), 157(A), 145(S) NSST8x8: 1664 (M), 1665 (A), 81(S) |  | NSST4x4: 256 (M), 240(A), 16(S) No NSST8x8 |  | 4x4: 160(M), 160(A), 80(S) | NSST4x4: 256 (M), 240(A), 16(S) NSST8x8: 2048 (M), 1984 (A), 64(S): | NSST4x4: 256 (M), 240(A), 16(S) NSST8x8: 1024 (M), 960 (A), 64(S): |
| Memory requirements for storing transform cores |  |  | NSST4x4: 5120 + 53760 bits NSST8x8: 1612800 + 40320 bits |  | VTM: 14.592 KB BMS: 26.880 KB | VTM: 6.144 KB BMS: 9.216 KB | VTM: 4.727 KB BMS: 8.852 KB | VTM: 139.21 KB BMS: 260.7 KB | VTM: 61.87 KB BMS: 115.87 KB |
| Specify the minimum bit-precision required by the transforms | To represent transform coefficients |  | Same as BMS |  | 8-bit |  | 9-bit | 9-bit | 9-bit |
|  | Precision of arithmetic operations needed during transform computation |  | Same as BMS |  | 16-bit |  | 32-bit |  |  |
|  | For storing intermediate data representation |  | Same as BMS |  | Same as BMS |  | Same as BMS |  |  |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. |  |  | 4 iterations, 2 with 512 parallelizable operations, and 2 with 256 parallelizable operations |  | No |  | 5 | No | No |
| Other operations and memory requirements |  |  | None |  | No |  |  |  |  |
| Encoder times | AI | V: B: 84% | V: 266% B: 102% | V: 331% B: 128% | V: 244% B: 99% | V: 244% B: 99% | V: 240% B: 98% | V: 250% B: 97% | V: 244% B: 93% |
|  | RA | V: B: 90% | V: 129% B: 102% | V: 146% B: 119% | V: 124% B: 100% | V: 126% B: 100% | V: 124% B: 99% | V: 127% B: 101% | V: 125% B: 99% |
|  | LDB | V: B: | V: 114% B: 101% | V: 121% B: 109% | V: 112% B: 100% | V: 113% B: 99% | V: 112% B: 99% | V: 113% B: 100% | V: 113% B: 100% |
| Decoder times | AI | V: B: 98% | V: 107% B: 100% | V: 110% B: 101% | V: 102% B: 98% | V: 101% B: 98% | V: 104% B: 99% | V: 105% B: 100% | V: 103% B: 98% |
|  | RA | V: B: 100% | V: 102% B: 101% | V: 104% B: 101% | V: 99% B: 100% | V: 100% B: 100% | V: 101% B: 100% | V: 102% B: 101% | V: 100% B: 100% |
|  | LDB | V: B: | V: 101% B: 101% | V: 101% B: 101% | V: 99% B: 97% | V: 99% B: 97% | V: 99% B: 99% | V: 99% B: 99% | V: 100% B: 100% |
| List of all combinations of transform types |  |  | Same as BMS |  | Same as BMS |  |  | Same as BMS | Same as BMS |
| List of all combinations of transforms block size and transform type used for secondary transformation |  |  | W<8 or H<8: NSST 4x4 W>=8 or H>= 8: NSST 8x8 |  | Same as BMS |  | The proposed LGT is only applied to top-left 4x4 region, which is a non-separable transform accepting 16 inputs and producing 16 outputs. The configuration of transform sets is the same as BMS NSST (i.e. total number of transform sets is 35, and each transform set consists of 2 or 3 transforms excluding the case that the LGT is not applied). | Min(W,H) = 4: NSST4x4 Min(W,H) >= 8: NSST8x8 | Min(W,H) = 4: NSST4x4 Min(W,H) >= 8: NSST8x8 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). |  |  | The 4x4 NSST is implemented with HyGT, same as for BMS, while 8x8 NSST is implemented with hierarchical structure proposed in this CE |  | Pure matrix multiplication |  | Implemented using Givens rotations as BMS NSST | Direct Matrix Multiplication | Direct Matrix Multiplication |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. |  |  |  |  | None |  |  | No | No |

## Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-K0100 (LGE) | 2.4 2.6 | CE6-Related : Matrix multiplication based NSST with reduced memory map (35 6map) -RST4x4 : -3.45% (VTM- AI, Total 4.5 KB memory required) RST4x4+RST16 8x8 : -4.32% (VTM- AI, Total 18 KB memory required) |
| JVET-K0110 (Cannon) |  | CE6-related: NSST restriction |

# CE6-3: Combinations and Signalling

The following table summarizes the results for CE6-3.

|  |  |  | All Intra - Over VTM1.1 |  |  |  |  | Random Access - Over VTM1.1 |  |  |  |  | Low Delay B - Over VTM1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Inter AMT | 8x8 NSST | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AMT1+NSST | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AMT3+NSST | Y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-3.1.1 | N |  | -4.17% | -2.09% | -2.03% | 219% | 123% | -1.90% | -0.69% | -0.73% | 133% | 107% | -0.79% | -0.36% | -0.34% | 117% | 106% |
| CE6-3.1.2 | N |  | -4.37% | -2.40% | -2.40% | 223% | 125% | -1.98% | -1.26% | -1.40% | 133% | 109% | -0.76% | -0.48% | -0.56% | 117% | 105% |
| CE6-3.2.1 | N |  | -4.65% | -3.42% | -3.40% | 239% | 121% | -2.08% | -1.92% | -2.02% | 125% | 105% | -0.82% | -1.01% | -0.78% | 111% | 99% |
| CE6-3.2.2 | N | Y | -5.81% | -4.37% | -4.37% | 255% | 122% | -2.80% | -2.41% | -2.48% | 128% | 105% | -1.16% | -1.24% | -1.00% | 112% | 101% |
| CE6-3.3.1 | Y |  | -4.59% | -4.07% | -4.21% | 344% | 118% | -2.71% | -2.27% | -2.44% | 188% | 108% | -1.73% | 0.28% | 0.01% | 174% | 109% |
| CE6-3.4.1 | Y |  | -3.45% | -1.55% | -1.55% | 213% | 124% | -2.25% | -0.25% | -0.28% | 166% | 108% | -1.80% | 0.60% | 0.58% | 165% | 107% |
|  |  |  | All Intra - Over BMS-1.1 |  |  |  |  | Random Access - Over BMS-1.1 |  |  |  |  | Low Delay B - Over BMS-1.1 |  |  |  |  |
| Test # |  | 8x8 NSST | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-3.1.1 | Y |  | 0.86% | 1.84% | 2.08% | 37% | 102% | 0.36% | 2.08% | 2.02% | 70% | 101% | 0.07% | 0.84% | 0.50% | 85% | 97% |
| CE6-3.1.2 | Y |  | 0.71% | 2.74% | 3.05% | 38% | 104% | 0.32% | 2.38% | 2.28% | 71% | 103% | 0.05% | 1.07% | 1.08% | 87% | 104% |
| CE6-3.2.1 | Y |  | 0.45% | 1.06% | 1.37% | 39% | 101% | 0.18% | 1.46% | 1.41% | 68% | 100% | 0.01% | 0.61% | 0.68% | 84% | 97% |
| CE6-3.2.2 | Y | Y | -0.44% | 0.24% | 0.55% | 41% | 101% | -0.32% | 1.04% | 0.96% | 69% | 100% | -0.20% | 0.28% | 0.35% | 86% | 100% |
| CE6-3.3.1 | Y |  | 0.59% | 0.65% | 0.72% | 60% | 98% | 0.26% | 0.77% | 0.54% | 78% | 101% | 0.06% | 0.19% | 0.39% | 90% | 100% |
| CE6-3.4.1 | Y |  | -0.01% | 0.03% | 0.01% | 100% | 101% | -0.07% | -0.17% | -0.27% | 101% | 99% | -0.33% | -0.53% | -0.14% | 102% | 97% |

## Crosscheck reports

| Test # | Doc. # | Crosscheck | Does crosschecking confirm whether the SW implementation is fully aligned with the algorithm description? | Does crosschecking confirm whether ALL data points coincide with the results provided by proponent? | Does crosschecking inspect other things that are not listed above? If yes, please describe. | Commentary on the potential benefits and disadvantages of the proposed technology (highly encouraged) |
| --- | --- | --- | --- | --- | --- | --- |
| 3.1.1 | JVET-K0085 (Tencent) | Technicolor |  |  |  |  |
|  |  | Samsung | Y | Y | CE6.2.4 is used for combination. | Useful for encoding time reduction. |
|  |  | Sony | Y | Y | Yes. It is noted that matrix multiplication based 4x4 NSST described in CE6.2.4 is used instead of HyGT-based 4x4 NSST in BMS, which is not described in CE6.3.1. For fair comparison, it is encouraged to check the coding performance of only enabling the unified AMT/NSST signaling, but not including matrix multiplication based 4x4 NSST (CE6.2.4). | On CE6.3.1.1&2, it seems beneficial to unify AMT/NSST signaling, and also reduce encoder complexity for RDCheck. On CE6.3.1.2, it seems beneficial to align the transform scheme for luma and chroma. |
| 3.1.2 |  | Technicolor |  |  |  |  |
|  |  | Samsung | Y | Y | CE6.2.4 is used for combination. | Useful for encoding time reduction. |
|  |  | Sony | Y | Y | Yes. It is noted that matrix multiplication based 4x4 NSST described in CE6.2.4 is used instead of HyGT-based 4x4 NSST in BMS, which is not described in CE6.3.1. For fair comparison, it is encouraged to check the coding performance of only enabling the unified AMT/NSST signaling, but not including matrix multiplication based 4x4 NSST (CE6.2.4). | On CE6.3.1.1&2, it seems beneficial to unify AMT/NSST signaling, and also reduce encoder complexity for RDCheck. On CE6.3.1.2, it seems beneficial to align the transform scheme for luma and chroma. |
|  |  | Brightcove |  |  |  |  |
| 3.2.1 | JVET-K0305 (HHI) | Qualcomm |  |  |  |  |
|  |  | Brightcove |  |  |  |  |
| 3.2.2 |  | Qualcomm |  |  |  |  |
|  |  | Brightcove |  |  |  |  |
| 3.3.1 | JVET-K0127 (Panasonic) | KDDI |  |  |  |  |
| 3.4.1 | JVET-K0250 (MediaTek) | Bcom |  |  |  |  |

## Complexity metrics

|  |  | 3.1.1 | 3.1.2 | 3.2.1 | 3.2.2 | 3.3.1 | 3.4.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operation counts |  | Primary transform: Same as BMS Secondary transform: NSST4x4: 256 (M), 240(A), 16(S) |  | NSST4x4: 256 (M), 256(A), 128(S) Breakdown analysis in 5.2.1 | NSST4x4: 256 (M), 157(A), 145(S) NSST 4x8/8x4: 1024 (M), 1056 (A), 32(S) NSST8x8: 4096 (M), 4160 (A), 64(S) Breakdown analysis in 5.2.2 | Same as BMS | Similar to BMS. Only need to count the non-zero coefficients |
| Memory requirements for storing transform cores |  | VTM: 14.592 KB BMS: 26.880 KB |  | 59328 bits | 1,454,080 bits + 7,127,040 bits | Same as BMS | no |
| Specify the minimum bit-precision required by the transforms | To represent transform coefficients | Primary transform: Same with BMS Secondary transform: 8-bit |  | Primary transform: Same with BMS Secondary transform: 64 x Givens-angles (8bit), 1 x permutation (16x4 bit) | Primary transform: Same with BMS Secondary transform: 10-bit | Same as BMS |  |
|  | Precision of arithmetic operations needed during transform computation |  |  | Same as BMS (i.e +15 bits) | Same with BMS | Same as BMS |  |
|  | For storing intermediate data representation | +0 bits |  | Same as BMS (i.e. +5 bits) | Same with BMS | Same as BMS |  |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. |  | No |  | Same as BMS (i.e. 8: 2 rounds, 4 layers each) | Quadratic block sizes: Same as BMS No-Quadratic: No | Same as BMS | Same as BMS |
| Other operations and memory requirements |  | No |  | The table of candidate lists requires 1byte per candidate (i.e. [shape x intra mode x #candidates] 17x35x5 bytes in total). | The table of transform candidate lists for non-quadratic block shapes requires 14bit per candidate in Test 3.2.2 | No | count the number of non-zero coefficients |
| Encoder times | AI | V: 219% B: 37% | V: 223% B: 38% | V: 239% B: 39% | V: 255% B: 41% | V: 344% B: 60% | V: 213% B: 100% |
|  | RA | V: 133% B: 70% | V: 133% B: 71% | V: 125% B: 68% | V: 128% B: 69% | V: 188% B: 78% | V: 166% B: 101% |
|  | LDB | V: 117% B: 85% | V: 117% B: 87% | V: 111% B: 84% | V: 112% B: 86% | V: 174% B: 90% | V: 165% B: 102% |
| Decoder times | AI | V: 123% B: 102% | V: 125% B: 104% | V: 121% B: 101% | V: 122% B: 101% | V: 118% B: 98% | V: 124% B: 101% |
|  | RA | V: 107% B: 101% | V: 109% B: 103% | V: 105% B: 100% | V: 105% B: 100% | V: 108% B: 101% | V: 108% B: 99% |
|  | LDB | V: 106% B: 97% | V: 105% B: 104% | V: 99% B: 97% | V: 101% B: 100% | V: 109% B: 100% | V: 107% B:97% |
| List of all combinations of transform types |  | AMT same as BMS |  | Provided in 5.2.1 | Provided in 5.2.2 | Provided in 5.2.3 | AMT same as BMS |
| List of all combinations of transforms block size and transform type used for secondary transformation |  | NSST1/2/3 coupled with AMT index 1/2/3 |  | Same as BMS | Provided in 5.2.2 | Same as BMS | Same as BMS |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). |  | AMT same as BMS Secondary transform is done by matrix multiplication |  | AMT same as BMS Secondary transform is done by HyGT | AMT same as BMS Secondary transform is done by HyGT | Same as BMS | Same as BMS |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. |  | No |  | No | No |  | No. Same as BMS |

### HyGT-4x4 secondary transform as in BMS-1.1 and CE6-3.2 [JVET-K0305]

| Type | HyGT – 4x4 |
| --- | --- |
| Number of passes: | 8 (2 rounds, 4 layers each) |
| Multiplications per pass: | 32 |
| Additions per pass: | 32 |
| Bit-shifts per pass: | 16 |
| Parameterized permutation layer: | 1 |
| Multiplications: | 256 |
| Additions: | 256 |
| Bit-shifts: | 128 |
| Parameter: | 64 x Givens-angles (8bit), 1 x permutation (16x4 bit) |
| Precision of arithmetic operations | +15 bits (rel. to precision of primary coefficients) |
| Precision intermediate data | +5 bits (rel. to precision of primary coefficients) |
| Storing single transform core: | 576 bits |
| Number of transforms: | 103 |
| Storing transform cores: | 59,328 bits |

| block size | primary transformation |  | secondary transformation |
| --- | --- | --- | --- |
|  | horizontal | vertical |  |
| all | DCT-[2,5,8], DST-[1,7] | DCT-[2,5,8], DST-[1,7] |  |
| all | DCT-[2,5,8], DST-[1,7] | DCT-[2,5,8], DST-[1,7] | 4x4-HyGT |

### Complexity metrics of additional secondary transform for non-quadratic block shapes in CE6-3.2 [JVET-K0305]

| Type | Matrix multiplication (32) | Matrix multiplication (64) |
| --- | --- | --- |
| Multiplications: | 1024 | 4096 |
| Additions: | 1056 | 4160 |
| Bit-shifts: | 32 | 64 |
| Parameter: | 1024 coefficients (10bit) | 4096 coefficients (10bit) |
| Precision of arithmetic operations | +10bits (rel. to precision of primary coefficients) | +10 bits (rel. to precision of primary coefficients) |
| Precision intermediate data | +0 bits (rel. to precision of primary coefficients) | +0 bits (rel. to precision of primary coefficients) |
| Storing single transform core: | 10,240 bits | 40,960 bits |
| Number of transforms: | 142 | 174 |
| Storing transform cores: | 1,454,080 bits | 7,127,040 bits |

| block size | primary transformation |  | secondary transformation |
| --- | --- | --- | --- |
|  | horizontal | vertical |  |
| all | DCT-[2,5,8], DST-[1,7] | DCT-[2,5,8], DST-[1,7] |  |
| width =height | DCT-[2,5,8], DST-[1,7] | DCT-[2,5,8], DST-[1,7] | 4x4-HyGT |
| height = 4 width > 4 | DCT-[2,5,8], DST-[1,7] | DCT-[2,5,8], DST-[1,7] | 4x8-Matmul |
| width = 4 height > 4 | DCT-[2,5,8], DST-[1,7] | DCT-[2,5,8], DST-[1,7] | 8x4-Matmul |
| width >4; height > 4 width != height | DCT-[2,5,8], DST-[1,7] | DCT-[2,5,8], DST-[1,7] | 8x8-Matmul |

### All combinations for primary and secondary transformation of CE6-3.3.1 [JVET-K0127]

| block size |  | primary transformation |  | secondary transformation |
| --- | --- | --- | --- | --- |
|  |  | horizontal | vertical |  |
| width <= 4 | height <= 4 | DCT2 | DCT2 | 4x4NSST |
|  |  | DST7 | DST7 | None |
| width <= 4 | height > 4 | DCT2 | DCT2 | 4x4NSST |
|  |  | DST7 | DCT5/DCT8/DST1/DST7 | None |
| width > 4 | height <= 4 | DCT2 | DCT2 | 4x4NSST |
|  |  | DCT5/DCT8/DST1/DST7 | DST7 | None |
| width > 4 | height > 4 | DCT2 | DCT2 | 4x4NSST |
|  |  | DCT5/DCT8/DST1/DST7 | DCT5/DCT8/DST1/DST7 | None |

## Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-K0306 (HHI) | 2.2 2.3 3.2 | CE6 - related: “Set of Transforms” selection and signaling scheme tested with different types of secondary transforms sets. |
| JVET-K0405 (Qualcom) | 3.1 3.2 | CE6-related: Secondary Transforms Coupled with a Simplified Primary Transformation |

# Experiment Summaries

## CE6.1.1: Extended AMT

This contribution presents results of CE 6.1.1 where an extension to primary transforms in BMS is tested. In addition to the transforms used in BMS, DST-4 and identity transforms are used to generate more combinations of separable transforms. A look-up table is designed for mapping different set of transforms based on intra-mode and block size information. The CE results show that

1)	On top of VTM 1.1, the proposed method provides 3.6%, 2.2% and 1.5% BD-rate gains for luma under AI, RA and LD configurations, respectively.

2)	On top of BMS 1.1, the proposed method provides 0.3%, 0.1% and 0.1% BD-rate gains for luma under AI, RA and LD configurations, respectively.

The proposed method does not introduce any additional encoding/decoding runtime complexity

This contribution also includes low-complexity implementations the proposed scheme using transform adjustment stages (TAS) proposed in JVET-J0066 and CE 6.1.2 (JVET-K0272). The results show that TAS implementation of the proposed scheme (i) results in less than 0.05% BD-rate loss for AI, RA and LD on VTM and BMS configurations and (ii) provides slight reduction in encoding and decoding time.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.60% | -1.59% | -1.62% | 210% | 127% | -0.31% | -0.17% | -0.26% | 100% | 100% |
| RA | -2.20% | 0.03% | -0.02% | 166% | 112% | -0.09% | 0.05% | -0.05% | 99% | 100% |
| LB | -1.50% | 0.93% | 0.91% | 159% | 111% | -0.09% | -0.03% | 0.13% | 99% | 99% |

## CE6.1.2: Efficient Implementations of AMT with Transform Adjustment Stages

This contribution reports simulation results of CE6 test 6.1.2. A unified transform approach is proposed to approximate the four types of DCTs and DSTs: DCT-8, DST-7, DCT-5, and DST-1 in AMT for primary transform, where each transform is approximated with an adjustment stage as pre-processing and a DCT-2 variant (sign changes and reflection of DCT-2). The results show approximately 2% and 5% reduction for encoding and decoding time against BMS1.1 anchor, and less than 0.05% loss in BD rate.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.41% | -1.56% | -1.57% | 208% | 110% | 0.00% | 0.09% | 0.01% | 98% | 91% |
| RA | -2.16% | -0.01% | -0.11% | 155% | 104% | 0.02% | 0.12% | -0.26% | 95% | 98% |
| LB | -1.53% | 1.02% | 0.81% | 150% | 102% | -0.09% | -0.10% | 0.04% | 94% | 96% |

## CE6.1.3: Transform reduction in AMT, with further reduction via DST-IV inheritance

This contribution relates to adaptive multiple transforms (AMT) design implemented in BMS1.1. It first reports results of transform reduction as described in CE 6.1 – Primary Transforms (test 6.1.3 Transform Reduction). It further proposes inheriting DST-IV from the existing implementation of DCT-II. This leads to reduced memory and/or hardware area for specific coding implementations requiring DST-IV computation.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.60% | -1.63% | -1.69% | 206% | 125% | -0.20% | -0.08% | -0.17% | 101% | 100% |
| RA | -2.21% | -0.02% | -0.07% | 163% | 118% | -0.05% | -0.02% | -0.06% | 101% | 100% |
| LB | -1.56% | 0.77% | 1.04% | 163% | 118% | -0.08% | -0.06% | 0.18% | 103% | 103% |

## CE6.1.4: DST-VII with residual flipping

This contribution reports the experimental results of CE6-1.4 test proposed in call for proposal (CfP) response document of JVET-J0013. In CE6-1.4 test, DST-VII with residual flipping for up to 64x64 transform is tested in order to reduce the required memory sizes for storing transform matrices compared to the adaptive multiple transform (AMT) in benchmark set (BMS). And, DST-VII with residual flipping for up to 32x32 transform is also tested. The CE6-1.4 method with up to 64x64 transform reportedly saves about 60% of memory for transform matrices compared to AMT and 75% of memory for additional transform matrices in addition to DCT-II compared to AMT. The CE6-1.4 method with up to 32x32 transform reportedly saves about 75% of memory for transform matrices compared to AMT and about 94% of memory for additional transform matrices in addition to DCT-II compared to AMT. The experimental results with up to 64x64 transform reportedly show average BD-rates of -3.24%, -2.06%, and -1.49% with encoding/decoding times of 198%/120%, 163%/109%, and 163%/111% for AI, RA, and LDB cases, respectively, compared to VTM 1.1 anchor. And, the experimental results with up to 64x64 transform reportedly show average BD-rates of 0.25%, 0.13%, and 0.01% with encoding/decoding times of 94%/99%, 96%/100%, and 98%/100% for AI, RA, and LDB cases, respectively, compared to BMS 1.1 anchor. And, the experimental results with up to 32x32 transform reportedly show average BD-rates of -3.23%, -1.94%, and -1.37% for AI, RA, and LDB cases, respectively, compared to VTM 1.1 anchor. And, the experimental results with up to 32x32 transform reportedly show average BD-rates of 0.27%, 0.26%, and 0.15% for AI, RA, and LDB cases, respectively, compared to BMS 1.1 anchor.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.24% | -1.71% | -1.68% | 198% | 120% | 0.25% | 0.13% | 0.06% | 94% | 99% |
| RA | -2.06% | -0.09% | -0.13% | 163% | 109% | 0.13% | 0.29% | -0.02% | 96% | 100% |
| LB | -1.49% | 0.95% | 1.01% | 163% | 111% | 0.01% | 0.05% | 0.20% | 98% | 100% |

## CE6.1.5: AMT with reduced transform types

This contribution presents a technology of reduced AMT transform types described in CE 6.1.5. The proposed method uses two kernel types defined as DCT-8 and DST-7 without intra mode dependency. Testing results on the proposed method show 3.3%, 2.1%, and 1.5% BD rate savings on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB) configurations, respectively, compared to VTM1.1, and 0.2%, 0.1%, and 0.0% BD rate loss on average for AI, RA, and LDB configurations, respectively, compared to BMS1.1.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.30% | -1.60% | -1.60% | 207% | 122% | 0.20% | 0.10% | 0.10% | 100% | 99% |
| RA | -2.10% | 0.00% | -0.10% | 167% | 112% | 0.10% | 0.30% | 0.10% | 100% | 100% |
| LB | -1.50% | 1.00% | 1.20% | 166% | 115% | 0.00% | 0.20% | 0.20% | 100% | 101% |

## CE6.1.6: AMT with block size restriction

This contribution presents a technology described in CE 6.1.6 that is AMT with size restriction. The proposed method performs AMT only when width or height is not over 32 pixels in length. The other blocks are not considered to be applied for AMT and not signaled to decoder. Testing results on the proposed method show 3.4%, 2.0%, and 1.4% BD rate savings on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB) configurations, respectively, compared to VTM1.1, and . 0.0%, 0.1%, and 0.1% BD rate loss on average for AI, RA, and LDB configurations, respectively, compared to BMS1.1.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.40% | -1.50% | -1.50% | 206% | 121% | 0.00% | 0.20% | 0.10% | 98% | 99% |
| RA | -2.00% | 0.00% | -0.10% | 149% | 108% | 0.10% | 0.30% | 0.20% | 90% | 99% |
| LB | -1.40% | 1.20% | 1.00% | 142% | 104% | 0.10% | -0.10% | 0.10% | 90% | 97% |

## CE6.1.7: Chroma AMT

This contribution reports the results of CE6.1.7.1 Chroma AMT, which extends Adaptive Multiple Core Transforms (AMT) on top of BMS to chroma transform block (TB) with additional flag (chromaEmtFlag) signaling whether to use DCT-2 or that of the collocated luma TB’s as chroma transform.

The experiments show that

- average BD-rate gains of each Y, U and V over VTM are 3.42%, 2.34% and 2.34% for AI, 2.13%,  1.19% and 1.33% for RA, 1.54%, 0.16% and -0.01% for LB with approximately 170~214% ENC runtime and 115~132% DEC runtime, respectively.

- average BD-rate gains of each Y, U and V over VTM with AMT enabled (--EMT=3 --EMTFast=3) are -0.04%, 0.81% and 0.82% for AI, -0.04%, 1.15% and 1.21% for RA, 1.54%, 0.16% and -0.01% for LB, with approximately 1~2% runtime increase for encoder and decoder , respectively.

- average BD-rate gains of each Y, U and V over BMS are -0.05%, 0.43% and 0.38% for AI, -0.08%, 0.77% and -0.90% for RA, -0.05%, 1.31% and 1.10% for LB, with approximately 1% runtime increase for encoder and decoder, respectively.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.42% | -2.34% | -2.34% | 214% | 132% | 0.05% | -0.43% | -0.38% | 101% | 101% |
| RA | -2.13% | -1.19% | -1.33% | 170% | 116% | 0.08% | -0.77% | -0.90% | 101% | 100% |
| LB | -1.54% | -0.16% | 0.01% | 167% | 115% | 0.05% | -1.31% | -1.10% | 101% | 101% |

## CE6.1.8: Transform Matrix Replacement

This contribution reports the results of CE6.1.8.1 Transform Matrix Replacement, which proposes replacing DCT-5, DCT-8 and DST-1 used in AMT with DCT-2, Flipped DST-7 and DST-6 with transform matrix sharing between DST-7, Flipped DST-7 and DST-6.

The experiments show that

- average BD-rate gains of each Y, U and V over VTM are 3.24%, 1.59% and 1.58% for AI, 2.09%, 0.09% and 0.06% for RA, 1.53%, -0.89% and -1.05% for LB with about 165%~211% ENC run-time and about 112~124% DEC run-time, respectively.

- average BD-rate losses of Y over VTM with AMT enabled (--EMT=3 --EMTFast=3) are 0.23% for AI, 0.08% for RA and 0.04% for LB, respectively.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.24% | -1.59% | -1.58% | 211% | 124% | 0.25% | 0.04% | 0.04% | 99% | 96% |
| RA | -2.09% | -0.09% | -0.06% | 167% | 112% | 0.11% | 0.11% | 0.03% | 99% | 99% |
| LB | -1.53% | 0.89% | 1.05% | 165% | 114% | 0.04% | 0.12% | 0.02% | 100% | 99% |

## CE6.1.9: Shrink Transform

This contribution presents results of a core experiment 6.1.9 included CE6 (JVET-J1026). BD-rates for luma are -0.05%, -0.09%, and -0.08% under the CTC AI, RA, and LDB conditions, respectively, in case of comparing with VTM. In a similar way, BD-rates for luma are -0.05%, -0.07%, and -0.16% under the CTC AI, RA, and LDB conditions, respectively, in case of comparing with BMS.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -0.05% | -0.19% | -0.15% | 100% | 100% | -0.05% | -0.18% | -0.14% | 100% | 101% |
| RA | -0.09% | -0.18% | -0.14% | 99% | 100% | -0.07% | -0.11% | -0.17% | 100% | 100% |
| LB | -0.08% | -0.69% | -0.58% | 100% | 100% | -0.16% | -0.48% | -0.70% | 100% | 100% |

## CE6.1.10: AMT simplification and improvement (with CE6.3.4)

Adaptive multi-core transform (AMT) in JEM-7.0 is applied to luma coding blocks (CBs) of nonzero residual when the luma CB width and height are smaller than or equal to 64. Three aspects of AMT modifications are proposed.

Aspect 1: Reducing the number of transform types.

The proposed AMT only utilizes DCT-II, DST-VII, and flipped DST-VII (FDST-VII).

Aspect 2: Reducing encoding time with normative decoder changes.

In intra slices, unlike explicitly signaling AMT flag for all intra modes in JEM-7.0 AMT, the AMT flag is explicitly signaled for DC and planar modes, implicitly set to 1 for even angular modes, and implicitly set to 0 for odd angular modes. Besides, for intra coding units (CUs) and inter CUs, when AMT flag is equal to 1, the number of AMT modes is set to 1, 2, 3, or 4 according to CU type (intra or inter) and CU size, unlike always 4 in JEM-7.0 AMT. AMT index is signaled when there are more than one AMT mode.

Aspect 3: Improving coding efficiency

If the number of nonzero transform coefficients is larger than a threshold, AMT flag and AMT index can be sent; otherwise they are derived at decoder.

In the following, VTM-1.1 anchor excludes AMT, while BMS-1.1 anchor includes JEM-7.0 AMT. Luma BD-rate, encoding time, and decoding time are reported.

Results of applying aspect 1 to JEM-7.0 AMT, as planned in CE6.1.10.1

VTM-1.1-AI: {Y-BD-rate = -3.15%, EncT=191%, DecT=117%}

VTM-1.1-RA: {Y-BD-rate = -2.08%, EncT=159%, DecT=108%}

VTM-1.1-LB: {Y-BD-rate = -1.51%, EncT=159%, DecT=110%}

BMS-1.1-AI: {Y-BD-rate = +0.21%, EncT=91%, DecT=97%}

BMS-1.1-RA: {Y-BD-rate = +0.08%, EncT=93%, DecT=99%}

BMS-1.1-LB: {Y-BD-rate = -0.05%, EncT=97%, DecT=99%}

Results of applying aspect 1 and aspect 2 to JEM-7.0 AMT, as planned in CE6.1.10.2

VTM-1.1-AI: {Y-BD-rate = -2.67%, EncT=169%, DecT=121%}

VTM-1.1-RA: {Y-BD-rate = -1.80%, EncT=152%, DecT=109%}

VTM-1.1-LB: {Y-BD-rate = -1.32%, EncT=155%, DecT=109%}

BMS-1.1-AI: {Y-BD-rate = +1.03%, EncT=80%, DecT=98%}

BMS-1.1-RA: {Y-BD-rate = +0.56%, EncT=84%, DecT=100%}

BMS-1.1-LB: {Y-BD-rate = +0.23%, EncT=91%, DecT=99%}

Results of applying aspect 3 to JEM-7.0 AMT, as planned in CE6.3.4

VTM-1.1-AI: {Y-BD-rate = -3.45%, EncT=213%, DecT=124%}

VTM-1.1-RA: {Y-BD-rate = -2.25%, EncT=166%, DecT=108%}

VTM-1.1-LB: {Y-BD-rate = -1.80%, EncT=165%, DecT=107%}

BMS-1.1-AI: {Y-BD-rate = -0.01%, EncT=100%, DecT=101%}

BMS-1.1-RA: {Y-BD-rate = -0.07%, EncT=101%, DecT=99%}

BMS-1.1-LB: {Y-BD-rate = -0.33%, EncT=102%, DecT=97%}

| 6.1.10.1 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.15% | -1.56% | -1.56% | 191% | 117% | 0.21% | 0.07% | 0.05% | 91% | 97% |
| RA | -2.08% | -0.02% | -0.16% | 159% | 108% | 0.08% | 0.14% | -0.04% | 93% | 99% |
| LB | -1.51% | 1.03% | 1.15% | 159% | 110% | -0.05% | 0.18% | 0.10% | 97% | 99% |

| 6.1.10.2 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -2.67% | -1.87% | -1.85% | 169% | 121% | 1.03% | 0.43% | 0.46% | 80% | 98% |
| RA | -1.80% | -0.12% | -0.20% | 152% | 109% | 0.56% | 0.48% | 0.32% | 84% | 100% |
| LB | -1.32% | 0.85% | 0.78% | 155% | 109% | 0.23% | 0.30% | 0.09% | 91% | 99% |

## CE6-1.11: AMT replacement and restriction

In JVET-J0017 [1] LGE’s response to the call for proposals, a simplified AMT (Adaptive Multiple Transforms) was introduced. In this contribution, we report test results of CE 6-1.11 and explain details of algorithm and architecture applied in the proposed AMT simplification, which is composed of four features: 1) employment of only two transforms, such as DST-7 and DCT-8, 2) AMT size restriction up to 32, 3) DST-7 design with DFT (Discrete Fourier Transform), and 4) AMT index coding. The first feature facilitates removal of DST-1 and DCT-5. The second and third ones contribute complexity reduction in both computation and memory usage. The last one leads noticeable coding gain increase for intra-coded blocks. Particularly, DFT-based DST-7 design enables XX% and XX% reduction of multiplication counts compared to direct multiply in 16-point and 32-point DST-7, respectively. Consequently, the multiplication counts become similar or even less than DCT-2 counterparts. As CE 6-1.11 test results, 3.31% (AI), 1.98% (RA), and 1.37% (LD) bit rate savings compared to VTM anchor are observed. Compared with original AMT (BMS AMT), BD-rate is degraded by 0.15% (AI), 0.19% (RA), and 0.20% (LD), while applying above mentioned four aspects of AMT complexity reduction. Moreover, encoding/decoding times are also reduced by 9%/17% (AI), 26%/8% (RA), and 31%/9% (LD), which were achieved especially due to the above second and third features.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.31% | -1.59% | -1.59% | 201% | 104% | 0.17% | 0.19% | 0.17% | 97% | 93% |
| RA | -1.98% | 0.01% | -0.06% | 142% | 101% | 0.21% | 0.41% | 0.09% | 86% | 98% |
| LB | -1.37% | 0.90% | 1.10% | 135% | 101% | 0.14% | 0.03% | -0.61% | 87% | 96% |

## CE6.1.12: Spatially Varying Transform

This contribution reports the results of integrating Spatially Varying Transform (SVT) proposed in JVET-J0024 to BMS 1.1. SVT is used for primary transform of inter-predicted blocks. Simulation results reportedly show on average –0.6%/–1.1% luma BD-rate in RA/LB configuration over VTM1.1, with 7%/15% EncT increase, and –0.3%/–0.6% luma BD-rate in RA/LB configuration over BMS1.1, with 3%/5% EncT increase.

Compared with InterEMT on top of VTM, SVT achieves around –0.1% luma BD-rate and –0.3% chroma BD rate with 25% EncT reduction in RA, and achieves –0.1% luma BD-rate and –0.9% chroma BD-rate with 31% EncT reduction in LB. Compared with InterEMT on top of BMS, SVT achieves 0.2% luma BD-rate and –0.3% chroma BD rate in RA, and –0.2% luma BD-rate and –0.9% chroma BD-rate in LB, while reduces EncT by 8% and 11% in RA and LB, respectively.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| RA | -0.62% | 0.19% | 0.16% | 107% | 102% | -0.27% | 0.07% | 0.05% | 103% | 101% |
| LB | -1.09% | 0.45% | 0.54% | 115% | 104% | -0.56% | -0.22% | -0.08% | 105% | 102% |

## CE6.1.13: Implicit transform design for intra and inter residual coding

This contribution reports test results of CE6.1.13 on implicit transform solution for intra and inter residual coding. The following two aspects have been evaluated in the CE: 1) Intra prediction mode dependent transform for luma and chroma residuals; 2) Position dependent transform for residual of HEVC merge mode. It is reported the proposed transform solution can keep most of coding performance with 49% encoding time reduction for AI configuration, compared to the existing AMT coding tool in BMS.

| 6.1.13.1 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI (a) | -1.73% | -1.51% | -1.47% | 108% | 115% |  |  |  |  |  |
| AI (b) | -0.08% | -0.01% | -0.05% | 102% | 100% |  |  |  |  |  |
| AI (c) | -1.88% | -1.71% | -1.77% | 107% | 115% |  |  |  |  |  |

| 6.1.13.2 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI |  |  |  |  |  |  |  |  |  |  |
| RA | -0.05% | 0.10% | 0.16% | 100% | 97% |  |  |  |  |  |
| LB | -0.28% | 0.45% | 0.45% | 100% | 99% |  |  |  |  |  |

| 6.1.13.3 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -1.88% | -1.71% | -1.77% | 107% | 115% | 1.21% | 2.00% | 2.09% | 47% | 100% |
| RA | -0.79% | -0.69% | -0.70% | 102% | 102% | 1.22% | 1.16% | 1.00% | 61% | 99% |
| LB | -0.57% | 0.15% | 0.04% | 100% | 100% | 0.73% | -0.21% | 0.21% | 70% | 98% |

## CE6.1.14: Block size dependent zero-out transform

This contribution reports the test results of CE6-1.14 on block size dependent zero-out transform. With the proposed method, it is reported that up to 75% multiplication operations in transform (with matrix multiplication implementation) are saved at the cost of average 0.00%, 0.00% and 0.04% BD-rate increase comparing to VTM-1.1, under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively, and average 0.00%, 0.01% and -0.03% BD-rate change comparing to BMS-1.1, under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 0.00% | 0.00% | 0.00% | 102% | 102% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| RA | 0.00% | 0.03% | 0.01% | 101% | 102% | 0.00% | 0.03% | 0.01% | 101% | 102% |
| LB | 0.04% | -0.03% | 0.01% | 102% | 103% | -0.03% | 0.02% | -0.04% | 101% | 103% |

## CE6.2.1: Secondary Transformation: NSST Signaling

This contribution reports the results of conditional NSST signaling, depending on the AMT transform.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -1.60% | -3.41% | -3.52% | 236% | 99% | 0.25% | 0.02% | 0.04% | 84% | 98% |
| RA | -0.74% | -3.02% | -3.04% | 128% | 99% | 0.11% | 0.26% | 0.10% | 90% | 100% |
| LB |  |  |  |  |  |  |  |  |  |  |

## CE6.2.2: Hierarchically Structured Matrix-based Transforms for NSST

This contribution presents results of CE 6.2.2 where hierarchically structured matrix-based transforms (HSMTs) are proposed as alternatives to 8x8 HyGT-based NSST implementations in JEM7. The CE results show that

1)	On top of VTM 1.1, the proposed method by enabling AMT (with EMT=3 and EMTFast=3 options) provides 6.7%, 3.8% and 2.2% average BD-rate gains for AI, RA and LD coding, respectively.

2)	On top of VTM 1.1, the proposed method with EMT=0 option provides 4.6%, 2.2% and 0.9% average BD-rate gains for AI, RA and LD coding, respectively.

3)	On top of BMS 1.1, the proposed method provides 1.3%, 0.7% and 0.3% average BD-rate gains for AI, RA and LD coding, respectively.

The proposed method does not introduce any additional encoding/decoding runtime complexity

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -4.61% | -4.59% | -4.78% | 266% | 107% | -1.33% | -0.84% | -0.70% | 102% | 100% |
| RA | -2.21% | -3.56% | -3.68% | 129% | 102% | -0.68% | -0.30% | -0.37% | 102% | 101% |
| LB | -0.86% | -1.09% | -1.27% | 114% | 101% | -0.27% | -0.16% | -0.25% | 101% | 101% |

## CE6.2.3: NSST with modified NSST sets and signaling

This contribution presents a technology described in CE 6.2.3 that is NSST with modified sets and signaling. The proposed method uses the following four parts: 1) reduced angle sets in total as half of that in NSST, 2) new angles sets considering four modes, 3) additional on/off flag to indicate NSST usage, and 4) enabling NSST 8x8 in BMS. Testing results on the proposed method show 5.4%, 2.6%, and 1.0% BD rate savings on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB) configurations, respectively, compared to VTM1.1, and 1.2%, 0.6%, and 0.3% BD rate savings on average for AI, RA, and LDB configurations, respectively, compared to BMS1.1.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -5.40% | -5.00% | -5.20% | 331% | 110% | -1.20% | -0.40% | -0.20% | 128% | 101% |
| RA | -2.60% | -3.80% | -4.00% | 146% | 104% | -0.60% | -0.30% | -0.30% | 119% | 101% |
| LB | -1.00% | -1.40% | -1.50% | 121% | 101% | -0.30% | -0.40% | -0.10% | 109% | 101% |

## CE6.2.4: Matrix multiplication based NSST

This contribution reports the test results of CE6-2.4 on matrix multiplication based Non-Separable Secondary Transform (NSST). With the proposed method, it is reportedly shown that average -0.30%, -0.13% and -0.05% coding gain can be achieved comparing to VTM-1.1 (with NSST enabled), under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively, and average -0.17%, -0.06% and -0.06% coding gain can be achieved comparing to BMS-1.1, under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively.

| 6.2.4.1 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.28% | -4.00% | -4.11% | 244% | 102% | -0.17% | -0.21% | -0.25% | 99% | 98% |
| RA | -1.44% | -3.41% | -3.48% | 124% | 99% | -0.06% | -0.16% | -0.27% | 100% | 100% |
| LB | -0.54% | -1.14% | -1.09% | 112% | 99% | -0.06% | 0.23% | 0.24% | 100% | 97% |

| 6.2.4.2 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.12% | -3.86% | -3.93% | 244% | 101% | -0.04% | -0.14% | -0.11% | 99% | 98% |
| RA | -1.39% | -3.36% | -3.39% | 126% | 100% | -0.01% | -0.18% | -0.30% | 100% | 100% |
| LB | -0.48% | -1.18% | -1.21% | 113% | 99% | -0.07% | -0.16% | -0.29% | 99% | 97% |

## CE6.2.5: Layered Givens Transform (LGT)

In JVET-J0017 LGE’s response to the call for proposals (CFP), Layered Givens Transform (LGT) is proposed, which consists of Givens rotations. In this contribution, LGT is applied to top-left 8x8 region that is divided into four 4x4 blocks each of which the LGT (also denoted as 4x4 LGT) is applied to. The 4x4 LGT consists of 5 Givens rotation layers (GRLs) and 6 permutations. Contrary to BMS 4x4 NSST where fixed shuffling patterns are used, the permutations in the 4x4 LGT are performed based on arrays which store shuffling information. With the help of the permutations, the number of GRLs is reduced from 8 (BMS 4x4 NSST) to 5, while increasing memory usage for the permutations. It is reported that the number of multiplication is reduced by 37.5%, and BD-rate savings are observed to be 3.12% (AI), 1.36% (RA), and 0.47% (LD) for VTM, and BD-rate difference of 0.13% (AI), 0.05% (RA), and -0.02% (LD) are observed compared to BMS 4x4 NSST on top of VTM.

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -3.12% | -3.64% | -3.72% | 240% | 104% | 0.01% | 0.10% | 0.03% | 98% | 99% |
| RA | -1.36% | -3.27% | -3.19% | 124% | 101% | 0.01% | -0.10% | -0.20% | 99% | 100% |
| LB | -0.47% | -1.17% | -1.16% | 112% | 99% | -0.05% | -0.09% | -0.23% | 99% | -0.05% |

## CE6.2.6: Reduced Secondary Transform (RST)

In JVET-J0017 LGE’s response to the call for proposals (CFP), a new Non-Separable Secondary Transform (NSST), called Reduced Secondary Transform (RST) is proposed. In this contribution, we report test results of CE 6.2.6 and explain details of algorithm applied in the proposed NSST design, which is composed of three features: 1) Applying 16x16 direct matrix multiplication for 4x4 NSST ; 2) Applying Rx64 direct matrix multiplication for 8x8 NSST, where the results for R= 16 and 32 are reported. 3) Modification in NSST index coding, where NSST index is not coded depending on the residual block. These features lead to a significant coding performance improvement. Also it reduces the multiplication complexity of 8x8 NSST. Finally it removes the multilayer complexity of the current NSST in BMS and JEM.

For VTM test condition, BD-rate savings of 5.44% (AI), 2.81% (RA), and 1.11% (LD) are observed for R = 32, and 4.97% (AI), 2.53% (RA), and 0.97% (LD) are observed for R = 16. For BMS test condition, BD-rate savings of 1.60% (AI), 0.95% (RA), and 0.45% (LD) are observed for R = 32, and 1.24% (AI), 0.77% (RA), and 0.34% (LD) are observed for R = 16.

| 6.2.6.1 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -5.44% | -5.13% | -5.40% | 250% | 105% | -1.60% | -0.84% | -0.82% | 97% | 100% |
| RA | -2.81% | -3.75% | -4.07% | 127% | 102% | -0.95% | -0.53% | -0.72% | 101% | 101% |
| LB | -1.11% | -1.72% | -1.73% | 113% | 99% | -0.45% | -0.87% | -0.42% | 100% | 99% |

| 6.2.6.2 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -4.97% | -4.54% | -4.80% | 244% | 103% | -1.24% | -0.45% | -0.46% | 93% | 98% |
| RA | -2.53% | -3.13% | -3.40% | 125% | 100% | -0.77% | -0.03% | -0.29% | 99% | 100% |
| LB | -0.97% | -1.49% | -1.51% | 113% | 100% | -0.34% | -0.49% | -0.41% | 100% | 100% |

## CE6.3.1: Coupled primary and secondary transform

This contribution reports the test results of CE6-3.1 on Coupled Primary and Secondary Transform (CPST). Instead of signaling primary and secondary transform independently, with the proposed CPST, primary and secondary transform are coupled and signaled jointly by one transform index. With the proposed method, it is reportedly shown that -4.37%, -1.98% and -0.76% coding gain can be achieved comparing to VTM-1.1, with 223%, 133% and 117% encoding time, under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively.

| 6.3.1.1 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -4.17% | -2.09% | -2.03% | 219% | 123% | 0.86% | 1.84% | 2.08% | 37% | 102% |
| RA | -1.90% | -0.69% | -0.73% | 133% | 107% | 0.36% | 2.08% | 2.02% | 70% | 101% |
| LB | -0.79% | -0.36% | -0.34% | 117% | 106% | 0.07% | 0.84% | 0.50% | 85% | 97% |

| 6.3.1.2 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -4.37% | -2.40% | -2.40% | 223% | 125% | 0.71% | 2.74% | 3.05% | 38% | 104% |
| RA | -1.98% | -1.26% | -1.40% | 133% | 109% | 0.32% | 2.38% | 2.28% | 71% | 103% |
| LB | -0.76% | -0.48% | -0.56% | 117% | 105% | 0.05% | 1.07% | 1.08% | 87% | 104% |

## CE6.3.2: Set of Transforms

A unified signaling and selection scheme for primary and secondary transforms [2] is tested together with two different sets of defined secondary transforms. Instead of signaling the secondary transform separately from the primary transform and allowing all combinations, a list of 5 combinations is provided for each TU as transform candidates and an index into this list is signaled. Test 3.2.1 achieves a luma BD bitrate saving of -4.65% at an encoder runtime of 239% over BMS1.1 with VTM-AI configuration and a loss of 0.45% at an encoder runtime of 39% with BMS-AI configuration. Test 3.2.2 achieves a luma BD bitrate saving of -5.81% at an encoder runtime of 255% over BMS1.1 with VTM-AI configuration and a luma BD bitrate saving of -0.44% at an encoder runtime of 55% with BMS-AI configuration. The performance gain of Test 3.2.2 over Test 3.2.1 comes with the cost of a significantly increased memory demand for storing the additional transform cores.

| 6.3.2.1 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -4.65% | -3.42% | -3.40% | 239% | 121% | 0.45% | 1.06% | 1.37% | 39% | 101% |
| RA | -2.08% | -1.92% | -2.02% | 125% | 105% | 0.18% | 1.46% | 1.41% | 68% | 100% |
| LB | -0.82% | -1.01% | -0.78% | 111% | 99% | 0.01% | 0.61% | 0.68% | 84% | 97% |

| 6.3.2.2 | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -5.81% | -4.37% | -4.37% | 255% | 122% | -0.44% | 0.24% | 0.55% | 41% | 101% |
| RA | -2.80% | -2.41% | -2.48% | 128% | 105% | -0.32% | 1.04% | 0.96% | 69% | 100% |
| LB | -1.16% | -1.24% | -1.00% | 112% | 101% | -0.20% | 0.28% | 0.35% | 86% | 100% |

## CE6.3.3: AMT and NSST complexity reduction

This contribution provides the detailed description and the results of CE6-3.3[1] that was originally proposed in JVET-J0020[2]. The proposed method skips some combinations of AMT and NSST which can reduce the encoding complexity. Simulation results reportedly show that the encoding run time using the proposed method is 78% of the encoding runtime of BMS-1.1 at the cost of 0.26% BD-rate loss for RA, and 60% of the encoding run time of BMS-1.1 at the cost of 0.59% BD-rate loss for AI. In addition, this contribution also reports results for a minor modification to the AMT encoder fast algorithm that is implemented in BMS. Simulation results reportedly show that the encoding run time with this modification is 92% of the encoding runtime of BMS-1.1 at the cost of 0.05% BD-rate loss for RA.

## CE6.3.4: AMT simplification and improvement (with CE6.1.10)

Adaptive multi-core transform (AMT) in JEM-7.0 is applied to luma coding blocks (CBs) of nonzero residual when the luma CB width and height are smaller than or equal to 64. Three aspects of AMT modifications are proposed.

Aspect 1: Reducing the number of transform types.

The proposed AMT only utilizes DCT-II, DST-VII, and flipped DST-VII (FDST-VII).

Aspect 2: Reducing encoding time with normative decoder changes.

In intra slices, unlike explicitly signaling AMT flag for all intra modes in JEM-7.0 AMT, the AMT flag is explicitly signaled for DC and planar modes, implicitly set to 1 for even angular modes, and implicitly set to 0 for odd angular modes. Besides, for intra coding units (CUs) and inter CUs, when AMT flag is equal to 1, the number of AMT modes is set to 1, 2, 3, or 4 according to CU type (intra or inter) and CU size, unlike always 4 in JEM-7.0 AMT. AMT index is signaled when there are more than one AMT mode.

Aspect 3: Improving coding efficiency

If the number of nonzero transform coefficients is larger than a threshold, AMT flag and AMT index can be sent; otherwise they are derived at decoder.

In the following, VTM-1.1 anchor excludes AMT, while BMS-1.1 anchor includes JEM-7.0 AMT. Luma BD-rate, encoding time, and decoding time are reported.

Results of applying aspect 1 to JEM-7.0 AMT, as planned in CE6.1.10.1

VTM-1.1-AI: {Y-BD-rate = -3.15%, EncT=191%, DecT=117%}

VTM-1.1-RA: {Y-BD-rate = -2.08%, EncT=159%, DecT=108%}

VTM-1.1-LB: {Y-BD-rate = -1.51%, EncT=159%, DecT=110%}

BMS-1.1-AI: {Y-BD-rate = +0.21%, EncT=91%, DecT=97%}

BMS-1.1-RA: {Y-BD-rate = +0.08%, EncT=93%, DecT=99%}

BMS-1.1-LB: {Y-BD-rate = -0.05%, EncT=97%, DecT=99%}

Results of applying aspect 1 and aspect 2 to JEM-7.0 AMT, as planned in CE6.1.10.2

VTM-1.1-AI: {Y-BD-rate = -2.67%, EncT=169%, DecT=121%}

VTM-1.1-RA: {Y-BD-rate = -1.80%, EncT=152%, DecT=109%}

VTM-1.1-LB: {Y-BD-rate = -1.32%, EncT=155%, DecT=109%}

BMS-1.1-AI: {Y-BD-rate = +1.03%, EncT=80%, DecT=98%}

BMS-1.1-RA: {Y-BD-rate = +0.56%, EncT=84%, DecT=100%}

BMS-1.1-LB: {Y-BD-rate = +0.23%, EncT=91%, DecT=99%}

|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | -4.59% | -4.07% | -4.21% | 344% | 118% | 0.59% | 0.65% | 0.72% | 60% | 98% |
| RA | -2.71% | -2.27% | -2.44% | 188% | 108% | 0.26% | 0.77% | 0.54% | 78% | 101% |
| LB | -1.73% | 0.28% | 0.01% | 174% | 109% | 0.06% | 0.19% | 0.39% | 90% | 100% |

# Participant List

| Name | Affiliation | Email | P/C |
| --- | --- | --- | --- |
| Amir Said Hilmi E. Egilmez Yung-Hsuan Chao Vadim Seregin Ted Hsieh | Qualcomm | asaid@qti.qualcomm.com hegilmez@qti.qualcomm.com yunghsua@qti.qualcomm.com vseregin@qti.qualcomm.com thsieh@qti.qualcomm.com | P/C |
| Xin Zhao Xiang Li Liang Zhao | Tencent | xinzzhao@tencent.com xlxiangli@tencent.com leolzhao@tencent.com | P/C |
| Mischa Siekmann Heiko Schwarz Santiago de Luxán Hernández | HHI | mischa.siekmann@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de santiago.de.luxan@hhi.fraunhofer.de | P/C |
| Pierrick Philippe | Orange | pierrick.philippe@orange.com | C |
| Fabrice Le Léannec Karam Naser Eduard Francois | Technicolor | fabrice.leleannec@technicolor.com karam.naser@technicolor.com Edouard.Francois@technicolor.com | P/C |
| Kiho Choi | Samsung | kiho14.choi@samsung.com | P/C |
| Kiyofumi Abe Kanoh Ryuichi | Panasonic | abe.kiyo@jp.panasonic.com kanoh.ryuichi@jp.panasonic.com | P/C |
| Jungwon Kang Sung-Chang Lim | ETRI | jungwon@etri.re.kr sclim@etri.re.kr | P/C |
| Yung-Lyul Lee Nam-Uk Kim | Sejong Univ. | yllee@sejong.ac.kr nukim@sju.ac.kr | P/C |
| Takeshi Tsukuba Masaru Ikeda | Sony | takeshi.tsukuba@sony.com masaru.ikeda@sony.com | P/C |
| Kei Kawamura Yoshitaka Kidani Sei Naito | KDDI | ki-kawamura@kddi.com yo-kidani@kddi.com se-naitou@kddi.com | P/C |
| Chih-Wei Hsu Yu-Wen Huang Ching-Yeh Chen Han Huang Tzu-Der Chuang Man-Shu Chiang Shih-Ta Hsiang | MediaTek | cw.hsu@mediatek.com yuwen.huang@mediatek.com chingyeh.chen@mediatek.com h.huang@mediatek.com peter.chuang@mediatek.com man-shu.chiang@mediatek.com shih-ta.hsiang@mediatek.com | P/C |
| Moonmo Koo Mehdi Salehifar Seung Hwan Kim Jaehyun Lim | LG Electronics | moonmo.koo@lge.com mehdi.salehifar@lge.com seunghwan3.kim@lge.com jaehyun.lim@lge.com | P/C |
| Yin Zhao Jianle Chen Haitao Yang Sergey Ikonin Alexey Filippov Vasily Rufitskiy Alexander Karabutov | Huawei | yin.zhao@huawei.com jianle.chen@huawei.com haitao.yang@huawei.com Sergey.Ikonin@huawei.com Alexey.Filippov@huawei.com Vasily.Rufitskiy@huawei.com karabutov.alexander@huawei.com | P/C |
| Yongbing Lin Quanhe Yu Jianhua Zheng Xuguang Zuo | HiSilicon | linyongbing@hisilicon.com yuquanhe@hisilicon.com zhengjianhua@hisilicon.com zuoxuguang@hikvision.com | P/C |
| Yukinobu Yasugi Takeshi Chujoh Tomohiro Ikai | Sharp | yasugi.yukinobu@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |  |
| Kiran Misra | Sharp Labs | misrak@sharplabs.com |  |
| Yuriy Reznik | Brightcove | yreznik@brightcove.com |  |
| Yu-Chen Sun | Alibaba | yuchen.s@alibaba-inc.com |  |
| Xianglin (Shawn) Wang Yi-Wen Chen | Kwai, Inc. | xianglinwang@kwai.com yiwenchen@kwai.com |  |

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-J1026, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.