| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0028-v2 |
| --- | --- |

| Title: | CE8: Summary report on current picture referencing |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Karsten Müller Li Wang | Tel: Email: | xiaozhongxu@tencent.com karsten.mueller@hhi.fraunhofer.de wangli7@hikvision.com |
| Source: | CE8 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 8 on current picture referencing. Four tests have been agreed to carry out in CE8 in between JVET-J and JVET-K meetings, to study and evaluate technologies related to current picture referencing. In this report, coding performance and complexity of these tests are reported and analyzed. In particular, test results against VTM anchor are provided to show the coding efficiency and complexity trade-off of each tool. Test results against BMS anchor are also provided to show the interaction with BMS coding tools. Crosschecking results for the performed tests are integrated in this contribution.

# Introduction

As is agreed in San Diego meeting, the following tests in table 1 are performed in CE8.

Table 1: tests that are evaluated and studied in this CE.

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| 8.1 | G. Venugopal (HHI) | JVET-J0039 | Intra region-based template matching | F. Racape (Technicolor) |
| 8.2.1 | X. Xu Tencent | JVET-J0050 | Current picture referencing, for intra and inter pictures, using CPR flag | G. Venugopal (HHI) Xiaozhen Zheng (DJI) |
| 8.2.2 | X. Xu (Tencent) | JVET-J0050 | Current picture referencing for intra and inter pictures, using refIdx approach | W. Zhang (Hulu) |
| 8.3 | X. Zuo (Hikvision) | JVET-J0042 | Current picture referencing for intra pictures | X. Ma (Huawei) |

The detailed description of each tools is provided in the next section.

# Description of CE tools

## JVET-K0048: Intra region-based template matching [1]

In intra region-based template matching, the prediction signal for the luma component is given by already reconstructed blocks inside the current picture. The location of the blocks used is specified by integer-precision displacement vectors. However, the displacement vectors are not explicitly coded.  Instead, a region index is transmitted, and the displacement vectors are determined by a defined decoder-side displacement search inside the indicated region. The basic concept of intra region-based template matching is illustrated in Fig. 1 below.

Fig. 1: Illustration of intra template matching search window and five regions

A template refers to the samples above and left of the current luma block. The template is 2-sample wide.  The best match inside a region is found by minimizing the sum of squared differences in the search region.

If two blocks yield the same SSD, the one that is processed first is considered the better match. Let , and  denote the three best matches, with .  The associated predictor blocks of , and  are denoted by , and , respectively.

The number of predictors used for final prediction is made dependent on the SSD error of the first predictor as following:

Let  .

, if and

= , if(i.e. discards )

= , otherwise (i.e. discards , )

The current Test 8.1 configuration uses the following settings:

- The region size parameter (RSP) is set to 0.

- The sizes of the regions are modified as in Table 1

Table 1: Modified search region sizes

|  |  | Down-sample search region |
| --- | --- | --- |
| A1 | 4 × (RSP + 1) | no |
| A2 | 2 × A1 | no |
| A3 | 4 × A2 | yes |

- The speed ups mentioned in [1] (i.e. No. 3 and 4 of section 3) are not considered.

The proposed method with the above configuration is called the normal mode of intra RTM.

The intra RTM method is applied to all block sizes available in VTM/BMS. For the case of BMS, the AMT and NSST tools are not applied to RTM intra predicted blocks.

## JVET-K0075/K0076: Current picture referencing for intra and inter pictures [2]

### CPR using block level flag (JVET-K0075)

Skip flag

Merge idx

PredMode

yes

no

End of CU

Residue coding

Intra coding

intra

inter

merge flag

yes

no

Merge idx

IBC flag

yes

no

BV

coding

MV

coding

Fig. 2: Syntax flowchart of proposed CPR in JVET-0050

Current picture referencing (CPR) is implemented as a block level coding mode (See Fig. 2). A block vector in integer resolution is used to indicate the displacement from the current block to a reference block inside the current picture. In the proposed method, the general syntax structure that has been used in HEVC and JEM is followed. The followings are a summary of the detailed implementation of CPR in the proposal:

- Unification of CPR merge candidate list and inter merge candidate list into one list.

  - The current block is coded in CPR mode if the merge index points to a merge candidate that is coded in CPR mode; if it points to an inter coded candidate, the current block is coded in inter mode

- When merge flag is false (non-merge case), a CPR flag is signaled after merge flag to indicate the usage of CPR mode. If a block is coded as a non-merge CPR mode:

  - The block vector is always at 1-pel integer resolution, both luma and chroma.

  - The block vector prediction method uses two candidates as predictors, one from left neighbor and one from above neighbor (if CPR coded). When either neighbor is not available, a default block vector will be used as a predictor. A flag is signaled to indicate the block vector predictor index.

  - Block vector difference is coded in the same way as a motion vector difference.

### CPR using reference picture index approach (JVET-K0076)

The operations of CPR mode and inter mode are unified, by considering the current partially decoded picture as a reference picture. In this approach, changes from the block level approach are:

- The current reference picture is put in the last position in List 0 and marked as a long-term reference picture.

- Block vector prediction and difference coding follow the same methods in inter coding.

- Chroma interpolation is allowed when luma block vector is an odd integer.

- BVD resolution is switchable between 1-pel integer and 4-pel integer at block level, when IMV flag is equal to 2.

## JVET-K0450: Current picture referencing for intra pictures [3]

In HEVC SCC, CPR mode is regarded as inter mode, and the original intra frame is changed to P frame.

In this test, first, for luma, the CPR mode is also regarded as inter mode. So in luma frame there are intra blocks and inter blocks. Then the luma frame can be regarded as inter frame and the coding method is basically inherited from the inter frame. Each block should be signaled to indicate its prediction type. Also for blocks predicted by CPR, the residual is transformed using only one transform set, which consists of DST-VII and DCT-VIII. This is just the same way that the inter prediction residual is transformed in JEM.

(a)                                       (b)

Fig.3 (a)The syntax of inter block of inter frame. (b)the syntax of inter block in proposed CPR method

However, considering the proportions of inter blocks and intra blocks, the syntax is changed. In the coding of inter picture, a skip_flag syntax is first coded for each block. If the value of the syntax is false, then the pred_mode_flag syntax is coded to indicate the prediction type of the block, as shown in Fig.3 (a). However, for the luma frame in intra frame, it is no longer efficient to code the skip_flag syntax for each block because the proportion of blocks choosing skip mode is rather low. Therefore, in the proposed method the skip_flag syntax is omitted when encoding luma frame. Then for each luma block, the pred_mode_flag syntax is first encoded. Also, in inter frame, it does not need to encode the cbf flag of block with merge mode because the flag value is inferred to be true, as shown in Fig. 3(a). (otherwise the block is with skip mode). Now in the proposed method, there is no skip mode block. For block with merge mode, a cbf flag is needed to be encoded to indicate the residual is zero or not, as shown in Fig. 3(b).

## Summary of technologies in the proposed tools

### Intra region-based template matching

The key aspects of this tool are:

- Considered RTM mode as an intra mode

- Signaling flag to indicate if a block is coded by intra prediction mode or the proposed mode

- Signal which of the five regions to perform template matching

- Proposed tool applies to intra and inter slices

- Proposed tool applies to all block sizes

- Proposed tool applies to luma sample only

- Template size is 2-smaple wide

### CPR mode using block level flag signaling

- Considered CPR mode as a third mode

- Signaling flag to indicate if a block is coded by CPR mode or inter mode

- BVP similar to MVP; BVD the same as MVD

- Proposed tool applies to intra and inter slices

- Proposed tool applies to block size 16 and below (Normative)

- Proposed tool applies to luma and chroma samples

### CPR mode using reference index signaling

- Considered CPR mode as an inter mode

- Use reference index pointing to current picture to indicate the usage

- Syntax and decoding process align with inter coding

- Proposed tool applies to intra and inter slices

- Proposed tool applies to all block sizes

- Proposed tool applies to luma and chroma samples

### CPR mode for intra pictures

- Considered CPR mode as an inter mode

- Use reference index pointing to current picture to indicate the usage

- Syntax modified from inter coding, where skip flag is removed

- Proposed tool applies to intra slices only

- Proposed tool applies to all block sizes

- Proposed tool applies to luma and chroma samples

Below is a summary of technologies used in the proposed tests in CE8:

| Feature | CE8-1 (K0048) | CE8-2.1 (K0075) | CE8-2.2 (K0076) | CE8-3 (K0450) |
| --- | --- | --- | --- | --- |
| Prediction mode | Intra mode | Third mode | Inter mode | Inter mode |
| Signaling flag | Block level | Block level | Reference index | Reference index |
| Syntax structure | Modified intra coding | Modified inter coding | Aligned with inter coding | Modified inter coding |
| Applied to slices | Intra/Inter | Intra/Inter | Intra/Inter | Intra |
| Block sizes allowed | All | <=16 | All | All |
| BV precision luma | Integer | Integer | Integer | Integer |
| BV precision chroma | Integer | Integer | Sub-pel | Sub-pel |
| Sample applicability | Luma | Luma/Chroma | Luma/Chroma | Luma/Chroma |
| Decoder operation | Template Matching | Motion Compensation | Motion Compensation | Motion Compensation |

# CE results summary

To reduce software implementation efforts on both VTM and BMS platforms, it is agreed that BMS version BMS-1.0 shall be used for these experiments, as specified in the corresponding output document from the last meeting [4].

- The proposed methods in this CE shall be tested with VTM configurations (without changing the JEM_TOOLS Macro).

- The proposed methods in this CE should be tested with BMS configurations.

The mandatory common test conditions (CTC) as specified in the JVET-J1010 document from this meeting [5] shall be followed to perform the simulations in this CE.

The mandatory test sequences as described in common test conditions [5] shall be used for these tests.

In addition to that, to evaluate the benefit of proposed tools on screen content materials, the sequences in TGM YUV 4:2:0 1080p category from HEVC SCC CTC JCT-VC-U1015 document [6] and class F in common test conditions [5] shall be tested. The details of these sequences are listed in the table below.

Table 2: screen content test sequences used in CE8

| Category | Resolution | Sequence name | fps | Bit depth | Frames to be encoded | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4:2:0 TGM | 1920x1080 1920x1080 1920x1080 1920x1080 | FlyingGraphics* Desktop Console ChineseEditing | 60 60 60 60 | 8 8 8 8 | 0-299* 0-599 0-599 0-599 | M M M M | M M M M | O O O O |
| Class F | 832x480 1024x768 1280x720 1280x720 | BasketballDrillText ChinaSpeed SlideEditing SlideShow | 50 30 30 20 | 8 8 8 8 | 0-499 0-499 0-299 0-499 | M M M M | M M M M | O O O O |
| *Note that only the first 300 frames of this sequence are used. M refers to mandatory test in the configuration; O refers to optional test in the configuration |  |  |  |  |  |  |  |  |

The followings are a summary table of the tests in this CE.

Table 3: CE8 test results against VTM/BMS anchors

# Input contributions

Table 4: input contributions for tests that are evaluated and studied in this CE

| Test | Title | Contribution # |
| --- | --- | --- |
| 8.1 | CE8: Intra Region-based Template Matching (Test 8.1) | JVET-K0048 |
| 8.2.1 | CE8-2.1: Current picture referencing using block level flag signaling | JVET-K0075 |
| 8.2.2 | CE8-2.2: Current picture referencing using reference index signaling | JVET-K0076 |
| 8.3 | CE8-3.1: Current picture referencing for intra pictures | JVET-K0450 |
|  | Crosscheck for CE8-2.2 on current picture referencing using reference index signaling (JVET-K0076) | JVET-K0436 |

# Crosscheck reports

Table 5: crosscheck reports that are associated with the tests in this CE.

| Test # | CE8.1 | CE8.2.1 | CE8.2.2 | CE8.3 |
| --- | --- | --- | --- | --- |
| Cross-checker | F. Racape | G. Venugopal | W. Zhang (Hulu) | X. Ma |
|  | (Technicolor) | (HHI) |  | (Huawei) |
| Software review/studied? (Y/N) | Yes | Yes | Yes | Yes |
| Any significant inconsistencies between the description and the provided software? | No | No | No | No |
| Results available (Full/Partial) | Partial | Partial | Partial | Cross-checker didn’t run class TGM. |
| Resutls matched? (Y/N) | Yes | Yes | Yes | Yes |
| Runtime behavior matched? (Y/N) | Yes | Yes | No | Yes |
| Additional comments | No | No | Encoding/decoding runtime is not accurate due to the workload imbalance when running the anchor jobs and the crosscheck jobs. | No special operation for Chroma component. |

# Related contributions

Table 6: Related contributions and crosschecks for this CE

| Title | Contribution # |
| --- | --- |
| CE8 related: Intra Region-based Template Matching for luma and chroma | JVET-K0050 |
| Crosscheck of JVET-K0050: CE8 related: Intra Region-based Template Matching for luma and chroma | JVET-K0401 |

# References

- G. Venugopal, H. Schwarz, D. Marpe and T. Wiegand, "Intra Region-based Template Matching", JVET-J0039, JVET-J1002, 10th JVET meeting, Apr. 2018.

- X. Xu, X. Li, G. Li and S. Liu, "Intra block copy improvement on top of Tencent’s CfP response", JVET-J0050, JVET-J1002, 10th JVET meeting, Apr. 2018.

- X. Zuo, L. Wang, F. Chen and X. Song, "Intra block copy for intra-frame coding", JVET-J0042, 10th JVET meeting, Apr. 2018.

- X. Xu, K. Müller and L. Wang, “Description of Core Experiment 8 (CE8): Current Picture Referencing”, JVET-J1028, 10th JVET meeting, Apr. 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin, "JVET common test conditions and software reference configurations", JVET-J1010, 10th JVET meeting, Apr. 2018.

- H. Yu, R. Cohen, K. Rapaka and J. Xu, "Common Test Conditions for Screen Content Coding, JCTVC-U1015", 19th JCT-VC meeting, Jun. 2015.