| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10 July – 18 July 2018 | Document: JVET-K0029_v1 |  | Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0010 |
| --- | --- | --- | --- | --- |

| Title: | CE9: Summary Report on Decoder Side MV Derivation |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Report |  |  |
| Author(s) or Contact(s): |  | Semih Esenlik, Yi-Wen Chen Xiaoyu Xiu, Antoine Robert, Xu Chen, Tzu-Der Chuang, Byeongdoo Choi, Jungwon Kang, Naeri Park | Email: | semih.esenlik@huawei.com yiwenchen@kwai.com Xiaoyu.Xiu@InterDigital.com antoine.robert@technicolor.com Anderson.chen@hisilicon.com Peter.chuang@mediatek.com choib@sharplabs.com jungwon@etri.re.kr naeri.park@lge.com |
| Source: |  | CE coordinators |  |  |

____________________________

# Abstract

The tools in the scope of this CE include bi-directional optical flow, template matching and bilateral matching based techniques for motion vector derivation and refinement at the decoder side.

The core experiment is organized into 5 sub-tests as follows:

- CE9.1 - Decoder Side Motion Vector Refinement (DMVR): 5 tests are performed in this subcategory.

- CE9.2 - Bilateral Matching: 8 tests.

- CE9.3 - Template Matching: 7 tests.

- CE9.4 - MV Candidate List Reordering by Template Matching: 3 tests.

- CE9.5 - BIO: 3 tests.

This report summarises the status of each experiment. Crosscheck results are integrated in the document.

# Introduction

The description of Core Experiment 9 (CE9) on Decoder Side Motion Vector Derivation [1] has been finalized on 14th of May 2018. A total 30 (6+9+8+3+4) tests among 5 sub-tests are defined in CE9 description. After the finalization of the CE description document, 4 tests were withdrawn by proponents, therefore results of 26 tests are reported in CE summary report.  The goal of this Core Experiment (CE) is to further investigate the techniques of decoder side motion vector derivation in VTM and BMS. Tested solutions for decoder side motion vector derivation techniques are evaluated according to BD-rate gain, runtime and additionally according to tool questionnaire that is developed for worst case complexity assessment.

# Experimental Conditions

# Software

Only one software base is going to be used which is the BMS software. The BMS subversion repository can be found at https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS. The final 1.0 release version of the software is going to be used.

# Anchors

- Proponents implement the tests in BMS SW, for both VTM and BMS tests.

- First anchor: BMS software is going to be used with all BMS tools switched on (default configuration) as anchor. If the proposed technique aims to replace an existing tool in the BMS, that tool is switched off in the test simulation but not in the anchor simulation (which must be same for all tests in all Subtests).

- Second anchor: BMS software is going to be used with all BMS tools switched off via configuring encoder parameters of BMS SW (no change of macro setting).

# Test Results

Table 1. Summary of the coding performance and runtime measurements.

| # | Tester | VTM |  |  |  |  | BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AHG 13 | DMVR in BMS according to AHG13 (Test is DMVR off) | -2.65% | -2.54% | -2.67% | 109% | 131% | 1.47% | 1.55% | 1.64% | 99% | 92% |
| 9.1.1 | Xu Chen (Hisilicon, Huawei,) | -1.46% | -1.44% | -1.47% | 106% | 116% | 0.59% | 0.58% | 0.65% | 99% | 95% |
|  |  | -2.80% | -2.58% | 2.69% | 107% | 117% | -0.16% | -0.07% | -0.11% | 99% | 95% |
| 9.1.2 | Yu-Chi Su (MediaTek), only RA | -2.65% | -2.52% | -2.65% | 108% | 127% | -0.01% | -0.01% | -0.02% | 101% | 99% |
| 9.1.3 | Xiaoyu Xiu (InterDigital) | -2.60% | -2.52% | -2.64% | 109% | 119% | 0.03% | 0.00% | 0.02% | 100% | 96% |
| 9.1.5 | Chun-Chi Chen (Qualcomm) | -2.66% | -2.54% | -2.67% | 108% | 127% | -0.02% | 0.00 | -0.01% | 100% | 99% |
| 9.1.6 | Semih Esenlik (Huawei , USTC) | -2.91% | -2.62% | -2.74% | 103% | 114% | -0.23% | -0.10% | -0.13% | 98% | 95% |
|  |  | -3.50% | -2.92% | -3.07% | 106% | 120% | -0.58% | -0.31% | -0.34% | 99% | 96% |
|  |  | -3.62% | -3.30% | -3.43% | 104% | 118% | -0.68% | -0.53% | -0.58% | 98% | 96% |
|  |  | -4.39% | -3.72% | -3.90% | 107% | 127% | -1.17% | -0.86% | -0.93% | 99% | 98% |
|  |  | -3.94% | -3.58% | -3.75% | 105% | 121% | -0.90% | -0.78% | -0.84% | 99% | 97% |
|  |  | -4.71% | -4.04% | -4.22% | 109% | 131% | -1.40% | -1.07% | -1.17% | 99% | 99% |
|  |  | -2.48% | -2.18% | -2.27% | 105% | 117% | -0.04% | 0.11% | 0.12% | 99% | 97% |
|  |  | -2.96% | -2.47% | -2.55% | 106% | 123% | -0.34% | -0.09% | -0.06% | 100% | 98% |
|  |  | -3.20% | -2.86% | -2.93% | 105% | 121% | -0.49% | -0.32% | -0.31% | 99% | 98% |
|  |  | -3.87% | -3.25% | -3.39% | 108% | 130% | -0.92% | -0.59% | -0.64% | 100% | 100% |
|  |  | -4.26% | -3.60% | -3.75% | 110% | 134% | -1.16% | -0.83% | -0.89% | 101% | 101% |
| 9.2.1 | Tzu-Der Chuang (MediaTek), RA/LB | -5.72% -0.43% | -4.64% -0.71% | -4.82% -0.86% | 172% 236% | 272% 151% | -2.35% -0.34% | -1.90% -0.64% | -2.00% -0.64% | 128% 149% | 168% 132% |
|  | (supplementary), no sub-block search | -4.76% -0.46% | -3.92% -0.51% | -4.06% -0.66% | 155% 172% | 168% 131% | -1.58% -0.41% | -1.27% -0.47% | -1.33% -0.59% | 118% 125% | 163% 118% |
| 9.2.2 | Xiaoyu Xiu (InterDigital) | -2.45% | -2.25% | -2.22% | 125% | 162% | -1.02% | -1.10% | -1.08% | 107% | 112% |
| 9.2.3 | Jingya Li (Panasonic) | -1.90% | -1.76% | -1.81% | 116% | 154% | -0.62% | -0.71% | -0.78% | 106% | 116% |
| 9.2.5 | Chun-Chi Chen (Qualcomm) | -5.28% | -4.70% | -4.88% | 116% | 164% | -1.76% | -1.55% | -1.62% | 102% | 112% |
| 9.2.6 | Chun-Chi Chen (Qualcomm) | -4.76% | -4.15% | -4.32% | 112% | 150% | -1.40% | -1.13% | -1.22% | 100% | 108% |
| 9.2.7 | Byeongdoo Choi (Sharp) | -6.68% | -6.11% | -6.37% | 157% | 277% | -3.32% | -3.10% | -3.26% | 125% | 162% |
| 9.2.8 | Yue Li (USTC) | -3.61% | -3.15% | -3.27% | 107% | 119% | -0.64% | -0.46% | -0.54% | 99% | 97% |
|  |  | -4.48% | -3.92% | -4.11% | 109% | 125% | -1.22% | -0.98% | -1.05% | 100% | 98% |
| 9.2.9 | Semih Esenlik (Huawei , USTC) | -3.18% | -2.95% | -3.09% | 103% | 115% | -0.38% | -0.33% | -0.35% | 97% | 96% |
|  |  | -3.68% | -3.17% | -3.33% | 106% | 124% | -0.68% | -0.48% | -0.52% | 98% | 98% |
|  |  | -3.80% | -3.57% | -3.70% | 105% | 119% | -0.79% | -0.73% | -0.78% | 99% | 97% |
|  |  | -4.59% | -3.97% | -4.14% | 107% | 132% | -1.29% | -1.02% | -1.10% | 98% | 101% |
|  |  | -4.07% | -3.86% | -4.03% | 105% | 121% | -0.97% | -0.96% | -0.98% | 99% | 97% |
|  |  | -4.92% | -4.31% | -4.50% | 109% | 136% | -1.50% | -1.28% | -1.35% | 99% | 102% |
|  |  | -2.72% | -2.48% | -2.56% | 103% | 118% | -0.18% | -0.06% | -0.05% | 98% | 97% |
|  |  | -3.11% | -2.67% | -2.76% | 105% | 125% | -0.41% | -0.22% | -0.21% | 99% | 100% |
|  |  | -3.38% | -3.13% | -3.23% | 106% | 121% | -0.59% | -0.49% | -0.50% | 99% | 98% |
|  |  | -4.08% | -3.47% | -3.62% | 109% | 133% | -1.03% | -0.78% | -0.78% | 100% | 101% |
|  |  | -4.47% | -3.88% | -4.02% | 111% | 138% | -1.28% | -1.04% | -1.06% | 100% | 103% |
|  |  | -2.80% | -2.57% | -2.63% | 109% | 132% | -0.30% | -0.23% | -0.20% | 100% | 102% |
| 9.3.1 | Hahyun Lee (ETRI) | -4.25% | -4.20% | -4.44% | 257% | 172% | -2.37% | -2.16% | -2.37% | 147% | 113% |
| 9.3.2 | Jingya Li (Panasonic) | -4.80% | -4.57% | -4.71% | 126% | 180% | -2.62% | -2.37% | -2.58% | 115% | 127% |
| 9.3.4 | Antoine Robert (Technicolor) | -6.57% -3.39% | -6.22% -3.62% | -6.44% -3.94% | 171% 219% | 295% 227% | -3.65% | -3.52% | -3.68% | 133% | 173% |
| 9.3.5 | Antoine Robert (Technicolor) | Combined results (CE9.3.4 and CE9.3.5) are reported by the proponent (see 9.3.4) |  |  |  |  |  |  |  |  |  |
| 9.3.6 | Xu Chen (HiSilicon) | -1.70% | -1.86% | -1.90% | 156% | 122% | -0.71% | -0.77% | -0.81% | 115% | 108% |
|  |  | -3.12% | -2.92% | -2.98% | 159% | 119% | -1.60% | -1.42% | -1.50% | 116% | 109% |
| 9.3.7 | Naeri Park (LGE) | -3.42% | -3.28% | -3.46% | 115% | 167% | -1.51% | -1.42% | -1.52% | 103% | 124% |
| 9.3.8 | Naeri Park (LGE) | -4.01% | -3.90% | -4.08% | 117% | 193% | -2.10% | -2.08% | -2.17% | 103% | 141% |
| 9.4.1 | Naeri Park (LGE) | -0.71% | -0.74% | -0.74% | 104% | 122% | -0.62% | -0.73% | -0.76% | 101% | 116% |
| 9.4.2 | Sergey Ikonin (Huawei) | -0.49% | -0.44% | -0.38% | 102% | 104% | -0.29% | -0.28% | -0.28% | 100% | 101% |
| 9.4.3 | Na Zhang (HiSilicon) | -0.56% | -0.51% | -0.48% | 103% | 115% | -0.71% | -0.81% | -0.80% | 101% | 113% |
|  |  | -0.81% | -0.46% | -0.63% | 104% | 119% | -0.90% | -0.67% | -0.83% | 101% | 113% |
| 9.5.2 | Ching-Yeh Chen (MediaTek) | -2.86% | -0.94% | -0.65% | 108% | 146% | -1.28% | -0.49% | -0.42% | 104% | 118% |
|  | With adaptive unit | -2.85% | -0.92% | -0.66% | 111% | 156% | -1.28% | -0.48% | -0.41% | 105% | 122% |
|  | Apply on chroma | -2.90% | -2.23% | -2.30% | 110% | 155% | -1.31% | -1.15% | -1.25% | 105% | 122% |
|  | With adaptive unit and apply on chroma | -2.90% | -2.22% | -2.30% | 113% | 168% | -1.31% | -1.16% | -1.23% | 106% | 127% |
| 9.5.3 | Xiaoyu Xiu (InterDigital) | -2.61% | -1.02% | -0.74% | 111% | 137% | -1.26% | -0.55% | -0.49% | 103% | 108% |
| 9.5.4 | Chao-Hsiung Hong(Qualcomm) | -2.94% | -1.07% | -0.76% | 116% | 166% | -1.49% | -0.58% | -0.49% | 106% | 130% |

Table 2. Summary of the complexity assessment questionnaire.

| # | Tester | Initial MV signaled | Sub-CU refinement | Neighboring recon. samples used | Max # of SAD calculation | Max. SR | Cost Function | Interpolation filter/tap no | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AHG 13 | DMVR in BMS according to AHG13* *(Anchor is BMS-DMVR) | yes | no | no | 18 | 1 | SAD | DCTIF/8 | SIMD = SSE42 anchor&test |
| 9.1.1 | Xu Chen (Hisilicon, Huawei,) | yes | no | no | 12 | 1 | MRSAD | DCTIF/8 | No SIMD for MRSAD calculation SIMD = AVX2 anchor&test Prediction from refined MV disabled |
|  |  | yes | no | no | 12 | 1 | MRSAD | DCTIF/8 | No SIMD for MRSAD calculation SIMD = AVX2 anchor&test |
| 9.1.2 | Yu-Chi Su (MediaTek), only RA | yes | no | no | 18 | 1 | SAD | DCTIF/8 | SIMD = SSE42 anchor&test MAX # of SAD for L0 is 9; Max # of SAD for L1 is 9 but its optional |
| 9.1.3 | Xiaoyu Xiu (InterDigital) | yes | no | no | 19 | 1 | SAD | DCTIF/8 | SIMD = AVX anchor&test |
| 9.1.5 | Chun-Chi Chen (Qualcomm) | yes | no | no | 18 | 1 | SAD | DCTIF/8 | SIMD = AVX anchor&test |
| 9.1.6 | Semih Esenlik (Huawei , USTC) | yes | no | no | 6 | 1 | MRSAD | DCTIF/8 | No SIMD for MRSAD calculation, SIMD = AVX2 anchor&test |
|  |  | yes | no | no | 10 | 1 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 12 | 2 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 16 | 2 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 24 | 4 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 28 | 4 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 6 | 1 | MRSAD | DCTIF/8 | No SIMD for MRSAD calculation, SIMD = AVX2 anchor&test, No reference to refined MV inside 32x32 grid |
|  |  | yes | no | no | 10 | 1 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 12 | 2 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 16 | 2 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 28 | 4 | MRSAD | DCTIF/8 |  |
| 9.2.1 | Tzu-Der Chuang (MediaTek), RA/LB | yes | Yes (optional) | no | CU-level: 9+32*5+8 = 177 Sub-CU-level: 5 + 16*3 + 4 = 57 | 8 | SAD | ME: Bilinear filter/2 MC: DCTIF/8 (same as JEM) | SIMD = SSE42 anchor&test |
|  | (supplementary), no sub-block search | yes | no | no | 9+32*5+8 = 177 | 8 | SAD | ME: Bilinear filter/2 MC: DCTIF/8 (same as JEM) | SIMD = SSE42 anchor&test |
| 9.2.2 | Xiaoyu Xiu (InterDigital) | no | Yes (optional) | no | 16 for CU-level | 12 | SAD | ME: Bilinear filter/2 MC: DCTIF/8 | SIMD = AVX anchor&test |
| 9.2.3 | Jingya Li (Panasonic) | no | no | no | Not defined | Within pre-determined memory block | SAD | ME: Bilinear filter/2 MC: DCTIF/8 (same as JEM) | SIMD = SSE42 anchor&test |
| 9.2.5 | Chun-Chi Chen (Qualcomm) | yes | no | no | 9+25*5+8 = 142 | 8 | >64:MRSAD ≤64:SAD | DCTIF/8 | SIMD for MRSAD calculation, SIMD = AVX anchor&test |
| 9.2.6 | Chun-Chi Chen (Qualcomm) | yes | no | no | 9+5+11*2+8 = 44 | 2 | >64:MRSAD ≤64:SAD | ME: Bilinear filter/2 MC: DCTIF/8 | SIMD for MRSAD calculation, SIMD = AVX anchor&test |
| 9.2.7 | Byeongdoo Choi (Sharp) | no | Yes (optional) | yes | same as JEM | 8 | SAD | ME: Bilinear filter/2 MC: DCTIF/8 (same as JEM) | SIMD = AVX2 anchor&test |
| 9.2.8 | Yue Li (USTC) | yes | no | no | 10 | 1 | MRSAD | DCTIF/8 | No SIMD for MRSAD calculation, SIMD = AVX2 anchor&test |
|  |  | yes | no | no | 13 | 2 | MRSAD | DCTIF/8 |  |
| 9.2.9 | Semih Esenlik (Huawei , USTC) | yes | no | no | 6 | 1 | MRSAD | DCTIF/8 | No SIMD for MRSAD calculation, SIMD = AVX2 anchor&test |
|  |  | yes | no | no | 10 | 1 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 9 | 2 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 13 | 2 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 15 | 4 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 19 | 4 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 6 | 1 | MRSAD | DCTIF/8 | No SIMD for MRSAD calculation, SIMD = AVX2 anchor&test, No reference to refined MV inside 32x32 grid |
|  |  | yes | no | no | 10 | 1 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 9 | 2 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 13 | 2 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 19 | 4 | MRSAD | DCTIF/8 |  |
|  |  | yes | no | no | 13 | 2 | MRSAD | DCTIF/8 | No SIMD for MRSAD calculation, SIMD = AVX2 anchor&test, No reference to refined MV inside the whole frame |
| 9.3.1 | Hahyun Lee (ETRI) | Yes | No | Yes | Not defined | 8 | SAD | Bilinear filter/2 | SIMD = AVX anchor&test (Encoder) = SSE42 anchor&test (Decoder) |
| 9.3.2 | Jingya Li (Panasonic) | no | no | yes | Not defined | Within pre-determined memory block | SAD | ME: Bilinear filter/2 MC: DCTIF/8 (same as JEM) | SIMD = SSE42 anchor&test |
| 9.3.4 | Antoine Robert (Technicolor) | no | yes | yes | Not defined | 8 | SAD | Bilinear filter/2 | SIMD = SSE42 anchor&test |
| 9.3.5 | Antoine Robert (Technicolor) | Combined results (CE9.3.4 and CE9.3.5) are reported by the proponent (see 9.3.4) |  |  |  |  |  |  |  |
| 9.3.6 | Xu Chen (HiSilicon) | no | no | yes | 9 | 1 | SAD | DCTIF/8 | SIMD = AVX2 anchor&test |
|  |  | no | no | yes | Not defined | 8 | SAD | Bilinear filter/2 | SIMD = AVX2 anchor&test |
| 9.3.7 | Naeri Park (LGE) | no | no | yes | Not defined | 8 | SAD | Bilinear filter/2 | SIMD = AVX2 anchor&test |
| 9.3.8 | Naeri Park (LGE) | no | no | yes | Not defined | 8 | SAD | Bilinear filter/2 | SIMD = AVX2 anchor&test |
| 9.4.1 | Naeri Park (LGE) | no | no | yes | 5 | N.A | SAD | Bilinear filter/2 | SIMD = AVX2 anchor&test |
| 9.4.2 | Sergey Ikonin (Huawei) | yes | no | yes | 4 | N.A. | SAD | Bilinear filter/2 | SIMD = AVX2 anchor&test |
| 9.4.3 | Na Zhang (HiSilicon) | no | no | yes | 2 | N.A | SAD | DCTIF/8 | SIMD = AVX2 anchor&test |
|  |  | no | no | yes | 2 | N.A | SAD | DCTIF/8 | SIMD = AVX2 anchor&test |
| 9.5.2 | Ching-Yeh Chen (MediaTek) | no | yes | no | 0 | N.A | N.A | 3 tap filter [-1, 0, 1] for gradient, DCTIF/8 for MC | SIMD = SSE42 anchor&test |
|  | With adaptive unit | no | yes | no | 0 | N.A | N.A | 3 tap filter [-1, 0, 1] for gradient, DCTIF/8 for MC | SIMD = SSE42 anchor&test |
|  | Apply on chroma | no | yes | no | 0 | N.A | N.A | 3 tap filter [-1, 0, 1] for gradient, DCTIF/8 for MC | SIMD = SSE42 anchor&test |
|  | With adaptive unit and apply on chroma | no | yes | no | 0 | N.A | N.A | 3 tap filter [-1, 0, 1] for gradient, DCTIF/8 for MC | SIMD = SSE42 anchor&test |
| 9.5.3 | Xiaoyu Xiu (InterDigital) | yes | yes | no | 1 | N.A. | SAD | 6 tap filter for gradient, 6 tap filter for MC (as in JEM7.0) | SIMD = AVX anchor&test |
| 9.5.4 | Chao-Hsiung Hung (Qualcomm) | yes | yes | no | 0 | N.A. | N.A. | 5 tap filter [2, -9, 0, 9, 2] for gradient DCTIF/8 for MC | SIMD = AVX anchor&test |

# Crosscheck Results

Table 3. Summary of the BD-rate and runtime measurements from crosscheck results.

| # | Cross-checker | VTM |  |  |  |  | BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AHG13 | Yuwen He (Interdigital) | -2.65% | -2.54% | -2.67% | 109% | 136% | 1.47% | 1.55% | 1.64% | 98% | 93% |
| 9.1.1 | Tzu-Der Chuang (MediaTek) |  |  |  |  |  |  |  |  |  |  |
| 9.1.2 | Ivan Krasnov (Huawei) | -2.65% | -2.52% | -2.65% | 108% | 123% | -0.01% | -0.01% | -0.02% | 100% | 99% |
| 9.1.3 | Yue Li (USTC) | -2.60% | -2.52% | -2.64% | 110% | 120% | 0.03% | 0.00% | 0.02% | 101% | 99% |
| 9.1.5 | Xiaoyu Xiu (InterDigital) |  |  |  |  |  |  |  |  |  |  |
| 9.1.6 | Xiaoyu Xiu (InterDigital) |  |  |  |  |  |  |  |  |  |  |
| 9.2.1 | Fangdong Chen (Hikvision) | -5.72% | -4.64% | -4.82% | 157% | 122% | -2.35% | -1.90% | -2.00% | 123% | 149% |
|  |  | -0.43% | -0.71% | -0.86% | 204% | 117% | -0.34% | -0.64% | -0.64% | 145% | 127% |
| 9.2.2 | Byeongdoo Choi (Sharp) | -2.45% | -2.25% | -2.22% | 123% | 178% | -1.26% | -1.36% | -1.48% | 144% | 173% |
| 9.2.3 | Chun-Chi Chen (Qualcomm) | -1.90% | -1.76% | -1.81% | 116% | 154% | -0.62% | -0.71% | -0.78% | 106% | 116% |
| 9.2.5 | Hahyun Lee (ETRI) | -5.28% | -4.70% | -4.88% | 116% | 185% | -1.76% | -1.55% | -1.62% | 101% | 109% |
| 9.2.6 | Wenhao Zhang (Hulu) |  |  |  |  |  |  |  |  |  |  |
| 9.2.7 | Tzu-Der Chuang (MediaTek) Wenhao Zhang (Hulu) |  |  |  |  |  |  |  |  |  |  |
| 9.2.8 | Jingya Li (Panasonic) | -3.61% | -3.15% | -3.27% | 105% | 119% | -0.64% | -0.46% | -0.54% | 99% | 95% |
|  |  | -4.48% | -3.92% | -4.11% | 106% | 125% | -1.22% | -0.98% | -1.05% | 99% | 97% |
| 9.2.9 | Chun-Chi Chen (Qualcomm) | -3.18% | -2.95% | -3.09% | 105% | 116% | -0.38% | -0.33% | -0.35% | 100% | 98% |
|  |  | -3.68% | -3.17% | -3.33% | 107% | 127% | -0.68% | -0.48% | -0.52% | 100% | 101% |
|  |  | -3.80% | -3.57% | -3.70% | 106% | 120% | -0.79% | -0.73% | -0.78% | 100% | 99% |
|  |  | -4.59% | -3.97% | -4.14% | 108% | 134% | -1.29% | -1.02% | -1.10% | 100% | 101% |
|  |  | -4.07% | -3.86% | -4.03% | 106% | 121% | -0.97% | -0.96% | -0.98% | 99% | 97% |
|  |  | -4.92% | -4.31% | -4.50% | 110% | 140% | -1.50% | -1.28% | -1.35% | 100% | 103% |
|  |  | -2.72% | -2.48% | -2.56% | 108% | 122% | -0.18% | -0.06% | -0.05% | 101% | 100% |
|  |  | -3.11% | -2.67% | -2.76% | 110% | 132% | -0.41% | -0.22% | -0.21% | 102% | 103% |
|  |  | -3.38% | -3.13% | -3.23% | 110% | 127% | -0.59% | -0.49% | -0.50% | 102% | 101% |
|  |  | -4.08% | -3.47% | -3.62% | 113% | 142% | -1.03% | -0.78% | -0.78% | 102% | 105% |
|  |  | -3.71% | -3.46% | -3.59% | 110% | 129% | -0.79% | -0.74% | -0.76% | 102% | 101% |
|  |  | -4.47% | -3.88% | -4.02% | 114% | 146% | -1.28% | -1.04% | -1.06% | 104% | 107% |
|  |  | -2.80% | -2.57% | -2.63% | 110% | 136% | -0.30% | -0.23% | -0.20% | 101% | 103% |
| 9.3.1 | Yi-Wen Chen (Qualcomm) | -4.25% | -4.20% | -4.44% | 247% | 154% | -2.22% | -1.97% | -2.20% | 158% | 122% |
| 9.3.2 | Fangdong Chen (Hikvision) | -4.80% | -4.57% | -4.71% | 127% | 179% | -2.62% | -2.37% | -2.58% | 112% | 120% |
|  |  | -2.12% | -1.78% | -1.68% | 155% | 157% | -1.78% | -1.45% | -1.22% | 130% | 128% |
| 9.3.4 | Sriram Sethuraman (Ittiam) | -6.35% | -5.98% | -6.22% | 190% | 301% | -3.45% | -3.32% | -3.48% | 148% | 196% |
|  |  | -5.79% | -5.27% | -5.53% | 207% | 329% | -3.86% | -3.71% | -3.80% | 149% | 230% |
| 9.3.5 | Sriram Sethuraman (Ittiam) | -6.59% | -6.24% | -6.48% | 198% | 331% | -3.66% | -3.55% | -3.70% | 152% | 215% |
|  |  | -6.14% | -5.66% | -5.83% | 208% | 346% | -4.12% | -4.03% | -4.21% | 150% | 242% |
| 9.3.4+ 9.3.5 | Sriram Sethuraman (Ittiam) | -6.57% | -6.22% | -6.44% | 191% | 300% | -3.65% | -3.52% | -3.68% | 149% | 196% |
|  |  | -6.12% | -5.55% | -5.83% | 207% | 328% | -4.13% | -3.86% | -4.11% | 150% | 231% |
| 9.3.6 | Antoine Robert (Technicolor) |  |  |  |  |  |  |  |  |  |  |
| 9.3.6 | Tomohiro Ikai (Sharp) | -1.70% | -1.86% | -1.90% | 152% | 128% | -0.71% | -0.77% | -0.81% | 113% | 110% |
|  |  | -0.91% | -1.03% | -1.01% | 106% | 126% | -0.33% | -0.35% | -0.40% | 101% | 109% |
|  |  | -1.05% | -1.13% | -1.14% | 147% | 103% | -0.40% | -0.48% | -0.51% | 113% | 101% |
|  |  | -3.12% | -2.92% | -2.98% | 157% | 125% | -1.60% | -1.42% | -1.50% | 116% | 110% |
| 9.3.7 | Byeongdoo Choi (Sharp) | -3.42% | -3.28% | -3.46% | 108% | 152% | -1.51% | -1.42% | -1.52% | 102% | 121% |
| 9.3.8 | Xu Chen (HiSilicon) | -4.01% | -3.90% | -4.08% | 115% | 176% | -2.10% | -2.08% | -2.17% | 102% | 138% |
| 9.4.1 | Sung-Chang Lim (ETRI) | -0.71% | -0.74% | -0.74% | 142% | 136% | -0.62% | -0.73% | -0.76% | 127% | 113% |
| 9.4.2 | Naeri Park (LGE) | -0.49% | -0.44% | -0.38% | 101% | 104% | -0.29% | -0.28% | -0.28% | 101% | 102% |
| 9.4.3 | Yu, Yue (Arris) | -0.56% | -0.51% | -0.48% |  |  | -0.71% | -0.81% | -0.80% |  |  |
|  |  | -0.81% | -0.46% | -0.63% |  |  | -0.90% | -0.67% | -0.83% |  |  |
| 9.5.2 | Chao-Hsiung Hung (Qualcomm) | -2.86% | -0.94% | -0.65% | 105% | 131% | -1.28% | -0.49% | -0.42% | 99% | 118% |
|  | With adaptive unit | -2.85% | -0.92% | -0.66% | 109% | 145% | -1.28% | -0.48% | -0.41% | 102% | 126% |
|  | Apply on chroma | -2.90% | -2.23% | -2.30% | 110% | 146% | -1.31% | -1.15% | -1.25% | 102% | 125% |
|  | With adaptive unit and apply on chroma | -2.90% | -2.22% | -2.30% | 111% | 156% | -1.31% | -1.16% | -1.23% | 103% | 131% |
| 9.5.3 | Tzu-Der Chuang (MediaTek) |  |  |  |  |  |  |  |  |  |  |
| 9.5.4 | Kiho Choi (Samsung) | -2.94% | -1.07% | -0.76% | 122% | 201% | -1.49% | -0.58% | -0.49% | 108% | 135% |

Table 4. Summary of the crosschecker comments from crosscheck results.

| # | Cross-checker | Crosschecker Comments |
| --- | --- | --- |
| AHG13 | Yuwen He (Interdigital) | Anchor&Test run with SSE42 |
| 9.1.1 | Tzu-Der Chuang (MediaTek) |  |
| 9.1.2 | Ivan Krasnov (Huawei) | Coding gain and runtime results are confirmed. Software match with description. Anchor&Test run with AVX2. |
| 9.1.3 | Yue Li (USTC) | Coding gain and runtime results are confirmed. Remove redundant MC is also introduced into DMVR. ( SIMD = AVX2 anchor&test) |
| 9.1.5 | Xiaoyu Xiu (InterDigital) |  |
| 9.1.6 | Xiaoyu Xiu (InterDigital) |  |
| 9.2.1 | Fangdong Chen (Hikvision) | EncT and DecT measured with AVX2 |
| 9.2.2 | Byeongdoo Choi (Sharp) |  |
| 9.2.3 | Chun-Chi Chen (Qualcomm) |  |
| 9.2.5 | Hahyun Lee (ETRI) | Coding gains are same with those provided by proponent. Encoding time is measured with AVX and Decoding time is measured with SSE42. VTM decoding time gap is observed. |
| 9.2.6 | Wenhao Zhang (Hulu) |  |
| 9.2.7 | Tzu-Der Chuang (MediaTek) Wenhao Zhang (Hulu) |  |
| 9.2.8 | Jingya Li (Panasonic) |  |
| 9.2.9 | Chun-Chi Chen (Qualcomm) |  |
| 9.3.1 | Yi-Wen Chen (Qualcomm) |  |
| 9.3.2 | Fangdong Chen (Hikvision) | EncT and DecT measured with AVX2 |
| 9.3.4 | Sriram Sethuraman (Ittiam) | EncT and DecT measured with AVX2 |
| 9.3.5 | Sriram Sethuraman (Ittiam) | EncT and DecT measured with AVX2 |
| 9.3.4+ 9.3.5 | Sriram Sethuraman (Ittiam) | EncT and DecT measured with AVX2 |
| 9.3.6 | Antoine Robert (Technicolor) |  |
| 9.3.6 | Tomohiro Ikai (Sharp) | CE9.3.6_op1 Coding gains are same with those provided by proponent. EncT and DecT measured with AVX2 |
|  |  | CE9.3.6_op1_Merge |
|  |  | CE9.3.6_op1_AM |
|  |  | CE9.3.6_op2 |
| 9.3.7 | Byeongdoo Choi (Sharp) |  |
| 9.3.8 | Xu Chen (HiSilicon) | Coding gain and runtime results are confirmed. |
| 9.4.1 | Sung-Chang Lim (ETRI) | Coding gain are confirmed. And, the software is consistent with the description. But, cross-check’s runtimes are unreliable. |
| 9.4.2 | Naeri Park (LGE) | The results are same with those provided by the proponent |
| 9.4.3 | Yu, Yue (Arris) |  |
| 9.5.2 | Chao-Hsiung Hung (Qualcomm) | Coding gain match those provided by the proponent. |
|  | With adaptive unit | Coding gain match those provided by the proponent. |
|  | Apply on chroma | Coding gain match those provided by the proponent. |
|  | With adaptive unit and apply on chroma | Coding gain match those provided by the proponent. |
| 9.5.3 | Tzu-Der Chuang (MediaTek) |  |
| 9.5.4 | Kiho Choi (Samsung) | EncT and DecT measured with AVX. Coding gains are same with those provided by proponent. Encoding time is measured with AVX and Decoding time is measured with AVX. VTM decoding time gap is observed. |

# List of Experiments

# CE9.1: Decoder Side Motion Vector Refinement (DMVR)

| # | Test | Input Documents/Tester |
| --- | --- | --- |
| CE9.1.1 | - Search Range is 1 - Adaptive search pattern (6 points instead of 9) - Mean removed SAD as cost function - Early termination: if motion vector is not changed after an iteration | JVET-K0199 X. Chen (Hisilicon, Huawei) |
| CE9.1.2 | - Early termination after L0 search if motion vector is not changed after one iteration | JVET-K0253 Yu-Chi Su (MediaTek) |
| CE9.1.3 | - Early termination based on initial SAD cost between prediction L0 and prediction L1 - High precision SAD (no clip and round) | JVET-K0342 Xiaoyu Xiu (InterDigital) |
| CE9.1.4 | - Usage of DMVR is signaled. - Set the initial MV as the first available bi-directional MVs found by checking spatial/temporal candidates in the pre-defined order - Half pel on | JVET-K0168 Sung-Chang Lim (ETRI) |
| CE9.1.5 | - DMVR not applied if MV difference between the selected candidate and any of the previous candidates in the merge list is larger than 1 pixel in horizontal or vertical direction. | JVET-K0358 Chun-Chi Chen (Qualcomm) |
| CE9.1.6 | - MV difference mirroring. - Results are to be provided for number of iterations 4, 2, 1, and half-pel on/off. - 6 point corner selective integer search and 4 point half pel search. - Results are to be provided by switching off spatial MV prediction from refined motion vectors in 32x32 grid. | JVET-K0216 Semih Esenlik (Huawei , USTC) |

# CE9.2: Bilateral Matching

| # | Test | Tester |
| --- | --- | --- |
| CE9.2.1 | - Explicitly signaled initial MV candidate. - Sub-CU search on - For the sub-CU-level search, only the MV determined from the CU-level search is evaluated. - Bounding window for Sub-CU search - Disabled for 4x4, 4x8, and 8x4 CUs. | JVET-K0254 Tzu-Der Chuang (MediaTek) |
| CE9.2.2 | - Clustering of initial candidates - The initial MV is rounded to the integer precision - Max number of iterations 12 - Half pel refinement: ½, 1/4, 1/8 pel in order - Early termination based on SAD cost | JVET-K0343 Xiaoyu Xiu (InterDigital) |
| CE9.2.3 | - SubCU level process is removed. - Candidate list size reduced - Predefined memory access windows relative to the current CTU (dependent on number of reference frames). - Adaptive search pattern to simplify search | JVET-K0177 Jingya Li (Panasonic) |
| CE9.2.5 | - Merge index is signaled. - Additional merge list - Adaptively apply Bilateral Matching when the following conditions are met - Uni-directional, ATMVP, STMVP, affine and candidates using IC mode are excluded. - (POCref0 – POCcur) and (POCref1 – POCcur) values is negative. - The MV difference between the selected candidate and any of the previous candidates is larger than a pre-defined one integer pixel. - Sub-block refinement is removed - Mean removed SAD is applied adaptively based on CU size | JVET-K0359 Chun-Chi Chen (Qualcomm) |
| CE9.2.6 | Based on CE9.2.5 two modifications are tested: - The DCTIF for search is replaced by bi-linear filter; - The Search range is reduced from 8 to 2 | JVET-K0359 Chun-Chi Chen (Qualcomm) |
| CE9.2.7 | - Implementation based on Bilateral Matching code in BMS1.0 software. - Motion vector difference is mirrored for forward and backward MVPs (bilateral matching disabled otherwise). - Sub-CU refinement off. - Merge candidates are used as origin MVs. | JVET-K0303 Byeongdoo Choi (Sharp) |
| CE9.2.8 | Implemented on top of CE9.2.9. - 4 points half-pel search is replaced by 2 point adaptive half-pel search pattern. | JVET-K0378 Yue Li (USTC) |
| CE9.2.9 | - Bilateral matching cost function instead of generating template. - MVD mirroring with initial MV candidate signaled as merge index. - Results are to be provided for search ranges 4, 2, 1, and half-pel off. - Results are to be provided by disabling spatial prediction from refined motion vectors within 32x32 grid. - Results are to be provided for disabling spatial prediction from refined MV completely. | JVET-K0217 Semih Esenlik (Huawei , USTC) |

# CE9.3: Template Matching

| # | Test | Tester |
| --- | --- | --- |
| CE9.3.1 | - Additional merge list is constructed which is different from regular merge list (Max 7 candidates). - Index signaled. - Sub-PU search off - Search range is restricted to 8 samples | JVET-K0168 Hahyun Lee (ETRI) |
| CE9.3.2 | - Sub-CU refinement process is removed. - Candidate list size reduced (unilateral candidates not inserted) - Adaptive pattern search - the search only one point in the direction of smallest cost, - if the cost does not get lower in the selected direction, other directions are searched. - Predefined memory access windows relative to the current CTU (dependent on number of reference frames). - 374x374 samples if 1 refPic - 264x264 samples if 2 refPic - … | JVET-K0178 Jingya Li (Panasonic) |
| CE9.3.4 | - Applied on top of JVET-J0021 - Unilateral candidates are not inserted in merge list. | JVET-K0214 Antoine Robert (Technicolor) |
| CE9.3.5 | - Applied on top of JVET-J0021 - MVP list is derived from candidates in merge list - Max 2 candidates are checked by template matching | JVET-K0214 Antoine Robert (Technicolor) |
| CE9.3.6 | - Merge: MV refinement applied only if first candidate is selected. - AMVP: refinement applied to both candidates - AM mode: applied only on the merge direction - Sub-CU search disabled | JVET-K0200 Xu Chen (HiSilicon) |
| CE9.3.7 | - Applied only to the first candidate in the merge list. - First candidate MV in merge list is refined using template matching. - Not applied to ATMVP candidate. | JVET-K0088 Naeri Park (LGE) |
| CE9.3.8 | - Combined results of CE9-4.1 and CE9-3.7 | JVET-K0088 Naeri Park (LGE) |

# CE9.4: MV Candidate List Reordering by Template Matching

| # | Test | Tester |
| --- | --- | --- |
| CE9.4.1 | - Merge list is constructed according to VTM and BMS rules. - The candidates are reordered according to template matching cost. | JVET-K0088 Naeri Park (LGE) |
| CE9.4.2 | - Applied to AMVP mode - MVD sign is signalled as an index, where table entries are ranked according to template matching cost | JVET-K0067 Sergey Ikonin (Huawei) |
| CE9.4.3 | - Merge list is constructed according to VTM and BMS rules. - The candidates are reordered according to template matching cost. - Sub-CU candidates are also considered in sorting. - Sorting is performed within groups (not full reorder). | JVET-K0143 Na Zhang (HiSilicon) |

# CE9.5: BIO

| # | Test | Tester |
| --- | --- | --- |
| CE9.5.2 | - Picture resolution dependent block size 2x2 and 4x4 based on the video resolution. - gradients are directly calculated based on interpolated prediction signal using a 3-tap filter ([1 0 -1]) - BIO applied to chroma. Luma displacement vectors are reused after scaling. - Reference block size is (w+7)x(h+7), padding is applied. - Division is replaced by shift operations | JVET-K0255 Tzu-Der Chuang (MediaTek) |
| CE9.5.3 | - Bio is conditionally disabled based on template matching cost on CU level, thr = 2(BDepth-9) - Same applied at Sub-CU level, thr = 3x2(BDepth-10) | JVET-K0344 Xiaoyu Xiu (InterDigital) |
| CE9.5.4 | - Reference block size is extended from (w+7)x(h+7) to (w+11)(h+11). - Gradients are directly calculated based on interpolated prediction signal using a single 5-tap filter ([2, -9, 0, 9, 2]) | JVET-K0119 Chao-Hsiung Hung (Qualcomm) |

# Related Non-CE Contributions

- JVET-K0041, S. Sethuraman, J. Raj, S. Kotecha (Ittiam)

  - Decoder Side MV Refinement/Derivation with CTB-level concurrency and other normative complexity reduction techniques

  - Crosscheck:

- JVET-K0093, N. Park, J. Nam, H. Jang, J. Lee, J. Lim, S. Kim (LGE)

  - CE9-Related : Restricted template matching schemes to mitigate pipeline delay

  - Crosscheck:

- JVET-K0105, H. Liu, L.Zhang, K. Zhang, Y. Wang, P. Zhao, D. Hong (Bytedance)

  - CE9-related: Simplification of Decoder Side Motion Vector Derivation

- JVET-K0187, F. Chen, L. Wang (Hikvision)

  - CE9-related: Low latency template based motion vector refinement

- JVET-K0256, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

  - CE9-related: MV prediction modifications for decoder-side MV derivation tools

- JVET-K0275, S. Esenlik, I. Krasnov, Z. Zhao, J. Chen (Huawei)

  - Non-CE9: DMVR without Intermediate Buffers and with Padding

- JVET-K0288, 	M. Xu, X. Li, S. Wenger, S. Liu (Tencent)

  - CE9-related: Memory bandwidth reduction for DMVR

- JVET-K0295, X. Li, M. Xu, S. Liu (Tencent)

  - CE9-related: Constrained Decoder Side Motion Vector Derivation

- JVET-K0347, X. Xiu, Y. He, Y. Ye (InterDigital)

  - CE9-related: Addressing the decoding latency issue for decoder-side motion vector refinement (DMVR)

- JVET-K0360, C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)

  - CE9-related: Bilateral Matching with Constrained Motion Vector Storage

- JVET-K0361, C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm), S. Esenlik, I. Krasnov, Z. Zhao, M. Xiang, H. Yang, J. Chen (Huawei)

  - CE9-related: Harmonization between CE9.2.6 (DMVR with Template-free Bilateral Matching, JVET-K0359) and CE9.2.9 (DMVR with Bilateral Matching, JVET-K0217)

# Recommendations

- Review ALL CE tests’ contributions and cross-checks.

- Make decisions on each part on technology under consideration by reviewing CE results.

- …

# Acknowledgement

- Thanks to all proponents for providing CE s/w in time with proper announcement and explanation about modification in the CE9 reflector.

- Thanks to all volunteers for providing the comprehensive cross-checks.

# References

[1] S. Esenlik, Y.-W. Chen, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation,” JVET-J10029, April 2018.

[2]