| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0030-v1 |
| --- | --- |

| Title: | CE10: Summary report of Core Experiment on combined and multi-hypothesis prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu Martin Winken Xiaoyu Xiu | Email: | cw.hsu@mediatek.com martin.winken@hhi.fraunhofer.de Xiaoyu.Xiu@InterDigital.com |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

A summary of Core Experiment 10 (CE10) on combined and multi-hypothesis prediction is reported. Four sub CEs are created to test different methods of combined predictions, including CE10.1: multi-hypothesis prediction, CE10.2: overlapped block motion compensation, CE10.3: non-rectangular partitions and CE10.4: diffusion filtering of inter- and intra-prediction signals. In CE10.1, one out of 10 tests was withdrawn and in CE10.2, one out of 3 tests was withdrawn. So there are 9, 2, 3 and 6 tests for each sub CE, respectively. All tests are evaluated based on the common test conditions defined in JVET-J1010. All tests and crosscheck results are integrated in this report.

Document list

CE proposals are listed as follows,

| Proposal Document # | Corresponding Tests | Author(s) | Title |
| --- | --- | --- | --- |
| JVET-K0257 | CE10.1.1 CE10.1.2 CE10.1.3 CE10.1.4 | M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10.1: Combined and multi-hypothesis prediction |
| JVET-K0269 | CE10.1.5 CE10.1.6 CE10.1.7 CE10.1.8 | M. Winken, H. Schwarz, D. Marpe, T. Wiegand (HHI) | CE10: Multi-hypothesis inter prediction (Tests 1.5-1.8) |
| JEVT-K0147 | CE10.1.10 | N. Zhang, Y. Lin, Q. Yu, J. Zheng(HiSilicon) | CE10.1.10: Dual Merge Mode |
| JVET-K0345 | CE10.2.2 | X. Xiu, Y. He, Y. Ye (InterDigital) | CE10.2.2: Complexity reduction for over-lapped block motion compensation (OBMC) |
| JVET-K0213 | CE10.2.3 | A. Robert, T. Poirier, F. LeLeannec (Technicolor) | CE10.2: Generalized OBMC (Test 10.2.3) |
| JVET-K0144 | CE10.3.1 CE10.3.2 | R.-L. Liao, C. S. Lim (Panasonic) | CE10: Triangular prediction unit mode (CE10.3.1 and CE10.3.2 |
| JVET-K0146 | CE10.3.3 | M. Bläser, Sauer | CE10: Results on Geometric block partitioning (Test 3.3) |
| JVET-K0323 | CE10.4.1 CE10.4.2 CE10.4.3 CE10.4.4 CE10.4.5 CE10.4.6 | J. Rasch,J. Pfaff, M. Schäfer, A. Henkel, H. Schwarz, M. Siekmann, M. Winken, P. Helle, D. Marpe, T. Wiegand (Fraunhofer HH), | CE 10: Signal Adaptive Diffusion Filters For Video Coding (Test 10.4.1-10.4.5) |

CE-related proposals are listed as follows,

| Proposal Document # | Author(s) | Title |
| --- | --- | --- |
| JVET-K0148 | R.-L. Liao, C. S. Lim (Panasonic) | CE10 related: Combining multi-hypothesis prediction with triangular prediction unit mode |
| JVET-K0258 | C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10-related: OBMC complexity reduction and parallel blending |
| JVET-K0259 | Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10-related: OBMC bandwidth reduction and line buffer reduction |
| JVET-K0270 | Y. Ahn, D. Sim (Digital Insights) | CE10-related: Diagonal motion partitions on top of MTT block structure |

Experimental Conditions

Software

One software code base is used which is the BMS software. The BMS subversion repository can be found at https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS. The final 1.0 release version of the software is used.

Anchors

- Two anchors are used to study the performance of the proposed technologies.

- First anchor: The BMS version 1.0 software is used with ‘bms’ configuration files to produce the BMS anchor. If the proposed technique aims to replace an existing tool in the BMS, that tool is switched off in the test simulation but not in the anchor simulation (which must be same for all tests in all Subtests).

- Second anchor: The BMS version 1.0 software is used with ‘vtm’ configuration files to produce the VTM anchor.

Summary of results

CE10.1: Multi-hypothesis prediction

In CE10.1, the goal is to test prediction to be combined coming from multiple hypotheses, where one hypothesis refers to prediction from inter mode or from intra mode. The tests and corresponding results are summarized as follows,

| # | Proposal | Tester | Supported modes | Hypothesis type | Signaling of hypothesis | # of extra hypothesis | Block constraint in luma samples |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.1.1 | JVET-J0018 | Chih-Wei Hsu (MediaTek) | AMVP uni prediction | inter | merge index | 1 | >= 8x8 |
| CE10.1.2 | JVET-J0018 | Chih-Wei Hsu (MediaTek) | skip | inter | implicitly derived | 1 or 2 |  |
|  |  |  | merge |  |  |  |  |
| CE10.1.3 | JVET-J0018 | Chih-Wei Hsu (MediaTek) | merge | intra | intra mode index | 1 |  |
| CE10.1.4 | JVET-J0018 | Chih-Wei Hsu (MediaTek) | skip | inter | merge index for uni-prediction + implicitly derived + intra mode index | 1 or 2 | >= 8x8 |
|  |  |  | merge | intra |  |  |  |
|  |  |  | AMVP uni prediction |  |  |  |  |
| CE10.1.5 | JVET-J0014 | Martin Winken (HHI) | merge | inter | ref index + MVP index + MVDs + weights | 1 |  |
|  |  |  | AMVP |  |  |  |  |
| CE10.1.6 | JVET-J0014 | Martin Winken (HHI) | merge | inter | ref index + MVP index + MVDs + weights | 1 or 2 |  |
|  |  |  | AMVP |  |  |  |  |
| CE10.1.7 | JVET-J0014 | Martin Winken (HHI) | merge | inter | ref index + MVP index + MVDs + weights | 1 | > 8x8 |
| CE10.1.8 | JVET-J0014 | Martin Winken (HHI) | merge | inter | ref index + MVP index + MVDs + weights | 1 or 2 | > 8x8 |
|  |  |  | AMVP |  |  |  |  |
| CE10.1.9 | JVET-J0053 | withdrawn |  |  |  |  |  |
| CE10.1.10 | JVET-J0059 | Na Chang (HiSilicon) | merge | inter | implicitly derived | 1 |  |

| # | Config. | VTM |  |  |  |  | BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE10.1.1 | RA | -0.26% | -0.26% | -0.20% | 109% | 102% | -0.28% | -0.33% | -0.29% | 105% | 101% |
|  | LB | -0.12% | -0.16% | -0.16% | 112% | 102% | -0.17% | -0.39% | -0.23% | 107% | 102% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.1.2 | RA | -0.76% | -0.75% | -0.71% | 110% | 105% | -0.68% | -0.87% | -0.87% | 105% | 107% |
|  | LB | -0.40% | -0.38% | -0.52% | 115% | 104% | -0.51% | -0.67% | -0.73% | 109% | 102% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.1.3 | RA | -0.68% | -0.68% | -0.51% | 112% | 104% | -0.62% | -0.30% | -0.33% | 104% | 102% |
|  | LB | -0.58% | -1.14% | -1.14% | 113% | 103% | -0.56% | -0.97% | -1.02% | 103% | 102% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.1.4 | RA | -1.29% | -1.45% | -1.29% | 119% | 106% | -1.13% | -1.15% | -1.15% | 107% | 105% |
|  | LB | -0.76% | -1.32% | -1.55% | 126% | 105% | -0.77% | -1.18% | -1.13% | 111% | 101% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.1.5 | RA | -1.63% | -1.07% | -1.07% | 127% | 104% | -0.92% | -0.83% | -0.83% | 107% | 100% |
|  | LB | -2.05% | -0.39% | -0.44% | 145% | 107% | -0.93% | -0.47% | -0.39% | 114% | 99% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.1.6 | RA | -1.91% | -1.24% | -1.19% | 130% | 104% | -1.07% | -0.91% | -0.95% | 108% | 100% |
|  | LB | -2.29% | -0.39% | -0.43% | 147% | 108% | -1.13% | -0.36% | -0.11% | 115% | 99% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.1.7 | RA | -1.55% | -1.07% | -1.02% | 122% | 104% | -0.89% | -0.78% | -0.84% | 106% | 100% |
|  | LB | -2.04% | -0.36% | -0.54% | 135% | 107% | -0.94% | -0.35% | -0.09% | 112% | 98% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.1.8 | RA | -1.84% | -1.29% | -1.21% | 125% | 105% | -1.03% | -0.88% | -0.91% | 107% | 100% |
|  | LB | -2.27% | -0.32% | -0.48% | 138% | 108% | -1.07% | -0.29% | -0.12% | 114% | 100% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.1.10 | RA | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | LB | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 97% |
|  | LP | -3.41% | -2.07% | -1.78% | 110% | 99% | -1.67% | -1.33% | -1.02% | 100% | 95% |

The simulation data have been crosschecked and crosscheck results are summarized as follows,

In this summary table, questions have been answered by the crosschecker, including

Level: 1 stands for simulation only, 2 for code review and simulation, 3 for independent implementation and simulation

Results: P stands for partial results by spot-checking, F for full results

EncT/DecT: M stands for match in general, G for gap observed

| # | Tester | Cross-checker | Level? (1/2/3) | Any significant inconsistencies between the description and the provided software? | Results? (P/F) | Match? (Y/N) | EncT? (M/G) | DecT? (M/G) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.1.1 | Chih-Wei Hsu | Martin Winken | 1 |  | F | Y | M | M |  |
|  | (MediaTek) | (HHI) |  |  |  |  |  |  |  |
| CE10.1.2 | Chih-Wei Hsu | Martin Winken | 1 |  | F | Y | M | M |  |
|  | (MediaTek) | (HHI) |  |  |  |  |  |  |  |
| CE10.1.3 | Chih-Wei Hsu | Martin Winken | 1 |  | F | Y | M | M |  |
|  | (MediaTek) | (HHI) |  |  |  |  |  |  |  |
| CE10.1.4 | Chih-Wei Hsu | Martin Winken | 1 |  | F | Y | M | M |  |
|  | (MediaTek) | (HHI) |  |  |  |  |  |  |  |
| CE10.1.5 | Martin Winken | Sung-Chang Lim | 2 | N | P | Y | M | M | Partial results for Class C, D, and E are confirmed. Full results will be confirmed before the meeting. |
|  | (HHI) | (ETRI) |  |  |  |  |  |  |  |
| CE10.1.6 | Martin Winken | Sung-Chang Lim | 2 | N | P | Y | M | M | Partial results for Class C, D, and E are confirmed. Full results will be confirmed before the meeting. |
|  | (HHI) | (ETRI) |  |  |  |  |  |  |  |
| CE10.1.7 | Martin Winken | Man-Shu Chiang | 2 | N | F | Y | M | M |  |
|  | (HHI) | (MediaTek) |  |  |  |  |  |  |  |
| CE10.1.8 | Martin Winken | Man-Shu Chiang | 2 | N | F | Y | M | M |  |
|  | (HHI) | (MediaTek) |  |  |  |  |  |  |  |
| CE10.1.10 | Na Chang | Weijia Zhu |  |  |  |  |  |  |  |
|  | (HiSilicon) | (SHARP) |  |  |  |  |  |  |  |

CE10.2: Overlapped block motion compensation

In CE10.2, the goal is to test prediction to be combined from using motions of neighboring coding units (CUs). The tests and corresponding results are summarized as follows,

| # | Proposal | Tester | # of blending lines | Blending order | Blending order (sequential/parallel) | Runtime reduction technique |
| --- | --- | --- | --- | --- | --- | --- |
| CE10.2.1 | JVET-J0024 | withdrawn |  |  |  |  |
| CE10.2.2 | JVET-J0015 | Xiu, Xiaoyu (InterDigital) | 2: CU area <64 or 4x4 sub CU 4: otherwise | Phase 1 : T->L (CU boundary) | Sequential | MV merge skip similar MVs |
|  |  |  |  | Phase 2 : T->L->B->R (other sub CU boundaries) |  |  |
| CE10.2.3 | JVET-J0022 | Antoine Robert (Technicolor) | 2: CU width or height < 8 4 for one side or 2 for both sides: otherwise | (T,B)->(L,R) | Sequential |  |
| JEM OBMC | JVET-K0258 |  | 2: CU area <64 or 4x4 sub CU 4: otherwise | T->L->B->R | Sequential | N/A |

| # | Config. | VTM |  |  |  |  | BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE10.2.2 | RA | -0.96% | -1.91% | -1.98% | 107% | 111% | -1.14% | -2.14% | -2.24% | 102% | 108% |
|  | LB | -1.37% | -1.80% | -1.90% | 110% | 112% | -1.80% | -2.92% | -2.60% | 105% | 115% |
| CE10.2.3 | RA | -1.01% | -2.03% | -2.09% | 114% | 125% | -1.24% | -2.22% | -2.37% | 107% | 131% |
|  | LB | -1.40% | -1.93% | -2.07% | 117% | 120% | -1.94% | -2.98% | -2.77% | 109% | 136% |
| JEM OBMC | RA | -1.03% | -2.03% | -2.08% | 113% | 123% | -1.22% | -2.18% | -2.28% | 105% | 128% |
|  | LB | -1.36% | -1.80% | -1.90% | 116% | 124% | -1.87% | -3.03% | -2.73% | 108% | 136% |

The simulation data have been crosschecked and crosscheck results are summarized as follows,

| # | Tester | Cross-checker | Level? (1/2/3) | Any significant inconsistencies between the description and the provided software? | Results? (P/F) | Match? (Y/N) | EncT? (M/G) | DecT? (M/G) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.2.2 | Xiu, Xiaoyu | ChunChia Chen | 2 | No | F | Y | M | M | Pros Of proposed algorithm : effective runtime reduction |
|  | (InterDigital) | (MediaTek) |  |  |  |  |  |  |  |
| CE10.2.3 | Antoine Robert | ChunChia Chen | 2 | No | F | Y | M | M | Pros of proposed algorithm : unified blending lines |
|  | (Technicolor) | (MediaTek) |  |  |  |  |  |  |  |

CE10.3: Non-rectangular partitions

In CE10.3, the goal is to test prediction to be combined from non-rectangular prediction partitions within one CU. The tests and corresponding results are summarized as follows,

| # | Proposal | Tester | Supported modes | Prediction type | Partitioning | Block constraint in luma samples | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.3.1 | JVET-J0020 | Ru-Ling Liao (Panasonic) | skip | inter | Diagonal and inverse diagonal triangular |  |  |
|  |  |  | merge |  |  |  |  |
| CE10.3.2 | JVET-J0020 | Ru-Ling Liao (Panasonic) | skip | inter | Diagonal and inverse diagonal triangular | >= 8x8 |  |
|  |  |  | merge |  |  |  |  |
| CE10.3.3 | JVET-J0023 | Max Blaeser (RWTH Aachen University) | all | inter | wedge shaped |  | Encoder / Decoder Complexity Measurements Still Undergoing Estimate based on VTM-1.0 / BMS-1.0 references that were provided |
|  |  |  |  | intra |  |  |  |

| # | Config. | VTM |  |  |  |  | BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE10.3.1 | RA | -1.00% | -1.52% | -1.53% | 119% | 103% | -0.96% | -1.34% | -1.44% | 109% | 99% |
|  | LB | -1.70% | -2.03% | -2.21% | 125% | 103% | -1.44% | -2.12% | -1.84% | 113% | 100% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.3.2 | RA | -0.95% | -1.44% | -1.43% | 118% | 103% | -0.91% | -1.25% | -1.33% | 108% | 100% |
|  | LB | -1.63% | -1.94% | -1.89% | 122% | 102% | -1.41% | -1.74% | -1.63% | 112% | 101% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.3.3 | RA | -0.80% | -1.32% | -1.19% | 426% | 196% | -0.73% | -1.17% | -1.08% | 242% | 164% |
|  | LB | -0.27% | -0.40% | -0.46% | 787% | 210% | -0.31% | -0.53% | -0.43% | 346% | 176% |
|  | LP |  |  |  |  |  |  |  |  |  |  |

The simulation data have been crosschecked and crosscheck results are summarized as follows,

| # | Tester | Cross-checker | Level? (1/2/3) | Any significant inconsistencies between the description and the provided software? | Results? (P/F) | Match? (Y/N) | EncT? (M/G) | DecT? (M/G) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.3.1 | Ru-Ling Liao | Yinji Piao | 2 | N | F | Y | M | M |  |
|  | (Panasonic) | (Samsung) |  |  |  |  |  |  |  |
| CE10.3.2 | Ru-Ling Liao | Yinji Piao | 2 | N | F | Y | M | M |  |
|  | (Panasonic) | (Samsung) |  |  |  |  |  |  |  |
| CE10.3.3 | Max Blaeser | JingYe |  |  |  |  |  |  |  |
|  | (RWTH Aachen University) | (Tencent) |  |  |  |  |  |  |  |

CE10.4: Diffusion filtering of inter- and intra-prediction signals

In CE10.4, the goal is to test prediction to be combined using filtering, where two types of diffusion filters (uniform and signal dependent) with two iteration parameters are included. The tests and corresponding results are summarized as follows,

| # | Proposal | Tester | Description |
| --- | --- | --- | --- |
| CE10.4.1 | JVET-J0038 | Jennifer Rasch (HHI) | • Fast Encoder Decisions and restrictions for All Intra |
| CE10.4.2 | JVET-J0038 | Jennifer Rasch (HHI) | • Fast Encoder Decisions and Restrictions • Merging diffusion parameters |
| CE10.4.3 | JVET-J0038 | Jennifer Rasch (HHI) | • Fast Encoder Decisions and restrictions • Additionally sending diffusion parameters in merge case |
| CE10.4.4 | JVET-J0038 | Jennifer Rasch (HHI) | • More Extensive Search • Additionally sending diffusion parameters in merge case |
| CE10.4.5 | JVET-J0038 | Jennifer Rasch (HHI) | • Fast Encoder Decisions and restrictions • Additionally sending diffusion parameters in merge case • No neighboring block samples used |
| CE10.4.6 | JVET-J0038 | Jennifer Rasch (HHI) | • Fast Encoder Decisions and restrictions • Additionally sending diffusion parameters in merge case • Restrict application of diffusion filter in inter mode |

| # | Config. | VTM |  |  |  |  | BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE10.4.1 | AI | -0.92% | -1.15% | -1.19% | 157% | 116% | -0.74% | -1.63% | -1.56% | 140% | 104% |
| CE10.4.2 | RA | -1.13% | -1.50% | -1.49% | 112% | 109% | -0.67% | -1.33% | -1.31% | 120% | 98% |
|  | LB | -0.58% | 0.01% | -0.15% | 113% | 110% | -0.37% | -0.16% | -0.18% | 116% | 98% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.4.3 | RA | -1.26% | -1.67% | -1.69% | 126% | 107% | -0.79% | -1.46% | -1.51% | 130% | 98% |
|  | LB | -0.67% | 0.54% | 0.23% | 138% | 109% | -0.51% | -0.09% | 0.14% | 136% | 99% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.4.4 | AI | -1.03% | -1.29% | -1.31% | 200% | 118% | -0.75% | -1.68% | -1.64% | 162% | 104% |
|  | RA | -1.41% | -1.89% | -1.87% | 142% | 109% | -0.83% | -1.62% | -1.62% | 140% | 99% |
|  | LB | -0.69% | -0.08% | 0.17% | 148% | 109% | -0.50% | 0.23% | 0.07% | 141% | 99% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.4.5 | RA | -1.13% | -1.43% | -1.48% | 126% | 106% | -0.67% | -1.47% | -1.50% | 130% | 96% |
|  | LB | -0.48% | 0.22% | 0.13% | 136% | 107% | -0.30% | -0.08% | 0.01% | 137% | 97% |
|  | LP |  |  |  |  |  |  |  |  |  |  |
| CE10.4.6 | RA | n/a | n/a | n/a | n/a | n/a | -0.72% | -1.47% | -1.42% | 127% | 98% |
|  | LB |  |  |  |  |  | -0.53% | -0.07% | 0.09% | 130% | 100% |
|  | LP |  |  |  |  |  |  |  |  |  |  |

The simulation data have been crosschecked and crosscheck results are summarized as follows,

| # | Test | Tester | Cross-checker | Level? (1/2/3) | Any significant inconsistencies between the description and the provided software? | Results? (P/F) | Match? (Y/N) | EncT? (M/G) | DecT? (M/G) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.4.1 | JVET-J0038 | Jennifer Rasch | Jacob Ström |  |  |  |  |  |  |  |
|  |  | (HHI) | (Ericsson) |  |  |  |  |  |  |  |
| CE10.4.2 | JVET-J0038 | Jennifer Rasch | Jacob Ström |  |  |  |  |  |  |  |
|  |  | (HHI) | (Ericsson) |  |  |  |  |  |  |  |
| CE10.4.3 | JVET-J0038 | Jennifer Rasch | Jacob Ström |  |  |  |  |  |  |  |
|  |  | (HHI) | (Ericsson) |  |  |  |  |  |  |  |
| CE10.4.4 | JVET-J0038 | Jennifer Rasch | Jacob Ström |  |  |  |  |  |  |  |
|  |  | (HHI) | (Ericsson) |  |  |  |  |  |  |  |
| CE10.4.5 | JVET-J0038 | Jennifer Rasch | Jacob Ström |  |  |  |  |  |  |  |
|  |  | (HHI) | (Ericsson) |  |  |  |  |  |  |  |
| CE10.4.6 | JVET-J0038 | Jennifer Rasch | Jacob Ström |  |  |  |  |  |  |  |
|  |  | (HHI) | (Ericsson) |  |  |  |  |  |  |  |

Reference