| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0031-v1 |
| --- | --- |

| Title: | CE11: Summary report on composite reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng, Guichun Li, Yue Li | Tel: Email: | xiaozhen.zheng@dji.com guichunli@tencent.com lytt@mail.ustc.edu.cn |
| Source: | CE11 coordinators |  |  |

_____________________________

- Abstract

This contribution is a summary report of Core Experiment 11 on composite reference picture. Three tests categories and 11 subtests were agreed to carry out in CE11 in between JVET-J and JVET-K meeting cycle, to study and evaluate technologies related to composite reference picture.

Follow by common test condition recommend from J meeting, BMS1.0 and BMS1.0 with VTM configurations are used to evaluate CE11 technologies. Test conditions are specified for each category. The corresponding coding performance of each coding tool under evaluated in CE11 are summarized in this contribution. To further evaluate CE11 tools, crosschecking reports are also integrated in this contribution.

Introduction

As is agreed in San Diego meeting, the following tests in table 1 are performed in CE11.

Table 1: tests that are evaluated and studied in CE11

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE11.1.1 | No limitation on block update refresh rate at J0011 | Xiaozhen Zheng (DJI) | Wenhao Zhang (Hulu) |
| CE11.1.2 | 1/2 block update refresh rate at J0011 | Xiaozhen Zheng (DJI) | G. Li (Tencent) |
| CE11.1.3 | 1/8 block update refresh rate at J0011 | Xiaozhen Zheng (DJI) | G. Li (Tencent) |
| CE11.1.4 | 1/10 block update refresh rate at J0011 | Xiaozhen Zheng (DJI) | Yue Li (USTC) |
| CE11.1.5 | No limitation on block update refresh rate at J0011 | Yue Li (USTC) | G. Li (Tencent) |
| CE11.1.6 | 1/2 block update refresh rate at J0032 | Yue Li (USTC) | G. Li (Tencent) |
| CE11.1.7 | 1/8 block update refresh rate at J0032 | Yue Li (USTC) | G. Li (Tencent) |
| CE11.1.8 | 1/10 block update refresh rate at J0032 | Yue Li (USTC) | Xiaozhen Zheng (DJI) |
| CE11.2.1 | Intra period as two seconds at J0011 | Xiaozhen Zheng (DJI) | G. Li (Tencent) |
| CE11.2.2 | Intra period as five seconds at J0011 | Xiaozhen Zheng (DJI) | Yue Li (USTC) |
| CE11.2.3 | Intra period as two seconds at J0032 | Yue Li (USTC) | G. Li (Tencent) |
| CE11.2.4 | Intra period as five seconds at J0032 | Yue Li (USTC) | Xiaozhen Zheng (DJI) |
| CE11.3.1 | HEVC encoder only long-term reference mechanism | Xiaozhen Zheng (DJI) / Yue Li (USTC) | Wei-Jung Chien (Qualcomm) |

The detailed description of each tools is provided in the next section.

Description of CE11 tools

JVET-K0156: Composite reference picture [1]

Usually, background areas have few motions in a long temporal window. Therefore, blocks with minor difference between the background and the current frame are picked up, and are used to replace the co-located blocks in a long-term reference. The proposed update method targets at the renewal of the background information. An indication flag is signaled at CTU level to indicate whether current CTU is used to update the long-term reference at decoder side. After a picture is decoded and reconstructed completely, the process of updating the long-term reference will be performed. For every CTU marked to update the long-term reference, its Luma and Chroma reconstructed pixels will be used to replace the co-located pixels in the long-term reference, as illustrated in Fig. 1.

Fig. 1: Illustration of block-level long-term reference update process

In the decoding process, if the long-term reference is used as reference, motion vector scaling and decoder motion refine operation that use motion trajectory are invalid because the distance between the long-term reference and the current slice is not available and motion trajectory model doesn’t work for long-term reference. Therefore, the tools like BIO、DMVR、FRUC are set to disable if any of the motion vector is referred to the long-term reference.

JVET-K0157: HEVC-like encoder only solution for composite reference picture [2]

Different to CE11 test 1 and test 2 that introduces a new composed virtual reference frame, CE11 test 3 targets at evaluating the use of the HEVC long-term reference mechanism, potentially in combination with signaling of no output coded pictures (pic_output_flag = 0). This combination could theoretically achieve similar functionality as that provided by composite reference pictures, e.g. by synthesizing and signaling a no-output reference picture that only contains background information which is illustrated at Fig. 1.

Fig. 1: Example use of no output reference pictures

As illustrated in Fig. 1, the frames I0, B1, B2, B3 and B4 are coded as short-term reference. During the encoding of those short-term frames, an alternative frame is composed by exploiting previous coded frames’ content. When an encoder determines the alternative frame has been completely constructed, such frame will be coded as long-term reference with pic_output_flag=0. To harmonize current coding tools in VTM and BMS, the tools with motion vector scaling and motion vector refinement at decoder side are modified when they use reference data from such long-term reference.

In order to implement encoder only solution, RPS cfg setting is changed from the setting shown in Fig. 2 to the setting shown in Fig. 3.

Fig2. RPS setting for short-term frames without long-term reference

Fig3. RPS setting for short-term frames with long-term reference

Meanwhile, HEVC syntax elements long_term_ref_pics_present_flag, num_long_term_ref_pics_sps, Output_flag_present_flag, deblocking_filter_control_present_flag, pps_deblocking_filter_disabled_flag, short_term_ref_pic_set_sps_flag, deltaRPS, ref_idcs, inter_ref_pic_set_prediction_flag, delta_rps, used_by_curr_pic_flag[j], use_delta_flag[j], num_long_term_pics, used_by_curr_pic_lt_flag, num_ref_idx_active_override_flag and num_ref_idx_l0_active_minus1are exploited and modified.

JVET-K0370: Block-composed background reference (BCBR) [3]

In the decoder side, if BCBR_enable_flag is true, a long-term reference picture, which is also the synthesized background reference picture, is appended into the reference picture list, i.e the number of reference picture list is increased by 1. The reference index for the added reference picture is equal to the number of reference picture minus 1, and all the motion compensation processes remain unchanged. The background reference picture is initialized using the reconstructed I frame. Then, a reconstructed CTU will be substitute the collocated one in the background reference picture if its background_flag_ctu is true.

In the encoder side, the flowchart is shown as follows, which contains three parts : background block selection, coding parameter decision, and background reference updating.  Temporal and spatial correlation constrains are used to select the background block. The coding parameter for the background CTU is decided according the following equation:

QPB=QP0-round3.5×logSl-cl-8.5

where QPB is the coding parameter of the decided background CTU, QP0 is the coding parameter of I picture, Sl is the sequence length for LDB and LDP configurations, the interval between adjacent I pictures for RA configurations, cl is the number of the encoded pictures after encoding one I picture. After encoding current picture, the background reference picture in the location of the selected CTUs will be updated with the reconstructed ones. A flag signalling the background CTU is transmitted to the decoder. Besides, the coding parameter of the background CTU is transmitted to the decoder through the DQP technology.

Fig. 3: Illustration of the proposed BCBR technology

CE results summary

To reduce software implementation efforts on both VTM and BMS platforms, it is agreed that BMS version BMS-1.0 shall be used for these experiments, as specified in the corresponding output document from the last meeting [4].

- The proposed methods in this CE shall be tested with VTM configurations (without changing the JEM_TOOLS Macro).

- The proposed methods in this CE should be tested with BMS configurations.

The mandatory common test conditions (CTC) as specified in the JVET-J1010 document from this meeting [5] shall be followed to perform the simulations in this CE. The mandatory test sequences as described in common test conditions [5] shall be used for these tests.

There are three test categories to be evaluated in CE11. Test 1 targets at the evaluation of the performance when CTU-level refresh rate is set to unlimited, 1/2, 1/8 and 1/10 per frame. Since composite reference might have higher coding performance when composite reference is replaced with IDR at a longer GOP, test 2 targets at the test of larger random access GOP whose IDR period is set to two seconds and five seconds. The best block update refresh rate conducted at test 1 is used as the default refresh rate at test 2. Test 3 explores the coding efficiency by using HEVC long-term reference mechanism.

The followings are a summary table of the tests in this CE.

Table 1: CE11 test results against VTM/BMS anchor (lowdelay B main10)

Table 2: CE11 test results against VTM/BMS anchor (random access main10)

The crosscheck results and crosscheckers’ comments are summarized below:

Comments from Alexandros Tourapis (Apple) on CE11.3.1 test :

- Encoder runtime depends on how to design mode decision and search method. Long-term reference is more utilized (maybe) with small search windows around zero.

- Encoder can create a more intelligent mode decision that gives some priority to such references and how these are updated and how also the partitioning and how other modes are considered depending on the resulting performance of these references.

Related contributions

CE11 related contribution are summarised at Table 4.

Table 4: CE11 related contributions

| Title | Contribution # |
| --- | --- |
| CE11-related: Hardware friendly composite reference picture solution | JVET-K0159 |
| Crosscheck for CE11-1.1 | JVET-K0439 |
| CE11 Related Work: Long-term Reference Simulated Implementation | JVET-K0447 |

References

- W. Li and X. Zheng, “CE11: Results on composite reference picture (test 11.1.1, 11.1.2, 11.1.3, 11.1.4, 11.2.1 and 11.2.2)”, JVET-K0156, 11th Meeting: Ljubljana, SI, 10–18 July 2018

- W. Li and X. Zheng, “CE11: HEVC-like encoder only solution for composite reference picture”, JVET-K0157, 11th Meeting: Ljubljana, SI, 10–18 July 2018

- C. Ma, D. Liu, Y. Liand, F. Wu, “CE11: Block-composed Background Reference (BCBR)”, JVET-K0370, 11th Meeting: Ljubljana, SI, 10–18 July 2018

- X. Zheng, G. Li and Y.Li, “Description of Core Experiment 11 (CE11): Composite reference pictures”, JVET-J1031, 10th JVET meeting, Apr. 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin, "JVET common test conditions and software reference configurations", JVET-J1010, 10th JVET meeting, Apr. 2018.