| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0032 |
| --- | --- |

| Title: | CE12: Summary report on mapping for HDR content |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Francois, D. Rusanovskyy, P. Yin | Email: | edouard.francois@technicolor.com dmytror@qti.qualcomm.com pyin@dolby.com |
| Source: | CE12 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 12 on mapping for HDR content. CE12 aims at evaluating approaches for mapping of HDR content. The considered technologies are adaptive quantization, out-of-loop reshaping and in-loop reshaping. Test results against VTM anchor are provided for each performed test. Crosschecking reports are integrated in this contribution.

# Introduction - summary of proposed technologies

The purpose of this CE was to investigate the performance of HDR coding tools based on content mapping (or reshaping). The considered technologies are adaptive quantization, used as anchor, out-of-loop reshaping and in-loop reshaping, with different variants.

The technologies and performance for out-of-loop approach are described in JVET-K0298. The technologies and performance for in-loop approach are described in JVET-K0308.

## Out-of-loop approach (JVET-K0298)

The following table summarizes the out-of-loop related tools considered in CE13.1 to 13.4 and described in JVET-0298.

| Tool | Acronym | Description |
| --- | --- | --- |
| Dynamic range adaptation | DRA | consists in applying a luma mapping and luma-based chroma scaling to the input video luma and chroma components, as a pre-encoding process, and to apply to inverse process as a post-decoding process |
| Harmonization of DRA chroma scale and QP | qpHarmoniz | consists in adjusting the luma-based chroma scaling to preserve a luma/chroma bit allocation ensured by the luma-based chroma QP offset derivation in the core HEVC design (cf Table 8-10 of HEVC specification) |
| DRA refinement | Refint_Flt / Refint_Fix | consists in applying a correction to the inverse DRA tables applying at the decoder to compensate the distortions resulting from the compression. The process may apply per slice. A floating-point was first delivered. A fixed-point version was further delivered. |

## In-loop approach (JVET-K0308)

The following table summarizes the in-loop related tools considered in CE13.5 to 13.8 and described in JVET-0308.

| Tool | Acronym | Description |
| --- | --- | --- |
| In-loop Luma reshaper | Reshap | maps an input luma code value to an altered value inside the coding loop. For intra slice, reshaping is explicitly performed to the input signal. Inverse reshaping applies after loop filtering and before DPB buffering. For an inter slice, only inverse reshaping applies to the prediction signal before loop filtering. |
| luma-based Chroma QP Offset | chrQpOffset | Adds a luma-dependent QP offset to chroma signal, to compensate the Luma signal interaction with the Chroma signal |
| luma-based Chroma scaling | chrScaling | luma-based Chroma QP Offset is alternatively implemented by scaling the chroma residue signal in each TU |
| Loop Filter application to original domain | LF intraOrig or LF interOrig | Intra: LF is applied in the original domain, after applying the inverse reshaping Inter: LF is applied in the original domain. |
| Loop Filter application to reshaped domain | LF intraResh or LF interResh | Intra: LF is applied in the reshaped domain, before applying the inverse reshaping Inter: LF is applied in the reshaped domain. A pair of forward and inverse reshaping is needed before and after LF. |
| Region of Interest | ROI | Adaptively turns on/off the reshaping of a CTU based on the average luma value in the CTU |

# Test results

## All intra

The table below summarizes the different results for the AI configuration, for the different considered tools combinations.

|  |  | AI - Over VTM1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | DE100 | PSNRL | wPsnrY | wPsnrU | wPsnrV |
| out-of-loop |  |  |  |  |  |  |
| CE12-1 | DRA + RefintFlt | -18.8% | -0.7% | 1.7% | -11.6% | -41.1% |
| CE12-3.1 | DRA + RefintFlt + qpHarmoniz | -11.7% | -3.1% | -0.8% | -11.3% | -28.8% |
| CE12-3.2 | DRA + RefintFix + qpHarmoniz | -11.7% | -3.1% | -0.8% | -11.4% | -28.8% |
| CE12-3.3 | DRA + qpHarmoniz | -8.0% | -3.1% | -0.7% | -0.2% | -14.7% |
| in-loop |  |  |  |  |  |  |
| CE12-5 | Reshap | 13.7% | -2.8% | -2.1% | 12.3% | 13.0% |
| CE12-6.1 | Reshap + chrQpOffset | -1.5% | -2.8% | -2.0% | -1.9% | -4.8% |
| CE12-6.2 | Reshap + chrScaling | 0.5% | -2.8% | -2.1% | 2.0% | 2.6% |
| CE12-7.1 | Reshap + LF_intraOrig_interOrig | -1.4% | -2.1% | -1.5% | -1.9% | -4.7% |
| CE12-7.2 | Reshap + LF_intraResh_interOrig | -1.5% | -2.8% | -2.0% | -1.9% | -4.8% |
| CE12-7.3 | Reshap + LF_intraResh_interResh |  |  |  |  |  |
| CE12-8 | Reshap + ROI |  |  |  |  |  |

Note: CE12-6.1, CE12-7.2 and CE12-7.3 are identical in AI configuration.

## Random access

The table below summarizes the different results for the RA configuration, for the different considered tools combinations.

|  |  | RA - Over VTM1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | DE100 | PSNRL | wPsnrY | wPsnrU | wPsnrV |
| out-of-loop |  |  |  |  |  |  |
| CE12-1 | DRA + RefintFlt | 10.6% | -6.2% | -4.0% | 35.1% | 212.0% |
| CE12-3.1 | DRA + RefintFlt + qpHarmoniz | 10.3% | -6.6% | -4.4% | 34.5% | 68.5% |
| CE12-3.2 | DRA + RefintFix + qpHarmoniz | 10.4% | -6.6% | -4.4% | 34.5% | 68.4% |
| CE12-3.3 | DRA + qpHarmoniz | 15.6% | -6.5% | -4.3% | 70.5% | 118.1% |
| in-loop |  |  |  |  |  |  |
| CE12-5 | Reshap | 11.0% | -2.6% | -2.0% | 12.0% | 10.3% |
| CE12-6.1 | Reshap + chrQpOffset | -1.3% | -2.6% | -2.0% | 0.0% | -0.1% |
| CE12-6.2 | Reshap + chrScaling | 2.0% | -2.6% | -2.0% | 5.9% | 8.9% |
| CE12-7.1 | Reshap + LF_intraOrig_interOrig | -1.1% | -1.7% | -1.3% | 0.1% | 0.0% |
| CE12-7.2 | Reshap + LF_intraResh_interOrig | -1.4% | -2.4% | -1.8% | -0.1% | -0.1% |
| CE12-7.3 | Reshap + LF_intraResh_interResh | -1.3% | -2.6% | -2.0% | 0.0% | -0.1% |
| CE12-8 | Reshap + ROI | -1.2% | -2.2% | -1.7% | 0.1% | -0.1% |

Note: CE12-6.1 and CE12-7.3 are identical in RA configuration.

# Crosscheck report

## Cross-check of CE12-1 and CE12-3.3

Proponent: Qualcomm/Technicolor

Cross-checker: Dolby

Objective metrics

The reported objective numbers in CE12-1 and 12-3.3 are confirmed by the cross-checker.

Encoder/Decoder complexity was not reported by JVET-K0298 for CE12-1. For CE12-3.3, decoding time for RA was reported. Decoding time was measured by the cross-checker on the same platform (stand-alone PC) used to measure timing for CE12-5, CE12-6, CE12-7 and CE12-8. For CE12-1, the decoding complexity vs. Anchor is 110% for AI, 116% for RA. For CE12-3.3, the decoding complexity is 108% for AI, 117% for RA. The decoding time for RA is not aligned with what Technicolor reported (101%). Note that the general issue of runtime variations issue is discussed in JVET-K0410.

Code verification

The code was also checked. The cross-checker confirms that the implementation is aligned with the tool description in JVET-K0298. For the Luma DRA mapping curve, the derivation is not that straightforward to conclude if the mapping curve matched Anchor dQP setting. And it is found to be different from CE12-5 curve which is straightforwardly derived from Anchor dQP. It is recommended to test exact same Luma mapping to evaluate the in-loop and out-loop architecture, potentially in future CE. As for chroma DRA curve, anchor uses slice adaptive chroma QP value while for chroma DRA, same curve is used for all slices. The in-loop reshaper can be used for slice adaptation. The idea of luma-based chroma QP derivation using HEVC chromaQP table seems more precise than chromaAdj=2 (chroma residue scaling) in CE12-6.2. The cross-checker recommends to test the HEVC chromaQP table based chroma residue scaling method potentially in future CE for in-loop reshaping.

The CE12-1 and CE12-3.3 tests show very different characteristics under AI and RA. For AI, chroma gain and luma loss is observed; while for RA, chroma loss and luma gain is observed. It is not clear what causes such inconsistency.

Visual tests

The visual test is done for solution 12-3.3 mainly for lower rate: R1 and R2. Visual improvement can be observed for the following clips:

- Market: texture on the wall is better than Anchor.

- Starting: the rope is the sky is better than Anchor.

- Hurdles: track texture is slightly better than Anchor.

## Cross-check of CE12-3.2

Proponent: Qualcomm/Technicolor

Cross-checker: Hisilicon

Objective metrics

Not reported yet

Code verification

Not reported yet

Visual tests

Not reported yet

## Cross-check of CE12-3.3

Proponent: Qualcomm/Technicolor

Cross-checker: NHK

Objective metrics

Not reported yet

Code verification

Not reported yet

Visual tests

Not reported yet

## Cross-check of 12-5

Proponent: Dolby

Cross-checker: Qualcomm

Objective metrics

Not reported yet

Code verification

Not reported yet

Visual tests

Not reported yet

## Cross-check of 12-6

Proponent: Dolby

Cross-checker: Sharp (cf  report in JVET-K0392)

Objective metrics

Test for CE12.6.1 and CE12.6.2 were performed following HDR test condition, VTM config, and tested for PQ content only.

During test, it is found that macro J0015_INLOOP_RESHAPER_SIMD generated different results on different platform, Therefore, the cross-checker is suggested to turn that macro off during cross-check. With J0015_INLOOP_RESHAPER_SIMD set to 0, the cross-checker was able to generate same objective result as those provided by Dolby.

Cross-checker tests for CE12.6.1 and CE12.6.2 were done on a heterogenous cluster, so the timing info is not very reliable.

Code verification

Not checked

Visual tests

Not checked

## Cross-check of CE12-7

Proponent: Dolby

Cross-checker: Technicolor

Objective metrics

The reported objective numbers in JVET-K0308 are confirmed by the cross-checker.

Runtimes were evaluated only for RA case of CE12-7.3 (same as 12-6.1, which is the one recommended by the proponent). Encoding runtime is estimated at 104% of the anchor runtime. Decoding runtime is estimated at 112% of the anchor runtime. It is observed that this is not aligned with the runtime metrics reported by the proponent (same issue as discussed in section 3.1).

Code verification

The code was also checked. The cross-checker confirms that the implementation is aligned with the tool description in JVET-K0308. It was checked that the reshaping function is directly derived from the dQP table used in the anchor (the slope of the reshaping function is based on the dQP values used in the anchor).

The cross-checker noted that for RA case the exact same slice-level chroma QP adaptation is used in the anchors and in CE12-5 to 12-8. Anchor slice-level chroma QP adaptation relies on the slice QP value and follows the QP cascading (namely QP increase for B frames) utilized in RA configuration. The cross-checker estimates that this chroma QP adaptation may lead to “over improve” the quality of the chroma in the anchor configuration, and that a different luma/chroma quality balance could be investigated.

Visual tests

Visual checks were done for solution CE12-7.3 (same as 12-6.1, which is the one recommended by the proponent). Tested rates were R2 and R3, for the 5 sequences used in RA fixed bitrate configuration defined in HDR CTCs. Visual improvements were observed on Market (sharper texture on the wall on the right) and Starting (cable in the sky better preserved). For Hurdles R3, very tiny improvements were observed in the ground texture. No significant difference in quality was observed for the other cases.

## Cross-check of CE12-8

Proponent: Dolby

Cross-checker: Technicolor

Objective metrics

The reported objective numbers in JVET-K0308 are confirmed by the cross-checker.

Code verification

Due to lack of time, the implementation was not verified.

Visual tests

No visual test was done.

The cross-checker suggests that visual tests could be done during the Ljubljana meeting if adequate HDR viewing equipment is available to assess the visual benefits of the ROI-feature. Otherwise the cross-checker suggests to possibly reconduct the region-based feature study in Core Experiment.

# CE12 and CE12-related documents

| JVET | MPEG | Title | Source |
| --- | --- | --- | --- |
| JVET-K0032 | m43229 | CE12: Summary report on HDR coding | E. Francois, D. Rusanovskyy, P. Yin |
| JVET-K0298 | m43231 | CE12: report of dynamic range adaptation (DRA) and DRA refinement | E. Francois (Technicolor), D. Rusanovskyy (Qualcomm) |
| JVET-K0308 | m43241 | CE12: HDR In-loop Reshaping (CE12-5, 12-6, 12-7 and 12-8) | T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen(Dolby) |
| JVET-K0309 | m43242 | In-loop Reshaping for SDR Video | F. Pu, T. Lu, P. Yin, W. Husak, S. McCarthy, T. Chen(Dolby) |
| JVET-K0392 | m43340 | Cross-check for CE12.6.1 and CE12.6.2 | J. Zhao, K. Misra (Sharp) |

# Conclusions

Results from CE12 tend to indicate that in-loop or out-of-loop mapping perform better than the anchor, both objectively and subjectively. Reported objective results also tend to show that out-of-loop approach performs better than in-loop for luma objective metrics, while the inverse is observed for chroma objective metrics. Some cross-checkers commented that the chroma quality is possibly over-improved in the anchor and in CE12-5 to 8. Further study is recommended to investigate the problem of luma/chroma rate allocation and balance and to better differentiate pros and cons of out-of-loop and in-loop mapping.

It is therefore recommended to reconduct the CE on mapping for HDR content, to further investigate in-loop or out-of-loop mapping.