| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0033-v1 |
| --- | --- |

| Title: | CE13: Summary report on projection formats |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Philippe Hanhart Jian-Liang Lin | Email: | Philippe.Hanhart@InterDigital.com jl.lin@mediatek.com |
| Source: | CE13 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 13 on projection formats. Seven projection formats are investigated in CE13, 1) padded equi-rectangular projection (PERP), 2) cubemap projection (CMP), 3) equi-angular cubemap projection (EAC), 4) hybrid angular cubemap projection (HAC), 5) modified cubemap projection (MCP), 6) parallel-to-axis uniform cubemap projection (PAU), and 7) rotated sphere projection (RSP). Test results against PERP, CMP, and EAC are provided to show the coding efficiency of each projection format. Test results are provided for both the VTM and BMS configurations. BD-rates are computed based on the end-to-end WS-PSNR metric. Crosschecking results are integrated in this contribution.

# Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.1.1 | PERP | - |
| 13.2.1 | CMP w/o padding | JVET-K0328 (InterDigital) |
| 13.2.2 | CMP w/ padding |  |
| 13.3.1 | EAC w/o padding | JVET-K0329 (InterDigital) |
| 13.3.2 | EAC w/ padding |  |
| 13.4.1.a | HAC (8 parameters) w/o padding | JVET-K0330 JVET-K0331 (InterDigital) |
| 13.4.1.b | HAC (3 parameters) w/o padding |  |
| 13.4.2 | HAC (8 parameters) w/ padding |  |
| 13.4.3 | EAC + adaptive frame packing |  |
| 13.4.4 | HAC (3 parameters) + adaptive frame packing |  |
| 13.5.1 | MCP w/o padding | JVET-K0131 (MediaTek) |
| 13.5.2 | MCP w/ padding |  |
| 13.7.1 | PAU w/o padding | JVET- K0182 (Zhejiang University) |
| 13.7.2 | PAU w/ padding |  |
| 13.8.1 | RSP | JVET-K0387 (Samsung) |
| 13.8.2 | RSP + rotation |  |
| 13.8.3 | RSP + rotation + in-active region filling + blending |  |
| K0332.a | CMP + adaptive frame packing | CE-related JVET-K0332 (InterDigital) |
| K0332.b | MCP + adaptive frame packing |  |
| K0332.c | PAU + adaptive frame packing |  |

# Test results

## Compared to PERP

|  | VTM |  |  |  |  | BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 13.1.1 | - | - | - | - | - | - | - | - | - | - |
| 13.2.1 | -3.49% | -1.17% | -1.21% | 106% | 102% | -3.39% | 0.08% | -1.01% | 106% | 100% |
| 13.2.2 | -2.86% | -0.94% | -1.11% | 107% | 107% | -2.73% | 0.03% | -0.80% | 106% | 101% |
| 13.3.1 | -11.53% | -6.81% | -7.09% | 106% | 103% | -11.24% | -5.92% | -6.70% | 106% | 102% |
| 13.3.2 | -11.06% | -6.90% | -7.13% | 107% | 104% | -10.78% | -6.31% | -6.87% | 106% | 101% |
| 13.4.1.a | -11.73% | -7.04% | -7.38% | 121% | 103% | -11.52% | -6.20% | -7.02% | 110% | 102% |
| 13.4.1.b | -11.67% | -6.95% | -7.28% | 123% | 104% | -11.46% | -6.10% | -7.01% | 110% | 102% |
| 13.4.2 | -11.26% | -7.04% | -7.33% | 122% | 104% | -11.03% | -6.55% | -7.17% | 110% | 102% |
| 13.4.3 | -11.84% | -7.88% | -8.20% | 107% | 106% | -11.57% | -7.17% | -7.75% | 106% | 102% |
| 13.4.4 | -11.98% | -8.06% | -8.43% | 123% | 105% | -11.77% | -7.44% | -8.07% | 110% | 103% |
| 13.5.1 | -11.73% | -7.42% | -7.72% | 105% | 102% | -11.80% | -6.77% | -7.68% | 106% | 100% |
| 13.5.2 | -11.56% | -7.58% | -8.01% | 105% | 104% | -11.64% | -7.42% | -8.19% | 105% | 101% |
| 13.7.1 | -11.65% | -6.92% | -7.26% | - | - | -11.50% | -6.14% | -7.12% | - | - |
| 13.7.2 | -11.58% | -7.15% | -7.56% | - | - | -11.41% | -6.84% | -7.55% | - | - |
| 13.8.1 | -10.37% | -6.56% | -6.92% | - | - | -10.71% | -6.83% | -7.46% | - | - |
| 13.8.2 | -11.66% | -7.94% | -8.99% | - | - | -11.49% | -7.87% | -8.94% | - | - |
| 13.8.3 | -11.10% | -7.43% | -8.60% | - | - | -10.95% | -7.43% | -8.39% | - | - |
| K0332.a | -3.81% | -2.16% | -2.39% | 101% | 108% | -3.71% | -1.12% | -1.96% | 96% | 101% |
| K0332.b | -12.01% | -8.41% | -8.83% | 105% | 105% | -12.06% | -7.95% | -8.76% | 105% | 102% |
| K0332.c | -11.83% | -7.82% | -8.13% | 107% | 108% | -11.65% | -7.21% | -7.97% | 106% | 102% |

## Compared to CMP

|  | VTM |  |  |  |  | BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 13.1.1 | 4.11% | 1.67% | 1.83% | 94% | 98% | 3.84% | 0.23% | 1.53% | 94% | 100% |
| 13.2.1 | - | - | - | - | - | - | - | - | - | - |
| 13.2.2 | 0.67% | 0.24% | 0.09% | 100% | 104% | 0.71% | -0.04% | 0.21% | 100% | 100% |
| 13.3.1 | -8.56% | -5.69% | -5.94% | 100% | 100% | -8.42% | -6.03% | -5.72% | 100% | 101% |
| 13.3.2 | -8.05% | -5.77% | -5.99% | 100% | 102% | -7.92% | -6.41% | -5.90% | 100% | 101% |
| 13.4.1.a | -8.75% | -5.96% | -6.26% | 114% | 101% | -8.70% | -6.34% | -6.07% | 104% | 102% |
| 13.4.1.b | -8.67% | -5.85% | -6.15% | 116% | 102% | -8.64% | -6.24% | -6.05% | 104% | 102% |
| 13.4.2 | -8.25% | -5.94% | -6.22% | 114% | 101% | -8.18% | -6.69% | -6.23% | 104% | 101% |
| 13.4.3 | -8.86% | -6.74% | -7.04% | 101% | 104% | -8.74% | -7.26% | -6.76% | 100% | 102% |
| 13.4.4 | -8.98% | -6.94% | -7.28% | 116% | 102% | -8.94% | -7.57% | -7.10% | 104% | 102% |
| 13.5.1 | -8.77% | -6.31% | -6.58% | 99% | 99% | -8.99% | -6.89% | -6.72% | 99% | 100% |
| 13.5.2 | -8.58% | -6.47% | -6.87% | 98% | 101% | -8.81% | -7.53% | -7.24% | 99% | 100% |
| 13.7.1 | -8.68% | -5.80% | -6.11% | - | - | -8.69% | -6.24% | -6.15% | - | - |
| 13.7.2 | -8.59% | -6.02% | -6.41% | - | - | -8.58% | -6.94% | -6.58% | - | - |
| 13.8.1 | -7.32% | -5.43% | -5.77% | - | - | -7.81% | -6.94% | -6.48% | - | - |
| 13.8.2 | -8.63% | -6.81% | -7.69% | - | - | -8.61% | -7.99% | -7.85% | - | - |
| 13.8.3 | -8.05% | -6.29% | -7.28% | - | - | -8.05% | -7.55% | -7.28% | - | - |
| K0332.a | -0.31% | -0.97% | -1.17% | 98% | 105% | -0.32% | -1.19% | -0.94% | 91% | 101% |
| K0332.b | -9.04% | -7.29% | -7.68% | 102% | 103% | -9.26% | -8.07% | -7.79% | 100% | 102% |
| K0332.c | -8.85% | -6.68% | -6.98% | 103% | 106% | -8.83% | -7.30% | -6.99% | 100% | 102% |

## Compared to EAC

|  | VTM |  |  |  |  | BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 13.1.1 | 13.68% | 8.09% | 8.49% | 94% | 97% | 13.13% | 6.90% | 7.90% | 94% | 98% |
| 13.2.1 | 9.40% | 6.15% | 6.43% | 100% | 100% | 9.24% | 6.51% | 6.18% | 100% | 99% |
| 13.2.2 | 10.10% | 6.41% | 6.53% | 100% | 104% | 9.98% | 6.47% | 6.39% | 100% | 99% |
| 13.3.1 | - | - | - | - | - | - | - | - | - | - |
| 13.3.2 | 0.58% | -0.08% | -0.05% | 100% | 101% | 0.58% | -0.41% | -0.19% | 100% | 100% |
| 13.4.1.a | -0.24% | -0.30% | -0.35% | 114% | 100% | -0.33% | -0.36% | -0.40% | 104% | 100% |
| 13.4.1.b | -0.15% | -0.19% | -0.24% | 116% | 101% | -0.25% | -0.25% | -0.37% | 104% | 101% |
| 13.4.2 | 0.34% | -0.28% | -0.32% | 115% | 101% | 0.27% | -0.73% | -0.58% | 104% | 100% |
| 13.4.3 | -0.31% | -1.09% | -1.14% | 101% | 103% | -0.32% | -1.28% | -1.06% | 100% | 100% |
| 13.4.4 | -0.46% | -1.32% | -1.41% | 116% | 102% | -0.56% | -1.63% | -1.45% | 104% | 101% |
| 13.5.1 | -0.25% | -0.65% | -0.69% | - | - | -0.63% | -0.92% | -1.06% | - | - |
| 13.5.2 | -0.03% | -0.83% | -1.00% | - | - | -0.41% | -1.61% | -1.61% | - | - |
| 13.7.1 | -0.15% | -0.12% | -0.18% | - | - | -0.30% | -0.23% | -0.46% | - | - |
| 13.7.2 | -0.03% | -0.35% | -0.50% | - | - | -0.16% | -0.97% | -0.91% | - | - |
| 13.8.1 | 1.39% | 0.32% | 0.21% | - | - | 0.74% | -0.94% | -0.79% | - | - |
| 13.8.2 | 0.00% | -1.11% | -1.79% | - | - | -0.12% | -2.03% | -2.21% | - | - |
| 13.8.3 | 0.63% | -0.56% | -1.35% | - | - | 0.49% | -1.56% | -1.60% | - | - |
| K0332.a | 9.06% | 5.16% | 5.23% | 96% | 105% | 8.90% | 5.29% | 5.24% | 93% | 99% |
| K0332.b | -0.53% | -1.66% | -1.82% | 99% | 102% | -0.90% | -2.14% | -2.17% | 102% | 101% |
| K0332.c | -0.32% | -1.02% | -1.07% | 101% | 105% | -0.44% | -1.33% | -1.31% | 102% | 101% |

# Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.1.1 |  |  |  |  |  | CTC anchor |
| 13.2.1 |  |  |  |  |  | CTC anchor |
| 13.2.2 | - |  |  |  |  |  |
| 13.3.1 | - |  |  |  |  |  |
| 13.3.2 | - |  |  |  |  |  |
| 13.4.1.a | B. Wang | 2 | F | Y | M |  |
| 13.4.1.a | C. Pujara | 1 | F | Y | - |  |
| 13.4.1.b | B. Wang | 2 | F | Y | M |  |
| 13.4.1.b | C. Pujara | 1 | F | Y | - |  |
| 13.4.2 | B. Wang | 2 | F | Y | M |  |
| 13.4.2 | C. Pujara | 1 | F | Y | - |  |
| 13.4.3 | C. Pujara | 1 | F | Y | - |  |
| 13.4.4 | C. Pujara | 1 | F | Y | - | We observed the time taken for deriving the adjustment parameters for HAC. Time taken to run pGeoDst->determineParam(this, iDstPOC) is up to 800 seconds for 3p+AFP for Skateboardinlot and Balboa sequences and similar time is observed for 8p+AFP case. These timings are only for the first frame processing. Similar time might be required for each RAP if done at RAP level. Though for Broadway, Landing2 sequences these timings is around 2 to 3 seconds. |
| 13.5.1 | P. Hanhart | 2 | F | Y | M |  |
| 13.5.2 | P. Hanhart | 2 | F | Y | M |  |
| 13.7.1 | C. Pujara | 1 | F | Y | - |  |
| 13.7.2 | C. Pujara | 1 | F | Y | - |  |
| 13.8.1 | Y. Sun | 1 | F | Y | - |  |
| 13.8.2 |  |  |  |  |  |  |
| 13.8.3 |  |  |  |  |  |  |
| K0332.a |  |  |  |  |  |  |
| K0332.b |  |  |  |  |  |  |
| K0332.c |  |  |  |  |  |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

# Brief description of tests

## CE13.1

Padded equi-rectangular projection (PERP). 360° CTC anchor.

## CE13.2 in JVET-K0328 (InterDigital)

Cubemap projection (CMP). 360° CTC anchor.

When padding is used, guard bands of 4 luma samples are added around each group of continuous faces. No blending is used.

## CE13.3 in JVET-K0329 (InterDigital)

Equi-angular cubemap projection (EAC).

When padding is used, guard bands of 4 luma samples are added around each group of continuous faces. No blending is used.

## CE13.4 in JVET-K0330/JVET-K0331 (InterDigital)

### Hybrid angular cubemap projection (HAC)

HAC is a generalization of the EAC projection format. The horizontal and vertical sampling functions can be adjusted for each face adaptively based on the content. Equations (1) (2) are the forward mapping functions from CMP to HAC for the horizontal and vertical directions, respectively.

| x'=fxx=4π∙atan-1x∙tanπ4∙a | (1) |
| --- | --- |
| y'=fyy=4π∙btan-1y∙tanπ4∙b | (2) |

where a and b are the HAC horizontal and vertical mapping function parameters, and are defined within the range of [0.3, 1.5]. The sampling distribution of HAC (that is, parameters a and b) for each face can be adaptively selected based on the content. As the parameter a or b gets closer to 0, the sampling distribution in that corresponding direction becomes closer to that of CMP.

The HAC parameters are constrained to keep sampling continuities at face boundaries in between two neighboring faces. There are two configurations

- For fixed frame packing through the whole sequence, resulting in 8 parameters to be determined

- For adaptive frame packing arrangement, resulting in 3 parameters to be determined

The HAC parameters are determined at the encoder side for the first frame of the sequence and signaled in the SPS.

When padding is used, guard bands of 4 luma samples are added around each group of continuous faces. No blending is used.

### Adaptive frame packing (AFP)

AFP periodically modifies the location of discontinuous face boundaries in the frame packed pictures to improve coding efficiency and reduce the visibility of face seam artifacts. AFP considers four frame packing configurations corresponding to yaw rotations of 0°, 90°, 180°, and 270°. Among the four combinations, the cost of each configuration is calculated based on gradient classification. For the first picture, the best combination is used. For the subsequent IRAP pictures, AFP alternates between the different configurations based on the cost of each combination computed for the given IRAP picture.

The frame packing parameters are determined at the encoder side at every IRAP and signaled in the PPS.

## CE13.5 in JVET-K0131(MediaTek)

MCP is based on the cubemap projection format adopting the compact 3x2 layout with a discontinuous edge in the middle. MCP introduces a coordinate mapping function to improve the sampling distribution compared to the conventional cubemap projection (CMP). For the Front, Back, Right, and Left faces, MCP applies the equal-angular mapping function on u-axis while on v-axis, the proposed mapping function is applied as follows:

| 2D-to-3D: | fv=v1+0.4(1-u2)(1-v2) | (3) |
| --- | --- | --- |
| 3D-to-2D: | gv=v , if t=01-1-4t(v-t)2t, otherwise , t=0.4v(gu2-1) | (4) |

For the Top and Bottom faces, the equal-angular mapping function is applied on both u-axis and v-axis to keep continuity with other faces as shown in Figure 1.

Figure 1. Mapping function on u-axis and v-axis for each face of MCP

To reduce the seam artifact caused by the discontinuous edge and frame boundary in the cubemap layout, padding is generated by the geometry mapping and applied on the boundary of the two 3x1 face rows. Each face with padding region is then shrunk by a scaling factor to make the frame size remained the same. Before converting back to ERP or generating viewports, blending is applied with a distance-based weighting function.

## CE13.7 in JVET-K0182 (Zhejiang University)

The PAU has the same face definition as CMP. Assuming (u’, v’) is defined in a CMP face, and (u, v) is the corresponding point in a PAU face. Each face is defined in the range of [-1, 1]. The equations (5)(6) and (7) are the forward adjustment function from CMP to PAU. Vertical direction is first adjusted by (5). For the horizontal direction, sampling adjustments of u are related to vertical sampling v, which is expressed in (6) and (7).

| v=4.0π*tan-1v' | (5) |
| --- | --- |
| α=tan-1v' | (6) |
| u=tan-1u'*cosαtan-1cosα | (7) |

For the backward adjustment function, the equations (8)(9) and (10) are used to map PAU to CMP.

| v'=tanvπ4.0 | (8) |
| --- | --- |
| β=tan-1v' | (9) |
| u'=1cos⁡(β)*tanu*tan-1cosβ | (10) |

Padding: To avoid obvious seam artifact near the discontinuous face boundary, padding is performed on the vertical face boundary. Padding width is 3 per face and the value of pixel are copied from the nearest pixel in vertical direction inside each face. The content of each face is scaled on an area of (face width) * (face height-3). So white area in each face is scaled to [-1, 1] in vertical direction.

Blending: No blending is used.

## CE13.8 in JVET-K0387 (Samsung)

Rotated sphere projection (RSP) in 3x2 packing arrangement. Refer to JVET-H1004 [1] for further details. In addition the following algorithms have been implemented:

### Rotation

It has been observed that rotation helps in improving the coding gain for 360° video. Rotation of the content before projection is performed for reducing the discontinuities in the complex region and aligning the edges in the content more suitably for the conventional coding scheme as well.

To arrive at the optimal rotation angle without encoding the video we devised the following algorithm.

- Downsize the image to 2K size and generate 36 RSP images at angles multiple of 10 degrees starting from 10 to 360 degrees. (only for X axis rotation)

- Compute gradient magnitudes and angles for each of these 36 images

- Find the image with maximum number of vertical and horizontal gradients from these images. Here we consider gradients between -5 to 5 degrees as horizontal and -85 to 95 degrees as vertical.

### In-active region filling

In RSP, we observe in-active regions near the ends of lobes that need to be encoded. It was observed that it would be better to fill the in-active regions with content similar to neighboring active region rather than just grey color. This can be also referred to as color filling. As shown in Figure 1, we try to fill up the non-active regions 1, 2 by using the whole or part of neighboring highlighted content. We can compute either mean/median/mode using the neighboring active pixels to fill up the non-active region. Setting all neighboring inactive pixels with this value improves prediction of active pixels on the boundary. Figure 2 shows one frame of Balboa sequence with inactive region filled as explained above.

Figure 2. Example RSP frame with inactive pixels set using colored filling

### Blending on the seam boundaries

In blending, boundary content is updated with weighted average of itself and equivalent content inside padded region. In RSP, circular boundary of the lobes is filled by using spherically neighboring content and those pixels are used for blending. On top of spherical padding, we add some extra padding for aligning boundary/seams with CU.

For each pixel inside the colored region of RSP content, we find its corresponding pixel values in spherical padding region using bilinear interpolation. Denote Pold as pixel intensity value inside RSP content before blending, Ppad as pixel intensity mapped inside spherical padding, Pnew as pixel intensity value inside RSP content after blending, we compute Pnew by the following

| Pnew= Pold*d+ Ppad*(D-d) D | (11) |
| --- | --- |

where d refers to distance of the pixel from boundary while D is number of pixels to be modified after blending within same column. We have observed significant improvement in subjective quality but loss in BD bitrate gain after introducing padding and blending for the discontinuous regions.

# Related contributions

## CE13-related: Adaptive frame packing on top of CMP, MCP, and PAU (JVET-K0332)

This contribution reports simulation results for adaptive frame packing (AFP, JVET-K0331) on top of CMP, modified cubemap projection (MCP, JVET-K0131), and parallel-to-axis uniform cubemap projection (PAU, JVET-K0182). Results based on end-to-end WS-PSNR show that AFP provides luma BD-rate savings of about 0.3%, 0.3%, and 0.15% on top of CMP, MCP, and PAU, respectively. There is no significant encoding or decoding complexity increase due to AFP.