| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0041-v1 |
| --- | --- |

| Title: | Decoder Side MV Refinement/Derivation with CTB-level concurrency and other normative complexity reduction techniques |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal [relates to CE9] |  |  |
| Author(s) or Contact(s): | S. Sethuraman, J. Raj A., S. Kotecha Consulate-1, #1 Richmond Road Bangalore-560025 India | Tel: Email: | +91-80-66601000 sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_______`______________________

# Abstract

This contribution presents a method for determining the availability of the refined motion vectors (MVs) of spatially neighboring coding units for use in AMVP process or as a starting point for DMVR/PMMVD process of a current coding unit. The proposed method enables concurrent reference data pre-fetch to be possible for all CUs within a CTB during one stage of a processing pipeline followed by concurrent decoder-side MV refinements, if required, and motion compensated prediction in a following stage of the pipeline. A configurable lag between consecutive CTB rows is proposed to allow more top CTB-row refined MVs to be available. When compared to considering all refined MVs within the current access unit to be unavailable, the proposed method offers BDRATE gains of up to 0.6% . In addition, two normative complexity reduction methods are proposed. The first is a modified search pattern based integer distance refinement procedure that attempts to strike a balance between the reduction of unconditional cost evaluations and the number of dependent stages during refinement  In the second method, a parametric error surface based sub-pixel displacement estimation procedure approach is proposed to reduce internal memory and computational requirements. VTM tool ON results are provided (and BMS tool OFF results will be provided in an updated version of this contribution).

# Introduction

The DMVR and PMMVD techniques in JEM [JVET-G1001] consider the refinement MVs of all coding units earlier in the processing order as available for use as an AMVP candidate or as a starting point for DMVR/PMMVD for a current coding unit. This brings about a strict serial dependency in the DMVR/PMMVD processes across coding units and affects the degree of parallelism in hardware and software implementations. An easy way to bring back a high degree of parallelism is by considering all refinement MVs earlier in the processing order to be not available for AMVP or DMVR/PMMVD processes. However, a significant part of the coding gains seen earlier are lost with such a change. This is because the lack of availability of the refinement MVs increase the MVD coding bits of INTER coded CUs and serve as inferior starting candidates during DMVR/PMMVD processes. Hence, there is a need for selectively making certain refinement MVs to be available while still offering a satisfactory degree of parallelism.

DMVR and PMMVD methods employ multiple iterations of refinement of a cost function around a starting search center. Given that the starting search center is a sub-pixel accurate center, one or more iterations of 1-pixel distance pattern searches such as square (3x3), plus-shaped, or cross-shaped searches with respect to this center is commonly employed. Hence, interpolation is required to derive these sample values from the reference frame’s integer pixel samples. The rhombus shaped search ranges under-utilize the interpolation investments (or take more iterations to reach diagonal displacements), while the square searches have more unconditional evaluations per iteration. The conditional evaluations increase the number of worsts-case dependent stages during refinement and hence impact the time available to perform each stage. Hence, there is a need for a method that keeps the number of dependent stages low, while leveraging the interpolated samples well and while limiting the unconditional evaluations at each stage to a lower number than the square search.

Sub-pixel accurate refinement motion vectors help in reducing the prediction residuals and help in averaging out the quantization noise in the reference samples through bi-prediction. The PMMVD techniques in JEM performed their refinement steps at a sub-pixel granularity, while multiple DMVR proposals during the last meeting [JVET-J0018, JVET-J0032] had extended DMVR to include sub-pixel accurate refinement stages to obtain higher coding gains. Since the center for refinement search itself is a sub-pixel accurate position, positions other than half-pixel distance refinement positions have different phases and require independent interpolations. Even the half-pixel distance positions fall on three different phases in the integer pixel grid. In spite of using bilinear interpolation, multiple interpolations either reduce parallelism or increase internal memory needs, in addition to having a higher computational complexity (when compared to only performing integer distance refinements). Hence, there is a need for a method that removes these drawbacks while providing most of the BDRATE gains possible with sub-pixel accurate refinements.

# Proposed algorithms

## A hardware implementation friendly search pattern

In this method, the first iteration performs evaluations on a plus-pattern (i.e. center position and its left, top, right, and bottom 1-pixel neighbour positions). If more than one iteration is allowed, each subsequent iteration evaluates at up to 5 additional positions around the least cost position of the preceding iteration in such a way that the cost function evaluations at all 8 1-pixel neighbour positions (same as in square search pattern) are available by the end of the current iteration. This can be realized by keeping track of the 1-pixel neighbour positions already evaluated around a given center in preceding iterations. Such evaluation ensures that the interpolated samples are fully utilized and that extreme diagonal movement lags the square-pattern search by only one iteration. The unconditional investment of 9 evaluations in the first iteration by the square pattern is reduced to 5, while nearly maintaining the same number of dependent stages as the square pattern search. Though this method results in a higher average number of cost evaluations than the greedy conditional evaluation methods, the time available for a given stage is higher and the number of worst-case parallel execution units for cost evaluation remains the same.

## A parametric error surface based sub-pixel offset determination

The proposed method attempts to leverage compute investments made during integer distance refinement iterations. When the integer-distance refinement early exits the iterative loop due to the center position having the least cost when compared to its other 1-pixel neighbors considered during a particular iteration, a parametric error surface is fitted using the cost function values of the positions considered. For example, given the sum of absolute difference (SAD) values evaluated at the center and its left, top, right and bottom neighbour 1-pixel positions that are available during the exiting iteration, a 5-parameter 2-D parabolic error surface can be fitted as described in CHENG_DU–CSVT-June-2003 [3]. The minimum position of this parabolic error surface is determined by solving a set of simultaneous equations. It is also possible to use more evaluated error values and obtain a least squares solution. To keep the computational complexity simple, only a simple 5-parameter model is used.

In a minor variant of this implementation, after exiting from the integer distance iterations, another iteration is performed using the averaged bilateral template as used in the DMVR currently present in BMS. The error values are independently evaluated against this averaged bilateral template for L0 and L1. Independent error surfaces are fitted using these two sets of error values to get independent sub-pixel offsets with the respect to the best integer-distance refinement positions in L0 and L1. Though this requires additional template and cost function evaluations even after early exit, a BDRATE gain in the range of 0.1 - 0.5% is observed in VTM tool ON cases with this.

Since no explicit half-pixel distance or higher precision refinement happens, the compute and memory requirements for the different interpolation planes is eliminated. Also, in one of the variants, no additional cost function evaluations are incurred.

## Concurrency across coding units within a CTB through refined MV availability rules

The proposed method aims to allow:

- concurrent data-prefetch of required reference samples for refinement and/or motion compensation of all coding units within a CTB that can occupy an entire pipeline stage; and,

- concurrent refinement and/or motion compensation of all coding units within a CTB that follows the data-prefetch stage in the pipeline and can occupy an entire pipeline stage.

To facilitate this, the following rules are applied to determine the availability of a spatial neighbor’s refined MV:

- The refined MVs of all coding units within the current CTB are assumed to be unavailable

- The refined MVs of neighbour coding units in the CTB to the left of the current CTB are assumed to be unavailable

- The refined MVs of neighbour coding units in the CTB to the top of the current CTB are considered available if those refined MVs are available before the start of the reference data pre-fetch stage of the current CTB, based on a configurable lag in the number of CTBs by which the data pre-fetch stage of the current CTB lags the data pre-fetch stage of its top neighbour CTB.

A lag value of zero is equivalent to all neighbor refined MVs being unavailable for AMVP or DMVR/PMMVD processes. A lag value of 2 is equivalent to the top-right dependency seen in intra prediction in HEVC/VVC and allows refined MVs of neighbour coding units falling within the top and top-left neighbour CTBs to be available. A lag value of 3 makes refined MVs of neighbour coding units falling within top-right CTB neighbour also to be available. The increased availabilities at non-zero lags improve the coding gains when compared to a lag value of zero. The impact is expected to be higher when the max CTB sizes are lower (e.g. 64 or 32, as compared to the default 128). When a CTB is force partitioned using a quad-tree split, the lags can be computed also along the z-scan order of the quad-tree partitions to determine availabilities.

# Test Results

The test results are provided in a progressive manner with the proposed algorithms incrementally applied. All simulations have been run with AVX2 SIMD intrinsics available for interpolation and SAD calculation on Linux with gcc-v4 compiler.

The first set of results (Tables 3.1 to 3.3) correspond to performing bilateral matching with equal and opposite delta motion vectors in L0 and L1 using the proposed hardware-friendly pattern search. All refined MVs of coding units preceding the current coding unit in encoding order are assumed to be available in these results. DCTIF based normative interpolation filters are used for obtaining the values at non-integer grid positions.

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.64% | -1.98% | -2.23% | 113% |  | NA | NA | NA | NA | NA |
| Class A2 | -4.63% | -4.55% | -4.63% | 113% |  | NA | NA | NA | NA | NA |
| Class B | -2.20% | -2.05% | -2.10% | 114% |  | NA | NA | NA | NA | NA |
| Class C | -1.33% | -1.63% | -1.69% | 112% |  | NA | NA | NA | NA | NA |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.54% | -2.43% | -2.53% | 113% |  | NA | NA | NA | NA | NA |
| Class D | -1.40% | -1.65% | -1.86% | 113% |  | NA | NA | NA | NA | NA |
| Class F (optional) | NA | NA | NA | NA |  | NA | NA | NA | NA | NA |

Table -3.1 Random Access Main 10, 1 iteration

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.72% | -3.10% | -3.46% | 114% |  | NA | NA | NA | NA | NA |
| Class A2 | -6.26% | -6.12% | -6.03% | 112% |  | NA | NA | NA | NA | NA |
| Class B | -2.76% | -2.77% | -2.82% | 114% |  | NA | NA | NA | NA | NA |
| Class C | -1.86% | -2.36% | -2.43% | 112% |  | NA | NA | NA | NA | NA |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.41% | -3.39% | -3.49% | 113% |  | NA | NA | NA | NA | NA |
| Class D | -1.83% | -2.17% | -2.39% | 113% |  | NA | NA | NA | NA | NA |
| Class F (optional) | NA | NA | NA | NA |  | NA | NA | NA | NA | NA |

Table -3.2 Random Access Main 10, 2 iterations

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.98% | -3.52% | -3.97% | 114% |  | NA | NA | NA | NA | NA |
| Class A2 | -6.72% | -6.52% | -6.39% | 113% |  | NA | NA | NA | NA | NA |
| Class B | -2.83% | -2.98% | -3.00% | 115% |  | NA | NA | NA | NA | NA |
| Class C | -2.07% | -2.57% | -2.67% | 113% |  | NA | NA | NA | NA | NA |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.64% | -3.69% | -3.78% | 114% |  | NA | NA | NA | NA | NA |
| Class D | -1.97% | -2.36% | -2.47% | 114% |  | NA | NA | NA | NA | NA |
| Class F (optional) | NA | NA | NA | NA |  | NA | NA | NA | NA | NA |

Table -3.3 Random Access Main 10, 4 iterations

It can be seen that the BDRATE gains increase with increasing iteration count, with the higher resolution sequences gaining more with increased iterations than the lower resolutions.

It should be noted that this method does not attempt to keep the average number of SAD evaluations low, but aims to reduce the number of dependent stages while capping the max SAD evaluations per iteration to 5. Hence, it may not be proper to measure the effectiveness of this method through the average decT numbers.

The second set of test results (Tables 3-4 to 3-6) report the cumulative effect of the proposed hardware-friendly pattern search and the 5-parameter error surface based sub-pixel offset determination method that employs the additional iteration of evaluations using the averaged bilateral template with independent sub-pixel offsets being determined for L0 and L1. The results for 1, 2, and 3 iterations of integer distance refinements using bilateral matching with and equal and opposite delta motion vectors in L0 and L1, followed by 1 additional iteration with averaged bilateral template based evaluations are presented below.

DCTIF based normative interpolation filter is used for obtaining the values at non integer-grid positions. Also, low MV precision is used in these simulations.

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.92% | -2.99% | -3.42% | 115% |  | NA | NA | NA | NA | NA |
| Class A2 | -7.10% | -6.45% | -6.48% | 114% |  | NA | NA | NA | NA | NA |
| Class B | -3.30% | -2.97% | -3.10% | 116% |  | NA | NA | NA | NA | NA |
| Class C | -2.20% | -2.50% | -2.63% | 115% |  | NA | NA | NA | NA | NA |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.89% | -3.55% | -3.71% | 115% |  | NA | NA | NA | NA | NA |
| Class D | -2.34% | -2.53% | -2.88% | 117% |  | NA | NA | NA | NA | NA |
| Class F (optional) | NA | NA | NA | NA |  | NA | NA | NA | NA | NA |

Table -3.4 Random Access Main 10, 2 iterations including Sub-Pixel error surface

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.39% | -3.54% | -4.06% | 115% |  | NA | NA | NA | NA | NA |
| Class A2 | -7.91% | -7.21% | -7.19% | 115% |  | NA | NA | NA | NA | NA |
| Class B | -3.52% | -3.36% | -3.41% | 117% |  | NA | NA | NA | NA | NA |
| Class C | -2.48% | -2.95% | -2.97% | 116% |  | NA | NA | NA | NA | NA |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -4.29% | -4.06% | -4.18% | 116% |  | NA | NA | NA | NA | NA |
| Class D | -2.58% | -2.82% | -3.09% | 118% |  | NA | NA | NA | NA | NA |
| Class F (optional) | NA | NA | NA | NA |  | NA | NA | NA | NA | NA |

Table -3.5 Random Access Main 10, 3 iterations including Sub-Pixel error surface

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.49% | -3.71% | -4.25% | 115% |  | NA | NA | NA | NA | NA |
| Class A2 | -8.10% | -7.33% | -7.28% | 115% |  | NA | NA | NA | NA | NA |
| Class B | -3.56% | -3.45% | -3.51% | 117% |  | NA | NA | NA | NA | NA |
| Class C | -2.58% | -3.01% | -3.16% | 116% |  | NA | NA | NA | NA | NA |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -4.39% | -4.16% | -4.32% | 116% |  | NA | NA | NA | NA | NA |
| Class D | -2.62% | -2.84% | -3.18% | 118% |  | NA | NA | NA | NA | NA |
| Class F (optional) | NA | NA | NA | NA |  | NA | NA | NA | NA | NA |

Table -3.6 Random Access Main 10, 4 iterations including Sub-Pixel error surface

All refined MVs of coding units preceding the current coding unit in encoding order are assumed to be available in these results.

It can be seen that the error surface based sub-pixel offset determination provides a significant BDRATE gain on top of the results with only the new proposed pattern search. The DecT numbers increase by ~10% over the corresponding results with only the new proposed pattern search.

In a subsequent revision of the document, the results for the variant without the additional cost function evaluations after early exit will be presented. Also, results with bilinear interpolation and mean-removed SAD are planned.

The third set of test results (Tables 3-7 and 3-8) report the cumulative effect of simulating lags of 0 and 2 according to the proposed refinement MV availability rules on top of the first two methods.

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.64% | -2.31% | -2.62% | 116% |  | NA | NA | NA | NA | NA |
| Class A2 | -5.00% | -4.73% | -4.67% | 118% |  | NA | NA | NA | NA | NA |
| Class B | -2.21% | -2.34% | -2.33% | 119% |  | NA | NA | NA | NA | NA |
| Class C | -1.80% | -2.22% | -2.17% | 118% |  | NA | NA | NA | NA | NA |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.74% | -2.78% | -2.81% | 118% |  | NA | NA | NA | NA | NA |
| Class D | -1.92% | -2.10% | -2.21% | 119% |  | NA | NA | NA | NA | NA |
| Class F (optional) | NA | NA | NA | NA |  | NA | NA | NA | NA | NA |

Table -3.7 Random Access Main 10, 4 iterations including Sub-Pixel error surface with lag=0 CTBs

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.13% | -2.68% | -2.99% | 117% |  | NA | NA | NA | NA | NA |
| Class A2 | -5.64% | -5.18% | -5.13% | 118% |  | NA | NA | NA | NA | NA |
| Class B | -2.47% | -2.52% | -2.51% | 120% |  | NA | NA | NA | NA | NA |
| Class C | -1.87% | -2.24% | -2.25% | 118% |  | NA | NA | NA | NA | NA |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.08% | -3.01% | -3.06% | 118% |  | NA | NA | NA | NA | NA |
| Class D | -1.95% | -2.26% | -2.37% | 120% |  | NA | NA | NA | NA | NA |
| Class F (optional) | NA | NA | NA | NA |  | NA | NA | NA | NA | NA |

Table -3.8 Random Access Main 10, 4 iterations including Sub-Pixel error surface with lag=2 CTBs

It can be seen that the BDRATE gains fall significantly with lag=0 when compared to allowing all refinement MVs from preceding coding units in encode order to be available. With lag=2, some of the drops in BDRATE are recovered. UHD sequences gain the most.

# Summary

In this contribution, firstly, a hardware friendly search pattern is also proposed that provides a good trade-off between positions evaluated in one iteration and the number of dependent stages. The results indicate that BDRATE gains start to plateau around 4 iterations with the proposed search pattern. Secondly, a parametric error surface based sub-pixel offset determination method is proposed during DMVR/PMMVD to reduce the computational complexity and reduce the internal memory requirements. The sub-pixel offsets provide a significant BDRATE gain over the results of the first set. Thirdly, a method for determining the availability of refined MVs of neighbor coding units is proposed to enable concurrent reference data pre-fetch and processing of refinement/inter-prediction in a regular CTB processing pipeline. It is shown that lag=0 has a significant BDRATE drop when compared to considering all refined MVs of preceding CUs to be available always. The proposed method with lag=2 partially recovers some of these BDRATE drops.

# Reference

- Algorithm Description of Joint Exploration Test Model 7 (JEM 7), JVET-G1001, Gwang Ju, Jan. 2018.

- Cheng Du, Yun He, and Junli Zheng, “PPHPS: A Parabolic Prediction-Based, Fast Half-Pixel Search Algorithm for Very Low Bit-Rate Moving-Picture Coding”, IEEE Transaction of circuits and systems for video technology, Vol. 13, No. 6, June 2003.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).