| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0042-v1 |
| --- | --- |

| Title: | A study on the overlap in functionality between SAO and ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Study [relates to CE2] |  |  |
| Author(s) or Contact(s): | S. Sethuraman, Nijil K. Consulate-1, #1 Richmond Road Bangalore-560025 India | Tel: Email: | +91-80-66601000 sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_______`______________________

# Abstract

This contribution provides a study report of disabling SAO in the presence of ALF to understand the coding loss, if any. The motivation is to understand the overlap in functionality between these two in-loop filtering stages with the aim to see whether SAO can be disabled when ALF is enabled so as to reduce the number of cascading in-loop filtering stages and thus reduce the internal memory needs. Since there have been some enhancements to SAO that have been proposed in CE2 experiments, three of the BDRATE improving enhancements have been included during the study. The study results show that the tool OFF BDRATE drop for this modified SAO averages 0.88% in luma under CTC, while the tool ON gain for SAO in the absence of ALF averages -2.0% in luma. The chroma BDRATE drops are higher (at ~3%). Since chroma ALF has only a single class, SAO seems to provide higher improvements in chroma than in luma. In informal visual evaluations that compared ALF-only against ALF+SAO based streams, the latter was seen to remove certain motion trail artifacts (due to the CTB level signaling), implying that SAO still provides perceivable visual quality improvements and additional efforts at improving ALF may be required before SAO can be dropped in the cascade of in-loop filtering stages.  Also, other objective quality metrics need to be tried to see which one correlates better with the subjective visual quality in order to reduce the subjectivity in conclusions going forward.

# Introduction

Sample adaptive offsets (SAO) has been a tool from HEVC that offers low decoder-side computational complexity and provides good objective and subjective coding gains. It provides an edge offset mode and a band offset mode where a set of offsets are signaled for each CTB. The edge offset mode helps in reduce ringing artifacts while the band offset mode helps in reducing banding or contouring artifacts. During the last meeting, some proposals (such as [JVET-J0022, JVET-J0018, and JVET-J0024]) have extended it through the concepts such SAO palette, additional edge offset categories, and the use of a threshold during the category determination to provide additional BDRATE gains.

Adaptive loop filter (ALF) performs gradient and spatial activity based classification at 2x2 level in BMS and obtains a set of filter coefficients for each class by solving the Wiener-Hopf equations on the encoding side that uses the original and degraded samples in each class. Significant BDRATE gains (when PSNR is used as the objective quality metric) averaging 5.5% has been reported during JEM development and in recent VTM + Tool ON and BMS with tool OFF studies. The action of the Wiener filter can also reduce the ringing artifacts when ringing is a common artifact present in many 2x2 patches of a given class.

Hence, it is useful to understand the overlap in functionality between ALF and SAO. If ALF can provide almost all the improvements that SAO can provide, then SAO can be removed from the cascade of in-loop filtering stages whenever ALF is enabled. The reduced number of dependent processing stages of in-loop filtering will reduce the internal memory requirements on the decoder side. The intent of this study is to understand the overlap in functionality, if any, between ALF and SAO.

# Details of the study

The BMS code base has been used for the study. To ensure that additional coding gains in newer SAO extensions are factored into the study, the SAO implementation in BMS has been augmented with the following:

- the extended SAO edge offset patterns concept of JVET-0018 with implementation provided as part of CE2.3.1,

- the SAO palette concept of JVET-0022 with implementation provided as part of CE2.3.2, and

- the modified edge offset category classification rules of JVET-0024 with implementation provided as part of CE2.3.4.

The ALF implemented in BMS is used without any modifications in the study.

For all the experiments, BMS-1.0 code was run in VTM mode with all extra tools turned off, except ALF which is turned ON or OFF depending on the experiment. All results are generated for the RA configuration.

# Study Results

Following experiments have been conducted during this study.

## Tool off drop for SAO

In this experiment, tool off BDRATE drop of VTM version of SAO is measured. VTM is used as the reference and VTM with SAO disabled is used as the test case. The experiments were conducted with ALF turned off.

|  | VTM vs VTM-SAO |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | 1.10% | 0.97% | 1.13% |
| Class A2 | 1.86% | 2.60% | 1.65% |
| Class B | 1.72% | 2.58% | 2.47% |
| Class C | 0.60% | 1.63% | 2.48% |
| Class E |  |  |  |
| Overall | 1.33% | 2.01% | 2.04% |
| Class D | 0.12% | 0.96% | 0.72% |
| Class F (optional) | NA | NA | NA |

Table -3.1 Random Access Main 10, SAO tool OFF

Results shows that the SAO tool off BDRATE drop is 1.33% for luma.

## Newer SAO extension gains

In this experiment, the additional coding gains of the 3 newer SAO extensions are measured. VTM is used as the reference and VTM with all the SAO extensions is used as the test case. The experiments were conducted with ALF turned off.

|  | VTM vs VTM+SAO_Ext |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | -0.88% | -1.81% | -2.01% |
| Class A2 | -0.85% | -0.70% | -0.95% |
| Class B | -0.81% | -2.04% | -1.21% |
| Class C | -0.39% | -0.29% | -0.35% |
| Class E |  |  |  |
| Overall | -0.72% | -1.26% | -1.09% |
| Class D | -0.25% | 0.03% | 0.05% |
| Class F (optional) | NA | NA | NA |

Table -3.2 Random Access Main 10, SAO extension gain

Results shows that the extensions provide a BDRATE gain of 0.72% for luma and ~1% for chroma.

## Tool on gain for SAO including newer SAO extensions

In this experiment, tool on coding gains of SAO including the newer SAO extensions is measured. VTM with SAO disabled is used as the reference. VTM with the extended SAO is compared against the reference to see the coding gains of SAO. The experiments were conducted with ALF turned off.

|  | Over VTM-SAO vs VTM+SAO_Ext |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | -1.95% | -2.78% | -3.14% |
| Class A2 | -2.65% | -3.22% | -2.56% |
| Class B | -2.47% | -4.43% | -3.54% |
| Class C | -0.98% | -1.87% | -2.72% |
| Class E |  |  |  |
| Overall | -2.00% | -3.18% | -3.05% |
| Class D | -0.37% | -0.92% | -0.66% |
| Class F (optional) | NA | NA | NA |

Table -3.3 Random Access Main 10, SAO with extension tool ON gain

Results shows that the tool ON BDRATE gain for the SAO with the 3 extensions over VTM is -2.00% for luma and ~3% for chroma.

## Tool off drop for SAO in the presence of ALF

In this experiment, tool off BDRATE drop of extended SAO in the presence of ALF is measured. VTM with the SAO extensions and ALF enabled is used as the reference. VTM with ALF enabled but SAO disabled is used as the test case. Both the experiments were conducted with ALF turned on.

|  | VTM+ALF+SAO_Ext vs VTM+ALF-SAO |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | 0.55% | 1.35% | 2.92% |
| Class A2 | 1.27% | 4.05% | 4.01% |
| Class B | 1.12% | 3.77% | 3.71% |
| Class C | 0.53% | 2.77% | 3.19% |
| Class E |  |  |  |
| Overall | 0.88% | 3.07% | 3.47% |
| Class D | 0.36% | 1.51% | 1.54% |
| Class F (optional) | NA | NA | NA |

Table -3.4 Random Access Main 10, tool OFF SAO with ALF

Results shows that the SAO drop in presence of ALF is 0.88% for luma. This implies that there is a considerable overlap in functionality between SAO and ALF when handling luma. But the gap in chroma BDRATE indicates that ALF’s chroma handling with a single class and 5x5 filter is not able to compete well with the chroma processing in SAO and methods such as in CE2.4.2.3 may be helpful.

## Visual evaluation

Informal visual evaluation of VTM+ALF streams against VTM+ALF-SAO streams were conducted. Many were not able to find major differences between them. Few cases VTM+ALF is picked as visually better against VTM+ALF-SAO. This implies that SAO still provides perceivable visual quality improvements. However, there are also cases where presence of SAO is introducing some visual artifacts.

In the following snapshots, Fig.1 shows a case where motion trail artifact is cleaned up in VTM+ALF, but visible when SAO is disabled. In Fig. 2 shows a case where VTM+ALF is having an artifact which is not present when SAO is disabled. In Fig. 3, one artifact each are prominent in both cases as shown through the circled area.

Figure 1BQTerrace Frame 599, QP = 37 (Left: VTM+ALF, Right: VTM+ALF-SAO)

Figure 2 BQTerrace Frame 326, QP = 37 (Left: VTM+ALF, Right: VTM+ALF-SAO)

Figure 3 BQTerrace Frame 377, QP = 37 (Left: VTM+ALF, Right: VTM+ALF-SAO)

# Summary

In this study, overlap in functionality between SAO and ALF have been studied to see whether it is possible to disable SAO in the presence of ALF in order to reduce the number of stages of in-loop filtering in VVC. For a fairer comparison, recent improvements proposed for SAO have been included in the study. It is seen that while there is a significant overlap in functionality, the CTB level signaling for SAO when compared to a set of filter coefficients in ALF for each class results in SAO still showing a 0.88% luma BDRATE gain and a significant chroma BDRATE gain. Additional improvements to ALF will be required in order to consider removal of SAO in the presence of ALF.

# Reference

- Algorithm Description of Joint Exploration Test Model 7 (JEM 7), JVET-G1001, Gwang Ju, Jan. 2018.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. does not have any current or pending patent rights relating to the technology described in this contribution.