| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0043-v1 |
| --- | --- |

| Title: | CE3: Bilateral reference sample filter (Test 2.7.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Merkle, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-433 philipp.merkle@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the results on the tool under test in CE3 [1] for studying the performance of the bilateral reference sample filter proposed in JVET-J0014 [2] on top of VTM and BMS.

# Introduction

With larger block sizes and versatile rectangular block shapes, filtering of reference samples for intra prediction has more effect on the intra coding performance. Compared to the standard 3-tap smoothing filter, the proposed bilateral filter reduces more noise while preserving edges, which is especially beneficial for predicting larger blocks.

# Proposed method

Given original (unfiltered) reference samples I(x) of an intra block arranged as illustrated in figure 1, the bilateral filter function is defined as

Ix=∆xi≤swpos(∆xi)∙wval(∆Ii)∙I(xi)∆xi≤swpos(∆xi)∙wval(∆Ii)

This filter is applied to all reference sample positions x∈[s .. (2W+H-s)], with filter size s. For avoiding complex calculations (exponential function) and floating point-arithmetic, filter weights wpos and wval are pre-calculated as 8-bit integers and stored in lookup tables.

Figure 1. Reference samples arrangement

The bilateral filter is applied to luma blocks with size W×H≥16×16 and all prediction modes m with minm-mHOR, m-mVER>1.

The bilateral filter is applied in two configurations, with s=4 for block sizes W×H≥64×64 and s=3 otherwise. Both configurations share the same lookup tables for wpos and wval, but use different subsets of wval that correspond to different filter strengths.

# Results

According to the CE3 description in JVET-J1023 [1] and the common test conditions in JVET-J1010 [3], the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with BMS-1.0 software in VTM and BMS configuration. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 1. Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.68% | -0.40% | -0.34% | 101% | 102% |
| Class A2 | -0.14% | -0.10% | -0.02% | 101% | 101% |
| Class B | -0.07% | 0.00% | -0.10% | 101% | 103% |
| Class C | -0.15% | -0.07% | -0.13% | 100% | 99% |
| Class E | -0.15% | -0.06% | -0.15% | 101% | 102% |
| Overall | -0.22% | -0.11% | -0.14% | 101% | 101% |
| Class D | -0.04% | 0.00% | 0.06% | 101% | 100% |
| Class F (optional) | -0.12% | -0.08% | -0.18% | 100% | 102% |

Table 2. Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.33% | -0.20% | -0.10% | 101% | 104% |
| Class A2 | -0.14% | 0.02% | 0.05% | 100% | 101% |
| Class B | -0.05% | 0.04% | -0.11% | 100% | 102% |
| Class C | -0.06% | 0.02% | 0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.02% | -0.03% | 100% | 102% |
| Class D | -0.02% | 0.07% | -0.04% | 101% | 101% |
| Class F (optional) | -0.05% | -0.09% | 0.10% | 100% | 100% |

Table 3. Result for BMS AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.47% | -0.64% | -0.54% | 101% | 102% |
| Class A2 | -0.19% | -0.10% | -0.05% | 100% | 102% |
| Class B | -0.09% | 0.00% | -0.18% | 101% | 102% |
| Class C | -0.33% | -0.35% | -0.28% | 100% | 101% |
| Class E | -0.21% | 0.03% | -0.15% | 101% | 103% |
| Overall | -0.24% | -0.20% | -0.24% | 101% | 102% |
| Class D | -0.04% | 0.13% | -0.02% | 101% | 101% |
| Class F (optional) | -0.24% | -0.19% | -0.32% | 100% | 101% |

Table 4. Result for BMS RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.23% | -0.38% | -0.19% | 101% | 101% |
| Class A2 | -0.13% | -0.10% | -0.03% | 101% | 101% |
| Class B | -0.08% | 0.05% | -0.23% | 102% | 99% |
| Class C | -0.05% | -0.07% | -0.46% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.10% | -0.24% | 101% | 100% |
| Class D | 0.01% | 0.04% | -0.30% | 102% | 100% |
| Class F (optional) | -0.07% | 0.00% | -0.13% | 100% | 99% |

# Cross check

The cross check of test 2.7.1 was done by Huawei.

# Conclusion

The results show that the proposed bilateral reference sample filter leads to good coding gains without increasing the encoding and decoding time. The performance improvement is consistent for VTM and BMS configuration. The filter is implemented efficiently and can easily be combined with the multi-reference line tools in CE3.5. In summary, the proposed tool increases the coding efficiency without increasing the complexity and is therefore recommended for adoption to VVC.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-J1023, San Diego, USA, April 2018.

- H. Schwarz et al., “Description of SDR, HDR and 360° video coding technology proposal by Fraunhofer HHI,” JVET-J0014, San Diego, USA, April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).