| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0044-v1 |
| --- | --- |

| Title: | CE7: Residual sign prediction in transform domain (Tests 7.5.1 and 7.5.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Alexander Karabutov, Vasily Rufitskiy Jianle Chen | Tel: Email: | +7-495-660-44-59 alexey.filippov@huawei.com karabutov.alexander@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

JVET-D0031 presented a technique for predicting signs of luma residue coefficients. It is based on minimizing the disparity between current and adjacent blocks in special domain. To decrease computational complexity of this technique, we propose to perform sign prediction in transform domain. This documents contains experimental results for 2 tests. In the 1st test, residual sign prediction in transform domain is applied to both intra- and inter-picture coded blocks. In the 2nd test, this technology is used to predict signs of residues only for intra-picture coded blocks.

On top of VTM, the simulation results for the 1st test reportedly show -1.30%, -1.04% and -1.05% BD-rate savings on Y, Cb and Cr components respectively for All Intra (AI) configuration with 119% encoding time on average, and -1.02%, -0.75% and -0.83% BD-rate savings on Y, Cb and Cr components respectively for Random Access (RA) configuration with 109% encoding time on average. On top of BMS, the simulation results reportedly show -0.45%, -0.33% and -0.39% BD-rate savings on Y, Cb and Cr components respectively for All Intra (AI) configuration with 109% encoding time on average, and -0.61%, -0.42% and -0.58% BD-rate savings on Y, Cb and Cr components respectively for Random Access (RA) configuration with 107% encoding time on average.

On top of VTM, the simulation results for the 2nd test reportedly show -1.30%, -1.04% and -1.05% BD-rate savings on Y, Cb and Cr components respectively for All Intra (AI) configuration with 119% encoding time on average, and -0.56%, -0.37% and -0.46% BD-rate savings on Y, Cb and Cr components respectively for Random Access (RA) configuration with 105% encoding time on average. On top of BMS, the simulation results reportedly show -0.45%, -0.33% and -0.39% BD-rate savings on Y, Cb and Cr components respectively for All Intra (AI) configuration with 109% encoding time on average, and -0.18%, -0.14% and -0.23% BD-rate savings on Y, Cb and Cr components respectively for Random Access (RA) configuration with 104% encoding time on average.

# Introduction

Signs of residual coefficients are processed by CABAC in by-pass mode. To take advantage of entropy coding, it was proposed in [1] to use residual sign prediction for coding only a sign prediction error flag instead of a sign itself. For this flag, two new CABAC contexts were introduced. The 1st context is used for flags that corresponds to coefficients with high magnitudes, since they can be predicted with higher probability of guessing. The 2nd one is for coefficients with low magnitudes. To distinguish low and high magnitudes, a thresholding procedure is applied to dequantized magnitudes at both encoder and decoder sides. This proposal is aimed at mitigating such a disadvantage of RSP as computational complexity caused by switching between spatial and transform domains while predicting residual signs. This contribution propose to move most operations to transform domain. Process of sign prediction could be divide to several steps:

- Preparation prediction of reconstructed boundary of the current block from neighbor blocks.

- Calculation hypothesis of boundary lines (left column and top line).

- Calculation costs of the hypothesis.

- Estimate sign prediction error flag on encoder or sign of coefficient on decoder.

# Technology description

In this contribution, it is proposed to reduce amount of inverse transforms by migrating from spatial to transform domain. The general idea is to use 1D transforms on 1D calculation of transform component instead processing the 2D transform. Processing of transform-domain residual sign prediction has similar four steps to above:

- Preparation linear prediction of the boundary residuals in transform domain.

- Calculation hypothesis of boundary transform components.

- Calculation of costs.

- Estimate sign prediction error flag on encoder or sign of coefficient on decoder.

The last step is not modified as compared with [1]. Intra-coded blocks which have non-zero value of the NSST index are skipped.

This documents covers 2 tests. In the 1st test, residual sign prediction in transform domain is applied to both intra- and inter-picture coded blocks. In the 2nd test, this technology is used to predict signs of residues only for intra-picture coded blocks.

## Preparation linear prediction of the boundary residuals in transform domain

Figure 1. Pixels used to derive cost value

The first step to get prediction of boundary residuals is to predict boundary pixels of current block. To realize it, a linear extrapolation of lines (Xi,j)/columns (Zi,j) is used:

PXi=2Xi,-1-Xi,-2PZi=2Z-1,i-Z-2,i

where PXi and PZi are linear extrapolations.

Pixels of a current block could be presented as a sum of residuals and an inter/intra-predictor:

Yi,j=Ri,j+Pi,j

where Ri,j and Pi,j denote residuals and a predictor, respectively.

The second step is to remove an inter/intra predictor and to perform forward transform (1D):

xn=TPXi-Pi,0zm=TPZi-P0,i

## Calculation hypothesis of boundary transform components

Transform component of boundary could be calculated from coefficients of TU, which lie on the same row/column by following formulas:

lyn=m=0M-1ωmRn,mcosπm2M,

tym=n=0N-1ωnRn,mcosπn2N,

where ωi=1M, i=02M, i>0, M is block height.

## Calculation of costs

Cost function values should be calculated for every combination of signs to be predicted. So, the number of hypotheses is 2n, where n is a number of predicted signs2max_num_pred_signs 2max_num_pred_signs. Sum of squared error (SSE) is used as cost function:

costk=n=0N-1xn-lyn2+m=0M-1zm-tym2

We use SSE instead of SAD for cost estimation since the Euclidian metric is preserved in both spatial and transform domain.

## Bitstream changes

Bitstream changes are as follows:

- Signalling of signs is performed after coding coefficient magnitudes of an entire coding block as well as after NSST and EMT flags.

- To harmonize this tool with EMT, number of contexts for sign prediction error was increased to take into account a dependency of probability of hypothesis guessing on EMT index.

# Experimental results

Both proposed tests are implemented on top of VTM-1.0 and BMS-1.0. The experimental results of All Intra, Random Access, Low Delay B and Low Delay P configurations are summarized in the Tables 1-2 for both tests respectively. The simulations were performed following the JVET CTC [3]. More detail results can be found in the accompanying Excel tables.

## Table 1. Experimental results for TD-RSP for intra- and inter-picture coded blocks (test CE7-5.1)

## Table 2. Experimental results for TD-RSP for intra-picture coded blocks only (test CE7- 5.2)

# References

- F. Henry, G. Clare, “Residual Coefficient Sign Prediction”, 4th JVET Meeting, Chengdu, Oct. 2016, JVET-D0031.

- F. Henry, G. Clare, “EE1: Residual Coefficient Sign Prediction”, 5th JVET Meeting, Geneva, January 2017, JVET-E0051.

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, April 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).