| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0046-v1 |
| --- | --- |

| Title: | CE3: Wide-angle intra prediction (Test 1.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document provides result for the CE3 Test 1.3.1 on wide-angle intra prediction. The proposed approach defines directional intra prediction modes with angles extending beyond the traditional 45-degree top-right and bottom-left directions. These modes are available only for non-square blocks and specific angle groups as follows:

- Angles going beyond 45 degrees in top-right direction if block width is larger than block height

- Angles going beyond 45 degrees in bottom-left direction if block height is larger than block width

Signaling for the tool is implemented by sending a 1-bin context coded identifier for those directions that have an available wide-angle alternative. The availability of a wide-angle alternatives is limited to angles with a positive angular parameter of 9/32 and above.

The results indicate that the proposed approach provides in AI tests BD-rate impacts of -0.28 % and -0.23 % for VTM and BMS configurations, respectively. In the case of RA tests, the BD-rate impacts are -0.18 % and -0.13 %, respectively.

# Introduction

This test was set up to verify performance of the wide-angle intra prediction tool proposed in JVET-J0012. In this test intra prediction from angles extending beyond the traditional 45-degree top-right and bottom-left directions was supported. In order to keep signaling overhead and computational complexity low, these angles were only used for non-square blocks and following angle groups:

- Angles going beyond 45 degrees in top-right direction if block width is larger than block height

- Angles going beyond 45 degrees in bottom-left direction if block height is larger than block width

Signaling for the tool is implemented by sending a 1-bin context coded identifier for those directions that have an available wide-angle alternative. In the case of the 65-direction angular intra prediction, the availability of the wide-angle modes is limited to the 10 directional modes closest to the 45-degree diagonal top-right and bottom-left modes. In the case of 33 angular directions, wide-angle modes are limited similarly to 5 directions closest to the diagonal ones. That is, the availability of a wide-angle alternatives is limited to angles with a positive angular parameter of 9/32 and above

The sample prediction process follows the one in reference software, but the effective prediction direction for the wide-angle modes is calculated by inverting the signaled prediction direction (using calculations identical to what is being done when projecting the side reference in the case of negative prediction angles) and switching the reference arrays - using top instead of left or left instead of top references.

Further details:

- Reference samples outside of the existing reference sample arrays are padded

- Context based arithmetic coding of the flag uses one context

- Side smoothing is enabled for wide-angle modes as in JVET-J0012 software

- If intra slices have their own chroma coding tree, chroma blocks have their own wide-angle mode signaling. In the case of inter slices (and if there is common coding tree for all components) chroma uses wide-angle mode of the corresponding luma block.

Software implementing the tool was made available in the following branch of the BMS:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE3/CE3-1.3.1/

# Test results

Full results are provided in the attached Excel sheets. A summary of the results can be found below:

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.22 % | -0.18 % | -0.27 % | 105 % | 102 % | -0.15 % | -0.07 % | -0.15 % | 105 % | 100 % |
| Class A2 | -0.24 % | -0.30 % | -0.29 % | 105 % | 101 % | -0.19 % | -0.21 % | -0.20 % | 105 % | 100 % |
| Class B | -0.26 % | -0.42 % | -0.41 % | 106 % | 97 % | -0.22 % | -0.32 % | -0.37 % | 103 % | 98 % |
| Class C | -0.29 % | -0.51 % | -0.65 % | 106 % | 106 % | -0.25 % | -0.23 % | -0.28 % | 100 % | 98 % |
| Class E | -0.41 % | -0.43 % | -0.42 % | 108 % | 101 % | -0.33 % | -0.24 % | -0.48 % | 99 % | 98 % |
| Overall | -0.28 % | -0.38 % | -0.42 % | 106 % | 101 % | -0.23 % | -0.23 % | -0.30 % | 102 % | 98 % |
| Class D | -0.35 % | -0.38 % | -0.49 % | 105 % | 101 % | -0.26 % | -0.29 % | -0.34 % | 100 % | 100 % |
| Class F | -0.20 % | -0.28 % | -0.44 % | 106 % | 102 % | -0.22 % | -0.27 % | -0.46 % | 103 % | 98 % |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.20 % | -0.30 % | -0.30 % | 102 % | 102 % | -0.14 % | -0.12 % | -0.22 % | 101 % | 100 % |
| Class A2 | -0.16 % | -0.39 % | -0.21 % | 102 % | 101 % | -0.12 % | -0.08 % | -0.08 % | 102 % | 100 % |
| Class B | -0.17 % | -0.12 % | -0.51 % | 103 % | 101 % | -0.15 % | -0.28 % | -0.31 % | 103 % | 100 % |
| Class C | -0.20 % | -0.19 % | -0.51 % | 103 % | 100 % | -0.10 % | -0.07 % | -0.36 % | 103 % | 100 % |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.18 % | -0.23 % | -0.41 % | 102 % | 101 % | -0.13 % | -0.15 % | -0.26 % | 103 % | 100 % |
| Class D | -0.15 % | -0.18 % | -0.60 % | 102 % | 100 % | -0.12 % | -0.17 % | -0.21 % | 103 % | 99 % |
| Class F | -0.14 % | -0.09 % | -0.25 % | 103 % | 100 % | -0.14 % | 0.00 % | -0.25 % | 103 % | 99 % |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.07 % | -0.01 % | 0.39 % | 100 % | 100 % | -0.09 % | -0.52 % | 0.11 % | 101 % | 101 % |
| Class C | -0.09 % | -0.14 % | 0.00 % | 102 % | 100 % | -0.07 % | -0.03 % | -0.48 % | 102 % | 100 % |
| Class E | -0.03 % | -0.63 % | -0.11 % | 100 % | 99 % | -0.26 % | -0.96 % | -0.66 % | 103 % | 102 % |
| Overall | -0.07 % | -0.21 % | 0.13 % | 101 % | 100 % | -0.12 % | -0.47 % | -0.28 % | 102 % | 101 % |
| Class D | -0.07 % | 0.14 % | -0.38 % | 102 % | 99 % | -0.04 % | 0.45 % | -0.57 % | 102 % | 93 % |
| Class F | -0.16 % | -0.15 % | -0.35 % | 105 % | 99 % | -0.01 % | 0.01 % | 0.59 % | 102 % | 98 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).