| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0047-v1 |
| --- | --- |

| Title: | CE4: Affine flexing (Test 1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document provides result for the CE4 Test 1.2 on affine flexing motion compensation. The proposed approach performs block-based affine motion compensation (identically to BMS) followed by a resampling or "flexing" filtering of the prediction block. The experiment included testing variants with both 4x4 and 8x8 luma sub-block size for motion compensation.

With respect to affine off anchors, the 4x4 luma block based method is reported to provide BD-rate impact of -3.31% and -2.14% for RA VTM and BMS tests, respectively. Similarly, BD-rate impacts for LB tests are reported to be -2.27% and -2.18%, for VTM and BMS, respectively.

With respect to affine off anchors, the 8x8 luma block based method is reported to provide BD-rate impact of -2.90% and -1.82% for RA VTM and BMS tests, respectively. Similarly, BD-rate impacts for LB tests are reported to be -1.94% and -1.94%, for VTM and BMS, respectively.

# Introduction

This test was set up to verify performance of the affine motion compensation approach proposed in JVET-J0012. The approach performs block-based motion compensation followed by a resampling or "flexing" filtering of the prediction block. The original proposal is based on 4x4 luma block motion compensation as in JEM 7.0. In addition, a version with 8x8 luma block compensation was tested. The test was using the 16-position JEM motion compensation filter for the flexing operation.

The affine flexing tool calculates the horizontal and vertical motion vector differences on the affine sub-PU boundaries. Based on whether the nature of motion is more rotational or zooming, the method decides to apply either lateral compensation (moving the sub-PU boundary samples laterally with respect to the neighboring sub-PU samples) or perpendicular compensation (moving the sub-PU boundary samples away or closer to the samples of the neighboring sub-PU), respectively. The remapping offsets applied to different lines are calculated based on motion vector difference and distance from the sub-PU border. A look-up table implementation is provided in the CE software. An example of lateral processing is illustrated in the figure below.

Figure 1: Example of horizontal lateral remapping of samples in a prediction unit in the case of 4x4 sub-PUs.

Further details:

- Flexing is applied to both horizontally and vertically, and the result of the flexing in first dimension is used as an input to the second dimension

- Flexing is applied only for the luma channel in the case of 4:2:0 video

- Line ends are padded with the last available samples on the line

Software implementing the tool was made available in the following branch of the BMS:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE4/CE4-1.2/

# Test results

Full results are provided in the attached Excel sheets. A summary of the results can be found below.

## Test result (affine OFF as anchor) – 4x4 sub-blocks

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 with affine off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.09 % | -1.65 % | -1.69 % | 129 % | 110 % | -1.17 % | -0.84 % | -0.86 % | 106 % | 97 % |
| Class A2 | -6.24 % | -4.85 % | -4.66 % | 133 % | 123 % | -3.73 % | -2.46 % | -2.41 % | 106 % | 100 % |
| Class B | -3.77 % | -2.55 % | -2.70 % | 132 % | 112 % | -2.54 % | -1.84 % | -1.75 % | 104 % | 99 % |
| Class C | -1.44 % | -0.85 % | -0.74 % | 131 % | 108 % | -1.16 % | -0.72 % | -0.82 % | 105 % | 102 % |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.31 % | -2.38 % | -2.37 % | 131 % | 113 % | -2.14 % | -1.47 % | -1.46 % | 105 % | 100 % |
| Class D | -2.04 % | -1.14 % | -1.48 % | 130 % | 108 % | -1.76 % | -1.55 % | -1.32 % | 105 % | 103 % |
| Class F (optional) | -1.69 % | -1.59 % | -1.65 % | 126 % | 105 % | -1.40 % | -1.44 % | -1.43 % | 103 % | 102 % |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 with affine off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.73 % | -1.85 % | -1.81 % | 155 % | 113 % | -2.75 % | -2.02 % | -2.02 % | 119 % | 101 % |
| Class C | -1.58 % | -0.78 % | -0.61 % | 146 % | 109 % | -1.46 % | -1.09 % | -1.06 % | 114 % | 105 % |
| Class E | -2.42 % | -1.79 % | -2.09 % | 195 % | 111 % | -2.20 % | -2.34 % | -1.64 % | 139 % | 100 % |
| Overall | -2.27 % | -1.48 % | -1.48 % | 161 % | 111 % | -2.18 % | -1.79 % | -1.61 % | 122 % | 102 % |
| Class D | -3.08 % | -1.95 % | -1.81 % | 139 % | 110 % | -3.04 % | -2.85 % | -2.99 % | 115 % | 107 % |
| Class F (optional) | -2.08 % | -2.08 % | -1.63 % | 154 % | 101 % | -2.09 % | -1.93 % | -2.50 % | 110 % | 102 % |

## Test result (affine OFF as anchor) – 8x8 sub-blocks

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 with affine off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.94 % | -1.54 % | -1.53 % | 122 % | 106 % | -1.05 % | -0.77 % | -0.78 % | 104 % | 96 % |
| Class A2 | -5.64 % | -4.06 % | -3.97 % | 124 % | 113 % | -3.21 % | -1.92 % | -1.86 % | 105 % | 98 % |
| Class B | -3.30 % | -2.08 % | -2.07 % | 128 % | 106 % | -2.17 % | -1.26 % | -1.18 % | 106 % | 97 % |
| Class C | -1.07 % | -0.58 % | -0.53 % | 127 % | 104 % | -0.90 % | -0.56 % | -0.64 % | 105 % | 100 % |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.90 % | -1.97 % | -1.93 % | 126 % | 107 % | -1.82 % | -1.11 % | -1.09 % | 105 % | 98 % |
| Class D | -1.44 % | -0.63 % | -0.77 % | 126 % | 103 % | -1.26 % | -0.96 % | -0.79 % | 105 % | 101 % |
| Class F (optional) | -1.12 % | -0.89 % | -0.89 % | 123 % | 102 % | -0.90 % | -0.83 % | -0.78 % | 102 % | 100 % |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 with affine off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.41 % | -1.55 % | -1.45 % | 148 % | 114 % | -2.46 % | -1.58 % | -1.78 % | 116 % | 101 % |
| Class C | -1.28 % | -0.47 % | -0.37 % | 138 % | 111 % | -1.30 % | -1.14 % | -0.88 % | 112 % | 103 % |
| Class E | -2.06 % | -1.10 % | -1.77 % | 182 % | 106 % | -1.92 % | -1.44 % | -2.46 % | 133 % | 103 % |
| Overall | -1.94 % | -1.08 % | -1.17 % | 152 % | 111 % | -1.94 % | -1.40 % | -1.65 % | 119 % | 102 % |
| Class D | -2.59 % | -1.67 % | -1.55 % | 133 % | 107 % | -2.68 % | -2.08 % | -2.37 % | 112 % | 104 % |
| Class F (optional) | -1.69 % | -1.51 % | -1.24 % | 148 % | 100 % | -1.45 % | -1.29 % | -1.34 % | 110 % | 100 % |

## Test result (BMS affine ON as anchor) – 4x4 sub-blocks

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 with 4x4 affine |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12 % | -0.07 % | -0.04 % | 101 % | 101 % | -0.03 % | -0.04 % | 0.03 % | 100 % | 100 % |
| Class A2 | -0.40 % | -0.12 % | -0.21 % | 102 % | 103 % | -0.32 % | 0.03 % | 0.03 % | 100 % | 101 % |
| Class B | -0.55 % | -0.41 % | -0.38 % | 100 % | 102 % | -0.41 % | -0.37 % | -0.25 % | 99 % | 100 % |
| Class C | -0.17 % | -0.09 % | 0.06 % | 100 % | 102 % | -0.08 % | -0.01 % | -0.12 % | 101 % | 100 % |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.33 % | -0.20 % | -0.16 % | 100 % | 102 % | -0.23 % | -0.13 % | -0.10 % | 100 % | 100 % |
| Class D | -0.26 % | -0.12 % | -0.27 % | 100 % | 100 % | -0.19 % | -0.27 % | -0.25 % | 101 % | 100 % |
| Class F (optional) | -0.22 % | -0.18 % | -0.22 % | 100 % | 101 % | -0.18 % | -0.15 % | -0.14 % | 100 % | 100 % |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 with 4x4 affine |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.26 % | -0.38 % | -0.05 % | 99 % | 97 % | -0.23 % | -0.55 % | 0.03 % | 100 % | 101 % |
| Class C | -0.14 % | -0.02 % | -0.01 % | 101 % | 102 % | -0.07 % | 0.10 % | -0.04 % | 100 % | 101 % |
| Class E | -0.24 % | 0.04 % | 0.25 % | 99 % | 100 % | -0.35 % | -0.29 % | 0.37 % | 102 % | 99 % |
| Overall | -0.21 % | -0.15 % | 0.04 % | 100 % | 99 % | -0.21 % | -0.27 % | 0.09 % | 101 % | 100 % |
| Class D | -0.14 % | 0.02 % | 0.54 % | 100 % | 99 % | -0.09 % | -0.09 % | -0.34 % | 102 % | 100 % |
| Class F (optional) | -0.38 % | -0.42 % | 0.03 % | 100 % | 100 % | -0.47 % | -0.07 % | -0.41 % | 98 % | 98 % |

## Test result (BMS affine with 8x8 sub-blocks ON as anchor) – 8x8 sub-blocks

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 with 8x8 affine |  |  |  |  | Over BMS-1.0 with 8x8 affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.23 % | -0.06 % | -0.05 % | 101 % | 101 % | -0.11 % | 0.11 % | 0.02 % | 100 % | 100 % |
| Class A2 | -0.75 % | -0.12 % | -0.24 % | 100 % | 105 % | -0.67 % | 0.00 % | 0.02 % | 100 % | 102 % |
| Class B | -0.74 % | -0.41 % | -0.35 % | 100 % | 102 % | -0.65 % | -0.24 % | -0.11 % | 100 % | 100 % |
| Class C | -0.19 % | -0.23 % | -0.20 % | 100 % | 101 % | -0.21 % | -0.10 % | -0.25 % | 100 % | 99 % |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.49 % | -0.23 % | -0.23 % | 100 % | 102 % | -0.43 % | -0.09 % | -0.10 % | 100 % | 100 % |
| Class D | -0.36 % | -0.31 % | -0.33 % | 100 % | 100 % | -0.44 % | -0.32 % | -0.17 % | 100 % | 98 % |
| Class F (optional) | -0.23 % | -0.12 % | -0.08 % | 100 % | 101 % | -0.21 % | -0.06 % | -0.08 % | 99 % | 98 % |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 with 8x8 affine |  |  |  |  | Over BMS-1.0 with 8x8 affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.48 % | -0.32 % | -0.43 % | 100 % | 101 % | -0.44 % | -0.57 % | -0.29 % | 100 % | 101 % |
| Class C | -0.16 % | 0.04 % | -0.15 % | 100 % | 99 % | -0.20 % | -0.51 % | -0.07 % | 100 % | 98 % |
| Class E | -0.33 % | 0.48 % | 0.07 % | 100 % | 97 % | -0.36 % | 0.17 % | -0.66 % | 100 % | 103 % |
| Overall | -0.34 % | 0.00 % | -0.21 % | 100 % | 99 % | -0.34 % | -0.37 % | -0.31 % | 100 % | 101 % |
| Class D | -0.28 % | -0.55 % | 0.34 % | 100 % | 97 % | -0.46 % | -0.23 % | -0.09 % | 100 % | 101 % |
| Class F (optional) | -0.68 % | -0.51 % | -0.63 % | 101 % | 97 % | -0.66 % | -0.70 % | -0.03 % | 100 % | 100 % |

## Test result – extra tests

In order to verify the performance of the proposed tool further, it was also tested using a testset with strong affine-like motion characteristics. This content is available on JVET ftp site in the following directory:

ftp://jvet@ftp.ient.rwth-aachen.de/testsequences/candidates/huawei/Affine/

In this extra test 17-frame (I-frame + one GOP) simulations were run using RA configuration. As the motion in “Shields” sequence is very small in the beginning of the clip, the encoding of that sequence was started from frame 32 (rest from 0). The additional testset is summarized in the table below:

Sequence    Width Height FPS 1st

Tractor      1920   1080  25   0

Station      1920   1080  25   0

Shields      1920   1080  50  32

BlueSky      1920   1080  25   0

SpinCalendar 1280    720  50   0

Jets         1280    720  25   0

### Extra tests (BMS affine ON as anchor) – 4x4 sub-blocks

|  | Random Access Main 10 / Extra testset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 with 4x4 affine |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Tractor | -4.14 % | -1.78 % | -1.92 % | 99 % | 107 % | -2.70 % | -2.01 % | -1.84 % | 99 % | 101 % |
| Station | -2.62 % | -1.10 % | -0.81 % | 100 % | 110 % | -1.81 % | -0.53 % | -0.51 % | 99 % | 100 % |
| Shields | -1.46 % | -0.76 % | -0.85 % | 100 % | 103 % | -1.20 % | -1.08 % | -1.06 % | 98 % | 100 % |
| BlueSky | -1.81 % | -1.49 % | -0.80 % | 101 % | 104 % | -1.17 % | -1.29 % | -0.66 % | 99 % | 98 % |
| SpinCalendar | -0.54 % | 0.81 % | -0.33 % | 101 % | 101 % | -0.62 % | 0.49 % | -0.44 % | 100 % | 106 % |
| Jets | -0.17 % | -0.35 % | 0.00 % | 100 % | 102 % | -0.31 % | -0.08 % | -0.47 % | 99 % | 99 % |
| Overall | -1.79 % | -0.78 % | -0.78 % | 100 % | 104 % | -1.30 % | -0.75 % | -0.83 % | 99 % | 101 % |

### Extra tests (BMS affine with 8x8 sub-blocks ON as anchor) – 8x8 sub-blocks

|  | Random Access Main 10 / Extra testset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 with 8x8 affine |  |  |  |  | Over BMS-1.0 with 8x8 affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Tractor | -8.12 % | -4.91 % | -5.19 % | 99 % | 112 % | -8.23 % | -5.06 % | -5.27 % | 97 % | 106 % |
| Station | -6.69 % | -3.14 % | -3.39 % | 99 % | 112 % | -6.23 % | -3.07 % | -2.84 % | 97 % | 108 % |
| Shields | -3.93 % | -3.32 % | -3.69 % | 99 % | 108 % | -4.32 % | -4.35 % | -4.58 % | 95 % | 101 % |
| BlueSky | -2.77 % | -2.85 % | -2.40 % | 101 % | 105 % | -3.55 % | -3.05 % | -2.72 % | 96 % | 103 % |
| SpinCalendar | -2.77 % | -2.27 % | -1.78 % | 99 % | 104 % | -2.65 % | -2.14 % | -1.39 % | 97 % | 102 % |
| Jets | -1.01 % | -1.07 % | -0.74 % | 101 % | 100 % | -1.20 % | -0.09 % | -0.99 % | 99 % | 107 % |
| Overall | -4.21 % | -2.93 % | -2.87 % | 100 % | 107 % | -4.36 % | -2.96 % | -2.96 % | 97 % | 104 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).