| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0048-v1 |
| --- | --- |

| Title: | CE8: Intra Region-based Template Matching (Test 8.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gayathri Venugopal Karsten Müller Heiko Schwarz Detlev Marpe Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002 287 gayathri.venugopal@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany |  |  |

_____________________________

# Abstract

This document presents the test results of intra region-based template matching, which is one of the methods considered for studying in the core experiment (CE8) group. The proposed method achieves an overall BD-rate gain of -1.41% for the all intra (AI) configuration with a 116% decoder run-time and a 214% encoder run-time against VTM-1.0. For the random access (RA) configuration, the overall coding gain is -0.44% with a 103% decoder run-time and a 117% encoder run-time while for the low delay (LB) configuration the overall gain is -0.10% with a 101% decoder run-time and a 109% encoder run-time. The experimental results against BMS-1.0 for the all intra (AI) configuration show a -1.01% overall BD-rate gain with a 104% decoder run-time and a 113% encoder run-time. For the random access (RA) configuration, the overall gain is -0.31% with a 101% decoder run-time and a 105% encoder run-time, while for the low delay (LB) configuration the overall gain is -0.15% with a 100% decoder run-time and a 102% encoder run-time. Improved results of RTM by applying it to both luma and chroma components are reported in JVET-K0050.

# Technical description

The prediction signal of the intra region-based template matching (RTM) mode is a linear combination of already reconstructed blocks inside the current picture [1] [2] [3]. The blocks required for this are called the predictors of the current block and they are found using template matching algorithm. A template refers to the samples present above and left of a block (Figure.1). Let the template of the current block be Tc and any possible template in the reconstructed picture be Tr. The basic idea of a template matching intra mode is to find the best match for Tc by minimizing an error metric. The error metric used in our proposal is the sum of squared differences (SSD). Thus, the best template match Tb is the template Tr that gives the minimum SSD error, as described in eq.1. The block associated with Tb is the predictor P.

(eq.1)

The proposed intra RTM mode searches for the best template match within a window in the immediate neighborhood of the block to be predicted. This search window has five regions as shown in Figure 2. However, at the decoder side only one region is considered. The information about the region under consideration is transmitted in the bitstream. A more detailed explanation of the aforementioned method can be found in [2].

Figure 1 Illustration of intra template matching

Figure 2 Search window and five regions of intra RTM

The current Test 8.1 configuration uses the following settings:

- The region size parameter (RSP) is set to 0.

- The size of the regions are modified as in Table 1

Table 1: Modified search region sizes

|  |  | Down-sample search region |
| --- | --- | --- |
| A1 | 4 × (RSP + 1) | no |
| A2 | 2 × A1 | no |
| A3 | 4 × A2 | yes |

- The speed ups mentioned in [2] (i.e. No. 3 and 4 of section 3) are not considered.

The proposed method with the above configuration is called the normal mode of intra RTM. The results shown in section.3 correspond to the normal RTM mode. However, the configuration with the speed ups enabled (called the fast mode) can be found in the APPENDIX.

The intra RTM method is applied to all block sizes available in VTM/BMS. For the case of BMS, the AMT and NSST tools are not applied to RTM intra predicted blocks.

# Results

According to the CE8 description in [3] and the common test conditions in [4], the proposed method is evaluated for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations with the BMS-1.0 software in the VTM and BMS configuration. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 4.8.5 compiler.

Table 2: Test8-1 against VTM-1.0 for AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.33% | 0.24% | 0.43% | 218% | 111% |
| Class A2 | -1.31% | 0.06% | -0.30% | 216% | 118% |
| Class B | -1.51% | -0.60% | -0.79% | 217% | 118% |
| Class C | -0.72% | 0.30% | -0.04% | 205% | 117% |
| Class E | -3.33% | -1.51% | -1.93% | 214% | 117% |
| Overall | -1.41% | -0.30% | -0.53% | 214% | 116% |
| Class D | -0.58% | -0.20% | -0.23% | 190% | 113% |
| Class F | -7.00% | -5.39% | -5.61% | 212% | 136% |
| Class TGM | -20.15% | -18.20% | -18.14% | 205% | 182% |

Table 3: Test8-1 against VTM-1.0 for RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.06% | 0.22% | 0.52% | 121% | 102% |
| Class A2 | -0.56% | 1.19% | 0.65% | 115% | 105% |
| Class B | -0.65% | 0.64% | 0.20% | 117% | 102% |
| Class C | -0.36% | 0.43% | 0.10% | 115% | 104% |
| Overall | -0.44% | 0.61% | 0.33% | 117% | 103% |
| Class D | -0.13% | 0.31% | -0.01% | 112% | 102% |
| Class F | -5.58% | -4.33% | -4.43% | 127% | 104% |
| Class TGM | -11.83% | -10.48% | -10.36% | 120% | 110% |

Table 4: Test8-1 against VTM-1.0 for LB

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | -0.10% | 0.48% | 0.33% | 112% | 102% |
| Class C | 0.02% | 0.09% | 0.30% | 109% | 100% |
| Class E | -0.28% | 1.22% | 0.57% | 103% | 99% |
| Overall | -0.10% | 0.54% | 0.38% | 109% | 101% |
| Class D | 0.07% | -0.01% | 0.24% | 106% | 100% |

Table 5: Test8-1 against BMS-1.0 for AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.20% | -0.08% | -0.05% | 115% | 101% |
| Class A2 | -1.09% | -1.20% | -1.07% | 113% | 104% |
| Class B | -0.95% | -0.78% | -1.16% | 113% | 104% |
| Class C | -0.81% | -0.98% | -1.01% | 111% | 107% |
| Class E | -2.11% | -1.64% | -2.00% | 114% | 105% |
| Overall | -1.01% | -0.92% | -1.07% | 113% | 104% |
| Class D | -0.38% | -0.34% | -0.33% | 109% | 102% |
| Class F | -7.29% | -6.95% | -7.28% | 111% | 114% |
| Class TGM | -22.26% | -21.03% | -20.95% | 102% | 136% |

Table 6: Test8-1 against BMS-1.0 for RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.03% | 0.13% | 0.08% | 106% | 100% |
| Class A2 | -0.47% | -0.28% | -0.05% | 105% | 101% |
| Class B | -0.42% | 0.22% | -0.32% | 105% | 101% |
| Class C | -0.26% | -0.15% | -0.45% | 105% | 100% |
| Overall | -0.31% | 0.01% | -0.22% | 105% | 101% |
| Class D | -0.04% | -0.08% | 0.04% | 106% | 101% |
| Class F | -5.64% | -5.29% | -5.23% | 107% | 104% |
| Class TGM | -12.49% | -11.64% | -11.78% | 108% | 104% |

Table 7: Test8-1 against BMS-1.0 for LB

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | -0.07% | -0.15% | 0.23% | 103% | 101% |
| Class C | -0.06% | 0.01% | -0.23% | 102% | 99% |
| Class E | -0.38% | 0.66% | -0.65% | 102% | 99% |
| Overall | -0.15% | 0.11% | -0.14% | 102% | 100% |
| Class D | 0.05% | 0.98% | 0.11% | 103% | 102% |

# Cross check

The cross check of the test 8.1 was done by Technicolor.

# Conclusions and recommendations

The experimental results of the proposed intra region-based template matching method are reported in this document. The intra RTM achieves a coding gain in all the classes of the testset for all the configurations. This indicates that the RTM method is suitable for all type of sequences. Further, the proposed method shows a much better performance for screen content and screen content-like sequences like those in class E, F and TGM than other classes. The coding gain of intra RTM on top of BMS-1.0 is smaller than the gain on top of VTM-1.0. This is possibly due to the presence of more tools in BMS-1.0 than in VTM-1.0. However, for class F and TGM, this behavior is inverted. Finally, the coding gain for the RA configuration is smaller than that for the AI configuration. Nevertheless, this is an expected behavior since the proposed mode is an intra prediction method.

The proposed method has been further studied by extending RTM to the chroma channel also yielding improved results as presented in JVET-K0050 [5]. Accordingly, we recommend further study of intra RTM in a CE.

# References

| [1] | M. Albrecht, S. Bosse, J. Brandenburg, M. C. Bartnik, B. Bross, J. Erfurt, V. George, P. Haase, P. Helle, C. Helmrich, A. Henkel, T. Hinz, S. d. L. Hernandez, S. Kaltenstadler, H. Kirchchoffer, C. Lehmann, W. Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, G. Venugopal, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, K. Stallenberger, J. Stegemann, K. Suehring, G. Tech, S. Walter, A. Wieckowski, T. Wiegand and M. Winken, "Description of SDR, HDR, and 360° video coding technology proposal by Fraunhofer HHI, JVET-J0014," San Diego, US, 2018. |
| --- | --- |
| [2] | G. Venugopal, H. Schwarz, D. Marpe and T. Wiegand, "Intra Region-based Template Matching, JVET-J0039," San Diego, US, 2018. |
| [3] | X. Xu, L. Wang and K. Müller, "Description of Core Experiment 8: Current Picture Referencing, JVET-J1028," San Diego, US, 2018. |
| [4] | J. Boyce, K. Suehring, X. Li and V. Seregin, "JVET common test conditions and software reference configurations, JVET-J1010," San Diego, US, 2018. |
| [5] | G. Venugopal, K. Müller, H. Schwarz, D. Marpe and T. Wiegand, "CE8 related: Intra Region-based Template Matching for luma and chroma, JVET-K0050," Ljubljana, SI, 2018. |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

APPENDIX

Table I: Overall coding gain of intra RTM for different tests against VTM-1.0 for AI

| Test | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Mode: Normal (Test 8.1) Region sizes:A1=4, A2=8,A3=32 Block size: Unconstrained Transform choice: Unconstrained | -1.41% | -0.30% | -0.53% | 214% | 116% |
| Mode: Fast Region sizes:A1=4, A2=8,A3=32 Block size: Unconstrained Transform choice: Unconstrained | -1.16% | -0.50% | -0.68% | 159% | 110% |
| Mode: Normal Region sizes:A1=A2=A3=4 Block size: Unconstrained Transform choice: Unconstrained | -0.95% | -0.04% | -0.23% | 163% | 105% |
| Mode: Normal Region sizes:A1=A2=A3=8 Block size: Unconstrained Transform choice: Unconstrained | -1.20% | -0.18% | -0.39% | 185% | 109% |
| Mode: Normal Region sizes:A1=A2=A3=16 Block size: Unconstrained Transform choice: Unconstrained | -1.47% | -0.37% | -0.57% | 241% | 124% |
| Mode: Normal Region sizes:A1=A2=A3=32 Block size: Unconstrained Transform choice: Unconstrained | -1.69% | -0.55% | -0.74% | 397% | 163% |
| Mode: Normal Region sizes:A1=A2=A3=8 Block size: Intra RTM allowed to blocks with width ≤ 16 and height ≤ 16 Transform choice: Unconstrained | -1.07% | -0.01% | -0.20% | 173% | 108% |
| Mode: Normal Region sizes:A1=A2=A3=8 Block size: Unconstrained Transform choice: DST-VII is applied as inverse transform instead of DCT-II | -1.56% | -0.20% | -0.37% | 197% | 116% |

Table II: Overall coding gain of intra RTM for different tests against VTM-1.0 for RA

| Test | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Mode: Normal (Test 8.1) Region sizes:A1=4, A2=8,A3=32 Block size: Unconstrained Transform choice: Unconstrained | -0.44% | 0.61% | 0.33% | 117% | 103% |
| Mode: Fast Region sizes:A1=4, A2=8,A3=32 Block size: Unconstrained Transform choice: Unconstrained | -0.35% | 0.34% | 0.10% | 110% | 100% |
| Mode: Normal Region sizes:A1=A2=A3=4 Block size: Unconstrained Transform choice: Unconstrained | -0.28% | 0.53% | 0.27% | 110% | 103% |
| Mode: Normal Region sizes:A1=A2=A3=8 Block size: Unconstrained Transform choice: Unconstrained | -0.36% | 0.56% | 0.27% | 113% | 101% |
| Mode: Normal Region sizes:A1=A2=A3=16 Block size: Unconstrained Transform choice: Unconstrained | -0.45% | 0.52% | 0.28% | 121% | 104% |
| Mode: Normal Region sizes:A1=A2=A3=32 Block size: Unconstrained Transform choice: Unconstrained | -0.55% | 0.56% | 0.27% | 141% | 107% |
| Mode: Normal Region sizes:A1=A2=A3=8 Block size: Intra RTM allowed to blocks with width ≤ 16 and height ≤ 16 Transform choice: Unconstrained | -0.30% | 0.64% | 0.31% | 107% | 102% |
| Mode: Normal Region sizes:A1=A2=A3=8 Block size: Unconstrained Transform choice: DST-VII is applied as inverse transform instead of DCT-II | -0.48% | 0.63% | 0.33% | 116% | 103% |

Table III: Overall coding gain of intra RTM for different tests against BMS-1.0 for AI

| Test | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Mode: Normal (Test 8.1) Region sizes:A1=4, A2=8,A3=32 Block size: Unconstrained Transform choice: Unconstrained | -1.01% | -0.92% | -1.07% | 113% | 104% |
| Mode: Fast Region sizes:A1=4, A2=8,A3=32 Block size: Unconstrained Transform choice: Unconstrained | -0.78% | -0.88% | -0.98% | 106% | 102% |
| Mode: Normal Region sizes:A1=A2=A3=8 Block size: Unconstrained Transform choice: Unconstrained | -0.87% | -0.83% | -0.92% | 110% | 102% |
| Mode: Normal Region sizes:A1=A2=A3=8 Block size: Unconstrained Transform choice: DST-VII is applied as inverse transform instead of DCT-II | -0.79% | -0.70% | -0.86% | 111% | 103% |

Table III: Overall coding gain of intra RTM for different tests against BMS-1.0 for RA

| Test | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Mode: Normal (Test 8.1) Region sizes:A1=4, A2=8,A3=32 Block size: Unconstrained Transform choice: Unconstrained | -0.31% | 0.01% | -0.22% | 105% | 101% |
| Mode: Fast Region sizes:A1=4, A2=8,A3=32 Block size: Unconstrained Transform choice: Unconstrained | -0.23% | -0.09% | -0.21% | 103% | 100% |
| Mode: Normal Region sizes:A1=A2=A3=8 Block size: Unconstrained Transform choice: Unconstrained | -0.25% | -0.04% | -0.17% | 104% | 100% |
| Mode: Normal Region sizes:A1=A2=A3=8 Block size: Unconstrained Transform choice: DST-VII is applied as inverse transform instead of DCT-II | -0.23% | -0.01% | -0.21% | 105% | 100% |