| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0049-v1 |
| --- | --- |

| Title: | CE3: Line-based intra coding mode (Tests 1.4.1, 1.4.2 and 1.4.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago De Luxán Hernández, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-511 santiago.de.luxan@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the simulation results of the Line-based intra coding mode (CE3, JVET-J1023, Tests 1.4.1, 1.4.2 and 1.4.3) on top of VTM and BMS. Modifications carried out on the algorithm with respect to JVET-J0014 and JVET-J1023 are reported. Besides, an alternative processing order for the decoding process is presented. The results for Test 1.4.2 are a 2,00% gain with 164% and 112% run times for the encoder and the decoder respectively for the AI configuration. In the case of the RA configuration it is reported a 0,82% gain with 110% and 102% run times for the encoder and the decoder respectively.

# Introduction

The Line-based Intra Prediction (LIP) coding mode divides luma intra-predicted blocks into 1-D partitions or lines. Hence, a W×H dimensional block (where H is the height and W is the width) can be partitioned into H W×1 (horizontal split) or W 1×H (vertical split) lines, as presented in Figure 1. This mode can be applied to any block size available in VTM/BMS.

Figure 1: Example of the horizontal or vertical division of a block in lines

For each of these lines, a residual signal is generated by entropy decoding the coefficients sent by the encoder and then inverse quantizing and inverse transforming them. Then, the line is intra predicted and finally the corresponding reconstructed samples are obtained by adding the residual signal to the prediction signal. Therefore, the reconstructed values of each line will be available to generate the prediction of the next line, which will repeat the process and so on. Figure 2 shows an example of the procedure.

Figure 2: Example of the decoding process of all lines within a block using the LIP algorithm

The LIP mode can be used with any of the intra prediction modes available in VTM/BMS. However, the intra mode will be the same for all lines within the block.

A more detailed description of the algorithm can be found in  [1] [2].

# Modifications with respect to JVET-J0014 and JVET-J1023

## Maximum transform size

Since the maximum transform size in VTM/BMS-1.0 has been set to 64×64 [3], the following restrictions apply for the LIP algorithm to ensure that the maximum transform size in the 1-D case is either 64×1 or 1×64:

- If W>64 and H≤64, then only the 1-D vertical split is allowed and therefore it is not necessary to signal the 1-D split type syntax element to the decoder.

- If W≤64 and H>64, then only the 1-D horizontal split is allowed and therefore it is not necessary to signal the 1-D split type syntax element to the decoder.

- If W>64 and H>64, then the LIP mode is not allowed for the block and therefore neither the LIP usage nor the 1-D split type syntax elements are signalled to the decoder.

## Fast encoder estimation with a single tree for luma and chroma

To avoid testing unnecessary lines, a fast encoder estimation algorithm for the LIP tool was presented in [2]. After coding each line of a block using the LIP mode, it compared the accumulated cost of all lines coded with the best cost so far found for the block. If the accumulated cost was larger, then the remaining lines to be coded within the block were skipped, since it was guaranteed that this particular LIP test was not going to win.

In the context of [2], separate trees were used for the luma and chroma components. For this reason, the costs that were used by the estimation algorithm were luma-only costs. In the BMS/VTM software, however, a single tree is utilized for all components. Therefore, whenever the LIP tool is tested for a block, the best cost so far that is used as a reference to stop coding lines would include the chroma component. For this reason, in VTM/BMS-1.0 only the luma part of that cost is used by the algorithm (if the best mode so far is intra).

## 1-D transform core

In [1] and [2], the 1-D transform choice for all blocks using the LIP algorithm was set to DST-VII for the planar mode and DCT-II for all remaining intra modes. In the BMS software the following changes apply:

- BMS configuration: the transform choice has been left unaltered if the Adaptive Multiple Transforms (AMT) tool is enabled. Otherwise, the DCT-II is used for all intra modes.

- BMS as VTM configuration: the DCT-II is used for all intra modes.

# Alternative implementation of the decoding stages

Figure 2 shows an implementation of the decoding stages in which entropy decoding, inverse quantization and inverse transform, intra prediction and reconstruction are carried out for each line in series. Nevertheless, the coefficient decoding, inverse quantization and inverse transform steps do not need to wait for the prediction to be calculated. Therefore, it is proposed to

- decode the coefficients of all lines ,

- inverse quantize and inverse transform all lines and

- calculate the intra prediction and the reconstruction values of all lines

to avoid switching between transform and prediction operations all the time. This process is depicted in Figure 3.

Figure 3: Alternative order proposed to perform the decoding, inverse transform and reconstruction stages

In this new order, the entropy decoding stage has a similar complexity as the one of a regular 2-D block. The transform step can also be optimized in the following way: In the 2-D normal case, the transform consists of three steps: row transform, transposition and finally column transform. In the case of the LIP algorithm and using the new order shown in Figure 3, this can be reduced to only one single step: either the row or the column transform (according to the 1-D split type), as shown in Figure 4. Therefore, the transform has 50% of the complexity of a regular 2-D separable transform.

Figure 4: a) 2-D separable transform stages, b) only the row or the column transform needs to be carried out for the 1-D case

# Experiments

According to the CE3 description [1] and the common test conditions [4], the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with BMS-1.0 software in VTM and BMS configuration. Class F has also been included in all the experiments carried out. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 4.8.5 compiler.

## Tests description

As described in [1], three different types of tests have been simulated:

- Test 1.4.1 : Usage of the Line-Based Intra Coding Mode without fast encoding decisions.

- Test 1.4.2 : Usage of the Line-Based Intra Coding Mode with fast encoding decisions.

- Test 1.4.3 : Usage of the Line-Based Intra Coding Mode with fast encoding decisions and restricting the minimum TU size to 1×8 or 8×1 (instead of 1×4 and 4×1).

## Results for the VTM configuration

Table I: Results for Test 1.4.1 with a VTM AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,79% | -0,14% | -0,17% | 276% | 112% |
| Class A2 | -1,14% | -0,98% | -0,84% | 311% | 113% |
| Class B | -2,02% | -1,95% | -2,43% | 287% | 117% |
| Class C | -3,95% | -3,56% | -4,02% | 324% | 135% |
| Class E | -3,46% | -3,79% | -4,42% | 268% | 120% |
| Overall | -2,34% | -2,15% | -2,47% | 293% | 120% |
| Class D | -4,25% | -3,69% | -4,07% | 336% | 132% |
| Class F (optional) | -8,09% | -7,42% | -7,62% | 299% | 130% |

Table II: Results for Test 1.4.1 with a VTM RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,27% | -0,22% | -0,21% | 131% | 106% |
| Class A2 | -0,37% | -0,29% | -0,13% | 124% | 104% |
| Class B | -0,93% | -0,82% | -1,48% | 125% | 104% |
| Class C | -2,03% | -1,82% | -2,00% | 132% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0,98% | -0,86% | -1,10% | 128% | 105% |
| Class D | -1,84% | -1,69% | -2,26% | 133% | 108% |
| Class F (optional) | -5,75% | -4,94% | -5,22% | 148% | 110% |

Table III: Results for Test 1.4.2 with a VTM AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,58% | -0,01% | -0,04% | 144% | 104% |
| Class A2 | -0,92% | -0,68% | -0,57% | 152% | 106% |
| Class B | -1,72% | -1,70% | -2,10% | 165% | 113% |
| Class C | -3,49% | -3,22% | -3,61% | 199% | 125% |
| Class E | -3,02% | -3,19% | -3,62% | 155% | 111% |
| Overall | -2,00% | -1,83% | -2,09% | 164% | 112% |
| Class D | -3,86% | -3,42% | -3,64% | 219% | 128% |
| Class F (optional) | -7,42% | -7,13% | -7,31% | 196% | 119% |

Table IV: Results for Test 1.4.2 with a VTM RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,18% | -0,14% | -0,08% | 106% | 101% |
| Class A2 | -0,33% | -0,18% | -0,05% | 107% | 102% |
| Class B | -0,80% | -0,71% | -1,20% | 109% | 102% |
| Class C | -1,71% | -1,45% | -1,74% | 115% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0,82% | -0,69% | -0,89% | 110% | 102% |
| Class D | -1,65% | -1,45% | -1,88% | 117% | 108% |
| Class F (optional) | -5,34% | -4,60% | -4,78% | 124% | 105% |

Table V: Results for Test 1.4.3 with a VTM AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,41% | -0,15% | -0,20% | 133% | 105% |
| Class A2 | -0,74% | -0,83% | -0,63% | 139% | 107% |
| Class B | -1,45% | -1,62% | -2,09% | 147% | 109% |
| Class C | -2,51% | -2,72% | -3,15% | 168% | 116% |
| Class E | -2,56% | -3,11% | -3,36% | 139% | 109% |
| Overall | -1,58% | -1,74% | -1,98% | 146% | 110% |
| Class D | -2,52% | -2,93% | -3,05% | 180% | 116% |
| Class F (optional) | -4,58% | -4,78% | -4,95% | 166% | 109% |

Table VI: Results for Test 1.4.3 with a VTM RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,13% | -0,05% | -0,10% | 106% | 101% |
| Class A2 | -0,29% | -0,23% | -0,19% | 105% | 101% |
| Class B | -0,66% | -0,76% | -1,30% | 107% | 103% |
| Class C | -1,25% | -1,45% | -1,70% | 110% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0,63% | -0,70% | -0,95% | 107% | 103% |
| Class D | -1,08% | -1,38% | -2,03% | 111% | 103% |
| Class F (optional) | -3,41% | -3,11% | -3,36% | 117% | 103% |

## Results for the BMS configuration

Table VII: Results for Test 1.4.1 with a BMS AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,22% | -0,09% | -0,19% | 123% | 102% |
| Class A2 | -0,24% | -0,44% | -0,32% | 128% | 101% |
| Class B | -0,60% | -0,77% | -1,12% | 124% | 104% |
| Class C | -1,27% | -2,03% | -2,07% | 126% | 110% |
| Class E | -1,04% | -1,44% | -1,77% | 123% | 104% |
| Overall | -0,70% | -0,99% | -1,15% | 125% | 105% |
| Class D | -1,32% | -2,07% | -2,19% | 127% | 110% |
| Class F (optional) | -5,10% | -6,68% | -7,08% | 135% | 109% |

Table VIII: Results for Test 1.4.1 with a BMS RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,13% | -0,15% | -0,18% | 108% | 101% |
| Class A2 | -0,13% | -0,29% | -0,14% | 109% | 101% |
| Class B | -0,27% | -0,28% | -0,60% | 108% | 99% |
| Class C | -0,61% | -1,16% | -1,29% | 110% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0,30% | -0,49% | -0,61% | 109% | 101% |
| Class D | -0,53% | -1,11% | -1,24% | 111% | 102% |
| Class F (optional) | -3,74% | -5,14% | -5,34% | 117% | 103% |

Table IX: Results for Test 1.4.2 with a BMS AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,09% | 0,02% | -0,04% | 104% | 101% |
| Class A2 | -0,09% | -0,20% | -0,10% | 105% | 100% |
| Class B | -0,30% | -0,40% | -0,57% | 106% | 101% |
| Class C | -0,73% | -1,25% | -1,28% | 108% | 105% |
| Class E | -0,43% | -0,77% | -0,62% | 105% | 102% |
| Overall | -0,35% | -0,55% | -0,57% | 106% | 102% |
| Class D | -0,84% | -1,54% | -1,43% | 110% | 107% |
| Class F (optional) | -4,06% | -5,48% | -6,07% | 117% | 107% |

Table X: Results for Test 1.4.2 with a BMS RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,01% | 0,08% | 0,09% | 101% | 101% |
| Class A2 | -0,02% | -0,08% | 0,04% | 102% | 100% |
| Class B | -0,15% | -0,13% | -0,39% | 103% | 100% |
| Class C | -0,34% | -0,60% | -0,96% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0,15% | -0,20% | -0,36% | 102% | 101% |
| Class D | -0,36% | -1,01% | -0,90% | 104% | 101% |
| Class F (optional) | -3,16% | -4,13% | -4,40% | 109% | 101% |

Table XI: Results for Test 1.4.3 with a BMS AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,06% | 0,02% | -0,02% | 104% | 101% |
| Class A2 | -0,08% | -0,17% | -0,13% | 104% | 100% |
| Class B | -0,24% | -0,33% | -0,54% | 105% | 100% |
| Class C | -0,50% | -0,88% | -0,94% | 107% | 105% |
| Class E | -0,33% | -0,45% | -0,64% | 104% | 103% |
| Overall | -0,26% | -0,39% | -0,49% | 105% | 102% |
| Class D | -0,52% | -1,07% | -1,19% | 108% | 103% |
| Class F (optional) | -2,19% | -3,35% | -3,88% | 112% | 102% |

Table XII: Results for Test 1.4.3 with a BMS RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0,01% | -0,05% | 0,06% | 100% | 100% |
| Class A2 | -0,03% | -0,10% | 0,08% | 102% | 100% |
| Class B | -0,09% | -0,05% | -0,38% | 103% | 100% |
| Class C | -0,19% | -0,53% | -0,77% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0,08% | -0,19% | -0,30% | 102% | 101% |
| Class D | -0,20% | -1,03% | -0,50% | 104% | 100% |
| Class F (optional) | -1,90% | -2,68% | -2,80% | 107% | 101% |

## Results summary

Table XIII: Results summary for the AI configuration

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | EncT | DecT | Y | EncT | DecT |
| Test 1.4.1 | CTC | -2,34% | 293% | 120% | -0,70% | 125% | 105% |
|  | Class F | -8,09% | 299% | 130% | -5,10% | 135% | 109% |
| Test 1.4.2 | CTC | -2,00% | 164% | 112% | -0,35% | 106% | 102% |
|  | Class F | -7,42% | 196% | 119% | -4,06% | 117% | 107% |
| Test 1.4.3 | CTC | -1,58% | 146% | 110% | -0,26% | 105% | 102% |
|  | Class F | -4,58% | 166% | 109% | -2,19% | 112% | 102% |

Table XIV: Results summary for the RA configuration

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | EncT | DecT | Y | EncT | DecT |
| Test 1.4.1 | CTC | -0,98% | 128% | 105% | -0,30% | 109% | 101% |
|  | Class F | -5,75% | 148% | 110% | -3,74% | 117% | 103% |
| Test 1.4.2 | CTC | -0,82% | 110% | 102% | -0,15% | 102% | 101% |
|  | Class F | -5,34% | 124% | 105% | -3,16% | 109% | 101% |
| Test 1.4.3 | CTC | -0,63% | 107% | 103% | -0,08% | 102% | 101% |
|  | Class F | -3,41% | 117% | 103% | -1,90% | 107% | 101% |

# Cross-Check

Cross-checks for the tests have been performed by BBC and Chips&Media.

# Conclusions

The reported results show gains in all the tests and configurations. Class F yields clearly the highest BD-Rates. This is probably due to the fact this screen content is typically richer in sharp edges, which can be better modelled with 1-D partitions. On the other hand, the gains reported in the RA configuration are smaller than in the AI one, since the LIP method is only applied to intra-predicted blocks. Furthermore, the gains in the BMS configuration are lower than in the VTM one, given that there are many more tools in the former than in the latter.

Regarding the individual tests behaviors, Test 1.4.1 obtains the highest gains, as expected. Test 1.4.2 has some losses with respect to 1.4.1 in terms of BD-Rate, but in turn, it has smaller encoding run-times, given that it enables fast encoder estimation decisions. Finally, Test 1.4.3 has lower gains than 1.4.2, since the LIP algorithm is applied on a smaller number of all the blocks available (1×4 and 4×1 TUs are not allowed), thereby reducing the total complexity.

# References

| [1] | G. Van der Auwera, J. Heo y A. Filippov, «Description of Core Experiment 3: Intra Prediction and Mode Coding,» Document JVET-J1023, San Diego, USA, 2018. |
| --- | --- |
| [2] | M. Albrecht, C. Bartnik, S. Bosse, J. Brandenburg, B. Bross, J. Erfurt, V. George, P. Haase, C. Herlmrich, A. Henkel, T. Hinz, S. de Luxán Hernández, S. Kaltenstadler, P. Keydel, H. Kirchhoffer, C. Lehmann, W.-Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, B. Stallenberger, J. Stegemann, K. Suehring, G. Tech, G. Venugopal, S. Walter, A. Wieckowski, T. Wiegand y M. Winken, «Description of SDR, HDR and 360º video coding technology proposal by Fraunhofer HHI,» Document JVET-J0014, San Diego, USA, 2018. |
| [3] | B. Bross, «Versatile Video Coding (Draft 1),» Document JVET-J1001-v2, San Diego, USA, 2018. |
| [4] | J. Boyce, K. Suehring, X. Li y V. Seregin, «JVET common test conditions and software reference configurations,» Document JVET-J1010, San Diego, USA, 2018. |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).