| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0050-v1 |
| --- | --- |

| Title: | CE8 related: Intra Region-based Template Matching for luma and chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gayathri Venugopal Karsten Müller Heiko Schwarz Detlev Marpe Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002 287 gayathri.venugopal@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany |  |  |

_____________________________

# Abstract

This document proposes the region-based template matching method for both luma and chroma for intra prediction. The proposed method is an extension of the algorithm explained in the JVET document JVET-K0048 to the chroma components.  The experimental results of the RTM method for luma and chroma for the all intra (AI) configuration against VTM-1.0 shows an overall BD-rate gain of -2.30% with 120% and 215% decoder and encoder run-time respectively. For the random access (RA) case, the overall coding gain is -0.76% with 103% decoder run-time and 117% encoder run-time, and for the low delay (LB) case it is -0.18% coding gain with 101% decoder run-time and 109% encoder run-time.

# Proposed method

The region-based template matching (RTM) is one of the intra methods considered for studying in the core experiment (CE8) group [1] [2]. The detailed explanation of intra RTM can be found in [3] and the experimental results are reported in [4]. This contribution extends RTM for chroma samples through the intra derived mode of chroma. For the case of the derived mode, the chroma PU uses the same intra prediction mode as the corresponding luma PU.

If the current intra mode of the chroma PU is derived and if the corresponding luma PU mode is RTM, the predictors of the chroma PU are also derived from the luma PU. They are then scaled for final prediction based on the chroma channel format. Thus, the predictors of chroma are not found through direct template matching but calculated from that of the corresponding luma block. Therefore, no separate template matching search is required for the method proposed in this document and the complexity remains the same as that of intra RTM without applying to the chroma channel.

The final prediction signal for chroma is generated in the same way as that of luma, as detailed in [4].

If the width or height of the luma block is greater than the maximum transform size, then the proposed method is not applied to the chroma block.

# Results

According to the CE8 description in [2] and the common test conditions in [5], the proposed method is evaluated for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations with the BMS-1.0 software in the VTM and BMS configuration. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 4.8.5 compiler.

Test8.1               – The intra RTM (luma only), reported in [4]

Test8.1-Related – The proposed intra RTM for luma and chroma components

Table 1: Test8.1-Related against VTM-1.0 for AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.06% | -0.73% | -0.58% | 219% | 119% |
| Class A2 | -2.78% | -2.41% | -2.10% | 217% | 122% |
| Class B | -2.10% | -2.33% | -2.49% | 218% | 120% |
| Class C | -1.86% | -1.82% | -2.16% | 206% | 119% |
| Class E | -3.97% | -3.16% | -3.62% | 214% | 118% |
| Overall | -2.30% | -2.10% | -2.22% | 215% | 120% |
| Class D | -0.74% | -0.84% | -0.78% | 190% | 113% |
| Class F | -8.71% | -8.38% | -8.68% | 213% | 135% |
| Class TGM | -24.30% | -23.96% | -24.04% | 203% | 177% |

Table 2: Test8.1-Related against VTM-1.0 for RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.32% | -0.50% | -0.56% | 122% | 105% |
| Class A2 | -1.10% | -1.14% | -0.89% | 115% | 103% |
| Class B | -0.89% | -1.36% | -1.68% | 117% | 102% |
| Class C | -0.68% | -0.91% | -1.17% | 115% | 103% |
| Overall | -0.76% | -1.02% | -1.16% | 117% | 103% |
| Class D | -0.21% | -0.34% | -0.84% | 112% | 103% |
| Class F | -6.55% | -6.52% | -6.68% | 128% | 106% |
| Class TGM | -14.04% | -13.95% | -14.01% | 120% | 111% |

Table 3: Test8.1-Related against VTM-1.0 for LB

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | -0.18% | -0.49% | -0.27% | 112% | 102% |
| Class C | -0.06% | -0.42% | -0.22% | 109% | 102% |
| Class E | -0.33% | -0.42% | -0.50% | 103% | 100% |
| Overall | -0.18% | -0.45% | -0.31% | 109% | 101% |
| Class D | -0.03% | -0.22% | -0.46% | 106% | 103% |

Table 4: Comparison of the overall gain of Test8.1-Related and Test8.1 against VTM-1.0

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| AI | Test8.1-Related | -2.30% | -2.10% | -2.22% | 215% | 120% |
|  | Test8.1 | -1.41% | -0.30% | -0.53% | 214% | 116% |
| RA | Test8.1-Related | -0.76% | -1.02% | -1.16% | 117% | 103% |
|  | Test8.1 | -0.44% | 0.61% | 0.33% | 117% | 103% |

# Cross check

The cross check of the test 8.1-Related was done by Tencent.

# Conclusions and recommendations

The experimental results of intra region-based template matching method for luma as well as chroma is presented in this document. The proposed approach improves the coding performance of intra RTM reported in JVET-K0048 [4] with more or less the same run-times. The new method shows a coding gain in all classes of the testset for all configurations. This indicates that the intra RTM method is suitable for all types of sequences.  We recommend further study of intra RTM in a CE.

# References

| [1] | M. Albrecht, S. Bosse, J. Brandenburg, M. C. Bartnik, B. Bross, J. Erfurt, V. George, P. Haase, P. Helle, C. Helmrich, A. Henkel, T. Hinz, S. d. L. Hernandez, S. Kaltenstadler, H. Kirchchoffer, C. Lehmann, W. Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, G. Venugopal, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, K. Stallenberger, J. Stegemann, K. Suehring, G. Tech, S. Walter, A. Wieckowski, T. Wiegand and M. Winken, "Description of SDR, HDR, and 360° video coding technology proposal by Fraunhofer HHI, JVET-J0014," San Diego, US, 2018. |
| --- | --- |
| [2] | X. Xu, L. Wang and K. Müller, "Description of Core Experiment 8: Current Picture Referencing, JVET-J1028," San Diego, US, 2018. |
| [3] | G. Venugopal, H. Schwarz, D. Marpe and T. Wiegand, "Intra Region-based Template Matching, JVET-J0039," San Diego, US, 2018. |
| [4] | G. Venugopal, K. Müller, H. Schwarz, D. Marpe and T. Wiegand, "CE8: Intra Region-based Template Matching (Test 8.1), JVET-K0048," Ljubjana, SI, 2018. |
| [5] | J. Boyce, K. Suehring, X. Li and V. Seregin, "JVET common test conditions and software reference configurations, JVET-J1010," San Diego, US, 2018. |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).