| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0051-v1 |
| --- | --- |

| Title: | CE3: Multiple reference line intra prediction (Test 5.4.1, 5.4.2, 5.4.3 and 5.4.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Benjamin Bross Heiko Schwarz Detlev Marpe Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results of sub-CE 3.5 on multi-reference line intra prediction (MRL), test 5.4.1 for MRL with explicit signaling for all block sizes, test 5.4.2 that restricts the encoder for small and narrow blocks as well as test 5.4.3 that additionally restricts the reference line index syntax for small and narrow blocks. The results show that restricting the block sizes increase the average BD-rate Y for VTM all intra from -0.9% in 5.4.1 to -0.5% in 5.4.2 while reducing the encoding runtime from 216% to 145%. Additionally, restricting the syntax in 5.4.3 decreases the BD-rate Y to -0.7% while keeping the encoder runtime almost unchanged. In addition to that, test 5.4.4 combines test 5.4.3 with 5.1.1 on mode dependent multiple reference line intra prediction by using extended reference lines only for angular intra modes. This test 5.4.4 provides -0.7% BD-rate Y with 147% encoding and 101% decoding time for VTM all intra configuration.

# Introduction

Sub-CE 3.5 of CE3 on intra prediction and mode coding as described in JVET-J1023 [1] specifies the tests for multi-reference line intra prediction (MRL). This document reports test results for MRL with explicit signaling as proposed in JVET-J0014 [2]. The originally proposed MRL allows DC, PLANAR and angular intra modes to use extended reference samples with mrl_idx equal to 1 and 3 as depicted in Figure 1. The reference line index mrl_idx is signaled per CU before the intra luma prediction mode. In CE 3.5, the following restrictions have been investigated:

- 5.4.1 no size restrictions, mrl_idx is signaled for each intra CU.

- 5.4.2 restricts the encoder to not use MRL for blocks with

  - one side smaller than half of the other side and

  - one side smaller than 8 luma samples.

- 5.4.3 additionally restricts the mrl_idx syntax (as proposed in CfP response JVET-J0014)

- 5.4.4 combines test 5.1.1 and 5.4.3 by restricting MRL to angular modes as further detailed in section 2.

The all intra and random access results for VTM and BMS configuration are summarized in Table 1 and Table 2.

Figure 1. Directly neighboring (nearest) reference samples in blue and extended reference samples in green used in MRL.

Table 1 Average BD-rates and relative runtimes for all CE3 5.4 tests in VTM configuration

| VTM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| 5.4.1 | -0.91 | -0.37 | -0.42 | 216% | 100% | -0.49 | -0.37 | -0.43 | 117% | 100% |
| 5.4.2 | -0.45 | -0.20 | -0.26 | 145% | 98% | -0.24 | -0.15 | -0.21 | 107% | 101% |
| 5.4.3 | -0.66 | -0.22 | -0.24 | 147% | 99% | -0.35 | -0.17 | -0.24 | 107% | 100% |
| 5.4.4 | -0.72 | -0.31 | -0.31 | 147% | 101% | -0.38 | -0.24 | -0.26 | 108% | 101% |

Table 2 Average BD-rates and relative runtimes for all CE3 5.4 tests in BMS configuration

| VTM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| 5.4.1 | -0.52 | -0.25 | -0.26 | 136% | 100% | -0.29 | -0.07 | -0.22 | 117% | 100% |
| 5.4.2 | -0.30 | -0.18 | -0.17 | 116% | 99% | -0.14 | -0.05 | -0.16 | 109% | 100% |
| 5.4.3 | -0.43 | -0.14 | -0.13 | 116% | 100% | -0.21 | 0.01 | -0.04 | 109% | 100% |
| 5.4.4 | -0.52 | -0.12 | -0.13 | 117% | 100% | -0.25 | -0.02 | -0.09 | 110% | 100% |

# Test 5.4.4 - combining 5.1.1 mode dependent MRL with 5.4.3

Test 5.4.4 adds the mode-dependency tested in 5.1.1. and proposed in JVET-J0065 [3] on top of test 5.4.3 as follows:

- Signal mrl_idx before intra luma pred mode (JVET-J0014)

- Not use extended reference samples (mrl_idx > 0) for DC and PLANAR modes (JVET-J0065)

- Modify MPM list derivation to not include DC or PLANAR mode in case of MRL (mrl_idx > 0) and use vertical, horizontal and bottom-left diagonal mode (2) instead

The difference to 5.1.1 is that the parsing of mrl_idx does not depend on the MPM list construction. In 5.1.1, the intra luma prediction mode has to be reconstructed before parsing mrl_idx because the parsing depends on the intra luma prediction mode. In 5.4.4, the parsing of mrl_idx is independent on the intra luma prediction mode reconstruction.

# Software and Settings

The source code was made available to all CE3 participants in the following branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE3/CE3-5.4

The tests can be carried out by setting the following macros in TypeDef.h:

#define JVET_J0014_MRL                 1 // multiple reference lines (MRL)

#define MRL_FAST_DISABLE_NARROW_BLOCKS 1 // MRL fast mode: exclude blocks with one side smaller

than half of the other side

#define MRL_FAST_ENCODER_ONLY          0 // MRL fast mode: apply restrictions only to search not

to signaling

#define MRL_NO_DC_PLANAR               1 // MRL do not test DC and PLANAR for mrlIdx > 0

#define MRL_MRLIDX_DEPENDENT_MPM       1 // MRL condition intra luma pred mode derivation on mrl_idx

and CommonDef.h:

static const int MRL_FAST_LOG2_MIN_SIZE = 3; // disable partitions with at least one side smaller

than 2^MRL_FAST_LOG2_MIN_SIZE (2: off, 3...5)

Table 3 Macro settings for the four CE3 5.4 tests

| Macro | 5.4.1 | 5.4.2 | 5.4.3 | 5.4.4 |
| --- | --- | --- | --- | --- |
| JVET_J0014_MRL | 1 | 1 | 1 | 1 |
| MRL_FAST_DISABLE_NARROW_BLOCKS | 0 | 1 | 1 | 1 |
| MRL_FAST_ENCODER_ONLY | 0 | 1 | 0 | 0 |
| MRL_NO_DC_PLANAR | 0 | 0 | 0 | 1 |
| MRL_MRLIDX_DEPENDENT_MPM | 0 | 0 | 0 | 1 |
| MRL_FAST_LOG2_MIN_SIZE | 2 | 3 | 3 | 3 |

# Test results

According to the CE3 description in JVET-J1023 [1] and the common test conditions in JVET-J1010 [3], simulations were performed for intra-only (AI) and random-access (RA) configuration with BMS-1.0 software in VTM and BMS configuration. The main parameters of the simulation environment are shown in Table 4.

Table 4 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

## Test 5.4.1 MRL with explicit signaling for all block sizes

Table 5 Result for 5.4.1 VTM configuration

| VTM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4.1 | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| Class A1 | -1.00% | -0.28% | -0.23% | 216% | 101% | -0.55% | -0.35% | -0.38% | 121% | 102% |
| Class A2 | -0.83% | -0.19% | -0.13% | 215% | 100% | -0.40% | -0.38% | -0.19% | 114% | 98% |
| Class B | -0.89% | -0.40% | -0.56% | 217% | 101% | -0.45% | -0.37% | -0.47% | 117% | 101% |
| Class C | -1.08% | -0.45% | -0.61% | 219% | 103% | -0.55% | -0.38% | -0.59% | 116% | 99% |
| Class E | -0.75% | -0.48% | -0.42% | 211% | 97% |  |  |  |  |  |
| Overall | -0.91% | -0.37% | -0.42% | 216% | 100% | -0.49% | -0.37% | -0.43% | 117% | 100% |
| Class D | -0.86% | -0.34% | -0.23% | 222% | 101% | -0.37% | -0.23% | -0.30% | 116% | 97% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 5 Result for 5.4.1 BMS configuration

| BMS | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4.1 | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| Class A1 | -0.41% | -0.29% | -0.29% | 134% | 101% | -0.28% | -0.29% | -0.24% | 119% | 102% |
| Class A2 | -0.45% | -0.09% | -0.02% | 135% | 101% | -0.26% | -0.15% | 0.00% | 116% | 100% |
| Class B | -0.57% | -0.28% | -0.35% | 136% | 99% | -0.32% | -0.01% | -0.26% | 117% | 99% |
| Class C | -0.64% | -0.27% | -0.26% | 136% | 98% | -0.27% | 0.07% | -0.32% | 118% | 100% |
| Class E | -0.46% | -0.26% | -0.28% | 137% | 99% |  |  |  |  |  |
| Overall | -0.52% | -0.25% | -0.26% | 136% | 100% | -0.29% | -0.07% | -0.22% | 117% | 100% |
| Class D | -0.37% | -0.16% | -0.13% | 136% | 100% | -0.15% | -0.41% | -0.38% | 117% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

## Test 5.4.2 MRL with explicit signaling and restricted block sizes (encoder only)

Table 5 Result for 5.4.2 VTM configuration

| VTM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4.2 | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| Class A1 | -0.65% | -0.39% | -0.32% | 148% | 99% | -0.34% | -0.34% | -0.27% | 110% | 107% |
| Class A2 | -0.43% | -0.16% | -0.07% | 145% | 99% | -0.23% | -0.30% | -0.12% | 106% | 99% |
| Class B | -0.48% | -0.05% | -0.27% | 145% | 98% | -0.23% | -0.02% | -0.27% | 106% | 101% |
| Class C | -0.30% | -0.09% | -0.30% | 143% | 98% | -0.17% | -0.08% | -0.14% | 106% | 98% |
| Class E | -0.37% | -0.44% | -0.30% | 144% | 97% |  |  |  |  |  |
| Overall | -0.45% | -0.20% | -0.26% | 145% | 98% | -0.24% | -0.15% | -0.21% | 107% | 101% |
| Class D | -0.14% | 0.02% | -0.09% | 143% | 101% | -0.02% | -0.24% | -0.60% | 106% | 96% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 5 Result for 5.4.2 BMS configuration

| BMS | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4.2 | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| Class A1 | -0.30% | -0.37% | -0.38% | 116% | 100% | -0.17% | -0.10% | -0.28% | 110% | 100% |
| Class A2 | -0.25% | -0.10% | 0.04% | 115% | 100% | -0.13% | -0.23% | -0.10% | 109% | 101% |
| Class B | -0.34% | -0.08% | -0.17% | 116% | 99% | -0.17% | 0.10% | -0.07% | 109% | 100% |
| Class C | -0.27% | -0.22% | -0.10% | 116% | 97% | -0.07% | -0.07% | -0.25% | 108% | 99% |
| Class E | -0.29% | -0.17% | -0.28% | 117% | 98% |  |  |  |  |  |
| Overall | -0.30% | -0.18% | -0.17% | 116% | 99% | -0.14% | -0.05% | -0.16% | 109% | 100% |
| Class D | -0.10% | 0.05% | 0.03% | 116% | 99% | -0.02% | -0.13% | -0.11% | 108% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

## Test 5.4.3 MRL with explicit signaling and restricted block sizes

Table 5 Result for 5.4.3 VTM configuration

| VTM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4.3 | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| Class A1 | -0.83% | -0.36% | -0.19% | 150% | 100% | -0.43% | -0.33% | -0.35% | 110% | 99% |
| Class A2 | -0.63% | -0.17% | -0.09% | 147% | 100% | -0.32% | -0.27% | -0.04% | 106% | 99% |
| Class B | -0.70% | -0.05% | -0.23% | 147% | 99% | -0.35% | -0.07% | -0.33% | 107% | 102% |
| Class C | -0.58% | -0.24% | -0.34% | 144% | 99% | -0.29% | -0.12% | -0.22% | 106% | 99% |
| Class E | -0.59% | -0.42% | -0.36% | 146% | 98% |  |  |  |  |  |
| Overall | -0.66% | -0.22% | -0.24% | 147% | 99% | -0.35% | -0.17% | -0.24% | 107% | 100% |
| Class D | -0.34% | -0.10% | -0.10% | 145% | 100% | -0.13% | -0.06% | -0.09% | 106% | 97% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 5 Result for 5.4.3 BMS configuration

| BMS | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4.3 | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| Class A1 | -0.38% | -0.29% | -0.25% | 116% | 101% | -0.21% | -0.26% | -0.14% | 111% | 101% |
| Class A2 | -0.35% | -0.04% | 0.02% | 116% | 100% | -0.20% | -0.06% | 0.01% | 108% | 101% |
| Class B | -0.48% | -0.05% | -0.10% | 116% | 100% | -0.24% | 0.14% | 0.16% | 109% | 101% |
| Class C | -0.46% | -0.22% | -0.13% | 116% | 98% | -0.17% | 0.10% | -0.26% | 109% | 99% |
| Class E | -0.43% | -0.14% | -0.19% | 118% | 100% |  |  |  |  |  |
| Overall | -0.43% | -0.14% | -0.13% | 116% | 100% | -0.21% | 0.01% | -0.04% | 109% | 100% |
| Class D | -0.22% | 0.03% | -0.17% | 115% | 101% | -0.09% | -0.27% | -0.34% | 108% | 101% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

## Test 5.4.4 Mode dependent MRL with explicit signaling and restricted block sizes

Table 5 Result for 5.4.4 VTM configuration

| VTM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4.4 | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| Class A1 | -0.84% | -0.40% | -0.33% | 150% | 103% | -0.48% | -0.50% | -0.41% | 110% | 100% |
| Class A2 | -0.69% | -0.30% | -0.15% | 147% | 102% | -0.36% | -0.27% | -0.13% | 106% | 101% |
| Class B | -0.74% | -0.11% | -0.29% | 147% | 100% | -0.38% | -0.07% | -0.29% | 108% | 101% |
| Class C | -0.63% | -0.27% | -0.37% | 145% | 100% | -0.32% | -0.26% | -0.20% | 107% | 101% |
| Class E | -0.68% | -0.66% | -0.42% | 145% | 100% |  |  |  |  |  |
| Overall | -0.72% | -0.31% | -0.31% | 147% | 101% | -0.38% | -0.24% | -0.26% | 108% | 101% |
| Class D | -0.36% | -0.11% | -0.26% | 145% | 102% | -0.15% | -0.01% | -0.25% | 107% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 5 Result for 5.4.4 BMS configuration

| BMS | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4.4 | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| Class A1 | -0.48% | -0.12% | -0.22% | 118% | 102% | -0.26% | -0.29% | -0.18% | 111% | 100% |
| Class A2 | -0.43% | -0.07% | 0.06% | 117% | 101% | -0.27% | -0.07% | -0.05% | 109% | 100% |
| Class B | -0.57% | -0.03% | -0.03% | 117% | 98% | -0.28% | 0.14% | 0.03% | 110% | 99% |
| Class C | -0.53% | -0.18% | -0.27% | 117% | 99% | -0.21% | 0.04% | -0.19% | 110% | 100% |
| Class E | -0.53% | -0.23% | -0.21% | 118% | 100% |  |  |  |  |  |
| Overall | -0.52% | -0.12% | -0.13% | 117% | 100% | -0.25% | -0.02% | -0.09% | 110% | 100% |
| Class D | -0.26% | 0.09% | 0.03% | 117% | 100% | -0.14% | -0.16% | -0.34% | 111% | 101% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# Conclusion

From all tested restrictions of MRL with explicit signaling as proposed in JVET-J0014, it can be concluded that, not using and signaling MRL for small and narrow block sizes as well not using MRL for DC and planar intra modes as tested in 5.4.4 provides a good trade-off between coding efficiency and encoder runtime compared to the unrestricted version tested in 5.4.1. For all intra in VTM configuration, 5.4.4 achieves -0.7% BD-rate Y compared to -0.9% for 5.4.1 with 147% encoder runtime compared to 216% for 5.4.1. Hence it is proposed to adopt this MRL technique as tested in 5.4.4 and further investigate MRL techniques w.r.t coding efficiency increase and runtime as well as memory reduction on that basis in a new CE.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-J1023, San Diego, USA, April 2018.

- M. Albrecht, C. Bartnik, S. Bosse, J. Brandenburg, B. Bross, J. Erfurt, V. George, P. Haase, P. Helle, C. Helmrich, A. Henkel, T. Hinz, S. de Luxan Hernandez, S. Kaltenstadler, P. Keydel, H. Kirchhoffer, C. Lehmann, W.-Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, B. Stallenberger, J. Stegemann, K. Suehring, G. Tech, G. Venugopal, S. Walter, A. Wieckowski, T. Wiegand, M. Winken, “Description of SDR, HDR, and 360° video coding technology proposal by Fraunhofer HHI,” JVET-J0014, San Diego, USA, April 2018.

- L. Zhao, X. Zhao, X. Li, S. Liu, “Further investigations on multi-line intra prediction,” JVET-J0065, San Diego, USA, April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).