| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0052-v1 |
| --- | --- |

| Title: | Non-CE4: A study on the affine merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo DR San Diego, CA-92127, USA | Tel: Email: | +1 858 521 5838 minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution studied line buffer usage of the affine merge mode in BMS1.0 and advocated the following three changes to simplify the design, namely, 1) directly using 4-parameter affine motion model to derive the seed vectors for the current PU of the affine merge mode; 3) disabling the affine merge mode for PUs whose width is less than 8 to enable motion data line buffer sharing, and 3) having the affine (merge) mode and the regular merge/skip and AMVP mode share a same motion data line buffer. For 4K video, the proposed changes reduce the line buffer size of the affine merge mode from roughly 18,688 bytes to 320 bytes, without comprising compression efficiency.  Relative to the BMS1.0 anchor, the overall BD-rate changes are 0.01% in RA, -0.05% in LD-B and -0.02% in LD-P, respectively.

# Introduction

In BMS1.0 a 4-parameter based affine motion compensation mode is introduced. The affine motion model supports different motion vectors at different sample positions inside a prediction unit (PU), which can effectively capture more complex motion for better coding efficiency. The BMS1.0 supports both the affine mode in which the seed vectors are explicitly signaled, and the affine merge mode in which the affine motion data is inherited from one of neighboring spatial PUs. This contribution studies the memory usage of the affine (merge) mode, and proposes to reduce the implementation cost by having the affine (merge) mode share a same motion data line with the regular merge/skip mode and AMVP mode,

# Algorithm description and problem statement of affine (merge) mode in BMS1.0

The 4-parameter based affine motion model can be described with the four parameters a, b, e, f as shown in Equation 1:

vx=ax-by+evy=bx+ay+f               Equation 1

Figure 1: 4-parameter affine motion model implemented in BMS1.0

In the 4-parameter affine motion model, the model parameters a,b,e,f are determined by signaling two control point (seed) vectors at the top-left and top-right corner of a PU. As shown in Figure 1, with two seed vectors v0=(v0x,v0y) at sample position (x0,y0) and v1=(v1x,v1y) at sample position(x1,y1), Equation 1 can be rewritten as:

vx=v1x-v0x(x1-x0)(x-x0)-v1y-v0y(x1-x0)(y-y0)+v0xvy=v1y-v0y(x1-x0)x-x0+v1x-v0x(x1-x0)(y-y0)+v0y               Equation 2

Note that in Figure 2,  (x1-x0) equals to the PU width and y1=y0.

Figure 2: Neighboring candidate positions used in the affine merge mode

The affine merge mode is designed to further improve the compression efficiency. Similar to the regular merge/skip mode described above, a PU can also inherit affine motion data from neighbors in the affine merge mode without explicitly signaling of the seed vectors. As shown Figure 2, in the proposed affine merge mode in BMS1.0, a PU searches through the five spatial neighbors in the order of A, B, C, D and E, and inherits the affine motion data from the first available neighboring block using the affine mode.

Figure 3: Inheriting affine motion data from neighbors in BMS1.0 4-parameter affine motion model

Figure 3 illustrates how affine motion data is inherited from a spatial neighbor in the case of 4-parameter affine motion model. In this example, block E is assumed to be the first available neighboring block using the affine mode. The derivation has two steps. In the first step, the seed vector of the bottom-left corner of the neighboring PU E, i.e. vE2=vE2x,vE2y at sample position xE2,yE2, derived by using the seed vectors vE0=vE0x,vE0y at the top-left sample position xE0,yE0, and  vE1=(vE1x,vE1y) at the top-right sample position xE1,yE1 of the neighboring PU containing block E, and by using the 4-parameter affine motion model:

vE2x=vE1x-vE0x(xE1-xE0)(xE2-xE0)-vE1y-vE0y(xE1-xE0)(yE2-yE0)+vE0xvE2y=vE1y-vE0y(xE1-xE0)(xE2-xE0)+vE1x-vE0x(xE1-xE0)(yE2-yE0)+vE0y               Equation 3

In the second step, the seed vectors for the current PU, i.e.  v0=(v0x,v0y) at the top-left corner position (x0,y0) and v1=(v1x,v1y) at the top-right corner position (x1,y1), are derived by using the seed vectors vE0=vE0x,vE0y, vE1=(vE1x,vE1y) and the derived seed vector vE2=vE2x,vE2y, and by using the 6-parameter affine motion model:

v0x=vE1x-vE0xxE1-xE0x0-xE0+vE2x-vE0x(yE2-yE0)(y0-yE0)+vE0xv0y=vE1y-vE0y(xE1-xE0)(x0-xE0)+vE2y-vE0y(yE2-yE0)(y0-yE0)+vE0y               Equation 4

v1x=vE1x-vE0xxE1-xE0x1-xE0+vE2x-vE0x(yE2-yE0)(y1-yE0)+vE0xv1y=vE1y-vE0y(xE1-xE0)(x1-xE0)+vE2y-vE0y(yE2-yE0)(y1-yE0)+vE0y               Equation 5

Figure 4: line buffer storage for affine merge mode of 4-parameter affine motion model

To support the affine merge mode, both PU sizes and seed vectors of neighboring PUs need to be stored. As a picture is divided into CTUs and coded CTU by CTU in raster scan order, this implies an additional line buffer, i.e. an affine motion data line buffer, is needed for storage of the seed vectors and PU sizes of the top neighboring blocks along the CTU boundary. In Figure 4 for example, the neighboring PUs of the previous CTU row containing block E, B and C use the affine mode; to support the affine merge mode of the current CTU, the affine motion data information of those neighboring PUs, which include the seed motion vectors, reference picture indices of the seed vectors and the PU sizes (or sample positions of seed vectors), will need to be stored in the affine motion data line buffer.

Compared to the regular motion data line buffer used for the regular merge/skip candidate list (for merge/skip mode) and AMVP candidate list derivation (for motion vector coding), the size of affine motion data line buffer is significant. If the minimum PU size is 4x4 and the maximum PU size is 128x128, in the regular motion data line buffer, a motion vector (e.g. 4 bytes) and associated reference picture index (e.g. 4 bits) per prediction list (list 0 and list 1) are stored for every four horizontal samples; in the affine motion data line buffer, two/three seed vectors (e.g. 8 bytes for the 4-parameter affine motion model) and associated reference picture index (e.g. 4 bits) per prediction list (list 0 and list 1), and PU width and height (e.g. 5+5 bits) are stored for every N horizontal samples (e.g. N=4 in BMS1.0, N is the minimum  PU width of PUs allowed for using affine merge mode).

For 4K video with horizontal picture size of 4096 luma samples, the size of the regular motion data line buffer is around 9,216 bytes (i.e. 4096*(4+0.5)*2/4); the size of the affine motion data line buffer will be 18,688 bytes (i.e. 4096*(8+0.5)*2/4 + 4096*10/8/4)) for the 4-parameter affine motion model.

# Proposed solution

To reduce memory footprint of the affine motion data line buffer, we propose to re-use the regular motion data line buffer for the affine merge mode. To make the proposed solution work, the following changes have been introduced.

- Using the 4-parameter model to directly compute the seed vectors for the current PU, as opposed to computing the third seed vector of the neighboring PU then using the 6-parameter affine motion model to compute the seed vectors for the current PU implemented in BMS1.0.

- Disabling the affine merge mode for PUs whose width is less than 8.

- Sharing a same motion data line buffer for affine mode, affine merge mode, regular merge/skip mode, and AMVP mode.

Figure 5: shared motion data line buffer for affine merge and regular merge/skip mode (4-parameter affine motion model)

In the proposed method as shown Figure 5, the positions of seed motion vectors of a PU coded in affine mode or affine merge mode are unchanged, i.e. still at the top-left and top-right corner position of the PU. If a PU is coded in affine mode in which seed motion vectors are explicitly signaled, the seed motion vectors at the top-left and top-right corner position of the PU are coded into bitstream; If a PU is coded in affine merge mode in which seed motion vectors are inherited from neighbors, the seed motion vectors at the top-left and top-right corner position of the PU are derived by using seed vectors and positions of the selected neighboring PU.

However, if the selected neighboring PU is located at the top CTU boundary, the motion vectors stored in the regular motion data line buffer rather than the seed motion vectors of the selected PU are used for derivation of the seed motion vectors of the current PU of the affine merge mode. For example in Figure 5, if the current PU uses the affine merge mode and inherits the affine motion data from the neighboring PU E located at top CTU boundary, then motion vectors in the regular motion data line buffer, i.e.  vLE0=vLE0x,vLE0y at sample position xLE0,yLE0 and   vLE1=(vLE1x,vLE1y) at the sample position xLE1,yLE1 with yLE1=yLE0, instead of affine seed motion vectors of the neighboring PU E, i.e. vE0=vE0x,vE0y at the top-left sample position xE0,yE0 and  vE1=(vE1x,vE1y) at the top-right sample position xE1,yE1, are used for the derivation of seed vectors v0 and v1 of the current PU.

In this case, motion vectors vLE0 and vLE1 used for motion compensation of the bottom-left and bottom-right sub-blocks of PU E is calculated by using the 4-parameter affine motion model, and by:

vLE0x=vE1x-vE0x(xE1-xE0)(xLE0-xE0)-vE1y-vE0y(xE1-xE0)(yLE0-yE0)+vE0xvLE0y=vE1y-vE0y(xE1-xE0)(xLE0-xE0)+vE1x-vE0x(xE1-xE0)(yLE0-yE0)+vE0y               Equation 6

vLE1x=vE1x-vE0x(xE1-xE0)(xLE1-xE0)-vE1y-vE0y(xE1-xE0)(yLE1-yE0)+vE0xvLE1y=vE1y-vE0y(xE1-xE0)(xLE1-xE0)+vE1x-vE0x(xE1-xE0)(yLE1-yE0)+vE0y               Equation 7

The seed vectors v0 and v1 of the current PU coded in affine merge mode are derived by

v0x=vLE1x-vLE0x(xLE1-xLE0)(x0-xLE0)-vLE1y-vLE0y(xLE1-xLE0)(y0-yLE0)+vLE0xv0y=vLE1y-vLE0y(xLE1-xLE0)(x0-xLE0)+vLE1x-vLE0x(xLE1-xLE0)(y0-yLE0)+vLE0y               Equation 8

v1x=vLE1x-vLE0x(xLE1-xLE0)(x1-xLE0)-vLE1y-vLE0y(xLE1-xLE0)(y1-yLE0)+vLE0xv1y=vLE1y-vLE0y(xLE1-xLE0)(x1-xLE0)+vLE1x-vLE0x(xLE1-xLE0)(y1-yLE0)+vLE0y               Equation 9

If the selected neighboring PU is not located at the top CTU boundary, e.g. located to the left side of the current PU or located inside the current CTU, then the seed vectors v0 and v1 of the current PU are derived by directly using the seed vectors of the selected neighboring PU and by using the 4-parameter affine motion model.

With the simplifications proposed above, the additional line buffer size required by the affine merge mode can be reduced from 18,688 bytes (i.e. 4096*(8+0.5)*2/4 + 4096*10/8/4)) in BMS1.0 to 320 bytes (i.e. 4096*5/8/8 for storing PU widths of neighboring PUs) for 4K video.

# Experimental results

Tests were carried out using the BMS1.0 software to study the feasibility of the proposed simplifications. The common test conditions defined in [1] were followed.

The following four tests were performed for BMS configurations:

- Affine merge disabled: disabling the affine merge mode in BMS1.0.

- 4-parameter model: using 4-parameter based affine motion model to directly compute the seed vectors for the current PU of affine merge mode.

- Min affine merge PU width 8: test 2) plus disabling affine merge mode for PUs whose width is less than 8.

- Line buffer sharing: test 3) plus line buffer sharing (MVs stored in high precision in the motion data line buffer)

The summary results are provided in Table 1, Table 2 and Table 3 below. As shown Table 1, on average the affine merge mode provides about 0.3% to 0.5% gain relative to the BMS1.0 anchor. The experimental results also revealed that the proposed changes do not compromise the compression efficiency of the affine merge mode.

The detailed results can be found attached spreadsheets. Note that the run time may not be accurate due to the heterogeneous nature of the LSF farm.

Table 1: Summary results of Test 1) & Test 2)

Table 2: Summary results of Test 3) & Test 4)

# Recommendation

The current design of the affine merge mode has a significant impact on memory footprint. It is recommended to study the simplification of the affine (merge) mode to make it more implementation friendly.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, the 10th JVET meeting, April 2018, San Diego, USA.

[2] H. Yang, S. Liu and K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-J1024, the 10th JVET meeting, April 2018, San Diego, USA.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).