| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0053 |
| --- | --- |

| Title: | CE2: Noise Suppression Filter (Test 2.5.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Roman Chernyak Stepin Victor Sergey Ikonin Jianle Chen | Tel: Email: | +7-925-008-12-86 chernyak.roman@huawei.com stepin.victor@huawei.com sergey.ikonin@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE2.5.3 non-local in-loop filter called Noise Suppressor Filter (NSF). Proposed filter is applied after SAO filter and before ALF. The NSF can be applied for both luma and chroma components in one test or for luma component only in another. It is reported that for the random access configuration applying filter for luma and chroma components NSF achieves 1.03% Y BD-rate saving, 2.04% Cb BD-rate saving and 1.89% Cr BD-rate saving with 100% encoding time and 134% decoding time compared to VTM 1.0. For luma-only NSF shows 0.99% Y BD-rate saving with 100% encoding time and 123% decoding time.

# Introduction

Non-local filters are well known and widely used for image de-noising. This proposal describes a method of applying non-local filter as an in-loop filter for video codec. Using non-local structure of each frame NSF suppress quantization noise, which was introduced during encoding process.

# Proposed method

The Noise Suppressor Filter is CTU-based in-loop non-local filter, which is applied after SAO and before ALF, if it is presented, and consists of following main steps:

- Simplified Block Matching

- Collaborative Filtering

- Backward Averaging

- Application Map

Firstly, current CTU is divided onto a set of non-overlapped blocks 8×8 pixels, then each block is classified based on variance value. Blocks with small variance marked as non-processed blocks, and with high – as blocks to be processed. Then for each block to processed, Simplified Block Matching procedure is launching. Figure 1 illustrates this process.

Figure 1 - Simplified Block Matching Procedure

For each block to be processed bi Simplified Block Matching Procedure tries to find up to k (8) the closest blocks to the current within search range 16×16 pixels and restricted by CTU boundaries. Block bi here called as root block. For decrease a number of operations in Block Matching procedure each block to be processed is separated into two sets, based on block location: 1) set for full search; 2) set for template search. In current design every other block at the CTU diagonal is marked for full search; the rest blocks in CTU are marked for template search. Actual block matching performs firstly for full-search marked blocks, where search procedure tries to find up to 8 the closest blocks for current one within search range. Predefined threshold is used for specify similarity level. It may happened that 8 similar blocks cannot be found using this threshold. In this case value k of blocks to be found will be decreased to 4 and then to 2 and 1 if necessary. Also at this step, candidates’ template is being filled by displacements of found blocks. Candidates’ template is used at the second step for blocks marked to template search. At the second step, the procedure checks all candidates from candidates’ template and select the best ones. Eventually, for each block to be processed Simplified Block Matching Procedure build a stack of 8 (or less) closest blocks.

At the second step each stack of blocks related to root blocks bi is being filtered by Collaborative Filter. Input, output and necessary denotations of this process are shown in Figure 2.

Figure 2 – Collaborative Filter

Noise Suppressor’s Collaborative Filter performs filtering along the blocks for each certain pixel’s location. Let S is a stack of similar blocks found by Simplified Block Matching Procedure, S=b0,b1,…,bk . Each block bi comprises of n2 of pixels, bi=p0i,p1i,…,pn2-1i. Then for each pixel position j, 1D array lj is built by following rule: lj=pj0,pj1,…, pjk. The filtering procedure flj,σ performs for each line lj and depends on one parameter – quantization noise level estimation σ. Filtering procedure f performs 1D Hadamard transform of lj and then for each frequency component ωj performs filtering using following equation:

fωj,σ=ωj2ωj2+σ2×ωj,       (1)

where σ=f(qp)

At the final stage, inverse Hadamard transform is performed and filtered pixels from the filtered line lj are storing into filtered blocks b. It should be noted, that filtered stack S in Figure 2 is used for demonstration of filter principle. In implementation accumulation buffer with CTU size is used. All information from different filtered stacks is collected back into the LCU and the collected value for each sample within the LCU is finally normalized. Is should be also noted, that according to equation (1) filtered value ωj is always less than unfiltered value  ωj, because ωj2ωj2+σ2 is always less than 1. Thus, the filtering procedure can be effectively implemented using look-up table.

The following pseudo code illustrates work of filtering step for 4 blocks in a stack as an example.

// reconstructed pixels scan

const int x0 = pIn[p0];

const int x1 = pIn[p1];

const int x2 = pIn[p2];

const int x3 = pIn[p3];

// 1D forward Hadamard transform

const int y0 = x0 + x2;

const int y1 = x1 + x3;

const int y2 = x0 - x2;

const int y3 = x1 - x3;

const int t0 = y0 + y1;

const int t1 = y0 - y1;

const int t2 = y2 + y3;

const int t3 = y2 - y3;

// frequency domain filtering

const int z0 = pTbl[t0];

const int z1 = pTbl[t1];

const int z2 = pTbl[t2];

const int z3 = pTbl[t3];

// backward Hadamard transform

const int iy0 = z0 + z2;

const int iy1 = z1 + z3;

const int iy2 = z0 - z2;

const int iy3 = z1 - z3;

// filtered pixels accumulation

pOut[p0] += iy0 + iy1

pOut[p1] += iy0 - iy1

pOut[p2] += iy2 + iy3

pOut[p3] += iy2 - iy3

Finally, application map is calculated at the encoder side and transmitted to the decoder side. Following options are supported in current design: 1) apply filter for whole frame; 2) do not apply filter for whole frame; 3) apply filter for certain regions. If last option is chosen, application map is being signaled in the bitstream.

# Experimental results

## Objective results

Performance of Noise Suppressor Filter was calculated according to Common Tests Conditions and summarized in following tables. Table 1 provides NSF CTC results, where filter is applied for both luma and chroma components. Table 2 shows results, where filter is used for luma only. Applying filter only for luma allows to decrease decoding complexity on more than 10%.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.71% | -1.85% | -2.04% | 100% | 145% | -0.24% | -0.64% | -0.87% | 100% | 123% |
| Class A2 | -0.52% | -1.24% | -0.91% | 100% | 133% | -0.30% | -0.50% | -0.48% | 101% | 118% |
| Class B | -0.48% | -1.42% | -1.80% | 100% | 133% | -0.25% | -0.81% | -1.03% | 100% | 118% |
| Class C | -0.45% | -1.23% | -1.49% | 100% | 128% | -0.32% | -1.02% | -1.16% | 100% | 116% |
| Class E | -1.02% | -2.38% | -2.82% | 100% | 141% | -0.64% | -1.03% | -0.76% | 101% | 123% |
| Overall | -0.61% | -1.58% | -1.79% | 100% | 135% | -0.34% | -0.81% | -0.90% | 100% | 119% |
| Class D | -0.24% | -0.94% | -1.16% | 100% | 124% | -0.08% | -0.57% | -0.71% | 100% | 114% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.49% | -3.52% | -3.58% | 100% | 157% | -0.59% | -2.04% | -2.53% | 100% | 122% |
| Class A2 | -1.14% | -1.70% | -0.45% | 100% | 131% | -0.87% | -2.61% | -2.21% | 100% | 112% |
| Class B | -1.16% | -1.88% | -1.86% | 100% | 133% | -0.75% | -2.85% | -2.56% | 100% | 115% |
| Class C | -0.46% | -1.37% | -1.75% | 100% | 124% | -0.29% | -1.99% | -2.33% | 100% | 111% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.03% | -2.04% | -1.89% | 100% | 134% | -0.62% | -2.41% | -2.42% | 100% | 114% |
| Class D | -0.12% | -0.83% | -1.13% | 101% | 115% | -0.01% | -1.46% | -1.71% | 100% | 108% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.89% | -1.58% | -1.50% | 100% | 135% | -0.78% | -2.92% | -2.47% | 100% | 120% |
| Class C | -0.48% | -1.74% | -2.17% | 100% | 130% | -0.56% | -2.45% | -2.97% | 100% | 119% |
| Class E | -0.56% | -1.01% | -0.98% | 100% | 115% | -0.89% | -2.17% | -1.30% | 100% | 108% |
| Overall | -0.67% | -1.49% | -1.59% | 100% | 128% | -0.73% | -2.58% | -2.35% | 100% | 117% |
| Class D | -0.28% | -1.31% | -2.27% | 100% | 122% | -0.26% | -1.92% | -3.01% | 100% | 114% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -3.06% | -3.87% | -4.23% | 100% | 147% | -1.05% | -4.11% | -4.14% | 100% | 125% |
| Class C | -1.18% | -2.52% | -3.60% | 100% | 136% | -0.53% | -3.35% | -4.36% | 100% | 120% |
| Class E | -3.76% | -3.78% | -2.03% | 100% | 125% | -1.27% | -4.80% | -2.65% | 100% | 110% |
| Overall | -2.61% | -3.40% | -3.47% | 100% | 138% | -0.93% | -4.03% | -3.84% | 100% | 120% |
| Class D | -0.75% | -2.23% | -3.20% | 100% | 131% | -0.28% | -2.57% | -3.40% | 100% | 116% |

Table 1 – Noise Suppressor Filter for luma and chroma CTC results

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.71% | 0.11% | 0.11% | 100% | 131% | -0.24% | 0.11% | 0.11% | 100% | 115% |
| Class A2 | -0.52% | 0.10% | 0.10% | 101% | 122% | -0.30% | 0.09% | 0.09% | 101% | 112% |
| Class B | -0.48% | 0.11% | 0.11% | 101% | 123% | -0.25% | 0.11% | 0.12% | 100% | 112% |
| Class C | -0.45% | 0.10% | 0.10% | 100% | 121% | -0.32% | 0.12% | 0.12% | 100% | 112% |
| Class E | -1.02% | 0.04% | 0.04% | 100% | 130% | -0.64% | 0.11% | 0.11% | 100% | 116% |
| Overall | -0.61% | 0.09% | 0.09% | 100% | 125% | -0.34% | 0.11% | 0.11% | 100% | 113% |
| Class D | -0.24% | 0.09% | 0.09% | 100% | 116% | -0.08% | 0.12% | 0.13% | 100% | 109% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.44% | 0.06% | 0.15% | 100% | 138% | -0.56% | 0.09% | 0.31% | 100% | 114% |
| Class A2 | -1.06% | -0.07% | -0.14% | 100% | 120% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -1.12% | -0.11% | 0.05% | 100% | 122% | -0.75% | 0.23% | 0.15% | 100% | 109% |
| Class C | -0.41% | 0.06% | -0.06% | 101% | 116% | -0.29% | 0.21% | 0.13% | 100% | 107% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.99% | -0.02% | 0.00% | 100% | 123% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.10% | 0.08% | -0.06% | 100% | 109% | -0.01% | 0.05% | -0.02% | 100% | 104% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.86% | 0.73% | 0.64% | 100% | 124% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.40% | 0.32% | 0.34% | 100% | 120% | -0.51% | 0.73% | 0.42% | 100% | 113% |
| Class E | -0.51% | 1.14% | 0.48% | 100% | 110% | -0.91% | 0.05% | 0.24% | 100% | 105% |
| Overall | -0.62% | 0.69% | 0.50% | 100% | 119% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.31% | 0.12% | 0.27% | 100% | 114% | -0.24% | 0.33% | 0.13% | 100% | 108% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.98% | 0.02% | -0.16% | 100% | 132% | -0.91% | 0.53% | 0.27% | 100% | 117% |
| Class C | -1.07% | -0.03% | -0.10% | 100% | 125% | -0.44% | 0.27% | 0.13% | 100% | 113% |
| Class E | -3.62% | -0.13% | -0.48% | 100% | 117% | -1.17% | 0.46% | -0.03% | 100% | 106% |
| Overall | -2.50% | -0.03% | -0.22% | 100% | 126% | -0.82% | 0.42% | 0.15% | 100% | 113% |
| Class D | -0.67% | -0.23% | 0.02% | 100% | 120% | -0.25% | 0.21% | 0.49% | 100% | 110% |

Table 2 - Noise Suppressor Filter for luma only CTC results

## Subjective results

Besides objective coding gain, the Nosie Suppressor Filter also demonstrates subjective quality improvement. Figure 5 - Figure 9 illustrate subjective improvement of Noise Suppressor Filter on different encoding configurations and video sequences.

| Figure 4 – VTM1.0 RA qp37 CampfireParty RAS 9, Frame#3 | Figure 5 - VTM1.0_NSF RA qp37 CampfireParty RAS 9, Frame#3 |
| --- | --- |
| Figure 6 - VTM1.0 LDB qp37 Johnny frame #205 | Figure 7 - VTM1.0_NSF LDB qp37 Johnny frame #205 |
| Figure 8 - VTM1.0 AI MarketPlace frame #56 | Figure 9 - VTM1.0_NSF AI MarketPlace frame #56 |

# Complexity analysis

Decoder complexity is studied on working station Intel Xeon E5-2687W, 32Gb using MSVC14 compiler and Intel V-TUNE profiler for one bitstream from classes A1, A2 and B decoded by VTM1.0 + NSF luma only processing on RA configuration. Bitstreams from each class are selected based on maximal Y-bd-rate value and decoding time close to average one.

| Sequence | Y-BD-rate | QP | NSF Decoder complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total | Block matching | Filtering | Averaging |
| CampfireParty | -1.11% | 22 | 12.9% | 6.8% | 4.8% | 1.3% |
| DaylightRoad2 | -1.76% | 37 | 11.1% | 5.5% | 4.6% | 0.9% |
| BQTerrace | -2.68% | 32 | 9% | 4.6% | 3.7% | 0.7% |

Table 3 – Noise Suppressor Filter complexity analysis

# Conclusions

Current design of Noise Suppressor in-loop filter reports objective and subjective coding improvements. The most gain is also kept on top of BMS1.0 anchor. It is proposed to adopt NSF in VTM.

# References

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

First RAS is used for tests