| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0059-v1 |
| --- | --- |

| Title: | CE3-related: Reference sample processing for wide-angle intra-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen | Tel: Email: | +7-925-008-23-08 alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

In test CE3-1.3.1 [1], intra prediction from angles extending beyond the traditional 45-degree top-right and bottom-left directions is supported. In this contribution, we propose to change reference sample processing for this technique as compared with CE3-1.3.1. In particular, reference sample buffer used for intra-prediction was extended and reference sample smoothing was added. On top of VTM, the simulation results for this technique reportedly show -0.35% luma BD-rate savings for All Intra (AI) configuration, and -0.20% luma BD-rate savings for Random Access (RA) configuration.

Another technique is aimed at encoder-side complexity reduction. The availability of reference samples can be checked to decide whether the directional intra-prediction mode set should be extended or not. On top of VTM, the simulation results for this modification reportedly show -0.36% of luma BD-rate savings for All Intra (AI) configuration with 2% encoding time reduction on average, and -0.22% of luma BD-rate savings for Random Access (RA) configuration with 1% encoding time reduction on average.

# Introduction

In test CE3-1.3.1 [1], intra prediction from angles extending beyond the traditional 45-degree top-right and bottom-left directions is supported. This contribution presents two mechanisms of reference sample processing applied to the tool studied in CE-1.3.1.

# Proposed techniques

The proposed mechanisms modify reference sample processing steps for wide-angular intra prediction.

2.1. Extending reference samples buffer

The tool in CE-1.3.1 pads reference samples that are outside of the existing reference sample arrays. Size of these arrays that have a length equal to a sum of width and height plus one.

However, for the case of wide-angular prediction, more neighboring samples could be utilized, as it is shown in Figure 1.

Fig. 1 Extending reference samples buffer

This mechanism increases the number of used reference samples to 2*max(width,height)+1.

Another modification proposed in this mechanism is enabling reference sample smoothing when wide angular flag value is enabled. This helps to alleviate the problem of discontinuity shown in Figure 2.

Fig. 2 Problem of discontinuity in case of wide angular prediction.

It could be noticed that two vertically-adjacent predicted samples may use two non-adjacent reference samples. Hence, applying low-pass reference samples filter reduces the negative effect of the increased gap ∆pα.

2.2. Checking availability of reference samples

This mechanism consists in disabling wide angular prediction for the cases when samples that are referenced

# Experimental Results

Both proposed mechanisms are implemented on top of VTM-1.0 and BMS-1.0. The experimental results of All Intra and Random Access configurations for extending reference samples buffer is shown in Table 1. Mechanism of checking reference samples availability being used jointly with the mechanism of extending reference samples buffer is reported to provide results as shown in Table 2.

All the simulations were performed in accordance with the JVET CTC [2].

Tables 1-2 are subject to be updated when simulation results are available. More detail results can be found in the accompanying Excel tables.

## Table 1. Experimental results for extending reference samples buffer (CTC)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.19% | -0.32% | 108% | 103% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | -0.30% | -0.34% | -0.35% | 108% | 103% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.33% | -0.56% | -0.51% | 108% | 104% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.34% | -0.54% | -0.63% | 107% | 102% | -0.26% | -0.26% | -0.32% | 103% | 101% |
| Class E | -0.53% | -0.59% | -0.49% | 107% | 104% | -0.38% | -0.48% | -0.47% | 104% | 102% |
| Overall | -0.35% | -0.46% | -0.47% | 108% | 103% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.35% | -0.55% | -0.62% | 106% | 99% | -0.26% | -0.31% | -0.31% | 103% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.23% | -0.28% | 103% | 103% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | -0.20% | -0.44% | -0.29% | 103% | 102% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.20% | -0.36% | -0.52% | 102% | 102% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.19% | -0.34% | -0.58% | 102% | 101% | -0.12% | -0.13% | -0.41% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.20% | -0.35% | -0.44% | 102% | 102% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.17% | -0.19% | -0.41% | 102% | 99% | -0.10% | -0.40% | -0.41% | 100% | 100% |

## Table 2. Experimental results for extending reference samples buffer and reference samples availability checking (CTC)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.26% | -0.36% | 105% | 102% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | -0.30% | -0.37% | -0.34% | 105% | 102% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.33% | -0.52% | -0.50% | 105% | 103% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.36% | -0.51% | -0.71% | 105% | 103% | -0.26% | -0.26% | -0.23% | 101% | 101% |
| Class E | -0.52% | -0.58% | -0.43% | 105% | 103% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.36% | -0.46% | -0.48% | 105% | 103% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.36% | -0.51% | -0.73% | 104% | 100% | -0.25% | -0.27% | -0.35% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.37% | -0.35% | 104% | 104% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | -0.21% | -0.26% | -0.26% | 104% | 104% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.21% | -0.37% | -0.53% | 104% | 104% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.22% | -0.34% | -0.49% | 104% | 105% | -0.12% | -0.01% | -0.42% | 100% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.22% | -0.34% | -0.43% | 104% | 104% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.19% | -0.48% | -0.73% | 104% | 104% | -0.11% | -0.18% | -0.15% | 100% | 102% |

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-B1010, “JVET common test conditions and software reference configurations”, April 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).