| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0060-v1 |
| --- | --- |

| Title: | CE3: Variable number of directional intra modes (Tests 1.1.1 and 1.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Vadim Seregin Adarsh Krishnan Ramasubramonian Marta Karczewicz | Email: | geertv@qti.qualcomm.com vseregin@qti.qualcomm.com aramasub@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The results of two CE3 tests are reported. In test CE3-1.1.1 the number of directional intra modes is increased to 129 for all block sizes, while in test CE3-1.1.2 a variable number of directional modes (33, 65, or 129) is used depending on two thresholds signalled in the SPS. The first threshold indicates the largest block area with 33 angular directions, and the second threshold indicates the largest block area with 65 angular directions, all other blocks use 129 angular directions.

For test CE3-1.1.1 the reported AI BD-rates are −1.30% (Y), −1.43% (U), −1.42% (V) with runtimes 119% (Enc) and 101% (Dec) over BMS-VTM-1.0, and the reported RA BD-rates are −0.60% (Y), −0.55% (U), −0.56% (V) with runtimes 104% (Enc) and 101% (Dec) over BMS-VTM-1.0. The reported AI BD-rates are 0.04% (Y), −0.10% (U), −0.13% (V) with runtimes 102% (Enc) and 100% (Dec) over BMS-1.0, and the reported RA BD-rates are 0.03% (Y), −0.05% (U), −0.13% (V) with runtimes 101% (Enc) and 99% (Dec) over BMS-1.0.

For test CE3-1.1.2 the reported AI BD-rates are −1.44% (Y), −1.45% (U), −1.48% (V) with runtimes 116% (Enc) and 101% (Dec) over BMS-VTM-1.0, and the reported RA BD-rates are −0.64% (Y), −0.51% (U), −0.54% (V) with runtimes 103% (Enc) and 101% (Dec) over BMS-VTM-1.0. The reported AI BD-rates are −0.14% (Y), −0.13% (U), −0.18% (V) with runtimes 101% (Enc) and 101% (Dec) over BMS-1.0, and the reported RA BD-rates are −0.04% (Y), −0.03% (U), −0.15% (V) with runtimes 100% (Enc), 100% (Dec) over BMS-1.0.

The results of test CE3-1.1.1 and its source code have been cross-checked by Sony [1] and the results of test CE3-1.1.2 and its source code by Sharp [1].

# Proposal

To capture the arbitrary edge directions presented in natural video, the number of directional intra prediction modes is increased. In this experiment, 33, 65 and 129 directional intra modes are tested. When 33 or 65 angular modes are used, they are respectively identical to the modes defined in HEVC and JEM7 [2], respectively. Included in this experiment is the case where the number of modes increases with the block size and are equal to 33, 65 or 129 depending on threshold values. This way small blocks use fewer angular modes resulting in a reduction of bits. These denser directional intra prediction modes apply for both luma and chroma intra predictions. The planar and DC modes remain the same as in HEVC. The directional mode increase from 33 to 65 is depicted as red dotted arrows in Figure 1.

Figure 1. 67 intra modes, including 65 directional modes + planar + DC

For variable directional intra modes, there are two block size threshold values, which are signaled in the SPS as log2 values minus 4 and minus 6, respectively. The first threshold indicates the largest block area with 33 angular directions, and the second threshold indicates the largest block area with 65 angular directions, all other blocks use 129 angular directions. In the default setting, thresholds are signalled as 4 and 6, respectively, and for higher resolution pictures (width greater than 2000 pixels) thresholds are set to 5 and 8.

| Location | Description |
| --- | --- |
| Sequence Header | A threshold indicates the largest block area with 33 angular modes. |
| Sequence header | A threshold indicates the largest block area with 65 angular modes. |

# Test scenarios

The following tests were included in CE3 [4]:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.1.1 | Use 129 directional modes for all blocks | Geert Van der Auwera (Qualcomm) | Steve Keating (Sony) |
| 1.1.2 | Use variable number of directional modes (33, 65, or 129) depending on block size | Geert Van der Auwera (Qualcomm) | Yukinobu Yasugi (Sharp) |

# Results

The following results are obtained according to the JVET common test conditions [7] and the CE3 test guidelines [8]. The BMS1.0 software is used for both the BMS and VTM results. The anchors are also generated using the BMS1.0 software.

## Test 1.1.1

The number of directional intra modes is fixed to 129 for all block sizes. The BD-rate gains and runtimes are summarized below.

### ‘All Intra’ and ‘Random Access’ results over BMS-VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.62% | -0.78% | -0.62% | 118% | 101% |
| Class A2 | -1.15% | -1.36% | -1.28% | 117% | 98% |
| Class B | -1.58% | -1.75% | -1.83% | 119% | 99% |
| Class C | -1.03% | -0.90% | -1.02% | 120% | 105% |
| Class E | -2.01% | -2.33% | -2.19% | 119% | 102% |
| Overall | -1.30% | -1.43% | -1.42% | 119% | 101% |
| Class D | -0.22% | -0.16% | -0.06% | 124% | 108% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.43% | -0.25% | 103% | 100% |
| Class A2 | -0.79% | -0.84% | -0.68% | 103% | 99% |
| Class B | -0.74% | -0.75% | -0.79% | 104% | 101% |
| Class C | -0.39% | -0.16% | -0.41% | 105% | 103% |
| Overall | -0.60% | -0.55% | -0.56% | 104% | 101% |
| Class D | -0.06% | 0.08% | -0.07% | 105% | 103% |

### ‘All Intra’ and ‘Random Access’ over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.22% | 0.07% | 0.04% | 101% | 101% |
| Class A2 | 0.07% | -0.10% | -0.05% | 102% | 101% |
| Class B | -0.09% | -0.30% | -0.35% | 103% | 102% |
| Class C | 0.12% | 0.05% | 0.05% | 103% | 100% |
| Class E | -0.07% | -0.16% | -0.23% | 99% | 97% |
| Overall | 0.04% | -0.10% | -0.13% | 102% | 100% |
| Class D | 0.29% | 0.28% | 0.32% | 102% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.05% | 0.02% | 102% | 100% |
| Class A2 | -0.05% | -0.19% | -0.05% | 101% | 100% |
| Class B | -0.03% | -0.14% | -0.29% | 102% | 101% |
| Class C | 0.13% | 0.15% | -0.11% | 100% | 96% |
| Overall | 0.03% | -0.05% | -0.13% | 101% | 99% |
| Class D | 0.19% | 0.24% | 0.04% | 101% | 100% |

## Test 1.1.2

Variable number of directional modes (33, 65, or 129) are used depending on block size. The BD-rate gains and runtimes are summarized below.

### ‘All Intra’ and ‘Random Access’ results over BMS-VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.83% | -0.76% | -0.72% | 114% | 101% |
| Class A2 | -1.24% | -1.27% | -1.27% | 113% | 100% |
| Class B | -1.65% | -1.71% | -1.84% | 118% | 101% |
| Class C | -1.22% | -1.11% | -1.23% | 118% | 105% |
| Class E | -2.17% | -2.34% | -2.20% | 116% | 100% |
| Overall | -1.44% | -1.45% | -1.48% | 116% | 101% |
| Class D | -0.51% | -0.38% | -0.45% | 121% | 108% |
| Class F (optional) | -1.68% | -1.40% | -1.42% | 119% | 105% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -0.50% | -0.39% | 102% | 101% |
| Class A2 | -0.80% | -0.71% | -0.60% | 103% | 99% |
| Class B | -0.77% | -0.57% | -0.68% | 104% | 102% |
| Class C | -0.49% | -0.29% | -0.41% | 104% | 103% |
| Overall | -0.64% | -0.51% | -0.54% | 103% | 101% |
| Class D | -0.21% | 0.04% | -0.19% | 104% | 102% |
| Class F (optional) | -1.04% | -0.88% | -0.76% | 105% | 101% |

### ‘All Intra’ and ‘Random Access’ results over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.10% | 0.01% | 100% | 101% |
| Class A2 | -0.09% | -0.12% | -0.09% | 100% | 101% |
| Class B | -0.20% | -0.25% | -0.40% | 101% | 101% |
| Class C | -0.12% | -0.13% | -0.08% | 101% | 100% |
| Class E | -0.31% | -0.20% | -0.21% | 101% | 101% |
| Overall | -0.14% | -0.13% | -0.18% | 101% | 101% |
| Class D | -0.08% | 0.11% | 0.15% | 101% | 101% |
| Class F (optional) | -0.59% | -0.45% | -0.48% | 101% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.13% | 0.09% | 100% | 100% |
| Class A2 | -0.09% | -0.19% | -0.03% | 101% | 101% |
| Class B | -0.09% | -0.07% | -0.25% | 101% | 101% |
| Class C | -0.01% | 0.20% | -0.29% | 100% | 97% |
| Overall | -0.04% | -0.03% | -0.15% | 100% | 100% |
| Class D | 0.02% | 0.01% | 0.01% | 100% | 100% |
| Class F (optional) | -0.28% | -0.23% | -0.31% | 101% | 101% |

# Acknowledgement

We are grateful to Sony and Sharp for conducting the cross-checks for these tests.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “CE3: Summary Report on Intra Prediction and Mode Coding,” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K0023

- J. Chen, E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1)”, 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1002

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7,” 7th JVET Meeting, Torino, Italy, July 2017, JVET-G1001

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1010

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1023

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).