| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0062-v1 |
| --- | --- |

| Title: | CE3: Intra reference sample interpolation (Tests 2.2.1, 2.3.1, 2.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Vadim Seregin Adarsh Krishnan Ramasubramonian Marta Karczewicz | Email: | geertv@qti.qualcomm.com vseregin@qti.qualcomm.com aramasub@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The result of test CE3-2.2.1, on using a 4-tap cubic interpolation filter for extending the main line of reference samples for directional intra prediction, is reported.

For test CE3-2.2.1 the reported BD-rates over BMS-VTM-1.0 are:

- AI: −0.05% (Y), −0.08% (U), −0.11% (V) with runtimes 102% (Enc) and 100% (Dec)

- RA: −0.04% (Y), −0.01% (U), −0.02% (V) with runtimes 101% (Enc) and 100% (Dec)

And the reported BD-rates over BMS-1.0 are:

- AI: −0.01% (Y), −0.04% (U), −0.03% (V) with runtimes 100% (Enc) and 99% (Dec)

- RA: 0.00% (Y), 0.00% (U), −0.07% (V) with runtimes 100% (Enc) and 99% (Dec)

The result of test CE3-2.3.1, on using a 6-tap cubic interpolation filter together with a 4-tap Gaussian interpolation filter for the directional prediction from intra reference samples, is reported.

For test CE3-2.3.1 the reported BD-rates over BMS-VTM-1.0 are:

- AI: −0.57% (Y), −0.39% (U), −0.43% (V) with runtimes 107% (Enc) and 103% (Dec)

- RA: −0.29% (Y), −0.64% (U), −0.75% (V) with runtimes 101% (Enc) and 100% (Dec)

And the reported BD-rates over BMS-1.0 are:

- AI: −0.44% (Y), −0.48% (U), −0.53% (V) with runtimes 101% (Enc) and 101% (Dec)

- RA: −0.18% (Y), −0.42% (U), −0.47% (V) with runtimes 101% (Enc) and 99% (Dec)

The result of test CE3-2.3.2, which is the combination of tests 2.2.1 and 2.3.1, is reported.

For test CE3-2.3.2 the reported BD-rates over BMS-VTM-1.0 are:

- AI: −0.63% (Y), −0.49% (U), −0.48% (V) with runtimes 107% (Enc) and 103% (Dec)

- RA: −0.32% (Y), −0.71% (U), −0.81% (V) with runtimes 102% (Enc) and 101% (Dec)

And the reported BD-rates over BMS-1.0 are:

- AI: −0.46% (Y), −0.58% (U), −0.58% (V) with runtimes 102% (Enc) and 100% (Dec)

- RA: −0.19% (Y), −0.47% (U), −0.51% (V) with runtimes 102% (Enc) and 100% (Dec)

The AI and RA results of test CE3-2.2.1 and its source code have been cross-checked by Sony [1]. The results of tests CE3-2.3.1 and CE-2.3.2 and the source code have been cross-checked by LGE [1].

# Proposal

## Test CE3-2.2.1: 4-tap cubic filter for extending reference samples

Interpolation filters are used for angular intra prediction in two cases. The first case is the derivation of the extended intra reference samples, and the second case is the derivation of the predicted samples from the references samples.

To derive the extended luma and chroma intra reference samples, the 4-tap cubic interpolation filter is used instead of the nearest neighbor rounding. As shown in the example in Figure 1, to derive the value of reference sample P, the 4-tap cubic interpolation filter is used, while in JEM or HEVC, the P sample is directly set as X1.

Figure 1. Example of interpolation to derive an extended intra reference sample

## Test CE3-2.3.1: 6-tap cubic interpolation filter

To generate a prediction sample from luma and chroma reference samples, two interpolation filters can be used. If a block’s width or height is greater than 8 pixels, the 4-tap Gaussian filter from JEM7 [2] is used; otherwise, a 6-tap cubic interpolation filter is used which is an extension of the 4-tap cubic filter in JEM7. The interpolation filter accuracy is equal to 1/32 fractional pixels.

## Test CE3-2.3.2: Combination of tests 2.2.1 and 2.3.1

In this test the combination of test 2.2.1 and 2.3.1 is evaluated.

# Test scenarios

The following tests were included in CE3 [4]:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.2.1 | 4-tap cubic interpolation filter for extending reference samples | Geert Van der Auwera (Qualcomm) | Steve Keating (Sony) |
| 2.3.1 | 6-tap cubic interpolation filter prediction from reference samples | Geert Van der Auwera (Qualcomm) | Sunmi Yoo (LGE) |
| 2.3.2 | Combine tests 2.2.1 + 2.3.1 | Geert Van der Auwera (Qualcomm) | Sunmi Yoo (LGE) |

# Results

The following results are obtained according to the JVET common test conditions [3] and the CE3 test guidelines [4]. The BMS1.0 software is used for both the BMS and VTM results. The anchors are also generated using the BMS1.0 software.

## Test CE3-2.2.1

### ‘All Intra’, ‘Random Access’ results over BMS-VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.06% | -0.12% | 102% | 100% |
| Class A2 | -0.05% | -0.06% | -0.06% | 102% | 98% |
| Class B | -0.05% | -0.07% | -0.03% | 102% | 99% |
| Class C | -0.02% | -0.12% | -0.17% | 103% | 103% |
| Class E | -0.08% | -0.11% | -0.17% | 101% | 103% |
| Overall | -0.05% | -0.08% | -0.11% | 102% | 100% |
| Class D | -0.05% | 0.00% | -0.03% | 106% | 108% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.03% | -0.03% | 101% | 100% |
| Class A2 | -0.06% | 0.03% | 0.08% | 101% | 99% |
| Class B | -0.03% | -0.02% | -0.05% | 101% | 101% |
| Class C | -0.02% | -0.06% | -0.05% | 101% | 102% |
| Overall | -0.04% | -0.01% | -0.02% | 101% | 100% |
| Class D | 0.00% | 0.03% | -0.01% | 102% | 103% |

### ‘All Intra’, ‘Random Access’ results over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.05% | -0.09% | 100% | 98% |
| Class A2 | 0.00% | -0.03% | -0.04% | 101% | 98% |
| Class B | -0.02% | 0.03% | 0.01% | 100% | 100% |
| Class C | 0.03% | -0.02% | 0.05% | 100% | 98% |
| Class E | -0.04% | -0.19% | -0.11% | 99% | 98% |
| Overall | -0.01% | -0.04% | -0.03% | 100% | 99% |
| Class D | 0.00% | 0.18% | 0.10% | 100% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.04% | 100% | 99% |
| Class A2 | -0.01% | 0.05% | 0.12% | 100% | 100% |
| Class B | -0.02% | -0.07% | -0.08% | 101% | 101% |
| Class C | 0.04% | 0.06% | -0.21% | 99% | 96% |
| Overall | 0.00% | 0.00% | -0.07% | 100% | 99% |
| Class D | 0.01% | 0.05% | -0.18% | 100% | 100% |

## Test CE3-2.3.1

### ‘All Intra’, ‘Random Access’ results over BMS-VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.12% | -0.27% | 107% | 102% |
| Class A2 | -0.14% | 0.00% | -0.20% | 106% | 100% |
| Class B | -0.53% | -0.41% | -0.36% | 106% | 101% |
| Class C | -1.11% | -1.34% | -1.47% | 108% | 107% |
| Class E | -0.75% | 0.24% | 0.42% | 106% | 105% |
| Overall | -0.57% | -0.39% | -0.43% | 107% | 103% |
| Class D | -0.74% | -0.71% | -0.86% | 110% | 109% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.29% | -0.42% | 102% | 98% |
| Class A2 | -0.14% | -0.49% | -0.63% | 102% | 100% |
| Class B | -0.32% | -0.64% | -0.77% | 102% | 102% |
| Class C | -0.50% | -1.01% | -1.05% | 100% | 100% |
| Overall | -0.29% | -0.64% | -0.75% | 101% | 100% |
| Class D | -0.35% | -0.67% | -0.90% | 101% | 100% |

### ‘All Intra’, ‘Random Access’ results over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.34% | -0.41% | 101% | 101% |
| Class A2 | -0.12% | -0.19% | -0.22% | 101% | 101% |
| Class B | -0.47% | -0.55% | -0.58% | 102% | 102% |
| Class C | -0.82% | -0.77% | -0.78% | 100% | 100% |
| Class E | -0.53% | -0.39% | -0.57% | 101% | 100% |
| Overall | -0.44% | -0.48% | -0.53% | 101% | 101% |
| Class D | -0.53% | -0.61% | -0.55% | 101% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.42% | -0.27% | 102% | 101% |
| Class A2 | -0.05% | -0.32% | -0.35% | 101% | 100% |
| Class B | -0.23% | -0.58% | -0.58% | 102% | 101% |
| Class C | -0.29% | -0.31% | -0.57% | 98% | 94% |
| Overall | -0.18% | -0.42% | -0.47% | 101% | 99% |
| Class D | -0.23% | -0.63% | -0.13% | 99% | 99% |

## Test CE3-2.3.2

### ‘All Intra’, ‘Random Access’ results over BMS-VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.17% | -0.34% | 107% | 102% |
| Class A2 | -0.19% | -0.08% | -0.24% | 107% | 99% |
| Class B | -0.57% | -0.49% | -0.42% | 108% | 102% |
| Class C | -1.15% | -1.47% | -1.54% | 108% | 105% |
| Class E | -0.85% | 0.11% | 0.45% | 107% | 104% |
| Overall | -0.63% | -0.49% | -0.48% | 107% | 103% |
| Class D | -0.78% | -0.70% | -0.94% | 111% | 109% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.35% | -0.44% | 102% | 100% |
| Class A2 | -0.15% | -0.51% | -0.61% | 102% | 99% |
| Class B | -0.34% | -0.79% | -0.96% | 102% | 101% |
| Class C | -0.52% | -1.03% | -1.05% | 103% | 104% |
| Overall | -0.32% | -0.71% | -0.81% | 102% | 101% |
| Class D | -0.39% | -0.71% | -1.08% | 104% | 103% |

### ‘All Intra’, ‘Random Access’ results over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.45% | -0.46% | 102% | 99% |
| Class A2 | -0.14% | -0.27% | -0.27% | 101% | 98% |
| Class B | -0.49% | -0.59% | -0.66% | 102% | 100% |
| Class C | -0.76% | -0.88% | -0.70% | 101% | 101% |
| Class E | -0.61% | -0.62% | -0.74% | 101% | 101% |
| Overall | -0.46% | -0.58% | -0.58% | 102% | 100% |
| Class D | -0.55% | -0.58% | -0.64% | 102% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.40% | -0.19% | 102% | 100% |
| Class A2 | -0.07% | -0.38% | -0.30% | 102% | 101% |
| Class B | -0.25% | -0.58% | -0.81% | 102% | 101% |
| Class C | -0.27% | -0.47% | -0.52% | 100% | 97% |
| Overall | -0.19% | -0.47% | -0.51% | 102% | 100% |
| Class D | -0.25% | -0.50% | -0.19% | 101% | 101% |

# Acknowledgement

We are very grateful to Sony and LGE for conducting the cross-checks for these tests.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “CE3: Summary Report on Intra Prediction and Mode Coding,” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K0023

- J. Chen, E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1)”, 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1002

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7,” 7th JVET Meeting, Torino, Italy, July 2017, JVET-G1001

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1010

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1023

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).