| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0063-v1 |
| --- | --- |

| Title: | CE3: Simplified PDPC (Test 2.4.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Vadim Seregin Amir Said Adarsh Krishnan Ramasubramonian Marta Karczewicz | Email: | geertv@qti.qualcomm.com vseregin@qti.qualcomm.com asaid@qti.qualcomm.com aramasub@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The results of CE3-2.4.1 on position dependent intra prediction combination (PDPC) are reported. PDPC is applied to the following intra modes without signalling: planar, DC, horizontal, vertical, bottom-left angular mode and adjacent angular modes, top-right angular mode and adjacent angular modes. PDPC computes a linear combination of neighboring reference samples with weights dependent on the position within the block. The weights are computed by using shift arithmetic only and no divisions nor special look-up tables are used. With PDPC prediction, it is stated that additional across the boundary filtering (secondary boundary) is not needed.

For test CE3-2.4.1 the reported BD-rates over BMS-VTM-1.0 are:

- AI: −0.97% (Y), −0.14% (U), −0.01% (V) with runtimes 109% (Enc) and 107% (Dec)

- RA: −0.48% (Y), 0.39% (U), 0.46% (V) with runtimes 101% (Enc) and 101% (Dec)

- LB: −0.24% (Y), 0.38% (U), −0.02% (V) with runtimes 102% (Enc) and 100% (Dec)

The reported BD-rates over BMS-1.0 are:

- AI: −1.35% (Y), −0.94% (U), −0.81% (V) with runtimes 103% (Enc) and 105% (Dec)

- RA: −0.70% (Y), −0.41% (U), −0.56% (V) with runtimes 102% (Enc) and 101% (Dec)

- LB: −0.36% (Y), −0.46% (U), −0.25% (V) with runtimes 101% (Enc) and 100% (Dec)

The AI and RA results of test CE3-2.4.1 and its source code have been cross-checked by Nokia [1].

# Proposal

Position dependent intra prediction combination (PDPC) was proposed in [1] and further simplified in [2][3][4]. PDPC is applied to the following intra modes without signalling: planar, DC, horizontal, vertical, bottom-left angular mode and its x adjacent angular modes, and top-right angular mode and its x adjacent angular modes. The value x depends on the number of angular modes and is equal to 4 or 8 for respectively 33 (VTM1) or 65 (BMS1) angular modes.

The prediction sample pred(x,y) located at position (x, y) is predicted using an intra prediction mode (DC, planar, angular) and a linear combination of reference samples according to the PDPC expression:

pred(x,y) = ( wL × R-1,y + wT × Rx,-1 – wTL × R-1,-1 +  (64 – wL – wT + wTL) × pred(x,y) + 32 )  >>  6,    (Eq. 1)

where Rx,-1, R-1,y represent the reference samples located at the top and left of current sample (x, y), respectively, and R-1,-1 represents the reference sample located at the top-left corner of the current block.

For the DC mode the weights are calculated as follows for a block with dimensions width and height:

wT = 32 >> ( ( y<<1 ) >> shift ), wL = 32 >> ( ( x<<1 ) >> shift ), wTL = ( wL>>4 ) + ( wT>>4 ),

with shift = ( log2( width ) – 2 + log2( height ) – 2 + 2 ) >> 2.

For planar mode wTL = 0, while for horizontal mode wTL = wT and for vertical mode wTL = wL. The PDPC weights can be calculated with adds and shifts only. The value of pred(x,y) can be computed in a single step using Eq. 1.

Figure 1 illustrates DC mode PDPC weights (wL, wT, wTL) for (0, 0) and (1, 0) positions inside one 4x4 block.

Figure 1: Examples of PDPC weights for sample positions (0,0) and (1,0) inside a 4x4 block.

If PDPC is applied to DC, planar, horizontal, and vertical intra modes, additional boundary filters are not needed, such as the HEVC DC mode boundary filter or horizontal/vertical mode edge filters.

Figure 2(a) illustrates the definition of reference samples Rx,-1, R-1,y and R-1,-1 for PDPC applied to the top-right diagonal mode. The prediction sample pred(x’, y’) is located at (x’, y’) within the prediction block. The coordinate x of the reference sample Rx,-1 is given by: x = x’ + y’ + 1, and the coordinate y of the reference sample R-1,y is similarly given by: y = x’ + y’ + 1.

The PDPC weights for the top-right diagonal mode are given by:

wT = 16 >> ( ( y’<<1 ) >> shift ), wL = 16 >> ( ( x’<<1 ) >> shift ), wTL = 0.

| - Diagonal top-right mode | - Diagonal bottom-left mode |
| --- | --- |
| - Adjacent diagonal top-right mode | - Adjacent diagonal bottom-left mode |

Figure 2: Definition of samples used by PDPC applied to diagonal and adjacent angular intra modes.

Similarly, Figure 2(b) illustrates the definition of reference samples Rx,-1, R-1,y and R-1,-1 for the application of PDPC to the bottom-left diagonal mode. The coordinate x of the reference sample Rx,-1 is given by:

x = x’ + y’ + 1, and the coordinate y of the reference sample R-1,y is: y = x’ + y’ + 1.

The PDPC weights for the bottom-left diagonal mode are:

wT = 16 >> ( ( y’<<1 ) >> shift ), wL = 16 >> ( ( x’<<1 ) >> shift ), wTL = 0.

The case of an adjacent top-right diagonal mode is illustrated in Figure 2(c). The PDPC weights for an adjacent top-right diagonal mode are: wT = 32 >> ( ( y’<<1 ) >> shift ), wL = 0, wTL = 0.

Similarly, the case of an adjacent bottom-left diagonal mode is illustrated in Figure 2(d). The PDPC weights for an adjacent bottom-left diagonal mode are: wL = 32 >> ( ( x’<<1 ) >> shift ), wT =0, wTL = 0.

As is the case for DC, planar, horizontal and vertical mode PDPC, there is no additional boundary filtering needed for diagonal and adjacent diagonal modes.

The reference sample coordinates for the last two cases are computed using the tables that are already used for angular mode intra prediction. In VTM1 and BMS1 tables are available for 65 angular modes in total. The tables can be reused for the PDPC extension to diagonal and adjacent modes, therefore, no additional tables are required in the implementation of PDPC. There are no multiplications for computing coordinates x and y. Multiplications are avoided by accumulating the table values corresponding with the angular mode with increasing x’ and y’ while traversing the prediction block. Linear interpolation of the reference samples is used if fractional reference sample coordinates are calculated.

# Test scenarios

The following test was included in CE3 [8]:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.4.1 | PDPC planar, DC, HOR/VER, diagonal, adjacent diagonal modes | Geert Van der Auwera (Qualcomm) | Jani Lainema (Nokia) |

# Results

The following results are obtained according to the JVET common test conditions [7] and the CE3 test guidelines [8]. The BMS1.0 software is used for both the BMS and VTM results. The anchors are also generated using the BMS1.0 software. Additional results for the LB configuration are reported.

## ‘All Intra’, ‘Random Access’, ‘Low Delay B’ results over BMS-VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.76% | -0.31% | 0.05% | 111% | 111% |
| Class A2 | -0.84% | -0.64% | -0.32% | 111% | 109% |
| Class B | -1.01% | 0.02% | 0.17% | 110% | 107% |
| Class C | -0.87% | 0.51% | 0.43% | 106% | 105% |
| Class E | -1.36% | -0.60% | -0.66% | 109% | 105% |
| Overall | -0.97% | -0.14% | -0.01% | 109% | 107% |
| Class D | -0.81% | 0.63% | 1.02% | 105% | 107% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.38% | -0.12% | 101% | 103% |
| Class A2 | -0.37% | 0.13% | 0.53% | 102% | 102% |
| Class B | -0.53% | 0.62% | 0.46% | 102% | 101% |
| Class C | -0.55% | 0.85% | 0.84% | 101% | 99% |
| Overall | -0.48% | 0.39% | 0.46% | 101% | 101% |
| Class D | -0.44% | 1.03% | 1.20% | 102% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.31% | -0.41% | -0.43% | 102% | 101% |
| Class C | -0.23% | 0.04% | 0.02% | 102% | 99% |
| Class E | -0.14% | 2.14% | 0.60% | 102% | 100% |
| Overall | -0.24% | 0.38% | -0.02% | 102% | 100% |
| Class D | -0.23% | 0.52% | 0.29% | 101% | 102% |

## ‘All Intra’, ‘Random Access’, ‘Low Delay B’ over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.35% | -1.96% | -1.44% | 103% | 108% |
| Class A2 | -1.44% | -1.20% | -1.20% | 104% | 106% |
| Class B | -1.43% | -0.88% | -0.76% | 103% | 102% |
| Class C | -0.93% | -0.16% | -0.07% | 102% | 106% |
| Class E | -1.70% | -0.81% | -0.87% | 102% | 104% |
| Overall | -1.35% | -0.94% | -0.81% | 103% | 105% |
| Class D | -0.78% | 0.28% | 0.51% | 103% | 106% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.73% | -1.22% | -1.12% | 101% | 100% |
| Class A2 | -0.65% | -0.40% | -0.39% | 102% | 102% |
| Class B | -0.82% | -0.28% | -0.48% | 102% | 101% |
| Class C | -0.58% | 0.01% | -0.36% | 102% | 100% |
| Overall | -0.70% | -0.41% | -0.56% | 102% | 101% |
| Class D | -0.42% | 0.03% | 0.10% | 103% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.37% | -0.97% | -0.11% | 102% | 100% |
| Class C | -0.29% | -0.27% | -0.78% | 101% | 102% |
| Class E | -0.42% | 0.15% | 0.22% | 101% | 99% |
| Overall | -0.36% | -0.46% | -0.25% | 101% | 100% |
| Class D | -0.15% | 0.23% | -0.24% | 101% | 102% |

# Acknowledgement

We are grateful to Nokia for conducting the cross-check for this test.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “CE3: Summary Report on Intra Prediction and Mode Coding,” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K0023

- ITU-T SG16/Q6 Doc. COM16-C1046, “Position Dependent intra Prediction Combination (PDPC)”

- X. Zhao, V. Seregin, A. Said, M. Karczewicz, “EE1 related: Simplification and extension of PDPC”, 8th JVET Meeting, Macau, Oct. 2018, JVET-H0057

- M. Karczewicz et al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm,” 10th JVET Meeting, San Diego, CA, USA, Apr. 2018, JVET-J0021

- G. Van der Auwera, V. Seregin, A. Said, M. Karczewicz, “Extension of simplified PDPC to diagonal intra modes,” 10th JVET Meeting, San Diego, CA, USA, Apr. 2018, JVET-J0069

- J. Chen, E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1)”, 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1002

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7,” 7th JVET Meeting, Torino, Italy, July 2017, JVET-G1001

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1010

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1023

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).