| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0064-v2 |
| --- | --- |

| Title: | CE3-related: On MDIS and intra interpolation filter switching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Adarsh Krishnan Ramasubramonian Vadim Seregin Ted Hsieh Marta Karczewicz | Email: | geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com thsieh@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The MDIS (Mode Dependent Intra Smoothing) filter in VTM1 and BMS1 is applied to the intra reference samples depending on the MDIS conditions. This requires storing unfiltered and filtered lines of intra reference samples. In JEM7, cubic and Gaussian interpolation filters for prediction of directional intra modes were included. However, depending on the MDIS conditions and interpolation filter switching conditions it may happen that two smoothing filtering operations, MDIS and Gaussian interpolation, are applied consecutively. Therefore, it is proposed to merge the two smoothing filtering operations by convolving the MDIS filter with the Gaussian interpolation filter, while the MDIS conditions are employed to decide between the two interpolation filters. This way only the unfiltered reference sample line needs to be stored for MDIS. The reported AI BD-rates are −0.65% (Y), −0.16% (U), −0.16% (V) with runtimes 109% (Enc) and 101% (Dec) over the BMS-VTM-1.0 anchor, −0.55% (Y), −0.36% (U), −0.47% (V) with runtimes 102% (Enc) and 100% (Dec) over the BMS-1.0 anchor. The reported RA BD-rates are −0.38% (Y), 0.06% (U), 0.05% (V) with runtimes 104% (Enc) and 102% (Dec) over BMS-VTM-1.0 anchor. A cross-check report is available in JVET-K0498.

# Introduction

The MDIS (Mode Dependent Intra Smoothing) filter in VTM1 [1], [1 2 1]/4, is applied to the intra reference samples depending on the MDIS conditions. This requires storing unfiltered and filtered lines of intra reference samples for intra prediction. For prediction of directional intra modes, the side reference samples are projected and extend the main reference sample line. Subsequently, linear interpolation is applied to the reference sample line in case of fractional pixel prediction for directional modes.

CE3 includes proposals [3][4] that replace the linear interpolation with a combination of two interpolation filters: cubic and Gaussian. The main objective of these proposals is to improve compression efficiency significantly. These interpolation filters may be applied to either the unfiltered or filtered reference samples depending on the separate conditions associated with interpolation filtering in [3][4]. In case Gaussian interpolation filtering is applied on top of MDIS filtered reference samples, two smoothing filtering operations are applied consecutively. This means complexity is increased and storage is required for the unfiltered and filtered intra reference sample lines.

In [6] an algorithm is proposed that disables MDIS filtering and MDIS conditions by convolving the MDIS filter [1 2 1]/4 with the 4-tap Gaussian interpolation filter from JEM7 [2] which results in a 6-tap Gaussian interpolation filter. This approach intends to solve the above stated problem of two-step smoothing filtering, however, in [6] the [1 2 1]/4 filter is still applied in case of non-fractional prediction of samples. The algorithm in [6] decides between the two interpolation filters by comparing the block width (for vertical modes) or the block height (for horizontal modes) against the fixed threshold value 8. By combining the MDIS filter with the Gaussian filter, the MDIS filtering before extension of the main reference line is also avoided, which is proposed separately in [5] by switching the order of projection and MDIS filtering.

# Proposal

As in [6] it is proposed to merge the two smoothing filtering operations for intra prediction of directional modes: MDIS filtering and Gaussian interpolation filtering. The main difference from [6] is that the MDIS conditions are employed to decide between the two interpolation filters while the separate interpolation filter decisions are removed. This way only the unfiltered reference sample line needs to be stored for MDIS, which reduces line memory requirements. A second difference from [6] is that the [1 2 1]/4 filtering in case of non-fractional sample prediction is avoided by using the zero-phase interpolation filter taps. The details of the proposed algorithm are described next.

In the Joint Video Experts Team (JVET) and its VVC Test Model version 1 (VTM1) [1] and Benchmark Set model version 1 (BMS1) software, the following MDIS table is included:

| sizeIndex | MDISthreshold[sizeIndex] |
| --- | --- |
| 0 | 20 |
| 1 | 20 |
| 2 | 20 |
| 3 | 14 |
| 4 | 2 |
| 5 | 0 |
| 6 | 20 → 0 (*) |
| 7 | 0 |

(*) In this proposal the MDISthreshold corresponding with sizeIndex=6 is modified to value 0.

The block size index is defined as follows in VTM1 and BMS1:

sizeIndex = (log2(BlockWidth) + log2(BlockHeight)) / 2

Define the minimum index difference between the directional intra mode and respectively the indices of horizontal and vertical modes as follows:

IntraModeDiff = min(abs(IntraModeIdx - HOR_IDX), abs(IntraModeIdx – VER_IDX))

The proposed condition for switching interpolation filters in directional intra prediction:

IF (IntraModeDiff > MDISthreshold[sizeIndex])

THEN “apply cubic interpolation filter”

ELSE “apply Gaussian interpolation filter”

In this algorithm the Gaussian interpolation filter, which has smoothing properties, is applied as the only smoothing filtering step.

Note that additional conditions are present in VTM1 and BMS1 to control the smoothing filtering of the intra reference samples, such as dependencies on luma or chroma, usage of the PDPC tool (Position Dependent Intra Prediction combination), usage of NSST (Non-separable Secondary Transform), etc.

In this proposal, the cubic interpolation filter is the following 32-phase filter:

intraCubic6TapFilter32[32][6] = {

{ 0,   0, 256,   0,   0, 0 },  //  0

{ 0,  -4, 253,   9,  -2, 0 },  //  1

{ 1,  -7, 249,  17,  -4, 0 },  //  2

{ 1, -10, 245,  25,  -6, 1 },  //  3

{ 1, -13, 241,  34,  -8, 1 },  //  4

{ 2, -16, 235,  44, -10, 1 },  //  5

{ 2, -18, 229,  53, -12, 2 },  //  6

{ 2, -20, 223,  63, -14, 2 },  //  7

{ 2, -22, 217,  72, -15, 2 },  //  8

{ 3, -23, 209,  82, -17, 2 },  //  9

{ 3, -24, 202,  92, -19, 2 },  // 10

{ 3, -25, 194, 101, -20, 3 },  // 11

{ 3, -25, 185, 111, -21, 3 },  // 12

{ 3, -26, 178, 121, -23, 3 },  // 13

{ 3, -25, 168, 131, -24, 3 },  // 14

{ 3, -25, 159, 141, -25, 3 },  // 15

{ 3, -25, 150, 150, -25, 3 },  // 16

{ 3, -25, 141, 159, -25, 3 },  // 17

{ 3, -24, 131, 168, -25, 3 },  // 18

{ 3, -23, 121, 178, -26, 3 },  // 19

{ 3, -21, 111, 185, -25, 3 },  // 20

{ 3, -20, 101, 194, -25, 3 },  // 21

{ 2, -19,  92, 202, -24, 3 },  // 22

{ 2, -17,  82, 209, -23, 3 },  // 23

{ 2, -15,  72, 217, -22, 2 },  // 24

{ 2, -14,  63, 223, -20, 2 },  // 25

{ 2, -12,  53, 229, -18, 2 },  // 26

{ 1, -10,  44, 235, -16, 2 },  // 27

{ 1,  -8,  34, 241, -13, 1 },  // 28

{ 1,  -6,  25, 245, -10, 1 },  // 29

{ 0,  -4,  17, 249,  -7, 1 },  // 30

{ 0,  -2,   9, 253,  -4, 0 },  // 31

};

The Gaussian filter is a 6-tap filter obtained by convolving the [1 2 1]/4 MDIS filter and the 4-tap Gaussian filter from JEM7 [2] resulting in the following filter (similar to [6]) with the zero-phase filter coefficients modified to obtain a symmetric filter:

intraGauss6TapFilter32[32][6] = {

{ 47, 256, 418, 256,  47,  0 },  //  0

{ 43, 247, 416, 264,  53,  1 },  //  1

{ 40, 240, 414, 270,  58,  2 },  //  2

{ 37, 233, 413, 277,  62,  2 },  //  3

{ 34, 226, 412, 284,  66,  2 },  //  4

{ 31, 218, 410, 292,  71,  2 },  //  5

{ 28, 210, 407, 299,  77,  3 },  //  6

{ 26, 203, 404, 306,  82,  3 },  //  7

{ 23, 195, 401, 313,  88,  4 },  //  8

{ 21, 188, 398, 320,  93,  4 },  //  9

{ 19, 180, 393, 327, 100,  5 },  // 10

{ 17, 173, 389, 333, 106,  6 },  // 11

{ 16, 167, 385, 339, 111,  6 },  // 12

{ 14, 159, 380, 346, 118,  7 },  // 13

{ 13, 153, 375, 351, 124,  8 },  // 14

{ 11, 145, 370, 358, 131,  9 },  // 15

{ 10, 138, 364, 364, 138, 10 },  // 16

{ 9,  131, 358, 370, 145, 11 },  // 17

{ 8,  124, 351, 375, 153, 13 },  // 18

{ 7,  118, 346, 380, 159, 14 },  // 19

{ 6,  111, 339, 385, 167, 16 },  // 20

{ 6,  106, 333, 389, 173, 17 },  // 21

{ 5,  100, 327, 393, 180, 19 },  // 22

{ 4,   93, 320, 398, 188, 21 },  // 23

{ 4,   88, 313, 401, 195, 23 },  // 24

{ 3,   82, 306, 404, 203, 26 },  // 25

{ 3,   77, 299, 407, 210, 28 },  // 26

{ 2,   71, 292, 410, 218, 31 },  // 27

{ 2,   66, 284, 412, 226, 34 },  // 28

{ 2,   62, 277, 413, 233, 37 },  // 29

{ 2,   58, 270, 414, 240, 40 },  // 30

{ 1,   53, 264, 416, 247, 43 },  // 31

};

Note that in case integer reference sample locations are computed, the zero-phase of both filters is used.

Note also that the MDIS condition enables smoothing of reference samples for planar mode in case sizeIndex is greater than 2. For planar mode the [1 2 1]/4 filtering can be implemented together with the planar prediction algorithm, hence, no additional storage of an additional filtered reference line is required.

# Results

The following results are obtained according to the JVET common test conditions [7] and the CE3 test guidelines [8]. The BMS1.0 software is used for both the BMS and VTM results. The anchors are also generated using the BMS1.0 software.

## ‘All Intra’ and ‘Random Access’ results over BMS-VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.27% | -0.22% | 109% | 100% |
| Class A2 | -0.11% | 0.27% | 0.22% | 109% | 101% |
| Class B | -0.59% | -0.09% | -0.06% | 108% | 100% |
| Class C | -1.19% | -0.41% | -0.41% | 108% | 104% |
| Class E | -0.95% | -0.28% | -0.33% | 110% | 100% |
| Overall | -0.65% | -0.16% | -0.16% | 109% | 101% |
| Class D | -0.76% | -0.25% | -0.19% | 109% | 106% |
| Class F (optional) | -0.51% | -0.19% | -0.27% | 110% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.29% | -0.16% | 114% | 112% |
| Class A2 | -0.21% | 0.15% | 0.26% | 102% | 99% |
| Class B | -0.41% | 0.13% | -0.04% | 105% | 103% |
| Class C | -0.57% | 0.16% | 0.16% | 99% | 98% |
| Overall | -0.38% | 0.06% | 0.05% | 104% | 102% |
| Class D | -0.35% | 0.51% | 0.24% | 100% | 101% |
| Class F (optional) | -0.16% | 0.18% | 0.21% | 101% | 101% |

## ‘All Intra’ and ‘Random Access’ results over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.74% | -0.66% | 103% | 100% |
| Class A2 | -0.17% | -0.02% | -0.04% | 102% | 99% |
| Class B | -0.53% | -0.25% | -0.39% | 102% | 99% |
| Class C | -0.91% | -0.49% | -0.67% | 101% | 104% |
| Class E | -0.77% | -0.33% | -0.60% | 102% | 99% |
| Overall | -0.55% | -0.36% | -0.47% | 102% | 100% |
| Class D | -0.56% | -0.25% | -0.38% | 100% | 102% |
| Class F (optional) | -0.36% | -0.18% | -0.61% | 100% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.45% | -0.44% | 101% | 98% |
| Class A2 | -0.10% | 0.05% | 0.15% | 101% | 101% |
| Class B |  |  |  |  |  |
| Class C | -0.30% | 0.27% | -0.18% | 100% | 98% |
| Overall |  |  |  |  |  |
| Class D | -0.24% | 0.13% | -0.02% | 101% | 102% |
| Class F (optional) | 0.02% | 0.01% | 0.08% | 100% | 99% |

# Acknowledgement

# We are grateful to Sony for performing a cross-check.

# References

- J. Chen, E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1)”, 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1002

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7,” 7th JVET Meeting, Torino, Italy, July 2017, JVET-G1001

- G. Van der Auwera, V. Seregin, M. Karczewicz, “CE3: Intra reference sample interpolation (Tests 2.2.1, 2.3.1, 2.3.2),” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K0062

- S. Yoo, J. Heo, J. Choi, L. Li, J. Lim, “CE3-2.3.3 and CE3-2.3.4: Interpolation filter selection regarding intra mode and block size,” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K0097

- V. Drugeon, “CE3: DC mode without divisions and modifications to intra filtering (Tests 1.2.1, 2.2.2 and 2.5.1),” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K0211

- J. Lee, H. Lee, S.-C. Lim, J. Kang, H. Y. Kim, “CE3: Combined filter (Test 2.6.1),” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K0165

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1010

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1023

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).