| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0066-v1 |
| --- | --- |

| Title: | CE3: Mode dependent de-ringing filter (Test 2.8.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Jianle Chen | Tel: Email: | +7-906-035-1823 sergey.ikonin@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution reports test results for CE3 test 2.8.2 of de-ringing filter for reference samples of intra prediction. It is reported that this technique provides 0.40% and 0.22% of luma BD-rate saving with the encoding time of 116% and 103% for AI and RA configurations, respectively, and no change in decoding time compared to VTM1.0. Compared to BMS 1.0 the proposed technique provides 0.40% and 0.17% luma BD-rate saving with the encoding time of 115% and 105% for AI and RA configurations, respectively, and no change in decoding time. It was observed that for screen content sequences (Class F) in AI configuration de-ringing filter provides 0.82% and 1.00% compared to VTM1.0 and BMS1.0 anchors respectively.

# Introduction

The de-ringing filter for reference samples of intra prediction was introduced in JVET-J0025. The filter allows to reduce ringing artefacts and increase sharpness of edges. The CE3-2.8.2 experiment evaluates coding efficiency of the filter.

# Technology description

The de-ringing filter applied to reference samples of intra prediction allows to increase prediction efficiency and improve subjective sharpness of edges. As the first step of the filtering edge pattern is calculated from reference samples by: taking first derivative ([-1, 0, 1]), obtaining absolute value, clipping with fixed threshold, blurring with Gaussian filter [1, 4, 7, 4, 1] and taking second derivative ([-1, 0, 1]). As the second step warping procedure is applied using scaled edge pattern as displacement vectors.

Figure 1 illustrates the filtering principle. On given example blue line denotes reference samples containing sharp edge and some ringing artifacts arisen around edge. Edge pattern calculated from reference samples after normalization scaling is interpreted as displacement vectors (black arrows). Using displacement vectors warping procedure replaces current pixel intensity by intensity of some neighboring pixel. If edge pattern is positive then neighboring pixel is taken from the right, if negative then taken from the left. Displacement vector may have fractional value. In that case linear interpolation between two neighboring pixels is used.

Figure 1. De-ringing of reference sample by warping

Appliance of the de-ringing filter is controlled by the 1 bin flag which is decided by RDO during encoding process. The filter is applied to both luma and chroma if prediction mode is angular. In BMS1.0 configuration the de-ringing filter can be applied only if NSST and EMT is disabled for the current block.

# Experimental results

## Objective results

The objective performance of the de-ringing filter is summarized in Table 1

Table 1. CE3-2.8.2 coding performance

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.02% | -0.09% | 109% | 100% | -0.10% | 0.03% | -0.15% | 116% | 100% |
| Class A2 | -0.37% | -0.15% | -0.20% | 114% | 100% | -0.34% | -0.22% | -0.30% | 115% | 100% |
| Class B | -0.29% | -0.17% | -0.17% | 116% | 100% | -0.25% | -0.20% | -0.27% | 115% | 100% |
| Class C | -0.71% | -0.66% | -0.82% | 122% | 100% | -0.78% | -0.84% | -0.78% | 114% | 99% |
| Class E | -0.50% | -0.20% | -0.23% | 115% | 101% | -0.48% | -0.57% | -0.44% | 117% | 100% |
| Overall | -0.40% | -0.26% | -0.32% | 116% | 100% | -0.40% | -0.37% | -0.39% | 115% | 100% |
| Class D | -0.30% | -0.18% | -0.31% | 125% | 100% | -0.27% | -0.23% | -0.26% | 112% | 99% |
| Class F (optional) | -0.82% | -0.56% | -0.61% | 119% | 101% | -1.00% | -0.74% | -0.95% | 115% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 with VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.02% | -0.11% | 103% | 101% | -0.08% | 0.00% | -0.12% | 105% | 101% |
| Class A2 | -0.34% | -0.29% | -0.19% | 103% | 100% | -0.25% | -0.31% | -0.14% | 104% | 100% |
| Class B | -0.17% | -0.01% | -0.31% | 103% | 100% | -0.12% | -0.03% | -0.23% | 105% | 100% |
| Class C | -0.30% | -0.42% | -0.43% | 104% | 100% | -0.22% | -0.33% | -0.61% | 105% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.22% | -0.18% | -0.28% | 103% | 100% | -0.17% | -0.16% | -0.29% | 105% | 100% |
| Class D | -0.14% | -0.10% | -0.53% | 104% | 99% | -0.06% | -0.34% | -0.35% | 105% | 99% |
| Class F (optional) | -0.47% | -0.35% | -0.38% | 105% | 101% | -0.50% | -0.36% | -0.47% | 107% | 99% |

It was observed that for screen content sequences (Class F) de-ringing filter provides -0.82% on VTM1.0 and -1.0% on BMS1.0 for AI configuration.

## Subjective results

Besides objective coding gain, the de-ringing filter also demonstrates improvement of subjective quality of edges by reducing ringing artefact and increasing sharpness. Figures 2-7 illustrate subjective improvement of de-ringing filter on different video sequences.

Figure 2. BasketballDrill, Frame#0, AI QP32, VTM1.0

Figure 3. BasketballDrill, Frame#0, AI QP32, VTM1.0 + de-ringing filter

Figure 4. DaylightRoad2, Frame#24, AI QP32, VTM1.0

Figure 5. DaylightRoad2, Frame#24, AI QP32, VTM1.0 + de-ringing filter

Figure 6. ChinaSpeed, Frame#0, AI QP32, VTM1.0

Figure 7. ChinaSpeed, Frame#0, AI QP32, VTM1.0 + de-ringing filter

# Conclusions

CE3 test 2.8.2 reports 0.40% of coding efficiency improvement with 116% of encoding time and 100% of decoding time compared to VTM1.0. The same gain (0.40%) is also kept on top of BMS1.0 with encoding and decoding time of 115% and 100% respectively. It was observed that for screen content sequences (Class F) in AI configuration de-ringing filter provides 0.82% and 1.00% compared to VTM1.0 and BMS1.0 anchors respectively. It is proposed to adopt de-ringing filter in VTM.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).