| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0067-v1 |
| --- | --- |

| Title: | CE9: Motion vector difference signs derivation (Test 4.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Jianle Chen | Tel: Email: | +7-906-035-1823 sergey.ikonin@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution reports results of CE9-4.2 test of motion vector difference signs derivation (MVSD) technique. It is reported that method provides 0.49% BD-rate saving with 102% encoding time and 104% decoding time compared to VTM1.0 for the random access configuration. Compared to BMS 1.0 method provides 0.29% BD-rate saving with 100% encoding time and 101% decoding time for the random access configuration.

# Introduction

In VTM1.0 and BMS1.0 signs of motion vector difference (MVD) are transmitted without compression with equal probability (EP) coding mode. The motion vector difference signs derivation method (MVSD) is addressed to improve compression efficiency of signs information by applying template matching to predict MVD signs at the decoder side. The method was introduces in J0025. CE9-4.2 test evaluates coding performance of this method.

# Proposed method

Instead of direct signaling of MVD signs in EP mode, possible MVD signs variants are sorted according to template matching cost and index of true MVD sign is coded with context model. At the decoder side signs are derived during reconstruction process by applying following steps:

- Generate list of MVD candidates based on absolute values of MVD (four candidates if both X and Y components of MVD are non-zero and 2 candidates if one component is zero);

- Calculate cost for each MVD candidate using template matching;

- Rank candidates by sorting them based on calculated cost;

- Taking MVD with an index in the sorted list according to the MVSD index obtained from bitstream;

During template matching bilinear filter is used for motion interpolation. The template matching cost is measured by the SAD between the neighbouring samples of the current CU and their corresponding reference samples. MVSD is applied only for translational (non-affine) blocks. Illustration of MVSD process is depicted on Figure 1.

PU

MVD candidates (A,B,C,D)

MVP

Current picture

Reference picture

A

B

C

D

Figure 1. Motion vector difference signs derivation

# Experimental results

Performance of motion vector difference signs derivation method was calculated according to Common Tests Conditions and summarized in Table 1.

Table 1. Coding performance of CE9 test 4.2

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.43% | -0.39% | 103% | 101% | -0.29% | -0.36% | -0.27% | 99% | 100% |
| Class A2 | -0.75% | -0.61% | -0.55% | 104% | 104% | -0.34% | -0.32% | -0.36% | 100% | 101% |
| Class B | -0.46% | -0.37% | -0.27% | 102% | 104% | -0.27% | -0.29% | -0.28% | 100% | 101% |
| Class C | -0.38% | -0.39% | -0.36% | 101% | 106% | -0.27% | -0.16% | -0.21% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.49% | -0.44% | -0.38% | 102% | 104% | -0.29% | -0.28% | -0.28% | 100% | 101% |
| Class D | -0.40% | -0.18% | -0.32% | 102% | 105% | -0.24% | -0.09% | -0.15% | 100% | 101% |
| Class F (optional) | -0.22% | -0.14% | -0.20% | 101% | 104% | -0.18% | -0.07% | -0.08% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 with VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.30% | -0.16% | -0.15% | 102% | 102% | -0.29% | -0.48% | -0.11% | 100% | 101% |
| Class C | -0.26% | -0.27% | -0.28% | 102% | 106% | -0.33% | -0.33% | -0.51% | 100% | 102% |
| Class E | -0.28% | -0.08% | -0.64% | 102% | 103% | -0.39% | -0.38% | -0.61% | 100% | 100% |
| Overall | -0.28% | -0.17% | -0.32% | 102% | 104% | -0.33% | -0.41% | -0.37% | 100% | 101% |
| Class D | -0.39% | -0.64% | -0.92% | 102% | 106% | -0.29% | 0.02% | -0.10% | 100% | 101% |
| Class F (optional) | -0.37% | -0.46% | 0.08% | 102% | 105% | -0.13% | 0.34% | -0.21% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 with VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.34% | -0.32% | -0.41% | 103% | 103% | -0.30% | -0.25% | -0.84% | 100% | 101% |
| Class C | -0.30% | -0.40% | -0.32% | 102% | 105% | -0.30% | -0.16% | -0.57% | 100% | 101% |
| Class E | -0.31% | -0.53% | 0.00% | 101% | 103% | -0.29% | -0.09% | -0.69% | 100% | 100% |
| Overall | -0.32% | -0.40% | -0.28% | 102% | 104% | -0.30% | -0.18% | -0.71% | 100% | 101% |
| Class D | -0.36% | -0.21% | 0.00% | 102% | 106% | -0.25% | -0.11% | -0.65% | 100% | 102% |
| Class F (optional) | -0.17% | -0.40% | -0.23% | 102% | 102% | -0.19% | -0.12% | -0.34% | 100% | 101% |

# Conclusions

CE9 test 4.2 reports 0.49% of coding efficiency improvement with 102% encoding time and 104% decoding time compared to VTM1.0 for RA configuration. On top of BMS1.0 for RA configuration method provides 0.29% with 100% encoding time and 101% decoding time. It is proposed to adopt motion vector difference signs derivation method in VTM.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).