| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0068-v3 |
| --- | --- |

| Title: | CE2 related: Hadamard Transform Domain Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Victor Stepin Sergey Ikonin Roman Chernyak Jianle Chen | Tel: Email: | +7-905-570-8423 stepin.victor@huawei.com sergey.ikonin@huawei.com chernyak.roman@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes in-loop filter in 1D Hadamard transform domain which is applied on CU level after reconstruction and has multiplication free implementation. Proposed filter is applied for all CU blocks that meet the predefined condition and filter parameters are derived from the coded information. It is reported that for the random access configuration the proposed method provides 0.50% of luma BD-rate saving with 105% encoding time and 104% decoding time compared to VTM 1.0.

# Introduction

# Documents JVET-D0029 [1] and JVET-J0022 [2] (tests CE2.1.1-2.1.3) describe bilateral filter which is applied to luma blocks after inverse transform and reconstruction. The filtered result is used both for output as well as for spatial and temporal prediction. Current proposal describes 1D Hadamard domain in-loop filter which is applied at the same place as bilateral filter and has multiplication free implementation.

# Proposed method

Proposed filtering is always applied to luma reconstructed blocks with non-zero transform coefficients, excluding 4x4 blocks and if slice quantization parameter is larger than 17. The filter parameters are explicitly derived from the coded information. Proposed filter, if applied, is performed on decoded samples right after inverse transform.

For each pixel from reconstructed block pixel processing comprises the following steps:

- Scan 4 neighboring pixels around processing pixel including current one according to scan pattern

- 4 point Hadamard transform of read pixels

Fi,σ=Ri2Ri2+σ2*R(i)

Spectrum filtering based on the following formula:

,wherein (i) is index of spectrum component in Hadamard spectrum, R(i)  is spectrum component of reconstructed pixels corresponding to index, σ is filtering parameter deriving from codec quantization parameter QP using following equation:

σ=2(1+0.126*QP-27)

It can be noted that filtering implies multiplication of spectrum component R(i) on scaling coefficient which is always less than 1. It can also be observed that at high values of R(i) scaling coefficient is close to 1. Based on these observation spectrum filtering is implemented using lookup table that allows to exclude multiplications and division from filtering operations:

- In current implementation THR is set to 128 that requires storage 128 of int32 entries in ROM per QP.Inverse 4 point Hadamard transform of filtered spectrum

- After filtering step the filtered pixels are placed to its original positions into accumulation buffer.

- After completing filtering of all pixels in CU the accumulated values are normalized by number of processing groups used for each pixel filtering. The normalization is implemented as add-and-shift method excluding divisions and multiplications.

The example of scan pattern is depicted on Figure 1.

Figure 1 Scan pattern. A – current pixel, BCD – surrounding pixels

For pixels laying on CU boundary the scan pattern is adjusted ensuring all required pixels are within current CU.

The following pseudo code illustrates work of proposed method in multiplication free implementation.

// reconstructed pixels scan

const int x0 = pIn[p0];

const int x1 = pIn[p1];

const int x2 = pIn[p2];

const int x3 = pIn[p3];    // p0-p3 define scan pattern

// 1D forward Hadamard transform

const int y0 = x0 + x2;

const int y1 = x1 + x3;

const int y2 = x0 - x2;

const int y3 = x1 - x3;

const int t0 = y0 + y1;

const int t1 = y0 - y1;

const int t2 = y2 + y3;

const int t3 = y2 - y3;

// frequency domain filtering

const int z0 = pTbl[t0];

const int z1 = pTbl[t1];

const int z2 = pTbl[t2];

const int z3 = pTbl[t3];

// backward Hadamard transform

const int iy0 = z0 + z2;

const int iy1 = z1 + z3;

const int iy2 = z0 - z2;

const int iy3 = z1 - z3;

// filtered pixels accumulation

pOut[p0] += iy0 + iy1    // // p0-p3 define scan pattern

pOut[p1] += iy0 - iy1

pOut[p2] += iy2 + iy3

pOut[p3] += iy2 - iy3

# Experimental results

In below tables simulation results of proposed method are summarised.

Table 1. Coding performance, RA, VTM

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.05% | -0.08% | 106% | 104% |
| Class A2 | -0.57% | -0.19% | -0.12% | 105% | 103% |
| Class B | -0.62% | -0.04% | -0.31% | 105% | 104% |
| Class C | -0.43% | -0.23% | -0.32% | 104% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.50% | -0.12% | -0.23% | 105% | 104% |
| Class D | -0.34% | -0.18% | -0.38% | 103% | 109% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 2. Coding performance, AI, VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.02% | 0.04% | 105% | 115% |
| Class A2 | -0.42% | -0.15% | 0.14% | 106% | 112% |
| Class B | -0.35% | 0.06% | -0.10% | 106% | 112% |
| Class C | -0.32% | -0.08% | -0.23% | 105% | 108% |
| Class E | -0.14% | -0.29% | -0.44% | 105% | 113% |
| Overall | -0.33% | -0.08% | -0.13% | 105% | 112% |
| Class D | -0.29% | -0.04% | -0.03% | 106% | 109% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 3. Coding performance, RA, BMS

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.44% | -0.33% | -0.46% | 106% | 99% |
| Class C | -0.38% | -0.22% | -0.51% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.34% | -0.34% | -0.52% | 105% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 4. Coding performance, AI, BMS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.45% | -0.30% | -0.30% | 106% | 104% |
| Class E | -0.24% | -0.37% | -0.33% | 106% | 106% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.25% | -0.09% | -0.05% | 106% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusions

It is reported that the proposed in-loop filter in 1D Hadamard domain provides gains in terms of coding efficiency with a limited amount of additions, subtractions and logical operations per pixel. The design of filter allows to avoid multiplications and divisions from filtering process by using lookup table. It is proposed to study this filter in Core Experiments.

# References

- Jacob Ström at al., “Bilateral Filter After Inverse Transform”, JVET, doc JVET-D0069, October 2016

- Li Zhang at al., “Description of Core Experiment 2 (CE2) on In-Loop Filters”, J1022, April 2018.

- Per Wennersten at al., ”Bilateral filtering for video coding”, VCIP 2017

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).