| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0069-v2 |
| --- | --- |

| Title: | CE7: Coefficient Coding (Subtest 7.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Jie Dong Ted Hsieh Marta Karczewicz | Email: | mcoban@qti.qualcomm.com jiedong@qti.qualcomm.com thsieh@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document describes CE 7.1.1 (JEM) coefficient coding scheme. It is based on bitplane coding of coefficient levels similar to HEVC. CE 7.1.1 coefficient coding provides -1.34% (AI), -0.87% (RA), -0.59% (LB) luma BD-Rate relative to VTM-1.0.

# Algorithm description

In HEVC, transform coefficients of one transform unit (TU) are coded with non-overlapped coefficient groups (CG), and each CG contains the coefficients of a 4x4 block of a TU. The CGs inside a TU, and the transform coefficients within a CG, are coded according to diagonal scan order. The coding of transform coefficient levels is separated into multiple scan passes. In the first pass, the first bin (denoted by bin0, also referred as significant_coeff_flag, which indicates the magnitude of the coefficient is larger than 0) is coded. Next, two scan passes for context coding the second/third bins (denoted by bin1 and bin2, respectively, also referred as coeff_abs_greater1_flag and coeff_abs_greater2_flag) may be applied. Finally, two scan passes for coding the sign information and the remaining values (also referred as coeff_abs_level_remaining) of coefficient levels are invoked, if necessary. Note that only bins in the first three scan passes are coded in a regular mode and named regular bins in the following descriptions. The last coefficient position coding, significance of CG coding follows that of HEVC.

In this proposal [1][3] the context model selection method of regular bins is modified w.r.t. HEVC. When coding bini in the i-th scan pass (i being 0, 1, 2), the context index is dependent on the values of the i-th bins of previously coded coefficients in the neighborhood covered by a local template. More specifically, the context index is determined based on the sum of the i-th bins of neighboring coefficients. The template is the same as the one used in [2], and depicted in Figure 1. To capture the characteristics of transform coefficients at different frequencies, one TU are split into up to three regions and the splitting method is fixed regardless of the TU sizes. For example, when coding bin0 of luma transform coefficients, as depicted in Figure 1, one TU is split into three regions marked with different colors, and the context index assigned to each region is listed. Luma and chroma components are treated in a similar way but with separate sets of context models. Moreover, the context model selection for bin0 (i.e., significant flags) of the luma component is further dependent on transform size. For the bypass coded bins of the remaining values of coefficient levels, the coding method of coeff_abs_level_remaining in [2] is applied.

In this scheme up to 8 gt1_flags and 1 gt2_flag is coded as regular bins. The rest of the bitplane information is coded as bypass bins. CG flags as well as last coefficient position is coded same as HEVC.

Figure 1: Definition of template used in transform coefficient context modeling.

## Significance flag coding:

Significance flag coding context for luma/chroma is described by the following method:

diag = posX + posY;

numPos = 0;

numPos += pData[ 1 ] != 0;

numPos += pData[ 2 ] != 0;

numPos += pData[ m_width + 1 ] != 0;

numPos += pData[ m_width ] != 0;

numPos += pData[ 2 * m_width ] != 0;

ctxIdx = numPos;

ctxOfs = diag < 2 ? 6 : 0;

if( m_chType == CHANNEL_TYPE_LUMA )

{

ctxOfs += diag < 5 ? 6 : 0;

}

if( m_log2BlockSize > 2 && m_chType == CHANNEL_TYPE_LUMA )

{

ctxOfs += 18 << std::min( 1, ( (int)m_log2BlockSize - 3 ) );

}

m_sigCtxSet( ctxOfs + ctxIdx );

where posX, posY define the (x,y) position a coefficient within a TU, pData[] represents the array containing the partial (accumulated level up to the last parsed coding pass) transform coefficient levels in a TU. If pData[] lies outside of a TU boundary, then it is assumed to have zero value.

For significance map of luma blocks 3 sets of 18 contexts are used corresponding to 4x4 TU, 8x8 TU and larger than 8x8 TU s. In the above algorithm description, if a neighboring

## Greater than 1 (gt1) flag coding:

gt1 flag coding context for luma/chroma is described by the following method:

diag = posX + posY;

numPos = 0;

numPos += abs(pData[ 1 ]) > 1;

numPos += abs(pData[ 2 ]) > 1;

numPos += abs(pData[ m_width + 1 ]) > 1;

numPos += abs(pData[ m_width ]) > 1;

numPos += abs(pData[ 2 * m_width ]) > 1;

ctxIdx = min( numPos, 4 ) + 1;

ctxOfs = 0;

if( m_chType == CHANNEL_TYPE_LUMA )

{

ctxOfs += diag < 3 ? 10 : ( diag < 10 ? 5 : 0 );

}

m_gt1FlagCtxSet( ctxOfs + ctxIdx );

where posX, posY define the (x,y) position a coefficient within a TU, pData[] represents the array containing the partial (accumulated level up to the last parsed coding pass) transform coefficient levels in a TU. If pData[] lies outside of a TU boundary, then it is assumed to have zero value.

## Greater than 2 (gt2) flag coding:

gt2 flag coding context for luma/chroma is described by the following method:

diag = posX + posY;

numPos = 0;

numPos += abs(pData[ 1 ]) > 2;

numPos += abs(pData[ 2 ]) > 2;

numPos += abs(pData[ m_width + 1 ]) > 2;

numPos += abs(pData[ m_width ]) > 2;

numPos += abs(pData[ 2 * m_width ]) > 2;

ctxIdx = min( numPos, 4 ) + 1;

ctxOfs = 0;

if( m_chType == CHANNEL_TYPE_LUMA )

{

ctxOfs += diag < 3 ? 10 : ( diag < 10 ? 5 : 0 );

}

m_gt1FlagCtxSet( ctxOfs + ctxIdx );

where posX, posY define the (x,y) position a coefficient within a TU, pData[] represents the array containing the partial (accumulated level up to the last parsed coding pass) transform coefficient levels in a TU. If pData[] lies outside of a TU boundary, then it is assumed to have zero value.

## remAbsLevel coding:

The remAbsLevel of the remaining part of the coefficient levels are coded same as HEVC except for the Rice parameter r calculation based on the levels of the coefficients on the local neighborhood of the coefficient being coded.

The Rice parameter r is derived as follows. For each scan position, the parameter is set to 0. Then, the sum_absolute_levelMinus1 is compared against a threshold set tR={4,12,28,…}. In other words, the Rice parameter is 0 if the sum_absolute_levelMinus1 falls into the first interval, is 1 if sum_absolute_levelMinus1 falls into the second interval and so on. The derivation of the Rice parameter r (order) is summarized in the following.

MAX_GR_ORDER_RESIDUAL = 10;

numPos = 0;

sumAbs = 0;

sumAbs += abs( pData[ 1 ] );

numPos += pData[ 1 ] != 0;

sumAbs += abs( pData[ 2 ] );

numPos += pData[ 2 ] != 0;

sumAbs += abs( pData[ m_width + 1 ] );

numPos += pData[ m_width + 1 ] != 0;

sumAbs += abs( pData[ m_width ] );

numPos += pData[ m_width ] != 0;

sumAbs += abs( pData[ 2 * m_width ] );

numPos += pData[ 2 * m_width ] != 0;

val   = sumAbs - numPos;

for( order = 0; order < MAX_GR_ORDER_RESIDUAL; order++ )

{

if( ( 1 << ( order + 3 ) ) > ( val + 4 ) )

{

break;

}

}

order == MAX_GR_ORDER_RESIDUAL ? ( MAX_GR_ORDER_RESIDUAL - 1 ) : order );

where posX, posY define the (x,y) position a coefficient within a TU, pData[] represents the array containing the transform coefficient levels in a TU. If pData[] lies outside of a TU boundary, then it is assumed to have zero value.

## Context summary:

Following table summarizes the contexts used in coefficient coding:

Table 1: Number of contexts used in coefficient coding of 7.1.1.

| CE-7.1.1 |  |  |
| --- | --- | --- |
| Syntax/Scan | Value | Notes |
| Scan method | Diag |  |
| SigCGFlag | 4 | 2+2 (lum+chr) |
| Last_xy | 58 (29x2) | 25+4 (lum+chr) |
| Sig Luma | 54 (18x3) | TU Size:4x4,8x8,>8x8 |
| Sig Chroma | 12 |  |
| gt1/gt2 Luma | 16 |  |
| gt1/gt2 Chroma | 6 |  |
| Reg. bins | 8GT1/16 |  |
|  | 1GT2/16 |  |
| RiceParam | BMS |  |
| Total # contexts | 150 |  |

# Simulation Results

CE7.1.1 is implemented in the BMS-1.0 software. Core coefficient coding only configuration as well as in combination with HEVC sign data hiding (SDH) results are presented. The simulations were carried out under the JVET common test conditions [4]. Encoding and decoding run times are obtained by running the simulations on XEON_E5_2673v2_16 and XEON_E5_2667v2 cores.

## VTM Configuration (AltResiComp=1)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.83% | -1.15% | -1.02% | 105% | 100% | 13.43% | 45.98% | 41.42% | 12% | 42% |
| Class A2 | -1.21% | -0.95% | -1.04% | 105% | 101% | 10.77% | 21.76% | 16.41% | 11% | 48% |
| Class B | -1.24% | -0.85% | -0.95% | 105% | 101% | 9.62% | 17.70% | 22.92% | 10% | 49% |
| Class C | -1.49% | -0.94% | -1.06% | 107% | 101% | 10.91% | 18.44% | 23.65% | 10% | 58% |
| Class E | -1.92% | -1.65% | -1.78% | 104% | 99% | 13.44% | 18.30% | 21.98% | 12% | 49% |
| Overall | -1.34% | -1.07% | -1.14% | 105% | 101% | 11.37% | 23.35% | 24.92% | 11% | 50% |
| Class D | -1.62% | -0.89% | -0.93% | 107% | 99% | 8.73% | 14.39% | 16.77% | 9% | 60% |
| Class F (optional) | -2.16% | -1.09% | -1.26% | 106% | 100% | 9.35% | 18.34% | 19.40% | 11% | 57% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.63% | -0.79% | -0.64% | 103% | 101% | 19.39% | 45.79% | 48.59% | 26% | 36% |
| Class A2 | -0.70% | -0.57% | -0.69% | 103% | 100% | 23.54% | 28.20% | 19.36% | 30% | 34% |
| Class B | -0.89% | -0.55% | -0.65% | 103% | 99% | 17.43% | 28.75% | 31.10% | 27% | 39% |
| Class C | -1.17% | -0.51% | -0.59% | 104% | 100% | 13.48% | 19.62% | 23.31% | 26% | 45% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.87% | -0.59% | -0.64% | 103% | 100% | 17.99% | 29.61% | 30.18% | 27% | 39% |
| Class D | -1.15% | -0.24% | -0.41% | 105% | 100% | 14.48% | 17.58% | 19.79% | 25% | 45% |
| Class F (optional) | -1.75% | -0.93% | -0.95% | 104% | 100% | 10.60% | 19.45% | 19.27% | 22% | 56% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.55% | -0.53% | -0.07% | 105% | 100% | 11.50% | 11.20% | 12.79% | 28% | 44% |
| Class C | -0.70% | -0.58% | -0.54% | 105% | 104% | 9.40% | 8.89% | 10.12% | 31% | 53% |
| Class E | -0.52% | -0.03% | -0.46% | 104% | 101% | 12.15% | 10.48% | 13.39% | 31% | 52% |
| Overall | -0.59% | -0.42% | -0.32% | 105% | 101% | 10.96% | 10.25% | 12.05% | 30% | 49% |
| Class D | -0.81% | -0.78% | -0.21% | 105% | 103% | 11.52% | 9.44% | 10.59% | 35% | 58% |
| Class F (optional) | -1.35% | -1.08% | -0.56% | 105% | 100% | 10.60% | 14.68% | 15.32% | 27% | 58% |

Table 2: [CE 7.1.1 VTM configuration] Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to the VTM-1.0 (using BMS-1.0 software in VTM configuration).

## BMS Configuration (AltResiComp=1)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -12.56% | -31.49% | -29.68% | 890% | 239% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -10.81% | -18.06% | -14.63% | 968% | 210% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -9.91% | -15.68% | -19.37% | 1003% | 206% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -11.05% | -16.14% | -19.67% | 1098% | 173% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | -13.57% | -16.91% | -19.53% | 849% | 204% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -11.37% | -19.02% | -20.39% | 970% | 203% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -9.50% | -13.18% | -15.09% | 1212% | 165% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -10.39% | -16.05% | -17.08% | 947% | 177% | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -16.77% | -30.74% | -32.64% | 392% | 286% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -19.55% | -21.76% | -16.52% | 343% | 292% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -15.53% | -22.68% | -24.02% | 383% | 253% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -12.90% | -16.79% | -19.35% | 407% | 219% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -15.88% | -22.54% | -23.00% | 383% | 257% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -13.44% | -15.06% | -16.88% | 418% | 224% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -11.09% | -16.67% | -16.76% | 462% | 178% | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -10.74% | -10.48% | -11.30% | 375% | 227% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -9.22% | -8.56% | -9.48% | 344% | 195% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | -11.26% | -9.36% | -12.15% | 337% | 194% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -10.36% | -9.56% | -10.91% | 355% | 207% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -10.68% | -9.13% | -9.40% | 302% | 178% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -10.70% | -13.54% | -13.58% | 389% | 172% | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 3: [CE 7.1.1 BMS configuration] Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to the BMS-1.0.

## VTM Configuration (AltResiComp=1 + SDH)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.73% | -2.49% | -2.37% | 107% | 104% | 12.42% | 43.97% | 39.47% | 12% | 43% |
| Class A2 | -2.10% | -2.42% | -2.31% | 110% | 102% | 9.77% | 19.92% | 14.91% | 11% | 49% |
| Class B | -2.10% | -2.63% | -2.78% | 110% | 100% | 8.70% | 15.60% | 20.66% | 11% | 49% |
| Class C | -2.16% | -2.78% | -2.90% | 112% | 102% | 10.17% | 16.24% | 21.41% | 10% | 59% |
| Class E | -2.52% | -3.51% | -3.50% | 108% | 101% | 12.74% | 16.04% | 19.82% | 13% | 50% |
| Overall | -2.12% | -2.75% | -2.78% | 109% | 102% | 10.50% | 21.27% | 22.86% | 11% | 50% |
| Class D | -2.32% | -2.84% | -2.89% | 113% | 103% | 7.96% | 12.19% | 14.51% | 9% | 62% |
| Class F (optional) | -2.17% | -2.51% | -2.59% | 111% | 101% | 9.32% | 16.66% | 17.77% | 12% | 57% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.37% | -1.39% | -1.40% | 106% | 100% | 18.50% | 44.88% | 47.40% | 27% | 35% |
| Class A2 | -1.22% | -1.32% | -1.43% | 107% | 100% | 22.88% | 27.26% | 18.49% | 31% | 34% |
| Class B | -1.48% | -1.64% | -1.87% | 106% | 99% | 16.74% | 27.40% | 29.49% | 28% | 39% |
| Class C | -1.64% | -1.83% | -1.88% | 107% | 101% | 12.95% | 18.04% | 21.64% | 26% | 46% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.45% | -1.57% | -1.69% | 107% | 100% | 17.31% | 28.37% | 28.78% | 28% | 39% |
| Class D | -1.65% | -1.56% | -1.81% | 108% | 101% | 13.90% | 16.01% | 18.06% | 26% | 45% |
| Class F (optional) | -1.85% | -2.10% | -2.07% | 107% | 100% | 10.50% | 18.03% | 17.90% | 23% | 56% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.27% | -1.87% | -1.48% | 108% | 101% | 10.70% | 9.69% | 11.17% | 29% | 44% |
| Class C | -1.38% | -1.85% | -1.75% | 108% | 103% | 8.65% | 7.46% | 8.77% | 31% | 53% |
| Class E | -0.76% | -1.89% | -1.39% | 107% | 105% | 11.89% | 8.38% | 12.27% | 32% | 54% |
| Overall | -1.18% | -1.87% | -1.55% | 108% | 103% | 10.31% | 8.62% | 10.65% | 30% | 50% |
| Class D | -1.35% | -1.59% | -2.25% | 108% | 100% | 10.93% | 8.54% | 8.25% | 36% | 57% |
| Class F (optional) | -1.53% | -2.48% | -1.99% | 108% | 101% | 10.40% | 13.18% | 13.53% | 28% | 58% |

Table 4: [CE 7.1.1 + SDH VTM configuration] Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to the VTM-1.0 (using BMS-1.0 software in VTM configuration).

## BMS Configuration (AltResiComp=1 + SDH)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -13.29% | -32.32% | -30.78% | 923% | 246% | -0.84% | -1.30% | -1.59% | 104% | 103% |
| Class A2 | -11.54% | -19.13% | -15.63% | 1009% | 211% | -0.81% | -1.33% | -1.18% | 104% | 101% |
| Class B | -10.58% | -17.14% | -20.78% | 1049% | 210% | -0.74% | -1.74% | -1.76% | 105% | 102% |
| Class C | -11.62% | -17.65% | -21.08% | 1160% | 173% | -0.63% | -1.79% | -1.72% | 106% | 101% |
| Class E | -14.03% | -18.26% | -20.95% | 882% | 208% | -0.55% | -1.64% | -1.78% | 104% | 102% |
| Overall | -12.00% | -20.30% | -21.68% | 1014% | 206% | -0.71% | -1.59% | -1.63% | 105% | 102% |
| Class D | -10.03% | -14.74% | -16.61% | 1285% | 167% | -0.58% | -1.79% | -1.79% | 106% | 101% |
| Class F (optional) | -10.46% | -17.20% | -18.42% | 1006% | 175% | -0.10% | -1.38% | -1.63% | 106% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -17.35% | -31.28% | -33.30% | 406% | 287% | -0.70% | -0.80% | -1.02% | 103% | 100% |
| Class A2 | -19.95% | -22.37% | -17.01% | 355% | 293% | -0.49% | -0.78% | -0.57% | 104% | 100% |
| Class B | -16.02% | -23.58% | -25.09% | 396% | 254% | -0.58% | -1.16% | -1.42% | 103% | 100% |
| Class C | -13.27% | -17.73% | -20.49% | 423% | 220% | -0.42% | -1.15% | -1.43% | 104% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -16.34% | -23.32% | -23.89% | 396% | 258% | -0.54% | -1.01% | -1.17% | 104% | 100% |
| Class D | -13.84% | -15.96% | -17.79% | 437% | 223% | -0.46% | -1.10% | -1.18% | 105% | 100% |
| Class F (optional) | -11.23% | -17.59% | -17.67% | 485% | 178% | -0.16% | -1.16% | -1.12% | 105% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -11.40% | -11.58% | -11.82% | 387% | 228% | -0.74% | -1.24% | -0.60% | 103% | 100% |
| Class C | -9.85% | -9.44% | -10.61% | 354% | 192% | -0.69% | -0.98% | -1.26% | 103% | 99% |
| Class E | -11.68% | -10.43% | -12.80% | 343% | 191% | -0.49% | -1.17% | -0.76% | 102% | 98% |
| Overall | -10.95% | -10.58% | -11.66% | 364% | 206% | -0.66% | -1.14% | -0.86% | 103% | 99% |
| Class D | -11.16% | -9.96% | -10.20% | 312% | 179% | -0.53% | -0.90% | -0.90% | 103% | 101% |
| Class F (optional) | -10.90% | -14.83% | -14.90% | 402% | 168% | -0.24% | -1.49% | -1.56% | 103% | 98% |

Table 5: [CE 7.1.1 + SDH BMS configuration] Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to the VTM-1.0.

## VTM Configuration (AltResiComp=1 with 18 significance contexts)

In this test significance flag coding context that splits the contexts according to the TU sizes is removed and only 18 contexts are used for coding of significance flag.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE7-1.1-VTM |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | 0.00% | 87% | 110% | 0.04% | -0.01% | -0.08% | 98% | 110% |
| Class A2 | 0.03% | -0.06% | -0.04% | 95% | 112% | 0.03% | 0.01% | -0.03% | 93% | 108% |
| Class B | 0.02% | -0.05% | -0.02% | 105% | 116% | 0.03% | 0.11% | 0.09% | 108% | 112% |
| Class C | 0.02% | 0.07% | 0.02% | 107% | 117% | -0.01% | 0.07% | 0.06% | 111% | 116% |
| Class E | 0.01% | 0.01% | 0.17% | 117% | 119% | 0.02% | 0.00% | -0.12% | 108% | 114% |
| Overall | 0.02% | -0.02% | 0.02% | 103% | 115% | 0.02% | 0.05% | 0.00% | 104% | 112% |
| Class D | 0.04% | 0.05% | 0.07% | 111% | 124% | 0.04% | 0.12% | 0.08% | 115% | 120% |
| Class F (optional) | 0.09% | 0.00% | 0.08% | 108% | 116% | 0.08% | 0.26% | -0.15% | 106% | 112% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over CE7-1.1-VTM |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.10% | 0.05% | 99% | 110% | 0.03% | -0.03% | -0.04% | 100% | 102% |
| Class A2 | 0.01% | -0.02% | 0.00% | 107% | 110% | 0.01% | 0.01% | -0.08% | 107% | 109% |
| Class B | 0.03% | -0.02% | -0.02% | 127% | 120% | 0.02% | -0.03% | -0.09% | 122% | 121% |
| Class C | 0.04% | 0.09% | 0.01% | 130% | 128% | 0.03% | 0.09% | -0.41% | 125% | 127% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.02% | -0.01% | 0.00% | 118% | 118% | 0.02% | 0.01% | -0.16% | 115% | 116% |
| Class D | 0.05% | -0.03% | 0.02% | 127% | 133% | 0.07% | 0.25% | -0.20% | 124% | 128% |
| Class F (optional) | 0.15% | 0.09% | -0.03% | 129% | 124% | 0.07% | 0.07% | 0.24% | 125% | 125% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over CE7-1.1-VTM |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.02% | -0.20% | 128% | 121% | 0.02% | -0.49% | 0.18% | 125% | 118% |
| Class C | -0.02% | 0.02% | -0.04% | 127% | 129% | -0.04% | -0.18% | -0.15% | 125% | 124% |
| Class E | 0.02% | -0.29% | -0.38% | 130% | 123% | -0.22% | -0.05% | 0.36% | 128% | 124% |
| Overall | 0.01% | -0.06% | -0.19% | 128% | 124% | -0.06% | -0.28% | 0.11% | 126% | 121% |
| Class D | 0.05% | 0.45% | -0.43% | 127% | 135% | 0.09% | 0.18% | 0.04% | 127% | 132% |
| Class F (optional) | 0.18% | -0.36% | -0.27% | 128% | 124% | 0.10% | -0.06% | 0.05% | 147% | 140% |

Table 6: Comparison of using 18 luma significance context versus 54 luma significance contexts for VTM and BMS configurations.

As can be seen from the results 3 sets of luma significance contexts for TU size of 4x4, 8x8 and >8x8 can be reduced to 1 set of 18 contexts with negligible BD-rate impact.

# Conclusion

CE7.1.1 coefficient coding scheme is described with accompanying coding results.

# References

- Y.-W Chen, et al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, JVET-J0021, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, San Diego, USA, April 2018.

- T. Nguyen, D. Marpe, T. Wiegand, “Non-CE11: Proposed Cleanup for Transform Coefficient Coding”, JCTVC-H0228, 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding”, JVET-J1027, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.1-0

- “JVET common test conditions and software reference configuration,” JVET-J1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).