| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0070-v2 |
| --- | --- |

| Title: | Non-CE7: Entropy Coding for Dependent Quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Jie Dong Marta Karczewicz | Email: | mcoban@qti.qualcomm.com jiedong@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this document an alternate dependent quantization entropy coding scheme that allows grouping of bypass coded bins at coefficient group level is described. The state machine driving the quantizater selection and coefficient coding algorithm is driven by the significance of a coefficient rather than the coefficient level parity allowing grouping of remaining level bypass coded bins for efficient parsing. The simulations results show -4.41% AI, -3.01% RA, -2.44% LB BD-Rate versus the VTM-1.0 anchor.

# Introduction

In dependent quantization algorithm of CE7.2.1 the coefficients are coded coefficient by coefficient [1]. The dependent quantization state machine states depend on the parity of the coefficient levels. The coefficient significance and greater than 1 (gt1) flags coding contexts as well as the quantizers used for a coefficient are selected by the state of the previously coded coefficient. This scheme results in interleaved regular coded bins and bypass coded bins at coefficient level within a coefficient group.

In this proposal an alternate dependent quantization entropy coding scheme that allows grouping of bypass coded bins at coefficient group level is presented.

# Proposed algorithm

In this algorithm a 4-state, two scalar quantizer Trellis Coded Quantization similar to the one descibed in JVET-J0014 is used [1].  Two scalar quantizers as shown below are used for performing the quantization.

Figure 1:  Illustration of the two scalar quantizers used: The first quantizer Q0 maps the transform coefficient levels (black numbers below the points) to even integer multiples of the quantization step size Δ.  The second quantizer Q1 maps the transform coefficient levels to odd integer multiples of the quantization step size Δ or to zero [JVET-J0014].

Transform coefficients are quantized in coding order and the quantizer that is used to perform the quantization is selected based on a state machine that is driven by its previous state and the parity of the level of the previously coded coefficient. The state machine is described by the following figure:

|  | current state; next state for …. sig == 0; sig==1. 0; 0; 2. 1; 2; 0. 2; 1; 3. 3; 3; 1. |
| --- | --- |

Figure 2: State machine diagram for coefficient significance-driven state machine.

On the encoder side, the trellis to decide the coefficient levels is illustrated below figure:

Figure 3: Encoder trellis diagram for coefficient significance driven state machine.

The states 0 and 1 use quantizer Q0, and states 2 and 3 use quantizer Q1. For significance map coding the context set used to code is selected based on the quantizer used, i.e. Qx where x=state>>1. Greater than 1flag (gt1) context set only uses one context set that is identical to the context set used for coding of gt1 flag used in coefficient coding in CE 7.1.2.

In the proposed scheme, in a coefficient group all regular coded level bins (sig, gt1, gt2, gt3, gt4) are coded with the same contexts as in CE 7.2.1 (except for gt1, i.e. single set) coefficient by coefficient in the first pass, this is followed by grouped remaining level for all coefficients in a coefficient group. The context index derivation is identical to the method used in CE7.2.1 where full coefficient levels are replaced by coefficient levels reconstructed using upto the highest coded regular coded bin (resulting index is equal to the index derived from full coefficient value). The proposed coefficient coding scheme without dependent quantization results in identical RD results except for reshuffled bins. The remaining level coding is identical to the scheme used in CE7.1.2 and CE7.2.1.

The change in bitstream format can be illustrated by the following figure:

Figure 4: Proposed syntax, grouped bypass coded bins.

# Results

The proposed scheme is implemented on top of CE 7.2.1 software [3].  The simulation was carried out under the JVET common test conditions [4].

|  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-1.0 |  |  |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  | -4.33% |  | -2.91% | -2.43% | 107% | 98% | -3.07% | -2.64% |  | -2.58% | 95% | 101% |
| Class A2 | -4.87% | -1.48% | -2.43% | 115% | 95% | -2.94% | -1.14% | -1.35% | 103% | 101% |
| Class B | -4.23% | -1.05% | -0.39% | 114% | 97% | -2.33% | -1.62% | 0.56% | 102% | 104% |
| Class C | -4.45% | 0.01% | -0.09% | 123% | 97% | -2.08% | -1.26% | 1.45% | 108% | 104% |
| Class E | -4.26% | -0.90% | -0.93% | 111% | 99% | -1.93% | -1.37% | 1.34% | 100% | 105% |
| Overall | -4.41% | -1.17% | -1.09% | 114% | 97% | -2.43% | -1.59% | 0.05% | 102% | 103% |
| Class D | -4.47% | 0.88% | 0.55% | 125% | 99% | -2.04% | -0.16% | 2.02% | 110% | 105% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Over VTM-1.0 |  |  |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.86% | -1.85% | -0.91% | 104% | 99% | -2.23% | -1.21% | -0.58% | 106% | 102% |
| Class A2 | -2.61% | -0.65% | -2.35% | 108% | 98% | -1.73% | -0.56% | -1.40% | 113% | 104% |
| Class B | -3.30% | -1.41% | -0.55% | 108% | 98% | -2.08% | -1.31% | 1.12% | 110% | 104% |
| Class C | -3.07% | -1.47% | -1.44% | 114% | 100% | -1.52% | -1.76% | 0.53% | 116% | 109% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.01% | -1.36% | -1.22% | 109% | 99% | -1.89% | -1.26% | 0.12% | 111% | 105% |
| Class D | -2.98% | 0.32% | -0.64% | 115% | 98% | -1.43% | -0.69% | 1.36% | 117% | 109% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Over VTM-1.0 |  |  |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.80% | -2.89% | -0.57% | 110% | 101% | -1.99% | -2.94% | 0.13% | 115% | 104% |
| Class C |  | -2.52% |  | -3.60% | -2.72% | 115% | 101% | -1.57% | -3.14% |  | -1.16% | 126% | 115% |
| Class E | -1.72% | 1.05% | 1.50% | 105% | 103% | -1.04% | 0.18% | 2.48% | 118% | 111% |
| Overall | -2.44% | -2.15% | -0.77% | 110% | 101% | -1.61% | -2.23% | 0.29% | 119% | 109% |
| Class D | -2.36% | -2.81% | -2.80% | 116% | 100% | -1.38% | -2.01% | -1.46% | 125% | 116% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 1: JVET-K0070 results for VTM and BMS configurations versus the VTM-1.0 and BMS-1.0 anchors.

Table below shows the JVET-K0070 VTM and BMS results compared to the CE7-2.1.0 VTM and BMS configuration results.

|  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over CE7-2.1-VTM |  |  |  |  |  |  | Over CE7-2.1-BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.57% | 0.76% | 0.86% | 101% | 102% | 0.44% | 0.67% | 0.80% | 89% | 102% |
| Class A2 | 0.62% | 0.76% | 0.75% | 99% | 99% | 0.47% | 0.71% | 0.62% | 91% | 102% |
| Class B | 0.65% | 0.87% | 0.90% | 100% | 100% | 0.47% | 0.87% | 0.90% | 91% | 106% |
| Class C | 0.60% | 0.79% | 0.89% | 100% | 101% | 0.37% | 0.86% | 0.78% | 91% | 108% |
| Class E | 0.61% | 0.93% | 0.96% | 101% | 102% | 0.39% | 0.92% | 1.00% | 91% | 103% |
| Overall | 0.61% | 0.83% | 0.88% | 100% | 101% | 0.43% | 0.82% | 0.83% | 91% | 105% |
| Class D | 0.64% | 0.91% | 0.79% | 99% | 100% | 0.36% | 0.90% | 0.82% | 92% | 109% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Over CE7-2.1-VTM |  |  |  |  |  |  | Over CE7-2.1-BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | 0.31% | 0.41% | 0.42% | 101% | 102% |
| Class A2 |  |  |  |  |  | 0.24% | 0.47% | 0.38% | 101% | 105% |
| Class B | 0.32% | 0.20% | -0.16% | 100% | 105% | 0.32% | 0.45% | 0.53% | 100% | 105% |
| Class C | 0.29% | 0.25% | -0.13% | 100% | 98% | 0.22% | 0.56% | 0.66% | 100% | 111% |
| Class E | 0.17% | 0.14% | 0.04% | 100% | 102% |  |  |  |  |  |
| Overall | 0.27% | 0.20% | -0.10% | 100% | 102% | 0.28% | 0.48% | 0.51% | 100% | 106% |
| Class D | 0.26% | -0.57% | 0.49% | 100% | 105% | 0.26% | 0.59% | 0.33% | 100% | 112% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Over CE7-2.1-VTM |  |  |  |  |  |  | Over CE7-2.1-BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.32% | 0.20% | -0.16% | 102% | 113% | 0.26% | -0.03% | 0.02% | 104% | 109% |
| Class C | 0.29% | 0.25% | -0.13% | 102% | 115% | 0.28% | 0.23% | 0.32% | 109% | 115% |
| Class E | 0.17% | 0.14% | 0.04% | 102% | 112% | 0.26% | 0.04% | -0.70% | 111% | 111% |
| Overall | 0.27% | 0.20% | -0.10% | 102% | 113% | 0.27% | 0.07% | -0.06% | 107% | 111% |
| Class D | 0.26% | -0.57% | 0.49% | 102% | 121% | 0.23% | -0.68% | -0.15% | 107% | 116% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2: JVET-K0070 results for VTM and BMS configurations versus the CE-7.2.1 VTM and BMS configurations.

As can be seen from Table 2, the performance drop form driving the trellis with significance map information is 0.61% AI, 0.40% RA, 0.27% LB BD-Rate for VTM configuration and 0.43% AI, 0.28% RA, 0.27% LB for BMS configuration with respect to level parity driven trellis in CE7-2.1.

For dependent quantization off case, i.e. “–DepQuant=0”, table shows that the results are identical to CE7-1.2.0 case, as expected.

|  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over CE7-1.2-0-VTM |  |  |  |  |  |  | Over CE7-1.2-0-BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 103% | 0.00% | 0.00% | 0.00% | 104% | 109% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% | 0.00% | 0.00% | 105% | 113% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% | 0.00% | 0.00% | 108% | 113% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 104% | 0.00% | 0.00% | 0.00% | 105% | 114% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% | 0.00% | 0.00% | 0.00% | 109% | 111% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 106% | 112% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% | 0.00% | 0.00% | 0.00% | 103% | 114% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Over CE7-1.2-0-VTM |  |  |  |  |  |  | Over CE7-1.2-0-BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 102% | 0.00% | 0.00% | 0.00% | 102% | 105% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 100% | 105% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% | 0.00% | 0.00% | 0.00% | 113% | 114% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 108% | 118% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 107% | 111% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 102% | 0.00% | 0.00% | 0.00% | 109% | 116% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Over CE7-1.2-0-VTM |  |  |  |  |  |  | Over CE7-1.2-0-BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 99% | 0.00% | 0.00% | 0.00% | 124% | 119% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 124% | 126% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 103% | 0.00% | 0.00% | 0.00% | 126% | 121% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 125% | 122% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 101% | 0.00% | 0.00% | 0.00% | 127% | 130% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3: JVET-K0070 results without dependent quantization versus the CE7-1.2-0 VTM and BMS configurations.

# Conclusion

In this document an alternate dependent quantization entropy coding scheme that allows grouping of bypass coded bins at coefficient group level wass described. The state machine driving the quantizater selection and coefficient coding algorithm is driven by the significance of a coefficient rather than the coefficient level parity allowing grouping of remaining level bypass coded bins for efficient parsing.

# References

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding”, JVET-J1027, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.2-0

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-2.1-0

- “JVET common test conditions and software reference configuration,” JVET-J1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).