| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0073-v1 |
| --- | --- |

| Title: | CE3: Multiple neighbor-based linear model (Tests 4.1.3, 4.1.4, and 4.1.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Hui-Yu Jiang | Email: | morriss.yj.chang@foxconn.com teresa.hy.jiang@foxconn.com |
| Source: | Foxconn Technology Group |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Test 4.1.3, Test 4.1.4, and Test 4.1.5 in Core Experiment (CE) 3. The experiments evaluate the proposed multiple neighbor-based linear model (MNLM), which uses multiple neighbor sets for LM, MMLM and MFLM to cover various linear relationships between luma samples and chroma samples in CU. Test 4.1.3 evaluates MNLM with 1 neighbor set for LM and 2 neighbor sets for MMLM/MFLM. Test 4.1.4 evaluates MNLM with 1 neighbor set for LM and 3 neighbor sets for MMLM/MFLM. Test 4.1.5 evaluates MNLM with 1 neighbor set for LM and 4 neighbor sets for MMLM/MFLM. On top of VTM AI configuration, Test 4.1.3 provides -1.74%/-9.78%/-10.73% YUV-BDrate with 102/102% coding time change, Test 4.1.4 provides -1.74%/-10.07%/-11.03% YUV-BDrate with 106/102% coding time change, and Test 4.1.5 provides -1.74%/-10.20%/-11.17% YUV-BDrate with 110/102% coding time change. To further study the performance of MNLM, supplementary results are provided by disabling the MFLM on top of Test 4.1.3, Test 4.1.4, and Test 4.1.5, respectively. On top of VTM AI configuration, Test 4.1.3 without MFLM provides -1.73%/-9.65%/-10.60% YUV-BDrate with 96/102% coding time change, Test 4.1.4 without MFLM provides -1.74%/-9.87%/-10.92% YUV-BDrate with 97/102% coding time change, and Test 4.1.5 without MFLM provides -1.74%/-10.02%/-11.09% YUV-BDrate with 101/102% coding time change.

Introduction

In JVET-J0026 [1], the multiple neighbor-based linear model (MNLM) was proposed to use multiple neighbor sets for LM, MMLM and MFLM to cover various linear relationships between luma samples and chroma samples in CU. It was included into Core Experiment (CE) 3 [2] to evaluate the performance on top of VTM and BMS configurations [3]. The following table shows the descriptions for each CE test reported in this document:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 4.1.3 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.4 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.5 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) + MMLM/MFLM (A, B, E, G) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |

Multiple Neighbor-based Linear Model (MNLM)

The MNLM uses multiple neighbor sets for LM, MMLM and MFLM to cover various linear relationships between luma samples and chroma samples in CU. As depicted in Fig. 1, five neighbor sets, N1, N2, N3, N4 and N5, are enabled in the proposed MNLM:

N1: B, C

N2: A, B, C, D

N3: B, C, E, F

N4: C, D, F, H

N5: A, B, E, G

Fig. 1. Multiple neighbors for LM, MMLM and MFLM

The different cross component linear model (CCLM) prediction modes in MNLM are listed in the table below:

| Mode index | Description |
| --- | --- |
| 0 | LM with N1 |
| 1 | MMLM with N2 |
| 2 | MMLM with N3 |
| 3 | MMLM with N4 |
| 4 | MMLM with N5 |
| 5 | MFLM with N2 |
| 6 | MFLM with N3 |
| 7 | MFLM with N4 |
| 8 | MFLM with N5 |

Mode 1, Mode 2, Mode 3, and Mode 4 use the same down-sampling filter, but different neighbor sets. Similarly, Mode 5, Mode 6, Mode 7, and Mode 8 use the same down-sampling filter, but different neighbor sets. Note that the down-sampling filters are different between Modes 1-4 and Modes 5-8.

At encoder side, a fast chroma mode decision is implemented by enabling only two chroma modes for RDO checks: one is DM mode, and the other one is the best mode selected by SATD process.

Experiment result

The MNLM is tested on top of VTM and BMS configurations under the common test conditions specified in [2] and [3]. Table 1, Table 2 and Table 3 show the experimental results for Test 4.1.3, Test 4.1.4 and Test 4.1.5 in VTM configuration, respectively. Table 5, Table 6 and Table 7 show the experimental results for Test 4.1.3, Test 4.1.4 and Test 4.1.5 in BMS configuration, respectively.

Table 1. Summary of experimental results (Test 4.1.3 v.s. VTM anchor)

Table 2. Summary of experimental results (Test 4.1.4 v.s. VTM anchor)

Table 3. Summary of experimental results (Test 4.1.5 v.s. VTM anchor)

Table 4. Summary of experimental results (Test 4.1.3 v.s. BMS anchor)

Table 5. Summary of experimental results (Test 4.1.4 v.s. BMS anchor)

Table 6. Summary of experimental results (Test 4.1.5 v.s. BMS anchor)

Supplemental experiment result

The MNLM is also investigated as MFLM is disabled. The different CCLM prediction modes in MNLM are listed in the table below:

| Mode index | Description |
| --- | --- |
| 0 | LM with N1 |
| 1 | MMLM with N2 |
| 2 | MMLM with N3 |
| 3 | MMLM with N4 |
| 4 | MMLM with N5 |

The MNLM with MFLM disabled is implemented on top of VTM and BMS configurations. The experiments were conducted under the common test conditions specified in [2] and [3]. Table 7, Table 8 and Table 9 show the experimental results for Test 4.1.3 without MFLM, Test 4.1.4 without MFLM and Test 4.1.5 without MFLM in VTM configuration, respectively.

Table 7. Summary of experimental results (Test 4.1.3 without MFLM v.s. VTM anchor)

Table 8. Summary of experimental results (Test 4.1.4 without MFLM v.s. VTM anchor)

Table 9. Summary of experimental results (Test 4.1.5 without MFLM v.s. VTM anchor)

Conclusion

This contribution shows the experimental results of Test 4.1.3, Test 4.1.4, Test 4.1.5. It is observed that the proposed MNLM could provide a better trade off in terms of bitrate reduction and the coding time among CE3.4 tests. Therefore, it is recommended to adopt MNLM into VTM and BMS.

Reference

- K. Misra, J. Zhao, A. Segall, W. Zhu, B. Choi, F. Bossen, M. Horowitz, P. Cowan, S. Deshpande, Y. Yasugi, T. Hashimoto, T. Zhou, T. Ikai, T. Chujoh, T. Aono, Y.-J. Chang, H.-Y. Jiang, T.-H. Li, and Y.-C. Yang, “Description of SDR and HDR video coding technology proposal by Sharp and Foxconn,” JVET-J0026, San Diego, USA, Apr. 2018.

- G. V. der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding,” JVET-J1023, San Diego, USA, Apr. 2018.

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, Apr. 2018.

Patent rights declaration(s)

Foxconn Technology Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).