| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0074-v1 |
| --- | --- |

| Title: | CE3-related: Combination of LAM (Test 4.1.1) and MNLM (Tests 4.1.3, 4.1.4, and 4.1.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Hui-Yu Jiang | Email: | morriss.yj.chang@foxconn.com teresa.hy.jiang@foxconn.com |
| Source: | Foxconn Technology Group |  |  |

_____________________________

- Abstract

This contribution reports the experimental results related to the combination of LM-Angular mode (LMA) in CE3 Test 4.1.1 and Multiple Neighbor-based Linear Model (MNLM) in CE3 Test 4.1.3, 4.1.4 and 4.1.5 without MFLM. On top of VTM AI configuration, Test 4.1.1 + 4.1.3 without MFLM provides -1.81%/-10.29%/-11.13% YUV-BDrate with 105/104% coding time change, Test 4.1.1 + Test 4.1.4 without MFLM provides -1.82%/-10.51%/-11.39% YUV-BDrate with 107/104% coding time change, and Test 4.1.1 + Test 4.1.5 without MFLM provides -1.83%/-10.63%/-11.57% YUV-BDrate with 110/104% coding time change.

Introduction

The LM-Angular mode (LMA) was proposed to generate the prediction by a combination of MMLM prediction and the prediction obtained using DC, PLANAR or angular modes. The Multiple Neighbor-based Linear Model (MNLM) was proposed to use multiple neighbor sets for LM, MMLM and MFLM to cover various linear relationships between luma samples and chroma samples in CU. Both LMA and MNLM were tested in CE3 [1], i.e., Test 4.1.1, Test 4.1.3, Test 4.1.4 and Test 4.1.5, as shown in the following table:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 4.1.1 | LM + MMLM + MFLM + LM-Angular | Adarsh Ramasubramonian (Qualcomm) | Jangwon Choi (LGE) |
| 4.1.2 | MMLM + MFLM + LM-angular | Adarsh Ramasubramonian (Qualcomm) | Jangwon Choi (LGE) |
| 4.1.3 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.4 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.5 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) + MMLM/MFLM (A, B, E, G) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |

This contribution simply combines the LMA and MNLM. There is no additional change on respective LMA and MNLM in the tests. The experimental results are provided in the next section.

Experiment result

The experiments were conducted on top of VTM1.0 and BMS1.0 under the common test conditions specified in [1] and [2]. Table 1, Table 2 and Table 3 show the experimental results for Test 4.1.1 + Test 4.1.3 without MFLM, Test 4.1.1 + Test 4.1.4 without MFLM and Test 4.1.1 + Test 4.1.5 without MFLM in VTM configuration, respectively.

Table 1. Summary results (Test 4.1.1 + Test 4.1.3 without MFLM v.s. VTM anchor)

Table 2. Summary results (Test 4.1.1 + Test 4.1.4 without MFLM v.s. VTM anchor)

Table 3. Summary results (Test 4.1.1 + Test 4.1.5 without MFLM v.s. VTM anchor)

Conclusion

This contribution shows the experiment results of the combination of LMA (CE3-4.1.1) and MNLM (CE3-4.1.3, 4.1.4, and 4.1.5). It is observed that the combination can provide better performance compared with VTM, original LMA (CE3-4.1.1) and original MNLM (CE3-4.1.3, 4.1.4, and 4.1.5). Therefore, it is recommended to consider this combination for potential adoption into VTM and BMS.

Reference

- G. V. der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding,” JVET-J1023, San Diego, USA, Apr. 2018.

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, Apr. 2018.

Patent rights declaration(s)

Foxconn Technology Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).