| hJoint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0074-v3 |
| --- | --- |

| Title: | CE3-related: Combination of LAM (Test 4.1.1) and MNLM (Tests 4.1.3, 4.1.4, and 4.1.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Hui-Yu Jiang | Email: | morriss.yj.chang@foxconn.com teresa.hy.jiang@foxconn.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

This contribution investigates the experimental results related to the combination of LM-Angular mode (LMA) in CE3 Test 4.1.1 and Multiple Neighbor-based Linear Model (MNLM) in CE3 Test 4.1.3, 4.1.4 and 4.1.5. On top of VTM AI configuration, LMA (CE3 Test 4.1.1) + MNLM (CE3 Test 4.1.5) with MFLM disabled provides -1.83%/-10.63%/-11.57% YUV-BDrate with 110/104% coding time change. This contribution further investigates the combined LMA and MNLM tool as the separate trees for luma and chroma are enabled as described in CE1 Test 5.1.1, i.e., the configuration parameter ‘DualITree’ set as 1. Compared with VTM AI configuration with ‘DualITree = 1’, LMA (CE3 Test 4.1.1) + MNLM (CE3 Test 4.1.5) with separate trees enabled (CE1 Test 5.1.1) and MFLM disabled provides -3.19%/-14.42%/-14.53% YUV-BDrate with 105/103% coding time change.

Introduction

The LM-Angular mode (LMA) was proposed to generate the prediction by a combination of MMLM prediction and the prediction obtained using DC, PLANAR or angular modes. The Multiple Neighbor-based Linear Model (MNLM) was proposed to use multiple neighbor sets for LM, MMLM and MFLM to cover various linear relationships between luma samples and chroma samples in CU. Both LMA and MNLM were tested in CE3 [1], i.e., Test 4.1.1, Test 4.1.3, Test 4.1.4 and Test 4.1.5, as shown in the following table:

| CE3 Test # | Description | Tester |
| --- | --- | --- |
| 4.1.1 | LM + MMLM + MFLM + LM-Angular | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.3 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) | Yao-Jen Chang (Foxconn) |
| 4.1.4 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) | Yao-Jen Chang (Foxconn) |
| 4.1.5 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) + MMLM/MFLM (A, B, E, G) | Yao-Jen Chang (Foxconn) |

Fig. 1. Multiple neighbors for LM, MMLM and MFLM

This contribution simply combines the LMA and MNLM. The separate trees tools ON/OFF are also configured and tested. Note that CE1-5.1.1 has proceeded the separate trees on top of VTM as follows:

| CE1 Test # | Description |
| --- | --- |
| 5.1.1 | This test applies the separate trees for I slices that is implemented in VTM to QTBT+TT by enabling the config parameter DualITree. |

The experimental results are provided in the next section.

Experiment result with separate trees disabled

The experiments were conducted on top of VTM1.0 and BMS1.0 under the common test conditions specified in [1] and [2]. Table 1, Table 2 and Table 3 show the experimental results for Test 4.1.1 + Test 4.1.3 without MFLM, Test 4.1.1 + Test 4.1.4 without MFLM and Test 4.1.1 + Test 4.1.5 without MFLM in VTM configuration, respectively. The separate trees are disabled for both anchors and the proposed combinations. Complete data are appended with this contribution.

Table 1. Summary results with separate trees disabled (Test 4.1.1 + Test 4.1.3 with MFLM disabled v.s. VTM anchor)

Table 2. Summary results with separate trees disabled (Test 4.1.1 + Test 4.1.4 with MFLM disabled v.s. VTM anchor)

Table 3. Summary results with separate trees disabled (Test 4.1.1 + Test 4.1.5 with MFLM disabled v.s. VTM/BMS anchor)

Experiment result with separate trees enabled

The same experiments were conducted on top of VTM1.0 and BMS1.0 under the common test conditions specified in [1] and [2]. Table 4 shows the experimental results for Test 4.1.1 + Test 4.1.5 without MFLM in VTM configuration, respectively. The separate trees are enabled for both anchors and the proposed combinations. Complete data are appended with this contribution.

Table 4. Summary results with separate trees enabled (Test 4.1.1 + Test 4.1.5 with MFLM disabled v.s. VTM anchor)

Conclusion

This contribution shows the experiment results of the combination of LMA (CE3-4.1.1) and MNLM (CE3-4.1.3, 4.1.4, and 4.1.5). It is observed that the combination can provide better performance compared with VTM, the original LMA (CE3-4.1.1) and the original MNLM (CE3-4.1.3, 4.1.4, and 4.1.5). Besides, the All-Intra results also show LMA + MNLM with dualITree enabled can reach 3.2% (Y), 14.4% (U), and 14.5% (V) gains with only 5%/3% coding time increase. Therefore, it is recommended to consider this combination and the separate trees for potential adoption into VTM and BMS.

Reference

- G. V. der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding,” JVET-J1023, San Diego, USA, Apr. 2018.

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, Apr. 2018.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).