| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0075-v4 |
| --- | --- |

| Title: | CE8-2.1: Current picture referencing using block level flag signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu 661 Bryant St Palo Alto, CA 94301 USA | Email: | xiaozhongxu@tencent.com |
| Source: | Tencent America |  |  |

_____________________________

# Abstract

In the contribution, the results of CE8 test 2.1: current picture referencing (CPR) using block level flag signaling are reported. In this approach, the use of CPR is signaled by using a block level flag, which is placed right after merge_flag and when merge_flag is not true. Other aspects of this method is similar as motion compensation. It is reported that under VTM configurations, the BD rate changes relative to the VTM anchors are:

- In AI, -0.95%/-20.25%/-52.94% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.50%/-15.83%/-38.92% for CTC/Class F/SCC 1080p classes, separately.

- In LB, -0.21%/-10.05%/-31.03% for CTC/Class F/SCC 1080p classes, separately.

It is also reported in this document that under BMS configurations, the BD rate changes relative to the BMS anchors are:

- In AI, -0.35%/-17.92%/-50.30% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.32%/-14.36%/-35.87% for CTC/Class F/SCC 1080p classes, separately.

- In LB, -0.24%/-9.04%/-28.12% for CTC/Class F/SCC 1080p classes, separately.

# Introduction

Current picture referencing (CPR) or Intra block copy (IBC) [1] has been proposed during the standardization of HEVC SCC extensions [2]. It has been proved to be efficient when coding of screen content video materials. This method was previously proposed in JVET-J0029 and JVET-J0050 to address the need for efficient screen content coding. In this contribution, the method is implemented on the BMS software version 1.0 [5] and tested under CE8 test conditions [4]. In this proposal, the signaling of CPR mode is done by using a block level flag, referred as CPR_flag.

# Algorithm details

Decoder aspect:

Skip flag

Merge idx

PredMode

yes

no

End of CU

Residue coding

Intra coding

intra

inter

merge flag

yes

no

Merge idx

CPR flag

yes

no

BV

coding

MV

coding

Fig. 1: Syntax flowchart of proposed CPR approach in CE8-2.1

When merge mode is true, a current block can be coded in CPR mode, if a CPR coded merge candidate is chosen; it can also be coded in regular inter mode, if an inter coded merge candidate is chosen. When merge flag is false (non-merge case), a CPR_flag is signaled after merge flag to indicate the usage of CPR mode. If a block is coded as a non-merge CPR mode:

- In CPR mode, both block width and height are smaller than or equal to 16.

- Block vector, the displacement vector for CPR mode, is always in 1-pel integer resolution. When performing chroma compensation, the chroma block vector is scaled from its luma block vector and the fractional part is removed.

- The block vector prediction method uses two candidates as predictors, one from left neighbor and one from above neighbor (if CPR coded). When either neighbor is not available, a default block vector ((-2*block_width, 0) or (-block_width, 0)) will be used as a predictor. A flag is signaled to indicate the block vector predictor index.

- Block vector difference is coded in the same way as a motion vector difference.

In Fig.1, a flowchart of the proposed CPR approach is shown.

Encoder aspect:

The encoder performs RD check for blocks with either width or height no larger than 16. For non-merge mode, the block vector search is performed using hash-based search first. If there is no valid candidate found from hash search, block matching based local search will be performed.

In the hash-based search, hash key matching (32-bit CRC) between the current block and a reference block is extended to all allowed block sizes. The hash key calculation for every position in current picture is based on 4x4 blocks. For the current block of a larger size, a hash key matching to a reference block happens when all its 4×4 blocks match the hash keys in the corresponding reference locations. If multiple reference blocks are found to match the current block with the same hash key, the block vector costs of each candidates are calculated and the one with minimum cost is selected.

In block matching search, the search range is set to be 64 pixels to the left and on top of current block.

# Simulations results

The proposed method is tested with both VTM and BMS configurations under common test conditions [3]. For tests in CE8, in addition to the sequences in CTC, both class F and additional 4 SCC 1080p sequences are to be tested.

Table I: CPR flag approach under VTM configurations

Table II: CPR flag approach under BMS configurations

# Conclusion

In this contribution, an implementation of CPR mode using block level signaling flag is proposed and tested. Large coding gains for screen content materials are observed from the simulation results. It is recommended to include this method into the test model for further consideration.

# References

[1] X. Xu, S. Liu, T. Chuang, Y. Huang, S. Lei, K. Rapaka, C. Pang, V. Seregin, Y. Wang, and M. Karczewicz, “Intra Block Copy in HEVC Screen Content Coding Extensions,” IEEE J. Emerg. Sel. Topics Circuits Syst., vol. 6, no. 4, pp. 409-419, 2016.

[2] High Efficiency Video Coding (HEVC), Rec. ITU-T H.265 and ISO/IEC 23008-2, Dec. 2016.

[3] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, CA, April 2018

[4] X. Xu, K. Müller and L. Wang, “Description of Core Experiment 8 (CE8): Current Picture Referencing”, JVET-J1028, San Diego, CA, April 2018

[5] https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).