| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0079-v1 |
| --- | --- |

| Title: | [CE4-2.6] Simplified ATMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang, Junghak Nam, Seung-Hwan Kim, Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 {hm.jang, junghak.nam, seunghwan3.kim, jaehyun.lim}@lge.com |
| Source: | LG Electronics inc. |  |  |

_____________________________

- Abstract

In JVET-J0017[1], a response of LGE to the call for proposals (CfP), Simplified ATMVP was applied to the merge candidate list construction instead of conventional (i.e. JEM) ATMVP. This contribution reports the experimental results on two CE test cases of the simplified ATMVP as follows.

- CE4-2.6(a): one fixed collocated picture is used to derive temportal motion information

- CE4-2.6(b): CE4-2.6(a) plus the adoptive sub-block decision in CE4-2.5(b).

Simulation results for CE4-2.6(a) shows on average 1.09% and 1.3% luma BD-rate gain compared to VTM1.0 in RA and LDB configurations, respectively. It is reported that the average achieved bit-savings for CE4-2.6(a) are approximately 0.73% and 1.08% compared to BMS1.0 without ATMVP as a place holder in RA and LDB configurations, respectively.

It is also reported that experimental results on CE4-2-6(b) showed 1.03% and 1.27% bit-savings compared to VTM1.0 in RA and LDB configurations and 0.69% and 1.05% bit-savings compared to BMS1.0 without ATMVP as a place holder in RA and LDB configurations, respectively.

Proposed method

The proposed simplified ATMVP is basically similar to ATMVP as place holder for the temporal motion vector derivation. However, in the proposed method, the usage of memory is effectively reduced because the reference picture to find the corresponding block is restricted to the collocated picture (designated in the slice segment header), while 4 reference pictures are required in the place holder method. To find the corresponding block, a temporal vector is derived from one of spatial candidates in a scanning order. If the current candidate has the reference picture same with the collocated picture, the search is finished. If all of candidates do not refer the collocated picture then zero motion is applied.

Experimental results

CE4-2.6 (a)

In CE4-2.6(a), it is applied that Simplified ATMVP refer one fixed collocated picture to derive temporal motion information.

Table 1: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.80% | -0.77% | -0.66% | 105% | 114% |
| Class A2 | -1.07% | -0.80% | -0.85% | 107% | 117% |
| Class B | -1.02% | -0.77% | -0.66% | 106% | 113% |
| Class C | -1.41% | -1.07% | -1.18% | 105% | 113% |
| Class E |  |  |  |  |  |
| Overall | -1.09% | -0.86% | -0.84% | 106% | 114% |
| Class D | -1.58% | -0.94% | -1.03% | 106% | 91% |

Table 2: Experimental results for random access test condition; anchor is BMS1.0 without conventional ATMVP

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -0.58% | -0.55% | 102% | 96% |
| Class A2 | -0.61% | -0.47% | -0.45% | 102% | 96% |
| Class B | -0.58% | -0.51% | -0.53% | 101% | 97% |
| Class C | -1.11% | -1.05% | -1.01% | 99% | 115% |
| Class E |  |  |  |  |  |
| Overall | -0.73% | -0.66% | -0.65% | 101% | 101% |
| Class D | -1.17% | -0.93% | -0.89% | 100% | 135% |

Table 3: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.65% | -0.67% | -0.39% | 107% | 110% |
| Class C | -1.30% | -1.06% | -0.86% | 104% | 113% |
| Class E | -2.37% | -1.88% | -2.57% | 104% | 119% |
| Overall | -1.30% | -1.10% | -1.10% | 105% | 113% |
| Class D | -1.84% | -1.47% | -0.62% | 104% | 110% |

Table 4: Experimental results for low-delay test condition; anchor is BMS1.0 without conventional ATMVP

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.53% | -0.53% | -0.78% | 102% | 106% |
| Class C | -0.93% | -0.96% | -1.08% | 99% | 105% |
| Class E | -2.21% | -1.77% | -2.42% | 98% | 108% |
| Overall | -1.08% | -0.98% | -1.29% | 100% | 106% |
| Class D | -1.16% | -1.39% | -1.19% | 98% | 106% |

CE4-2.6 (b)

In CE4-2.6(a), it is applied that Simplified ATMVP refers only one fixed collocated picture to derive temporal motion information, and the sub-block size is decided between 4x4 and 8x8 based on slice level syntax. This sub-block size decision algorithm is identical with CE4-2.5(b). The sub-block size decision algorithm decides sub-block size for ATMVP based on statistic of the prediction unit size and pre-defined threshold value for each slice and each temporal layer in encoder side. In decoder side, adaptive sub-block decision on/off flag is signaled and sub-block size is also signaled when the on/off flag is set as true in slice header.

Table 5: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.78% | -0.74% | -0.61% | 102% | 103% |
| Class A2 | -1.01% | -0.75% | -0.80% | 102% | 104% |
| Class B | -0.96% | -0.72% | -0.61% | 102% | 104% |
| Class C | -1.31% | -0.95% | -1.07% | 102% | 108% |
| Class E |  |  |  |  |  |
| Overall | -1.03% | -0.79% | -0.77% | 102% | 105% |
| Class D | -1.48% | -0.83% | -0.90% | 102% | 99% |

Table 6: Experimental results for random access test condition; anchor is BMS1.0 without conventional ATMVP

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -0.59% | -0.52% | 103% | 100% |
| Class A2 | -0.57% | -0.45% | -0.43% | 104% | 100% |
| Class B | -0.53% | -0.46% | -0.48% | 109% | 100% |
| Class C | -1.03% | -0.95% | -0.91% | 120% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.69% | -0.62% | -0.59% | 110% | 100% |
| Class D | -1.06% | -0.81% | -0.77% | 144% | 115% |

Table 7: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.65% | -0.61% | -0.34% | 104% | 106% |
| Class C | -1.30% | -1.02% | -0.90% | 102% | 106% |
| Class E | -2.27% | -2.10% | -2.63% | 99% | 106% |
| Overall | -1.27% | -1.12% | -1.10% | 102% | 106% |
| Class D | -1.84% | -1.46% | -0.60% | 101% | 106% |

Table 8: Experimental results for low-delay test condition; anchor is BMS1.0 without conventional ATMVP

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.54% | -0.54% | -0.83% | 101% | 104% |
| Class C | -0.92% | -0.96% | -0.99% | 98% | 104% |
| Class E | -2.06% | -2.38% | -2.69% | 98% | 102% |
| Overall | -1.05% | -1.14% | -1.35% | 99% | 104% |
| Class D | -1.17% | -1.53% | -1.17% | 98% | 101% |

Summary

This contribution proposes a simplified ATMVP where only one reference picture (i.e. collocate picture) is required to derive temporal motion vector.

Simulation results for CE4-2.6(a) showed on average 1.09% and 1.3% luma BD-rate gain compared to VTM1.0 in RA and LDB configurations, respectively. It is reported that the average achieved bit-savings for CE4-2.6(a) are approximately 0.73% and 1.08% compared to BMS1.0 without ATMVP as a place holder in RA and LDB configurations, respectively.

It is also reported that experimental results on CE4-2-6(b) showed 1.03% and 1.27% bit-savings compared to VTM1.0 in RA and LDB configurations and on average 0.69% and 1.05% bit-savings compared to BMS1.0 without ATMVP as a place holder in RA and LDB configurations, respectively.

Reference

- M. Koo, et al., “Description of SDR video coding technology proposed by LG Electronics,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-J0017, 10th meeting, San Diego, CA, 10-20 April 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).