| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0080-v1 |
| --- | --- |

| Title: | [CE4-2.6 related] Simplified ATMVP with fixed sub-block size. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang, Junghak Nam, Seung-Hwan Kim, Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 {hm.jang, junghak.nam, seunghwan3.kim, jaehyun.lim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution is related to CE4-2.6 but proposes a new simplified ATMVP by applying following three aspects;

- One fixed collocated picture is used to derive temporal motion information

- Sub-block size is fixed to 8x8

- ATMVP is disallowed for merge candidate when block size is smaller than sub-block size

For the suggestion, three experimental results are reported in this contribution. The first experimental test (test(a)) represent the test results on simplified ATMVP with fixed 4x4 sub-block size. This result is identical with CE4.2.6(a). The second experimental test(test(b)) represent the test results on simplified ATMVP with fixed 8x8 sub-block size instead of the adoptive sub-block decision and the third test (test(c)) represents second experimental test with ATMVP disabling when block size is smaller than sub-block size. Simulation results for this contribution is summarized as below.

- Test(a) : -1.09%(RA-toolon), -1.3%(LDB-toolon)

- Test(b) : -1.02%(RA-toolon), -1.15%(LDB-toolon)

- Test(c) : -1.02%(RA-toolon), -1.08%(LDB-toolon)

Proposed method

In CE4-2.6(b), the simplified ATMVP is studied with adaptive sub-block size decision method. The adaptive sub-block decision technology signal an on/off flag syntax and sub-block size syntax in slice header. The sub-block size for ATMVP is decided between 4x4 and 8x8 according to an indicator in each slice. Therefore the worst case ATMVP sub-block size is still 4x4. In this contribution, ATMVP is only allowed when block is larger than sub-block size and the sub-block size is fixed as 8x8 block. From this proposal, ATMVP candidate is disallowed for 4x4, 8x4 and 4x8 block to improve H/W throughput.

Experimental results

Test(a) : CE4-2.6 (a) - simplified ATMVP with fixed 4x4 sub-block size.

Table 1: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.80% | -0.77% | -0.66% | 105% | 122% |
| Class A2 | -1.07% | -0.80% | -0.85% | 107% | 129% |
| Class B | -1.02% | -0.77% | -0.66% | 106% | 123% |
| Class C | -1.41% | -1.07% | -1.18% | 105% | 127% |
| Class E |  |  |  |  |  |
| Overall | -1.09% | -0.86% | -0.84% | 106% | 125% |
| Class D | -1.58% | -0.94% | -1.03% | 106% | 128% |

Table 1: Experimental results for random access test condition; anchor is BMS1.0 without conventional ATMVP

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -0.58% | -0.55% | 102% | 106% |
| Class A2 | -0.61% | -0.47% | -0.45% | 102% | 107% |
| Class B | -0.58% | -0.51% | -0.53% | 101% | 106% |
| Class C | -1.11% | -1.05% | -1.01% | 99% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.73% | -0.66% | -0.65% | 101% | 107% |
| Class D | -1.17% | -0.93% | -0.89% | 100% | 107% |

Table 1: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.65% | -0.67% | -0.39% | 107% | 120% |
| Class C | -1.30% | -1.06% | -0.86% | 104% | 117% |
| Class E | -2.37% | -1.88% | -2.57% | 104% | 125% |
| Overall | -1.30% | -1.10% | -1.10% | 105% | 120% |
| Class D | -1.84% | -1.47% | -0.62% | 104% | 115% |

Table 1: Experimental results for low-delay test condition; anchor is BMS1.0 without conventional ATMVP

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.53% | -0.53% | -0.78% | 102% | 111% |
| Class C | -0.93% | -0.96% | -1.08% | 99% | 111% |
| Class E | -2.21% | -1.77% | -2.42% | 98% | 114% |
| Overall | -1.08% | -0.98% | -1.29% | 100% | 112% |
| Class D | -1.16% | -1.39% | -1.19% | 98% | 107% |

Test(b) : Simplified ATMVP with fixed 8x8 sub-block size.

Table 1: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | -0.76% | -0.67% | 104% | 108% |
| Class A2 | -1.02% | -0.75% | -0.83% | 103% | 110% |
| Class B | -0.96% | -0.71% | -0.68% | 103% | 109% |
| Class C | -1.27% | -0.94% | -1.03% | 102% | 111% |
| Class E |  |  |  |  |  |
| Overall | -1.02% | -0.79% | -0.80% | 103% | 109% |
| Class D | -1.41% | -0.69% | -0.87% | 102% | 110% |

Table 1: Experimental results for random access test condition; anchor is BMS1.0 without conventional ATMVP

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -0.56% | -0.54% | 101% | 103% |
| Class A2 | -0.59% | -0.46% | -0.39% | 101% | 103% |
| Class B | -0.54% | -0.43% | -0.50% | 100% | 101% |
| Class C | -1.00% | -0.90% | -0.82% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.69% | -0.59% | -0.57% | 100% | 101% |
| Class D | -1.05% | -0.70% | -0.88% | 99% | 100% |

Table 1: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.62% | -0.52% | -0.49% | 104% | 109% |
| Class C | -1.17% | -0.76% | -0.89% | 102% | 107% |
| Class E | -2.00% | -2.15% | -2.11% | 100% | 109% |
| Overall | -1.15% | -1.01% | -1.03% | 102% | 108% |
| Class D | -1.59% | -0.87% | -0.60% | 101% | 105% |

Table 1: Experimental results for low-delay test condition; anchor is BMS1.0 without conventional ATMVP

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.44% | -0.40% | -0.53% | 101% | 106% |
| Class C | -0.79% | -0.79% | -0.66% | 98% | 106% |
| Class E | -1.73% | -1.62% | -2.56% | 98% | 106% |
| Overall | -0.88% | -0.83% | -1.08% | 99% | 106% |
| Class D | -0.89% | -1.05% | -0.63% | 98% | 102% |

Test(b) and disabling ATMVP when block size is smaller than sub-block.

In CE4-2.6(a), it is applied that Simplified ATMVP refer one fixed collocated picture to derive temporal motion information and the sub-block size is decided between either 4x4 or 8x8 based on slice level syntax. This sub-block size decision algorithm is identical with CE4-2.5(b).

Table 1: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | -0.71% | -0.69% | 102% | 104% |
| Class A2 | -1.04% | -0.78% | -0.82% | 102% | 114% |
| Class B | -0.97% | -0.73% | -0.66% | 102% | 113% |
| Class C | -1.23% | -0.91% | -1.04% | 101% | 107% |
| Class E |  |  |  |  |  |
| Overall | -1.02% | -0.79% | -0.80% | 102% | 110% |
| Class D | -1.34% | -0.75% | -0.79% | 102% | 102% |

Table 1: Experimental results for random access test condition; anchor is BMS1.0 without conventional ATMVP

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -0.58% | -0.50% | 101% | 94% |
| Class A2 | -0.61% | -0.50% | -0.50% | 100% | 94% |
| Class B | -0.54% | -0.44% | -0.50% | 100% | 95% |
| Class C | -0.95% | -0.88% | -0.86% | 99% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.68% | -0.60% | -0.60% | 100% | 98% |
| Class D | -0.94% | -0.80% | -0.89% | 99% | 116% |

Table 1: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.59% | -0.61% | -0.39% | 103% | 97% |
| Class C | -1.09% | -0.96% | -0.72% | 101% | 100% |
| Class E | -1.86% | -2.21% | -1.82% | 100% | 97% |
| Overall | -1.08% | -1.13% | -0.86% | 101% | 98% |
| Class D | -1.50% | -0.54% | -0.47% | 100% | 124% |

Table 1: Experimental results for low-delay test condition; anchor is BMS1.0 without conventional ATMVP

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.44% | -0.31% | -0.60% | 101% | 98% |
| Class C | -0.70% | -0.97% | -0.94% | 98% | 102% |
| Class E | -1.73% | -1.75% | -1.55% | 97% | 97% |
| Overall | -0.85% | -0.89% | -0.95% | 99% | 99% |
| Class D | -0.79% | -0.53% | -0.92% | 98% | 106% |

Summary

This contribution proposes a simplified ATMVP with fixed 8x8 sub-block size and disable ATMVP candidate when block size is smaller than sub-block instead of adoptive sub-block decision in CE4.2.6 (b). Experimental results of test(b) show 0.xx%, 0.xx% and 0.xx% BD-rate increases for RA, LDB and LDP of tool-on test configurations respectively. Experimental results of test(c) show 0.xx%, 0.xx% and 0.xx% BD-rate increases for RA, LDB and LDP of tool-on test configurations respectively.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).