| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0083-v1 |
| --- | --- |

| Title: | CE6: Block size dependent zero-out transform (Test 1.14) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE6-1.14 on block size dependent zero-out transform. With the proposed method, it is reported that up to 75% multiplication operations in transform (with matrix multiplication implementation) are saved at the cost of average 0.00%, 0.00% and 0.04% BD-rate increase comparing to VTM-1.1, under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively, and average 0.00%, 0.01% and -0.03% BD-rate change comparing to BMS-1.1, under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively.

# Introduction

In BMS-1.1 [1], the maximum transform size is 64-point. In case a block width (height) being larger than 64, the block is implicitly split horizontally (or vertically) into multiples of sub-blocks which are smaller than or equal to 64x64. Specifically, for a W×H block, it is split into multiples of min(W, 64)×min(H, 64) sub-blocks.

For 64-point transform, only the first 32 coefficients are kept and the remaining coefficients are not calculated and constrained to 0. Specifically, for a W×H transform block, only the top-left min(W, 32)×min(H, 32) transform coefficients are calculated and the remaining coefficients are constrained as 0.

# Proposed method

Despite the transform block size, it is noticed that large block partitions typically present less residuals, and residual energy is more concentrated in low-frequency coefficients in transform domain. Therefore, it is proposed to apply a block size dependent transform zero-out, where the block size is indicated by coding block size. That is, for a W×H coding block, for each transform block, it is constrained that only the top-left ( W==128 ? 16 : (W==64 ? 32 : W) ) × ( H==128 ? 16 : (H==64 ? 32 : H) ) transform coefficients are kept and the remaining coefficients are not calculated and set as 0.

# Experimental results

The proposed scheme is implemented on top of BMS-1.1 software [1], and simulations are performed according to JVET-J1026 [4]. Four sets of tests are performed, as described below. Test #1 reports the coding performance of CE6-1.14 under VTM and BMS configurations defined in [3], respectively. Test #2 and #3 are requested in CE6[4]. Test #2 reports the coding performance of CE6-1.14 under VTM with NSST enabled, and Test #3 reports the coding performance of CE6-1.14 under BMS with NSST disabled.

Test #1a

Anchor: BMS-1.1, VTM configuration

Tested: BMS-1.1 w/ CE6-1.14, VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 101% | 0.00% | 0.00% | 0.00% | 103% | 102% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 104% | -0.01% | 0.04% | -0.01% | 102% | 103% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 102% | 0.02% | 0.03% | 0.02% | 101% | 103% | 0.00% | -0.13% | -0.15% | 102% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 102% | -0.01% | 0.06% | 0.00% | 101% | 101% | 0.03% | -0.10% | 0.17% | 102% | 105% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 101% |  |  |  |  |  | 0.11% | 0.24% | 0.08% | 101% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 102% | 0.00% | 0.03% | 0.01% | 101% | 102% | 0.04% | -0.03% | 0.01% | 102% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% | -0.01% | -0.02% | -0.06% | 99% | 100% | -0.03% | -0.16% | -0.02% | 99% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 96% | 0.01% | 0.08% | 0.02% | 100% | 101% | 0.02% | 0.12% | 0.23% | 101% | 103% |

Test #1b

Anchor: BMS-1.1, BMS configuration

Tested: BMS-1.1 w/ CE6-1.14, BMS configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 103% | 102% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 97% | -0.01% | 0.04% | -0.01% | 102% | 103% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 98% | 0.02% | 0.03% | 0.02% | 101% | 103% | 0.02% | -0.10% | -0.25% | 100% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 106% | -0.01% | 0.06% | 0.00% | 101% | 101% | -0.03% | 0.27% | 0.08% | 102% | 107% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 105% |  |  |  |  |  | -0.13% | -0.11% | 0.14% | 102% | 107% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.03% | 0.01% | 101% | 102% | -0.03% | 0.02% | -0.04% | 101% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% | -0.01% | -0.02% | -0.06% | 99% | 100% | 0.05% | -0.17% | 0.02% | 100% | 104% |
| Class F | 0.00% | 0.00% | 0.00% | 103% | 106% | 0.01% | 0.08% | 0.02% | 100% | 101% | -0.10% | -0.08% | -0.19% | 101% | 103% |

Test #2

Anchor: BMS-1.1, VTM configuration, --NSST=1

Tested: BMS-1.1 w/ CE6-1.14, VTM configuration, --NSST=1

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 99% | 0.02% | -0.01% | -0.07% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 102% | 0.00% | 0.03% | 0.02% | 102% | 102% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 102% | 0.00% | 0.04% | 0.02% | 100% | 99% | 0.01% | -0.13% | 0.15% | 101% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 105% | -0.01% | 0.02% | 0.00% | 101% | 102% | 0.02% | -0.14% | -0.08% | 101% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 97% |  |  |  |  |  | 0.05% | -0.18% | -0.41% | 100% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% | 0.00% | 0.02% | 0.00% | 100% | 100% | 0.02% | -0.14% | -0.07% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 96% | 0.01% | -0.03% | 0.00% | 99% | 98% | -0.02% | 0.54% | -0.04% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 99% |  |  |  |  |  | -0.21% | -0.14% | -0.05% | 100% | 105% |

Test #3

Anchor: BMS-1.1, BMS configuration, --NSST=0

Tested: BMS-1.1 w/ CE6-1.14, BMS configuration, --NSST=0

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.02% | 0.03% | -0.02% | 101% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 97% | 0.01% | 0.06% | -0.04% | 102% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 98% | -0.01% | 0.01% | 0.06% | 102% | 105% | 0.02% | -0.10% | -0.25% | 100% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 106% | 0.01% | -0.06% | -0.07% | 101% | 102% | -0.03% | 0.27% | 0.08% | 102% | 107% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 105% |  |  |  |  |  | -0.13% | -0.11% | 0.14% | 102% | 107% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.01% | 0.01% | -0.01% | 101% | 102% | -0.03% | 0.02% | -0.04% | 101% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.01% | 0.05% | 0.05% | 100% | 100% | 0.05% | -0.17% | 0.02% | 100% | 104% |
| Class F | 0.00% | 0.00% | 0.00% | 103% | 106% | 0.00% | -0.07% | 0.00% | 100% | 101% | -0.10% | -0.08% | -0.19% | 101% | 103% |

# Complexity analysis

## Arithmetic operation counts

The proposed method only has an impact on block W×H with either W or H being larger than 64, i.e., 128. The operation counts saving ratio is listed in below table

| Transform size | Mult | Add/Sub | Shift |
| --- | --- | --- | --- |
| 128×N, N×128 where N = 2, 4, 8, 16, 32, 64 | 50% | 50% | 50% |
| 128×128 | 75% | 75% | 75% |

## Other metrics

In this section, other complexity measuring metric recommended in JVET-J1026 has been reported.

| Metrics | Reported results |
| --- | --- |
| Additional memory requirements for storing transform cores | No |
| Minimum bit-precision | Unchanged |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output | No |
| Other operations and memory requirements relevant to the proposed tool | No |
| Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided | Reported in Section 3 |
| List of all combinations of transforms block size and transform type used for secondary transformation. | Same with BMS-1.1 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms | Proposed method is based on matrix multiplications |
| If the proposal requires additional computations at the encoder or decoder | No |

# Conclusions

In this contribution, a block size dependent zero-out transform method has been proposed. For larger coding block, more transform coefficients are not calculated and set to 0. It is reportedly shown that the proposed method has almost no impact on coding performance or encoder/decoder run-time, and the arithmetic operations counts are saved by 50% for 128×N and N×128 blocks, and by 75% for 128×128 blocks.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1/

- X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, “EE1: Improvements on non-separable secondary transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-D0120, Chengdu, CN, 15–21 Oct. 2016.

- X. Zhao, J. Chen, M. Karczewicz, “Mode-dependent non-separable secondary transform”, ITU-T SG16 Doc. COM16-C1044, Sep. 2015.

- A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1026, San Diego, US, 10–20 Apr. 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, San Diego, US, 10–20 Apr. 2018.