| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0084-v1 |
| --- | --- |

| Title: | CE6-2.4: Matrix multiplication based NSST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE6-2.4 on matrix multiplication based Non-Separable Secondary Transform (NSST). With the proposed method, it is reportedly shown that average -0.30%, -0.13% and -0.05% coding gain can be achieved comparing to VTM-1.1 (with NSST enabled), under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively, and average -0.17%, -0.06% and -0.06% coding gain can be achieved comparing to BMS-1.1, under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively.

# Introduction

In BMS-1.1 [1], the design of NSST is based on Hyper-Givens transform (HyGT) as proposed in JVET-D0120 [2] which significantly reduced the memory cost of 8x8 NSST. However, in BMS-1.1 where NSST is performed only in unit of 4x4, i.e., 4x4 NSST, the memory cost reduction ratio of HyGT comparing to the matrix multiplication as proposed in [3] becomes largely reduced.

# Proposed method

In CE6-2.4, it is proposed to apply matrix multiplication based 4x4 NSST. Two alternatives have been implemented, the first method (CE6-2.4.1) applies 19x3 NSST cores when 35 luma intra prediction modes (e.g., VTM configuration) are applied, and 35x3 NSST cores when 67 luma intra prediction modes (e.g., BMS configuration) are applied.The second method (CE6-2.4.2) applies 8x3 NSST cores when 35 luma intra prediction modes (e.g., VTM configuration) are applied, and 12x3 NSST cores when 67 luma intra prediction modes (e.g., BMS configuration) are applied.

In CE6-2.4.2, the same look-up-table proposed in [3] is used to identify a set of NSST cores given the intra prediction mode.

# Experimental results

The proposed scheme is implemented on top of BMS-1.1 software [1], and simulations are performed according to JVET-J1026 [4]. Four sets of tests are performed, as described below. Test #1 reports the coding performance of CE6-2.4.1 under VTM and BMS configurations defined in [3], respectively. Test #2 reports the coding performance of CE6-2.4.2 under VTM and BMS configurations defined in [3], respectively. Test #3 and Test #4, as requested as additional tests in CE6, report the coding performance of CE6-3.1.1 and CE6-3.1.2 using VTM configuration with AMT enabled, respectively.

Test #1a

Anchor: BMS-1.1, VTM configuration

Tested: BMS-1.1 w/ CE6-2.4.1, VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 | -3.26% | -5.39% | -6.29% | 238% | 99% | -1.59% | -4.90% | -5.54% | 130% | 98% |  |  |  |  |  |  |
| Class A2 | -2.53% | -3.90% | -4.16% | 245% | 102% | -1.00% | -2.53% | -2.42% | 121% | 98% |  |  |  |  |  |  |
| Class B | -2.82% | -4.01% | -3.60% | 247% | 100% | -1.39% | -3.78% | -3.52% | 125% | 102% | -0.44% |  | -1.44% | -1.34% | 115% | 98% |
| Class C | -3.74% | -3.49% | -3.70% | 259% | 106% | -1.72% | -2.49% | -2.70% | 122% | 95% | -0.44% |  | -0.87% | -0.98% | 114% | 98% |
| Class E | -4.21% | -3.36% | -3.26% | 225% | 101% |  |  |  |  |  | -0.84% |  | -0.99% | -0.80% | 104% | 100% |
| Overall | -3.28% | -4.00% | -4.11% | 244% | 102% | -1.44% | -3.41% | -3.48% | 124% | 99% | -0.54% |  | -1.14% | -1.09% | 112% | 99% |
| Class D | -3.84% | -2.71% | -2.64% | 263% | 104% | -1.52% | -1.44% | -1.89% | 120% | 91% | -0.31% |  | -0.41% | -0.65% | 109% | 98% |
| Class F | -2.37% | -2.34% | -2.45% | 229% | 97% | -1.42% | -1.54% | -1.50% | 130% | 97% | -0.65% |  | -1.29% | -0.90% | 117% | 98% |

Test #1b

Anchor: BMS-1.1, VTM configuration, --NSST=1

Tested: BMS-1.1 w/ CE6-2.4.1, VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.40% | -0.69% | 94% | 94% | -0.19% | -0.42% | -0.79% | 96% | 93% |  |  |  |  |  |
| Class A2 | -0.22% | -0.37% | -0.36% | 95% | 94% | -0.10% | -0.24% | -0.21% | 97% | 95% |  |  |  |  |  |
| Class B | -0.28% | -0.40% | -0.40% | 94% | 92% | -0.15% | -0.40% | -0.35% | 98% | 97% | -0.05% | -0.17% | -0.41% | 96% | 92% |
| Class C | -0.21% | -0.47% | -0.46% | 96% | 101% | -0.08% | -0.26% | -0.46% | 96% | 92% | -0.04% | 0.05% | -0.28% | 98% | 95% |
| Class E | -0.42% | -0.52% | -0.37% | 97% | 95% |  |  |  |  |  | -0.07% | -0.47% | -0.59% | 97% | 97% |
| Overall | -0.30% | -0.43% | -0.45% | 95% | 95% | -0.13% | -0.33% | -0.44% | 97% | 94% | -0.05% | -0.17% | -0.41% | 97% | 94% |
| Class D | -0.15% | -0.27% | -0.28% | 94% | 90% | -0.04% | -0.13% | -0.16% | 94% | 86% | 0.00% | 0.12% | 0.03% | 97% | 95% |
| Class F | -0.03% | -0.14% | -0.23% | 95% | 91% | -0.05% | 0.05% | -0.20% | 95% | 93% | -0.21% | -0.47% | 0.32% | 96% | 95% |

Test #1c

Anchor: BMS-1.1, BMS configuration

Tested: BMS-1.1 w/ CE6-2.4.1, BMS configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.30% | -0.41% | 99% | 99% | -0.10% | -0.34% | -0.34% | 100% | 99% |  |  |  |  |  |
| Class A2 | -0.12% | -0.15% | -0.18% | 98% | 97% | -0.05% | -0.05% | -0.14% | 99% | 100% |  |  |  |  |  |
| Class B | -0.16% | -0.30% | -0.35% | 99% | 100% | -0.10% | -0.14% | -0.29% | 100% | 98% | -0.04% | -0.06% | -0.17% | 101% | 98% |
| Class C | -0.11% | -0.16% | -0.10% | 99% | 97% | 0.00% | -0.11% | -0.28% | 99% | 102% | -0.04% | 0.04% | 0.15% | 99% | 96% |
| Class E | -0.20% | -0.11% | -0.19% | 98% | 97% |  |  |  |  |  | -0.11% | 0.99% | 1.03% | 99% | 95% |
| Overall | -0.17% | -0.21% | -0.25% | 99% | 98% | -0.06% | -0.16% | -0.27% | 100% | 100% | -0.06% | 0.23% | 0.24% | 100% | 97% |
| Class D | -0.04% | -0.11% | -0.10% | 98% | 97% | 0.05% | -0.20% | -0.16% | 98% | 94% | 0.03% | -0.12% | 0.46% | 100% | 102% |
| Class F | -0.04% | -0.08% | -0.25% | 99% | 96% | 0.06% | 0.12% | 0.07% | 98% | 99% | -0.11% | 0.17% | 0.50% | 100% | 98% |

Test #2a

Anchor: BMS-1.1, VTM configuration

Tested: BMS-1.1 w/ CE6-2.4.2, VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.23% | -5.36% | -6.19% | 237% | 100% | -1.59% | -4.96% | -5.48% | 130% | 100% |  |  |  |  |  |
| Class A2 | -2.39% | -3.72% | -3.98% | 246% | 101% | -0.97% | -2.37% | -2.35% | 121% | 98% |  |  |  |  |  |
| Class B | -2.69% | -3.84% | -3.47% | 246% | 99% | -1.35% | -3.72% | -3.48% | 127% | 102% | -0.40% | -1.45% | -1.32% | 115% | 98% |
| Class C | -3.48% | -3.25% | -3.39% | 260% | 104% | -1.60% | -2.44% | -2.50% | 128% | 101% | -0.39% | -1.02% | -0.93% | 115% | 100% |
| Class E | -3.99% | -3.36% | -3.10% | 226% | 101% |  |  |  |  |  | -0.74% | -0.94% | -1.39% | 106% | 100% |
| Overall | -3.12% | -3.86% | -3.93% | 244% | 101% | -1.39% | -3.36% | -3.39% | 126% | 100% | -0.48% | -1.18% | -1.21% | 113% | 99% |
| Class D | -3.55% | -2.58% | -2.44% | 269% | 103% | -1.42% | -1.46% | -1.75% | 124% | 96% | -0.28% | -0.28% | -1.71% | 111% | 101% |
| Class F | -2.19% | -2.19% | -2.26% | 232% | 103% | -1.34% | -1.54% | -1.50% | 132% | 100% | -0.49% | -1.00% | -0.68% | 118% | 99% |

Test #2b

Anchor: BMS-1.1, VTM configuration, --NSST=1

Tested: BMS-1.1 w/ CE6-2.4.2, VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.36% | -0.58% | 94% | 95% | -0.20% | -0.47% | -0.74% | 96% | 95% |  |  |  |  |  |
| Class A2 | -0.09% | -0.18% | -0.17% | 96% | 93% | -0.06% | -0.07% | -0.14% | 97% | 95% |  |  |  |  |  |
| Class B | -0.15% | -0.23% | -0.27% | 94% | 91% | -0.10% | -0.33% | -0.30% | 99% | 96% | -0.02% | -0.18% | -0.39% | 96% | 92% |
| Class C | 0.06% | -0.22% | -0.14% | 97% | 99% | 0.04% | -0.20% | -0.26% | 100% | 97% | 0.01% | -0.10% | -0.21% | 99% | 96% |
| Class E | -0.20% | -0.53% | -0.21% | 98% | 95% |  |  |  |  |  | 0.03% | -0.41% | -1.16% | 98% | 97% |
| Overall | -0.13% | -0.29% | -0.26% | 95% | 94% | -0.08% | -0.27% | -0.35% | 98% | 96% | 0.00% | -0.21% | -0.52% | 98% | 95% |
| Class D | 0.15% | -0.14% | -0.07% | 96% | 90% | 0.06% | -0.16% | -0.02% | 97% | 92% | 0.02% | 0.25% | -1.05% | 99% | 98% |
| Class F | 0.16% | 0.02% | -0.04% | 96% | 96% | 0.03% | 0.05% | -0.20% | 97% | 95% | -0.06% | -0.15% | 0.54% | 97% | 95% |

Test #2c

Anchor: BMS-1.1, BMS configuration

Tested: BMS-1.1 w/ CE6-2.4.2, BMS configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.36% | -0.43% | 99% | 98% | -0.10% | -0.37% | -0.39% | 99% | 99% |  |  |  |  |  |
| Class A2 | -0.01% | 0.02% | 0.01% | 99% | 97% | -0.01% | 0.01% | 0.01% | 99% | 99% |  |  |  |  |  |
| Class B | -0.05% | -0.14% | -0.20% | 99% | 98% | -0.03% | -0.15% | -0.27% | 100% | 98% | -0.02% | -0.14% | -0.41% | 99% | 98% |
| Class C | 0.08% | -0.12% | 0.02% | 99% | 97% | 0.07% | -0.20% | -0.51% | 100% | 103% | -0.01% | 0.16% | -0.10% | 99% | 97% |
| Class E | -0.02% | -0.10% | 0.06% | 99% | 98% |  |  |  |  |  | -0.22% | -0.62% | -0.36% | 99% | 95% |
| Overall | -0.04% | -0.14% | -0.11% | 99% | 98% | -0.01% | -0.18% | -0.30% | 100% | 100% | -0.07% | -0.16% | -0.29% | 99% | 97% |
| Class D | 0.15% | 0.10% | 0.12% | 99% | 98% | 0.06% | 0.02% | -0.30% | 99% | 97% | -0.02% | 0.24% | 0.24% | 100% | 104% |
| Class F | 0.12% | 0.02% | -0.25% | 99% | 96% | 0.09% | 0.07% | 0.11% | 100% | 101% | 0.00% | 0.25% | 0.15% | 99% | 99% |

Test #3

Anchor: BMS-1.1, VTM configuration, --EMT=3, --EMTFast=3

Tested: BMS-1.1 w/ CE6-2.4.1, VTM configuration, --EMT=3, --EMTFast=3

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.05% | -5.40% | -6.13% | 295% | 102% | -1.08% | -5.10% | -5.55% | 164% | 104% |  |  |  |  |  |
| Class A2 | -1.63% | -3.70% | -3.76% | 293% | 102% | -0.66% | -2.79% | -2.44% | 150% | 105% |  |  |  |  |  |
| Class B | -1.74% | -3.87% | -3.77% | 294% | 106% | -0.92% | -4.08% | -3.58% | 155% | 106% | -0.26% | -1.40% | -0.61% | 134% | 106% |
| Class C | -2.53% | -3.07% | -3.31% | 305% | 110% | -1.12% | -2.42% | -2.73% | 153% | 102% | -0.28% | -0.88% | -0.85% | 130% | 109% |
| Class E | -2.53% | -2.94% | -2.98% | 262% | 105% |  |  |  |  |  | -0.53% | -1.89% | -2.48% | 111% | 107% |
| Overall | -2.08% | -3.76% | -3.93% | 291% | 105% | -0.95% | -3.58% | -3.52% | 155% | 104% | -0.33% | -1.35% | -1.16% | 127% | 107% |
| Class D | -2.61% | -2.34% | -2.46% | 319% | 109% | -1.01% | -2.05% | -1.79% | 152% | 98% | -0.17% | -0.37% | -1.06% | 122% | 100% |
| Class F | -1.76% | -2.06% | -2.02% | 286% | 103% | -0.99% | -1.50% | -1.34% | 168% | 102% | -0.24% | -0.58% | -0.39% | 139% | 109% |

Test #4

Anchor: BMS-1.1, VTM configuration, --EMT=3, --EMTFast=3

Tested: BMS-1.1 w/ CE6-2.4.2, VTM configuration, --EMT=3, --EMTFast=3

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.04% | -5.28% | -5.95% | 288% | 105% | -1.09% | -4.97% | -5.42% | 163% | 103% |  |  |  |  |  |
| Class A2 | -1.52% | -3.54% | -3.53% | 294% | 102% | -0.65% | -2.68% | -2.39% | 150% | 105% |  |  |  |  |  |
| Class B | -1.63% | -3.66% | -3.55% | 302% | 109% | -0.85% | -3.83% | -3.49% | 155% | 105% | -0.24% | -1.50% | -0.89% | 134% | 104% |
| Class C | -2.29% | -2.83% | -3.13% | 308% | 102% | -1.04% | -2.24% | -2.51% | 154% | 102% | -0.26% | -0.90% | -1.10% | 130% | 110% |
| Class E | -2.37% | -2.90% | -2.90% | 279% | 107% |  |  |  |  |  | -0.55% | -3.13% | -2.03% | 110% | 105% |
| Overall | -1.95% | -3.60% | -3.75% | 296% | 105% | -0.91% | -3.40% | -3.39% | 155% | 104% | -0.33% | -1.71% | -1.25% | 127% | 106% |
| Class D | -2.35% | -2.26% | -2.29% | 318% | 105% | -0.91% | -1.72% | -1.63% | 156% | 102% | -0.11% | -0.59% | -0.97% | 122% | 101% |
| Class F | -1.55% | -1.79% | -1.81% | 278% | 100% | -0.92% | -1.28% | -1.33% | 167% | 103% | -0.41% | -0.87% | -0.47% | 139% | 106% |

# Complexity analysis

## Memory for storing NSST cores

In CE6-2.4.1, the proposed method applies 19x3 and 35x3 NSST cores when 35 and 67 luma intra modes are used, respectively. In CE6-2.4.2, the proposed method applies 8x3 and 12x3 NSST cores when 35 and 67 luma intra modes are used. Each NSST core is a 16x16 matrix composed by 8-bit elements.

The following table summarizes the memory needed for storing NSST cores.

|  | VTM cfg (35 luma intra modes) | BMS cfg (67 luma intra modes) |
| --- | --- | --- |
| BMS-1.1 | 4.872 Kbytes [3] | 8.328 Kbytes [3] |
| CE6-2.4.1 | 19×3×256 = 14.592 KBytes | 35×3×256 = 26.880 KBytes |
| CE6-2.4.2 | 8×3×256 = 6.144 KBytes | 12×3×256 = 9.216 KBytes |

## Arithmetic operation counts

Each 4x4 NSST requires a 16x16 matrix multiplication, which includes 256 multiplications and 240 adds and 16 shifts.

## Minimum bit-precision

The proposed method requires a 16-bit minimum bit-precision before and after applying NSST.

## Other metrics

In this section, other complexity measuring metric recommended in JVET-J1026 has been reported.

| Metrics | Reported results |
| --- | --- |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output | No |
| Other operations and memory requirements relevant to the proposed tool | No |
| Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided | Reported in Section 3 |
| List of all combinations of transforms block size and transform type used for secondary transformation. | Same with BMS-1.1 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms | Proposed method is based on matrix multiplications |
| If the proposal requires additional computations at the encoder or decoder | No |

# Conclusions

In this contribution, a matrix multiplication based NSST method has been proposed. With the proposed method, it is reportedly shown that small coding gain, i.e., -0.1% ~ -0.3% has been achieved depending on different configurations with almost no run-time difference for encoder or decoder.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.

# References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1/

- X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, “EE1: Improvements on non-separable secondary transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-D0120, Chengdu, CN, 15–21 Oct. 2016.

- X. Zhao, J. Chen, M. Karczewicz, “Mode-dependent non-separable secondary transform”, ITU-T SG16 Doc. COM16-C1044, Sep. 2015.

- A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1026, San Diego, US, 10–20 Apr. 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, San Diego, US, 10–20 Apr. 2018.