| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0085-v1 |
| --- | --- |

| Title: | CE6-3.1: Coupled primary and secondary transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE6-3.1 on Coupled Primary and Secondary Transform (CPST). Instead of signaling primary and secondary transform independently, with the proposed CPST, primary and secondary transform are coupled and signaled jointly by one transform index. With the proposed method, it is reportedly shown that -4.37%, -1.98% and -0.76% coding gain can be achieved comparing to VTM-1.1, with 223%, 133% and 117% encoding time, under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively.

# Introduction

In Adaptive Multiple Transform (AMT) and Non-Separable Secondary Transform (NSST) of BMS-1.1, primary and secondary transforms are signaled and utilized independently. Since both AMT and NSST allow multiple candidates to be selected at the encoder side, checking all possible combinations of AMT and NSST significantly increases encoder run-time, e.g, roughly 6x of overall encoder run-time of VTM.

# Proposed method

In CE6-3.1, it is proposed to couple AMT and NSST, so that the usage of the two tools is indicated by a single transform index at the encoder.

The proposed method is illustrated in Figure 1.

Figure 1: Proposed coupled primary and secondary transform (CPST).

Totally 35x3 NSST cores are applied, and the selection is indicated by the intra prediction mode and signaled transform index. The method for selecting transform type is unchanged and kept as same as the original design of AMT in BMS-1.1. When transform index is 0, only DST-7 is applied as both horizontal and vertical transform, and secondary transform is not applied, otherwise, a secondary transform set is indicated by the intra prediction mode and the NSST core is further identified by the NSST index. The proposed method can be applied on luma only, or both luma and chroma. When the proposed method is applied on chroma and LM mode is being used, the primary and secondary transforms are selected from the transform set associated with Planar mode.

# Experimental results

The proposed scheme is implemented on top of BMS-1.1 software [1], and simulations are performed accordingly to JVET-J1026 [2]. Four sets of tests are performed, as described below. Test #1a and #1b report the coding performance of CE6-3.1.1 under VTM and BMS configurations defined in [3], respectively. Test #2a and #2b report the coding performance of CE6-3.1.2 under VTM and BMS configurations defined in [3], respectively. Test #3 and Test #4 report the coding performance of CE6-3.1.1 and CE6-3.1.2 comparing to AMT using VTM configuration, respectively, to see the additional coding gain of NSST using proposed method which does not increase rate-distortion checks at encoder.

As defined in JVET-J1026, in CE6-3.1.1, the proposed CPST method is applied only on luma component. In CE6-3.1.2, the proposed CPST method is applied on both luma and chroma components.

Test #1a

Anchor: BMS-1.1, VTM configuration

Tested: BMS-1.1 w/ proposed method (luma only), VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.72% | -1.52% | -1.42% | 209% | 131% | -1.67% | -0.70% | -0.66% | 138% | 111% |  |  |  |  |  |
| Class A2 | -3.52% | -1.66% | -1.62% | 221% | 124% | -1.29% | -0.12% | -0.07% | 129% | 108% |  |  |  |  |  |
| Class B | -4.06% | -2.08% | -1.92% | 225% | 128% | -2.14% | -0.98% | -0.95% | 134% | 110% | -0.73% | -0.61% | -0.31% | 121% | 106% |
| Class C | -4.28% | -1.88% | -2.00% | 227% | 113% | -2.22% | -0.76% | -1.00% | 129% | 100% | -0.59% | -0.30% | -0.50% | 118% | 104% |
| Class E | -5.33% | -3.36% | -3.25% | 205% | 120% |  |  |  |  |  | -1.16% | -0.04% | -0.18% | 107% | 109% |
| Overall | -4.17% | -2.09% | -2.03% | 219% | 123% | -1.90% | -0.69% | -0.73% | 133% | 107% | -0.79% | -0.36% | -0.34% | 117% | 106% |
| Class D | -4.19% | -1.59% | -1.53% | 236% | 110% | -1.84% | -0.54% | -0.74% | 123% | 97% | -0.46% | -0.48% | -0.60% | 110% | 99% |
| Class F | -2.47% | -1.51% | -1.59% | 214% | 109% | -1.72% | -0.68% | -0.61% | 142% | 105% | -0.53% | -0.51% | -0.63% | 127% | 105% |

Test #1b

Anchor: BMS-1.1, BMS configuration

Tested: BMS-1.1 w/ proposed method (luma only), BMS configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.86% | 2.58% | 3.01% | 37% | 98% | 0.48% | 2.13% | 2.35% | 66% | 99% |  |  |  |  |  |
| Class A2 | 0.90% | 2.34% | 2.60% | 36% | 97% | 0.33% | 2.05% | 1.96% | 72% | 99% |  |  |  |  |  |
| Class B | 0.72% | 1.84% | 1.98% | 36% | 99% | 0.29% | 2.37% | 2.29% | 71% | 102% | 0.03% | 0.86% | 0.44% | 81% | 96% |
| Class C | 0.91% | 1.24% | 1.47% | 36% | 111% | 0.38% | 1.68% | 1.48% | 69% | 102% | 0.00% | 0.67% | 0.49% | 83% | 97% |
| Class E | 0.98% | 1.39% | 1.61% | 40% | 104% |  |  |  |  |  | 0.25% | 1.02% | 0.62% | 95% | 99% |
| Overall | 0.86% | 1.84% | 2.08% | 37% | 102% | 0.36% | 2.08% | 2.02% | 70% | 101% | 0.07% | 0.84% | 0.50% | 85% | 97% |
| Class D | 0.99% | 1.01% | 1.03% | 32% | 98% | 0.39% | 1.02% | 1.08% | 70% | 101% | 0.05% | 0.02% | 0.61% | 87% | 103% |
| Class F | 0.76% | 0.92% | 0.60% | 37% | 100% | 0.48% | 1.05% | 0.72% | 64% | 101% | 0.11% | 0.44% | 0.45% | 79% | 101% |

Test #2a

Anchor: BMS-1.1, VTM configuration

Tested: BMS-1.1 w/ proposed method (both luma and chroma), VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.25% | -1.81% | -2.95% | 213% | 136% | -1.98% | -1.26% | -2.42% | 140% | 112% |  |  |  |  |  |
| Class A2 | -3.74% | -2.78% | -3.00% | 222% | 125% | -1.33% | -0.45% | -0.76% | 128% | 107% |  |  |  |  |  |
| Class B | -4.16% | -2.49% | -1.59% | 226% | 129% | -2.17% | -1.68% | -1.19% | 133% | 111% | -0.72% | -0.76% | -0.54% | 123% | 106% |
| Class C | -4.38% | -2.32% | -2.41% | 234% | 114% | -2.23% | -1.34% | -1.40% | 131% | 105% | -0.58% | -0.62% | -0.57% | 118% | 105% |
| Class E | -5.46% | -2.57% | -2.60% | 217% | 123% |  |  |  |  |  | -1.07% | 0.17% | -0.59% | 107% | 103% |
| Overall | -4.37% | -2.40% | -2.40% | 223% | 125% | -1.98% | -1.26% | -1.40% | 133% | 109% | -0.76% | -0.48% | -0.56% | 117% | 105% |
| Class D | -4.22% | -1.71% | -1.53% | 241% | 114% | -1.81% | -0.71% | -1.16% | 128% | 102% | -0.46% | -0.33% | -0.20% | 114% | 102% |
| Class F | -2.54% | -1.80% | -1.81% | 217% | 109% | -1.80% | -1.02% | -0.94% | 146% | 106% | -0.64% | -0.65% | -0.37% | 128% | 107% |

Test #2b

Anchor: BMS-1.1, BMS configuration

Tested: BMS-1.1 w/ proposed method (both luma and chroma), BMS configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.57% | 4.53% | 4.36% | 39% | 104% | 0.39% | 3.24% | 2.70% | 68% | 102% |  |  |  |  |  |
| Class A2 | 0.72% | 2.68% | 2.30% | 37% | 98% | 0.29% | 2.59% | 1.83% | 75% | 101% |  |  |  |  |  |
| Class B | 0.62% | 2.39% | 3.23% | 38% | 103% | 0.26% | 2.45% | 3.00% | 72% | 105% | 0.05% | 0.71% | 1.05% | 83% | 98% |
| Class C | 0.82% | 1.51% | 2.08% | 35% | 109% | 0.37% | 1.50% | 1.38% | 70% | 102% | -0.02% | 0.64% | 0.31% | 85% | 109% |
| Class E | 0.88% | 3.21% | 3.47% | 40% | 104% |  |  |  |  |  | 0.16% | 2.26% | 2.15% | 97% | 107% |
| Overall | 0.71% | 2.74% | 3.05% | 38% | 104% | 0.32% | 2.38% | 2.28% | 71% | 103% | 0.05% | 1.07% | 1.08% | 87% | 104% |
| Class D | 0.92% | 1.29% | 1.54% | 33% | 98% | 0.41% | 1.43% | 1.68% | 70% | 101% | 0.08% | 0.52% | -0.21% | 89% | 104% |
| Class F | 0.69% | 1.09% | 0.87% | 38% | 102% | 0.47% | 1.00% | 0.95% | 65% | 103% | -0.06% | 0.44% | 0.38% | 80% | 105% |

Test #3

Anchor: BMS-1.1, VTM configuration, --EMT=1, --EMTFast=1

Tested: BMS-1.1 w/ CE6-3.1.1 (luma only), VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.20% | -0.21% | 103% | 100% | -0.26% | -0.21% | -0.11% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.38% | 0.00% | -0.19% | 105% | 101% | -0.15% | 0.15% | -0.04% | 103% | 103% |  |  |  |  |  |
| Class B | -0.62% | -0.65% | -0.51% | 104% | 103% | -0.35% | -0.32% | 0.03% | 103% | 103% | -0.20% | -0.23% | 0.35% | 101% | 101% |
| Class C | -1.18% | -1.00% | -0.86% | 100% | 100% | -0.62% | -0.28% | -0.29% | 99% | 94% | -0.21% | -0.04% | -0.29% | 97% | 98% |
| Class E | -0.97% | -0.63% | -0.62% | 102% | 101% |  |  |  |  |  | -0.14% | 0.14% | 0.30% | 100% | 104% |
| Overall | -0.73% | -0.54% | -0.50% | 103% | 101% | -0.37% | -0.19% | -0.10% | 102% | 100% | -0.19% | -0.08% | 0.12% | 99% | 101% |
| Class D | -1.22% | -0.94% | -0.80% | 101% | 97% | -0.55% | -0.60% | 0.09% | 97% | 93% | -0.11% | -0.22% | 0.18% | 98% | 90% |
| Class F | -0.60% | -0.69% | -0.71% | 101% | 102% | -0.26% | -0.16% | -0.03% | 99% | 96% | 0.07% | 0.19% | -0.09% | 101% | 102% |

Test #4

Anchor: BMS-1.1, VTM configuration, --EMT=1, --EMTFast=1

Tested: BMS-1.1 w/ CE6-3.1.2 (both luma and chroma), VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.99% | -0.47% | -1.74% | 104% | 104% | -0.57% | -0.76% | -1.88% | 103% | 103% |  |  |  |  |  |
| Class A2 | -0.59% | -1.14% | -1.59% | 105% | 102% | -0.20% | -0.17% | -0.73% | 102% | 102% |  |  |  |  |  |
| Class B | -0.72% | -1.07% | -0.19% | 105% | 104% | -0.39% | -1.03% | -0.21% | 102% | 103% | -0.19% | -0.38% | 0.11% | 102% | 102% |
| Class C | -1.29% | -1.44% | -1.26% | 103% | 101% | -0.64% | -0.85% | -0.70% | 101% | 99% | -0.21% | -0.36% | -0.37% | 97% | 99% |
| Class E | -1.11% | 0.18% | 0.05% | 108% | 104% |  |  |  |  |  | -0.06% | 0.35% | -0.11% | 100% | 99% |
| Overall | -0.94% | -0.85% | -0.88% | 105% | 103% | -0.45% | -0.76% | -0.78% | 102% | 102% | -0.16% | -0.19% | -0.10% | 100% | 100% |
| Class D | -1.25% | -1.06% | -0.80% | 103% | 100% | -0.52% | -0.76% | -0.33% | 101% | 98% | -0.11% | -0.07% | 0.60% | 101% | 93% |
| Class F | -0.67% | -0.98% | -0.93% | 102% | 102% | -0.34% | -0.51% | -0.34% | 101% | 97% | -0.04% | 0.04% | 0.19% | 102% | 103% |

# Complexity analysis

In this section, other complexity measuring metric recommended in JVET-J1026 has been reported.

| Metrics | Reported results |
| --- | --- |
| Additional memory requirements for storing transform cores | No |
| Minimum bit-precision | Unchanged |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output | No |
| Other operations and memory requirements relevant to the proposed tool | No |
| Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided | Reported in Section 3 |
| List of all combinations of transforms block size and transform type used for secondary transformation. | Same with BMS-1.1 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms | Proposed method applies matrix multiplication based NSST |
| If the proposal requires additional computations at the encoder or decoder | No |

# Conclusions

In this contribution, instead of signaling and optimizing primary and secondary transform separately, a coupled primary and secondary transform scheme is proposed, and only one transform index is signaled. With the proposed method, the complexity of secondary transform is reportedly reduced, and the transform scheme is also unified between luma and chroma.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1/

- A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-J1026, San Diego, US, 10–20 Apr. 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, US, 10–20 Apr. 2018.