| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-Kxxxx-v1 |
| --- | --- |

| Title: | [non-CE] Temporal motion vector data storage reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang, Junghak Nam, Seung-Hwan Kim, Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 {hm.jang, junghak.nam, seunghwan3.kim, jaehyun.lim}@lge.com |
| Source: | LG Electronics inc. |  |  |

_____________________________

- Abstract

This contribution reports coding performance impact with respect to the size of temporal motion vector storage. It is observed that 16x memory size results in 0.1% BD-rate saving for random access and 0.1% BD-rate saving for low delay test configurations for tool-on test, 32x memory size results in 0.1% BD-rate loss for random access and 0.4% BD-rate loss for low delay test configuration and 64x memory reduction shows 0.56% BD-rate loss for random access and 1.1% BD-rate loss for low delay test configurations.

Introduction

In JCTVC-D072[1], the temporal motion vector storage reduction was proposed and it is adopted that temporal memory is reduced by a factor of sixteen even though BD-rate loss was observed. This contribution suggests temporal motion vector storage reduction based on motion vector data of spatial candidates.

Proposed method

The proposed method basically reduces temporal mv data (i.e. motion information) storage. In this proposal temporal motion vector data is compressed based on motion data of spatial candidates. The searching order for spatial candidates is pre-defined as Figure 1.

Figure 1. Block diagram of proposed compression method

If the reference spatial candidate is predicted by inter-prediction then the motion vector data of spatial candidate is set as default motion vector data for compression. The maximum 5 spatial candidates are referred to derive default temporal motion data. It is depicted in Figure 2.

Figure 2

Experimental results

16x memory reduction

When the temporal motion information is stored by the 16x16 block. It is achieved that memory reduction with BD rate reduction also.

Table 1: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.09% | -0.05% | 101% | 105% |
| Class A2 | -0.07% | -0.04% | -0.04% | 101% | 105% |
| Class B | -0.06% | -0.07% | -0.08% | 100% | 105% |
| Class C | -0.17% | -0.15% | -0.14% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.09% | -0.08% | 100% | 105% |
| Class D | -0.14% | 0.00% | -0.20% | 100% | 105% |

Table 2: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.08% | 0.16% | 101% | 106% |
| Class C | -0.09% | -0.20% | -0.12% | 101% | 106% |
| Class E | -0.05% | -0.09% | 0.17% | 101% | 108% |
| Overall | -0.06% | -0.06% | 0.07% | 101% | 106% |
| Class D | -0.17% | -0.09% | -0.29% | 100% | 104% |

32x memory compression

When the temporal motion information is stored by the 32x32 block. It is achieved that memory reduction with negligible BD-rate loss.

Table 3: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | 0.02% | 102% | 105% |
| Class A2 | 0.04% | 0.06% | 0.10% | 101% | 104% |
| Class B | 0.11% | 0.01% | 0.11% | 101% | 105% |
| Class C | 0.23% | 0.25% | 0.27% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.08% | 0.13% | 101% | 105% |
| Class D | 0.44% | 0.35% | 0.40% | 101% | 103% |

Table 4: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.29% | 0.40% | 0.54% | 101% | 106% |
| Class C | 0.60% | 0.55% | 0.67% | 101% | 103% |
| Class E | 0.35% | 0.00% | -0.07% | 100% | 106% |
| Overall | 0.41% | 0.35% | 0.43% | 101% | 105% |
| Class D | 0.57% | 0.43% | 0.37% | 101% | 103% |

64x memory compression

When the temporal motion information is stored by the 64x64 block. It is achieved that memory reduction and it is also observed similar ratio BD-rate loss compared with memory compression mechanism of HEVC standard.

Table 5: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.29% | 0.18% | 0.30% | 101% | 105% |
| Class A2 | 0.43% | 0.44% | 0.49% | 101% | 104% |
| Class B | 0.55% | 0.38% | 0.44% | 102% | 105% |
| Class C | 0.87% | 0.76% | 0.85% | 103% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.56% | 0.45% | 0.53% | 102% | 105% |
| Class D | 1.25% | 1.10% | 1.01% | 102% | 103% |

Table 6: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.88% | 0.95% | 1.20% | 101% | 105% |
| Class C | 1.47% | 1.49% | 1.58% | 101% | 102% |
| Class E | 1.08% | 0.80% | 0.77% | 101% | 105% |
| Overall | 1.13% | 1.09% | 1.22% | 101% | 104% |
| Class D | 1.37% | 1.28% | 1.31% | 101% | 102% |

Summary

This contribution describes temporal motion vector data storage reduction that provide less storage and coding gain. By 16x memory compression, the results show 0.1% and 0.1% BD-rate saving for random access and low delay configuration, tool-on test, respectively. We propose to study temporal motion data reduction in CE.

Reference

- Y.su, Andrew segall, “CE9: Reduced resolution storage of motion vector data”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).