| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0086-v4 |
| --- | --- |

| Title: | [non-CE] Temporal motion vector data storage reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang, Junghak Nam, Seung-Hwan Kim, Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 {hm.jang, junghak.nam, seunghwan3.kim, jaehyun.lim}@lge.com |
| Source: | LG Electronics inc. |  |  |

_____________________________

- Abstract

This contribution reports coding performance impact with respect to the compression unit size of temporal motion vector storage base on spatial candidates located in compression unit. It is observed that the proposed compression method shows 0.1% BD-rate coding gain from 16x16 compression unit and 0.1% BD-rate loss with a compression unit of 32x32, respectively in VTM Random access configuration.

Introduction

In JCTVC-D0072[1], the temporal motion vector storage reduction was proposed and it is adopted that temporal memory is reduced by a factor of 16 even though BD-rate loss was observed. This contribution suggests temporal motion vector storage reduction based on motion vector data of spatial candidates for temporal motion data compression. In the VTM, a 16x16 unit based temporal memory compression is applied. On the other hand, there is any memory compression is not applied because sub block motion vector prediction; ATMVP is enabled.

Proposed method

The proposed method basically reduces temporal mv data (i.e. motion information) storage. In this proposal temporal motion vector data is compressed based on motion data of spatial candidates. The searching order for spatial candidates is pre-defined as Figure 1.

Figure 1. Block diagram of proposed compression method

If the reference spatial candidate is predicted by inter-prediction then the motion vector data of spatial candidate is set as default motion vector data for compression. The maximum 5 candidates is used as reference temporal motion information to derive default temporal motion data. It is depicted in Figure 2.

Figure 2

Experimental results

16x16 unit based temporal motion compression.

When the temporal motion information is stored by the 16x16 block. It is achieved that memory reduction with BD rate reduction also.

Table 1: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.09% | -0.05% | 101% | 105% |
| Class A2 | -0.07% | -0.04% | -0.04% | 101% | 105% |
| Class B | -0.06% | -0.07% | -0.08% | 100% | 105% |
| Class C | -0.17% | -0.15% | -0.14% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.09% | -0.08% | 100% | 105% |
| Class D | -0.14% | 0.00% | -0.20% | 100% | 105% |

Table 2: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.08% | 0.16% | 101% | 106% |
| Class C | -0.09% | -0.20% | -0.12% | 101% | 106% |
| Class E | -0.05% | -0.09% | 0.17% | 101% | 108% |
| Overall | -0.06% | -0.06% | 0.07% | 101% | 106% |
| Class D | -0.17% | -0.09% | -0.29% | 100% | 104% |

32x32 unit based temporal motion compression.

When the temporal motion information is stored by the 32x32 block. It is achieved that memory reduction with negligible BD-rate loss.

Table 3: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | 0.02% | 102% | 105% |
| Class A2 | 0.04% | 0.06% | 0.10% | 101% | 104% |
| Class B | 0.11% | 0.01% | 0.11% | 101% | 105% |
| Class C | 0.23% | 0.25% | 0.27% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.08% | 0.13% | 101% | 105% |
| Class D | 0.44% | 0.35% | 0.40% | 101% | 103% |

Table 4: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.29% | 0.40% | 0.54% | 101% | 106% |
| Class C | 0.60% | 0.55% | 0.67% | 101% | 103% |
| Class E | 0.35% | 0.00% | -0.07% | 100% | 106% |
| Overall | 0.41% | 0.35% | 0.43% | 101% | 105% |
| Class D | 0.57% | 0.43% | 0.37% | 101% | 103% |

64x64 unit based temporal motion compression.

When the temporal motion information is stored by the 64x64 block. It is achieved that memory reduction and it is also observed similar ratio BD-rate loss compared with memory compression mechanism of HEVC standard in Random access configuration.

Table 5: Experimental results for random access test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.29% | 0.18% | 0.30% | 101% | 105% |
| Class A2 | 0.43% | 0.44% | 0.49% | 101% | 104% |
| Class B | 0.55% | 0.38% | 0.44% | 102% | 105% |
| Class C | 0.87% | 0.76% | 0.85% | 103% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.56% | 0.45% | 0.53% | 102% | 105% |
| Class D | 1.25% | 1.10% | 1.01% | 102% | 103% |

Table 6: Experimental results for low-delay test condition; anchor is VTM1.0.

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.88% | 0.95% | 1.20% | 101% | 105% |
| Class C | 1.47% | 1.49% | 1.58% | 101% | 102% |
| Class E | 1.08% | 0.80% | 0.77% | 101% | 105% |
| Overall | 1.13% | 1.09% | 1.22% | 101% | 104% |
| Class D | 1.37% | 1.28% | 1.31% | 101% | 102% |

Summary

This contribution describes temporal motion vector data storage reduction. From the proposed method, it is achieved that 0.1% coding gain compared with HEVC’s motion data compression in VTM. Additionally it is observed that 0.1% coding loss with a factor of 64 storage reduction with negligible coding performance change.

Reference

- Y.su, Andrew segall, “CE9: Reduced resolution storage of motion vector data”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).