| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0087 |
| --- | --- |

| Title: | CE3-3.3.1: MPM list construction based on dependency between neighboring intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Jin Heo Jangwon Choi Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 l.li@lge.com jin78.heo@lge.com jangwon84.choi@lge.com sunmi.yoo@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a MPM list construction method which depends on similarity of intra modes from neighbor blocks. In the proposed method, as shown in Figure 1, when the intra mode of E is similar with that of A, the MPM list is constructed with following orders: B, A, Planar, C, E, DC, D. Otherwise, the MPM list is constructed with the order of A, B, Planar, D, E, DC, C. In the test CE3-3.3.1, abovementioned method is tested on top of VTM [1] and BMS [2].The experiment results reportedly show xx%, xx%, xx% and xx% bit rate savings for VTM-AI, VTM-RA, BMS-AI and BMS-RA, respectively.

Proposed method

At the last meeting, a MPM list construction method was proposed in JVET-J0017 [3]. In the proposed method, as shown in Figure 1, when the intra mode of E is similar with that of L, the MPM list is constructed with following orders: B, A, Planar, C, E, DC, D. Otherwise, the MPM list is constructed with the order of A, B, Planar, D, E, DC, C. The similarity is defined such that the differences between intra mode E and A is smaller than 2. The similarity is taken into consideration only when the intra modes of E and A are both angular modes. The rest of MPM construction is the same with JVET-C0055 [4] including derived mode and default mode and so on.

Figure 1. Neighbor blocks of current block

Experimental results

The proposed method is implemented on top of BMS-1.0, and evaluated for both VTM and BMS according to the common test conditions [5] and Core Experiment [6]. For the VTM test, the VTM configuration file with Intra65Ang set to 1 is used, since the proposed method works in the case when intra 65 angular mode is enabled.

Table 1 Experiment result for all intra (AI) using VTM 1.0 as anchor

|  | Over VTM-1.0 AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

Table 2 Experiment result for random access (RA) using VTM 1.0 as anchor

|  | Over VTM-1.0 RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

Table 2 Experiment result for all intra (AI) using BMS 1.0 as anchor

|  | Over BMS-1.0 AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.14% | -0.08% | 101% | 103% |
| Class A2 | -0.03% | 0.01% | -0.05% | 100% | 102% |
| Class B | -0.03% | 0.05% | 0.12% | 101% | 103% |
| Class C | -0.04% | 0.09% | 0.05% | 101% | 101% |
| Class E | -0.09% | 0.12% | -0.10% | 101% | 101% |
| Overall | -0.04% | 0.08% | 0.01% | 101% | 102% |
| Class D | -0.01% | 0.17% | 0.03% | 101% | 102% |

Table 4 Experiment result for random access (RA) using BMS 1.0 as anchor

|  | Over BMS-1.0 RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.06% | 0.04% | 101% | 102% |
| Class A2 | 0.00% | -0.04% | -0.01% | 101% | 102% |
| Class B | 0.00% | 0.09% | -0.16% | 101% | 101% |
| Class C | 0.02% | 0.16% | -0.26% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | -0.11% | 101% | 101% |
| Class D | 0.00% | 0.12% | -0.04% | 101% | 101% |

References

- VVC Test Model (VTM), https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_VTM/tags/VTM-1.0/

- Benchmark Set (BMS), https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- M. Koo, J. Heo, J. Nam, N. Park, J. Lee, J. Choi, S. Yoo, H. Jang, L. Li, J. Lim, S. Paluri, M. Salehifar, S. Kim, “Description of SDR video coding technology proposal by LG Electronics,” Document of Joint Video Experts Team (JVET), JVET-J0017, 10th Meeting: San Diego, CA, 10–20 April 2018.

- V. Seregin, X. Zhao, A. Said, M. Karczewicz (Qualcomm), “Neighbor based intra most probable modes list derivation” Document of Joint Video Experts Team (JVET), JVET-C0055, 3rd Meeting: Geneva, Switzerland, May 26-June 1, 2016.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” Document of Joint Video Experts Team (JVET), JVET-J1023, 10th Meeting: San Diego, CA, 10–20 April 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).