| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: San Diego, CA, 10–20 April 2018 | Document: JVET-K0090 |
| --- | --- |

| Title: | CE3-7.3.1 and CE3-7.3.2: Linear interpolation intra prediction (LIP) based on LIP and MPM flag signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Jangwon Choi Sunmi Yoo Ling Li Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 jin78.heo@lge.com jangwon84.choi@lge.com sunmi.yoo@lge.com l.li@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In JVET-J0017 [1] LGE’s response to the call for proposals (CFP) linear interpolation intra prediction (LIP) was introduced. In this contribution, two test scenarios were tested for LIP: (1) CE3-7.3.1 where MPM flag is coded after LIP flag signaling, and (2) CE3-7.3.2 where LIP flag coding is coded after MPM flag signaling.

Simulation results for CE3-7.3.1 reportedly showed average luma BD-rate savings of 0.55%, 0.27%, 0.67%, and 0.43% compared to VTM1.0-AI, VTM1.0-RA, BMS1.0-AI, and BMS1.0-RA, respectively. It is also reported that the average BD-rate savings for CE3-7.3.2 are approximately 0.57%, 0.28%, 0.68%, and 0.44% compared to VTM1.0-AI, VTM1.0-RA, BMS1.0-AI, and BMS1.0-RA, respectively.

Proposed method

Linear interpolation intra prediction (LIP) was proposed in LGE’s submission to call for proposals [1]. LIP is applied on top of intra prediction. An LIP flag for an intra coded CU is signalled at the CU level to indicate whether LIP intra prediction mode is used or not. The proposed intra prediction method can be classified into two prediction types namely (i) the conventional intra prediction method, and (ii) the LIP method. The conventional intra prediction method uses 35 and 67 modes including 2 non-angular (Planar and DC) predictions in VTM1.0 and BMS1.0. However, the LIP prediction method was applied to 33 and 65 prediction modes except for Planar and DC, respectively.

The LIP intra prediction method first computes the intensity value of the bottom right (BR) neighboring sample, as described in Equation (1). Then, it generates the intensity values of the bottom row (Pb(x,y)) and the right column (Pr(x,y)) of the neighboring samples. These samples serve as references on the opposite side of the CU and are derived using the intensity values of the BR neighboring sample and the intensity values of other corner neighboring samples, i.e., the top right neighboring sample, the bottom left neighboring sample.

|  | BRW,H= W×R2W-1,-1+H×R(-1, 2H-1)(W+H) | (1) |
| --- | --- | --- |

where, R(x, y) denotes the intensity value of the reconstructed neighboring sample at (x, y) coordinate, W and H are the block width and height, respectively.

Figure 1 illustrates the sample prediction process using LIP for an arbitrary angular prediction mode. The red line represents the prediction direction. The points ‘A’ and ‘B’ represent the samples at the integer sample positions and the point ‘P’ indicates the sample at 1/32 sample position in the reference sample array. The point ‘A'´’ and ‘B´’ represent the samples at the integer sample positions and the point ‘P´’ indicates the sample at 1/32 sample position in the opposite side reference sample array.

Figure 1 Sample prediction process for LIP

The first stage of the intra angular mode prediction involves extrapolating samples from a projected reference sample location based on a given direction. The projected reference sample location is computed with bilinear interpolation which is used to obtain the value of the projected reference sample using two closest reference samples located at integer position.

The second stage in the prediction process requires an identical approach to generate another predicted sample value at the same sample location. The second predicted sample value is obtained by projecting the location of the sample onto the opposite side of the reference sample array by applying the selected prediction direction and interpolating the value to 1/32 sample position accuracy. The bilinear interpolation is performed linearly using the two closest opposite side reference samples in the direction of prediction.

The LIP takes advantage of the difference in accuracy between the input intensities in the final interpolation process. Specifically, larger weights are applied to the positions that are closer to more reliable neighboring positions. Final predicted sample value is calculated by using Equation (2).

|  | C= (W1×P+W2×P')(W1+W2) | (2) |
| --- | --- | --- |

Where, C is the final predicted sample value, ‘P’ and ‘P´’ are the predictors generated from the reference sample location and the opposite side reference sample location, respectively. W1 and W2 are weighting factors determined according to the distance between the current sample position and the reference (the opposite side reference) sample position.

Experimental results

The proposed method has been implemented based on BMS1.0 reference software and experimentally evaluated both for VTM and BMS test according to common test conditions defined in JVET-J1010 [2] and Core Experiment description in JVET-J1023 [3]. For the VTM test, the VTM configuration was enabled, while the BMS configuration was used for the BMS test using the same software.

CE3-7.3.1

In CE3-7.3.1, first the LIP flag is parsed and then, a most probably mode (MPM) flag is parsed. When LIP is applied, the LIP mode is determined in accordance with the MPM index information without MPM flag parsing. If the LIP flag is false, an MPM flag is parsed and an MPM index is parsed when the MPM flag is enabled (i.e. MPM mode). Otherwise, intra mode information is parsed.

Table 1 Test 7.3.1 VTM-AI results

Table 2 Test 7.3.1 VTM-RA results

Table 3 Test 7.3.1 BMS-AI results

Table 4 Test 7.3.1 BMS-RA results

CE3-7.3.2

In CE3-7.3.2, first the MPM flag is parsed and then, LIP flag is parsed. If the MPM flag is enabled, the LIP flag is parsed and the mode information is determined according to the MPM index. Otherwise, intra mode information is parsed.

Table 5 Test 7.3.2 VTM-AI results

Table 6 Test 7.3.2 VTM-RA results

Table 7 Test 7.3.2 BMS-AI results

Table 8 Test 7.3.2 BMS-RA results

Reference

- M. Koo, et al., “Description of SDR video coding technology proposed by LG Electronics,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-J0017, 10th meeting, San Diego, CA, 10-20 April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” Document of Joint Video Experts Team (JVET), JVET-J1023, 10th Meeting: San Diego, CA, 10–20 April 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).