| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0092-v1 |
| --- | --- |

| Title: | CE3 : Extended LM angular prediction (Test 4.1.6 and 4.1.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi, Jin Heo, Sunmi Yoo, Ling Li, Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, sunmi.yoo, l.li, jaehyun.lim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of extended LM-angular prediction, which is test 4.1.6 and 4.1.7 of CE3. This technology was proposed in the contributions of JVET-J0017 - LGE’s response to the call for proposals, and it introduced the adaptive CCLM mode selection method in the LM-angular chroma intra prediction. CE3-4.1.6 and CE3-4.1.7 represent the test results with MFLM tool and without MFLM tool, respectively. Experimental results for CE3-4.1.6 reportedly show 0.03%, 0.30%, and 0.23% bit rate savings on Y, Cb, and Cr components respectively with 101% encoding time, compared to BMS1.0 in All Intra configuration. It is also reported that experimental results on CE3-4.1.7 show 0.01%, 0.01%, and -0.02% bit rate savings on Y, Cb, and Cr components respectively with 90% encoding time, compared to the BMS1.0 in All Intra configuration.

Proposed method

With LAP (LM-angular prediction), which was introduced in JVET-D0110[1], two chroma intra prediction patterns named P1 and P2 are generated for the chroma block first, and then they are combined together as follows:

|  | P=0.5×P1+0.5×P2 | (1) |
| --- | --- | --- |

where, P1 is generated with the chroma intra modes other than the CCLM modes (including angular, DC and Planar mode), and P2 is generated with the CCLM mode. Unlike the original LAP algorithm, which generates P2 with the multiple model CCLM mode only, two CCLM modes (single model CCLM mode or multiple model CCLM mode) are selectively used for P2 prediction in the proposed method [2].

If the current chroma block is not coded with the CCLM mode, i.e., it is coded the angular, DC and Planar intra modes, a flag is followed to indicate whether LAP is applied or not with this mode. The encoder chooses P2 in the two CCLM modes (single model CCLM or multiple model CCLM) with the RDO process and signals the selected CCLM mode of P2 if the LAP flag is on.

When the LAP flag is on and the number of pixels in chroma block is over than 129, then the selected CCLM mode (single model CCLM or multi model CCLM) is determined from 1 bit flag and applied to the LAP. If the LAP flag is on and the number of pixels in chroma block is less than 129, the default mode (multi model CCLM) is selected for the LAP. For the signaling optimization, the proposed method is not applied when the number of pixels in chroma block is under 65.

Experimental results

The proposed method has been implemented using BMS-1.0 as software base and experimentally evaluated both for VTM and BMS test according to the common test conditions defined in JVET-J1010 [3] and Core Experiment description in JVET-J1023 [4]. For VTM based test, the VTM configuration was enabled, while the BMS configuration was used for BMS test using the same software.

CE3-4.1.6

In CE3-4.1.6, the extended LAP is applied to the BMS1.0 and VTM1.0.

Table 1 Experimental results for all-intra (AI) test condition; anchor is BMS1.0

|  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -1.03% | -0.50% | 101% | 100% |
| Class A2 | -0.06% | -0.29% | -0.20% | 101% | 101% |
| Class B | -0.01% | -0.14% | -0.25% | 101% | 102% |
| Class C | -0.02% | -0.07% | -0.04% | 101% | 101% |
| Class E | 0.01% | -0.18% | -0.20% | 101% | 102% |
| Overall | -0.03% | -0.30% | -0.23% | 101% | 101% |
| Class D | 0.03% | -0.01% | 0.02% | 101% | 102% |

Table 2 Experimental results for random-access (RA) test condition; anchor is BMS1.0

|  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -1.08% | -0.59% | 105% | 100% |
| Class A2 | -0.02% | -0.31% | 0.00% | 102% | 101% |
| Class B | 0.00% | -0.32% | -0.37% | 102% | 100% |
| Class C | 0.04% | 0.01% | -0.33% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.38% | -0.33% | 103% | 101% |
| Class D | 0.09% | 0.14% | 0.00% | 102% | 102% |

Table 3 Experimental results for all-intra (AI) test condition; anchor is VTM1.0

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.59% | -24.90% | -22.49% | 139% | 107% |
| Class A2 | -1.82% | -10.06% | -5.81% | 142% | 106% |
| Class B | -0.83% | -8.44% | -11.61% | 141% | 105% |
| Class C | -1.50% | -9.81% | -12.66% | 139% | 108% |
| Class E | -0.46% | -3.96% | -6.42% | 142% | 105% |
| Overall | -1.71% | -11.01% | -11.82% | 141% | 106% |
| Class D | -1.02% | -7.79% | -9.02% | 138% | 104% |

Table 4 Experimental results for random-access (RA) test condition; anchor is VTM1.0

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.01% | -21.85% | -23.84% | 110% | 103% |
| Class A2 | -0.70% | -10.28% | -4.80% | 107% | 102% |
| Class B | -0.40% | -11.30% | -13.94% | 107% | 103% |
| Class C | -0.59% | -9.42% | -11.30% | 107% | 103% |
| Class E |  |  |  |  |  |
| Overall | -1.03% | -12.70% | -13.39% | 107% | 103% |
| Class D | -0.36% | -7.77% | -8.98% | 106% | 102% |

Table 5 Experimental results for all-intra (AI) test condition; anchor is VTM1.0 with LM, MMLM, and MFLM

|  | Over VTM-1.0 with LM,MMLM,MFLM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -1.57% | -0.62% | 99% | 101% |
| Class A2 | -0.10% | -0.29% | -0.10% | 100% | 101% |
| Class B | -0.03% | -0.30% | -0.25% | 99% | 99% |
| Class C | 0.00% | -0.08% | -0.16% | 100% | 100% |
| Class E | 0.04% | -0.18% | -0.33% | 100% | 100% |
| Overall | -0.03% | -0.44% | -0.28% | 99% | 100% |
| Class D | 0.00% | -0.11% | 0.09% | 100% | 100% |

Table 6 Experimental results for random-access (RA) test condition; anchor is VTM1.0 with LM, MMLM, and MFLM

|  | Over VTM-1.0 with LM,MMLM,MFLM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -1.34% | -0.37% | 102% | 100% |
| Class A2 | -0.06% | -0.26% | -0.10% | 101% | 100% |
| Class B | -0.02% | -0.11% | -0.23% | 101% | 100% |
| Class C | -0.01% | 0.11% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.33% | -0.15% | 101% | 100% |
| Class D | 0.04% | -0.35% | -0.15% | 100% | 100% |

CE3-4.1.7

In CE3-4.1.7, the extended LAP is applied to the BMS1.0 without MFLM and VTM1.0.

Table 7 Experimental results for all-intra (AI) test condition; anchor is BMS1.0

|  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.91% | -0.44% | 88% | 100% |
| Class A2 | 0.03% | 0.33% | 0.85% | 90% | 100% |
| Class B | 0.00% | 0.22% | -0.11% | 90% | 101% |
| Class C | -0.01% | 0.24% | 0.01% | 91% | 99% |
| Class E | 0.02% | -0.19% | -0.11% | 90% | 101% |
| Overall | -0.01% | -0.01% | 0.02% | 90% | 100% |
| Class D | 0.03% | 0.26% | 0.17% | 92% | 101% |

Table 8 Experimental results for random-access (RA) test condition; anchor is BMS1.0

|  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -1.08% | -0.61% | 99% | 100% |
| Class A2 | 0.06% | 0.40% | 0.54% | 98% | 100% |
| Class B | 0.01% | 0.13% | -0.29% | 98% | 100% |
| Class C | 0.04% | 0.08% | -0.31% | 98% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.07% | -0.19% | 98% | 100% |
| Class D | 0.04% | 0.14% | -0.17% | 98% | 102% |

Table 9 Experimental results for all-intra (AI) test condition; anchor is VTM1.0

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.59% | -24.84% | -22.42% | 121% | 101% |
| Class A2 | -1.74% | -9.58% | -5.03% | 123% | 103% |
| Class B | -0.81% | -8.10% | -11.48% | 122% | 103% |
| Class C | -1.51% | -9.66% | -12.47% | 121% | 105% |
| Class E | -0.48% | -4.01% | -6.26% | 122% | 104% |
| Overall | -1.70% | -10.80% | -11.58% | 122% | 103% |
| Class D | -1.03% | -7.60% | -9.08% | 121% | 103% |

Table 10 Experimental results for random-access (RA) test condition; anchor is VTM1.0

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.02% | -21.73% | -23.76% | 107% | 101% |
| Class A2 | -0.62% | -9.66% | -4.16% | 105% | 102% |
| Class B | -0.39% | -10.94% | -13.83% | 104% | 102% |
| Class C | -0.59% | -9.29% | -11.27% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -1.01% | -12.40% | -13.20% | 105% | 102% |
| Class D | -0.33% | -7.48% | -9.06% | 104% | 102% |

Reference

- K. Zhang, et al., “Enhanced Cross-component Linear Model Intra-prediction” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-D0110, 4th meeting, Chengdu, CN, 15-21 October 2016.

- M. Koo, et al., “Description of SDR video coding technology proposed by LG Electronics,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-J0017, 10th meeting, San Diego, CA, 10-20 April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” Document of Joint Video Experts Team (JVET), JVET-J1023, 10th Meeting: San Diego, CA, 10–20 April 2018.

Patent rights declaration

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).