| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0096 |
| --- | --- |

| Title: | CE 6-1.11: AMT replacement and restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Mehdi Salehifar Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In JVET-J0017 [1] LGE’s response to the call for proposals, a simplified AMT (Adaptive Multiple Transforms) was introduced. In this contribution, we report test results of CE 6-1.11 and explain details of algorithm and architecture applied in the proposed AMT simplification, which is composed of four features: 1) employment of only two transforms, such as DST-7 and DCT-8, 2) AMT size restriction up to 32, 3) DST-7 design with DFT (Discrete Fourier Transform), and 4) AMT index coding. The first feature facilitates removal of DST-1 and DCT-5. The second and third ones contribute complexity reduction in both computation and memory usage. The last one leads noticeable coding gain increase for intra-coded blocks. Particularly, DFT-based DST-7 design enables XX% and XX% reduction of multiplication counts compared to direct multiply in 16-point and 32-point DST-7, respectively. Consequently, the multiplication counts become similar or even less than DCT-2 counterparts. As CE 6-1.11 test results, 3.31% (AI), 1.98% (RA), and 1.37% (LD) bit rate savings compared to VTM anchor are observed. Compared with original AMT (BMS AMT), BD-rate is degraded by 0.15% (AI), 0.19% (RA), and 0.20% (LD), while applying above mentioned four aspects of AMT complexity reduction. Moreover, encoding/decoding times are also reduced by 9%/17% (AI), 26%/8% (RA), and 31%/9% (LD), which were achieved especially due to the above second and third features.

Proposed method

Transform combinations with only DST-7 and DCT-8

Table 2 indicate the AMT indices that correspond to the separable transform for the horizontal and vertical direction for intra and inter transform units respectively.

Table 1 Supported transform pairs for intra predicted residual

| EMT index | Hor. transform | Ver. Transform |
| --- | --- | --- |
| 0 | DST7 | DST7 |
| 1 | DCT8 | DST7 |
| 2 | DST7 | DCT8 |
| 3 | DCT8 | DCT8 |

Table 2 The supported transform pairs for inter predicted residual

| EMT index | Hor. transform | Ver. Transform |
| --- | --- | --- |
| 0 | DCT8 | DCT8 |
| 1 | DST7 | DCT8 |
| 2 | DCT8 | DST7 |
| 3 | DST7 | DST7 |

Table 1 states that, a uniform mapping is used regardless of the intra mode used. This is contrary to BMS AMT where each intra prediction mode group uses its own unique mapping. Furthermore, DCT-8 is known to have the following relationship with DST-7:

, where ,

and                          (1)

The  and  in the Equation (1) are inverse transform matrices for DCT-8 and DST-7, and i and j are row and column indices, respectively. In the Equation (1), JN is the matrix is represented by 1s along its anti-diagonal line, and the matrix DN alternates between 1 and -1 on its diagonal line. Therefore, DCT8 can be derived from DST7 with sign changes and reordering just before and after the DST7 computation. Hence, DST7 is reused in this implementation for DCT8. The sign changes and shuffling do not add any additional overhead to DST7, so that the computational complexity of DCT8 is identical to that of DST7. This avoids the usage of any additional memory in DCT8 as well as DST-1 and DCT-5.

AMT size restriction

The maximum size of AMT supported in this proposal is 32×32. The 64×64 AMT that exists in BMS was omitted due to its computational complexity and memory usage.

DST-7 design with DFT

The inverse transform matrix for DST-7 is presented in the Equation (2), where n and k are row and column indices, respectively.

(2)

The N x N forward DST-7 matrix can be formulated with (2N+1)-point DFT in the Equation (3).

, where ,

, and                 (3)

The Equation (3) holds true only if N is even. It should be noted that the authors in [4] describe the conditions when N is either odd or even. The  takes imaginary part after performing the (2N+1)-point DFT whose input vector is supplied from right side. The N x N inverse DST-7 matrix is identical to N x N forward DST-6, of which mathematical formulae is similarly constructed as presented in the Equation (4).

,   where ,

, and                             (4)

For N = 8, 16 and 32, 17-point, 33-point, and 65-point DFTs are required for designing DST-7 from the Equations (3) and (4). If N is factored into two mutually prime numbers, i.e. N = N1N2 (here, N is used instead of 2N+1 for notational simplicity), a 1-dimensional N-point DFT can be transformed into its equivalent 2-dimensional DFT. The Equations (5-a) and (5-b) show the associated DFT formulae.

(5-a)

(5-b)

To establish the equivalence of (5-a) and (5-b), some specified mappings between 1-D and 2-D signals are required. The Equation (6) shows the mappings employed in the proposed DST-7 designs.

(6-a)

(6-b)

The Equation (6-a) ((6-b)) is applied in each 2-D DFT whose corresponding 1-D DFT is 33-point DFT (65-point DFT). From now on, for simplicity, we will denote each 2-D DFT as its equivalent 1-D DFT, i.e. 33-point DFT (65-point DFT). The notation  means , the remainder value of  divided by .

The 2-D signals in the Equation (5-b) are basically complex numbers. Thus, they can be decomposed into real and imaginary parts as in the Equation (7).

,

(7)

The matrices  and  in the Equations (3) and (4), inherent symmetric relations in trigonometric function values (e.g. ), etc. induce redundant calculations, among which only necessary ones need be performed according to the following equations (here, (N1, N2) = (3, 11) for 33-point DFT, and (N1, N2) = (5, 13) for 65-point DFT):

For  and ,

(8)

(9)

The Equations (8) and (9) appear in 33-point DFT and 65-point DFT, respectively.

For  and ,

,              (10)

By exploiting the symmetric relations in trigonometric function values, the amount of multiplication in the Equation (10) can be further decreased.

,

where                                            (11)

For  and ,

(12)

For  and ,

(13)

In summary, we can reach pictorial representations in Fig. 1 and Fig. 2, incorporating all the computational reductions mostly benefited from all the symmetries.

Figure 1  The 16-point DST-7 block diagram applying 33-point DFT with all the existent symmetries

Figure 2  The 32-point DST-7 block diagram applying 65-point DFT with all the existent symmetries

In Fig. 1 and Fig. 2, Simplified 3-point DFT Type 1, Simplified 3-point DFT Type 2, Simplified 11-point DFT Type 1, and Simplified 11-point DFT Type 2 correspond to the Equations (8), (10), (12), and (13), respectively. Similarly, Simplified 5-point DFT Type 1, Simplified 5-point DFT Type 2, Simplified 13-point DFT Type 1, and Simplified 13-point DFT Type 2 are implemented according to the Equations (9), (10), (12), and (13), respectively.

Practically, the Equations (8), (9), (10), (12), and (13) should be implemented with integer operations. To realize this, the sufficient amount of scaling needs to be imposed on trigonometric function values in all the equations in order to maintain enough precision. For instance, the Equation (12) turns into the following:

,                                     (19)

The  should be a scaling factor to make cosine and sine values sufficiently large. If  is a power of 2,  can be implemented with only right shift.

AMT index coding optimization

The following three modifications are incorporated on AMT index coding:

1) All intra-prediction modes are classified into four classes, i.e. Planar, DC, Horizontal, and Vertical, each of which is mapped to 0, 1, [2, 34], and [35, 66], respectively. For Horizontal class, indices 1 and 2 are exchanged before encoding and after decoding. In other words, the coded value 1 (2) indicate index 2 (1) for Horizontal class.

2) Truncated unary binarization is used instead. Up to 3 bins are coded for one index, each of which has its own context.

3) Different context sets for AMT index coding are assigned for the four classes.

Experimental results

The proposed method has been implemented based on BMS1.1 reference software and experimentally evaluated both for VTM and BMS test according to common test conditions defined in JVET-J1010 [2] and Core Experiment description in JVET-J1026 [3].

VTM-based results

Comparing the results of 3.1.1 and 3.1.2, It is figured out that, 1) the coding gain is decreased by 0.15% due to the proposed simplifications, 2) the simulation times are decreased by 9% (AI), 26% (RA), and 31% (LD) in encoding, and 17% (AI), 8% (RA), and 9% (LD) in decoding.

BMS + the proposed AMT + disabling JEM tools

- Anchor: BMS + disabling JEM tools

BMS + BMS AMT + disabling JEM tools

- Anchor: BMS + disabling JEM tools

BMS + the proposed AMT + disabling JEM tools except secondary transform

- Anchor: BMS + disabling JEM tools except secondary transform

BMS-based results

BMS + the proposed AMT

- Anchor: BMS

BMS + the proposed AMT + disabling secondary transform

- Anchor: BMS + disabling secondary transform

Results on Low QP condition

Comparing the results of 2.3.1 and 2.3.2, it is observed that the performance difference between the proposed AMT and BMS AMT on Low QP condition is similar with that on normal QP condition.

BMS + the proposed AMT + disabling JEM tools

- Anchor: BMS + disabling JEM tools

BMS + BMS AMT + disabling JEM tools

- Anchor: BMS + disabling JEM tools

Break-down analysis of the four features in the proposed AMT

- Test description: BMS + Config. + disabling JEM tools

(A) Employment of only DST-7 and DCT-8

(B) AMT size restriction up to 32

(C) DST-7 design with DFT

(D) Improvement of AMT index coding

- Anchor: BMS + disabling JEM tools

From the above table, it turns out that much of coding loss caused by the proposed simplifications are due to (A) and (B) in AI and RA, respectively. The DST-7 design with DFT ((C)) causes little impact on coding performance. The proposed AMT index coding scheme ((D)) contributes noticeable coding gain improvement in AI.

Conclusion

It is observed that all the simplifications and the new AMT index coding keep 96%, 91%, and 87% of coding gains of BMS AMT for AI, RA, and LD, respectively. Furthermore, it is reportedly showed that our proposed DFT-based DST-7 design can reduce computational complexity in terms of operation counts compared with existing matrix-based DST-7 and butterfly DCT-2. Due to the employment of only DST-7 and DCT-8 and simple derivation of DCT-8 reusing DST-7, all matrix-based implementations and the associated memory usage of DST-1, DCT-5, and DCT-8 can be removed. The performance of the proposed AMT is 3.31%, 1.98%, and 1.37% in AI, RA, and LD respectively on top of VTM, together with encoding/decoding time reduction by 9%/17%, 26%/8%, and 31%/9% in AI, RA, and LD respectively. It is strongly recommended that the proposed AMT should be integrated into next version of VTM and be further investigated in following CEs for more improvements and better trade-offs between performance and complexity.

Reference

- M. Koo, et al., “Description of SDR video coding technology proposed by LG Electronics,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-J0017, 10th meeting, San Diego, CA, 10-20 April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

- A. Said and X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling,” Document of Joint Video Experts Team (JVET), JVET-J1026, 10th Meeting: San Diego, CA, 10–20 April 2018.

- R. K. Chivukula, and Y. A. Reznik, “Fast computing of discrete cosine and sine transforms of types VI and VII,” SPIE Optical Engineering+ Applications, 2011, pp. 813505-313505

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).