| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0097 |
| --- | --- |

| Title: | CE3-2.3.3 and CE3-2.3.4:Interpolation filter selection regarding intra mode and block size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jin Heo Jangwon Choi Ling Li Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com jin78.heo@lge.com jangwon84.choi@lge.com l.li@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a method to select a suitable interpolation filter for intra prediction. In the proposed method, one of two different interpolation filters is chosen – cubic and Gaussian filter according to a current intra prediction direction and the block size. In Test CE3-2.3.3, the interpolation filters in JEM were used, and the 6-tap cubic filter and 4-tap Gaussian filter were used in Test CE3-2.3.4. The simulation results reportedly show that Y BD-rate decreases 0.54%, 0.28%, 11.72% and 16.02% for VTM-AI, VTM-RA, BMS-AI and BMS-RA, respectively, in Test CE3-2.3.3. Also, the coding efficiency is achieved approximately 0.58%, 0.30%, 11.76% and 16.04% for VTM-AI, VTM-RA, BMS-AI and BMS-RA, respectively.

Proposed method

At the last meeting, an interpolation selection algorithm was proposed in JVET-J0017 [1]. In the proposed method, a four-tap or six-tap intra interpolation filter is used for directional intra prediction filtering. Two types of interpolation filters are used: (i) Cubic interpolation filters for blocks with the size smaller than or equal to 64 samples with the intra modes whose absolute angles are less than 11 (±6 modes from horizontal and vertical modes), and (ii) Gaussian interpolation filters for blocks with the size larger than 64 samples with the intra modes whose absolute angles are equal to or larger than 11.

In Test 2.3.3 in CE3, the interpolation filters identical to the ones in JEM are used; In Test 2.3.4, a six-tap cubic interpolation filter proposed in JVET-J0021 [2] is used.

Experimental results

The proposed method has been implemented using BMS-1.0 as software base and experimentally evaluated both for VTM and BMS test according to the common test conditions defined in JVET-J1010 [3] and Core Experiment description in JVET-J1023 [4]. For VTM based test, the VTM configuration was enabled, while the BMS configuration was used for BMS test using the same software. In addition, the encoding option of intra 4-tap interpolation was enabled (Intra4Tap=1) to evaluate the proposed scheme.

Table 1 Test 2.3.3 VTM results

|  | All Intra Main10 – VTM configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  | Over BMS-1.0 |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -0.26% | -0.22% | -0.32% | 14.10% | 47.43% | 42.51% |
| Class A2 | -0.20% | -0.13% | -0.27% | 11.91% | 22.77% | 17.30% |
| Class B | -0.52% | -0.39% | -0.40% | 10.45% | 18.22% | 23.59% |
| Class C | -0.88% | -1.01% | -1.18% | 11.59% | 18.36% | 23.58% |
| Class E | -0.74% | -0.07% | 0.09% | 14.83% | 20.26% | 24.31% |
| Overall | -0.54% | -0.41% | -0.46% | 12.29% | 24.22% | 25.81% |
| Class D | -0.61% | -0.38% | -0.66% | 9.87% | 15.06% | 17.16% |
| Enc Time [%] | 106% |  |  | 11% |  |  |
| Dec Time [%] | 103% |  |  | 51% |  |  |
|  | Random Access Main 10 – VTM configuration |  |  |  |  |  |
|  | Over VTM-1.0 |  |  | Over BMS-1.0 |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -0.17% | -0.28% | -0.37% | 19.95% | 46.49% | 48.98% |
| Class A2 | -0.15% | -0.36% | -0.48% | 24.21% | 28.37% | 19.57% |
| Class B | -0.32% | -0.58% | -0.71% | 18.11% | 28.73% | 31.04% |
| Class C | -0.39% | -0.75% | -0.75% | 14.39% | 19.42% | 23.17% |
| Class E |  |  |  |  |  |  |
| Overall | -0.28% | -0.52% | -0.61% | 18.71% | 29.73% | 30.24% |
| Class D | -0.31% | -0.52% | -0.77% | 15.50% | 17.26% | 19.47% |
| Enc Time [%] | 102% |  |  | 25% |  |  |
| Dec Time [%] | 102% |  |  | 37% |  |  |

Table 2 Test 2.3.3 BMS results (All intra)

|  | All Intra Main10 – BMS configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  | Over BMS-1.0 |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -12.70% | -31.73% | -29.91% | -0.16% | -0.38% | -0.33% |
| Class A2 | -10.95% | -18.22% | -14.82% | -0.15% | -0.21% | -0.24% |
| Class B | -10.31% | -16.04% | -19.76% | -0.43% | -0.42% | -0.48% |
| Class C | -11.61% | -16.61% | -20.04% | -0.64% | -0.54% | -0.48% |
| Class E | -13.99% | -17.28% | -20.00% | -0.49% | -0.46% | -0.58% |
| Overall | -11.72% | -19.35% | -20.73% | -0.40% | -0.41% | -0.43% |
| Class D | -9.90% | -13.58% | -15.45% | -0.44% | -0.47% | -0.41% |
| Enc Time [%] | 998% |  |  | 106% |  |  |
| Dec Time [%] | 212% |  |  | 104% |  |  |
|  | Random Access Main 10– BMS configuration |  |  |  |  |  |
|  | Over VTM-1.0 |  |  | Over BMS-1.0 |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -16.83% | -30.92% | -32.73% | -0.07% | -0.24% | -0.14% |
| Class A2 | -19.61% | -22.02% | -16.76% | -0.07% | -0.37% | -0.30% |
| Class B | -15.73% | -23.09% | -24.32% | -0.25% | -0.50% | -0.41% |
| Class C | -13.06% | -17.03% | -19.74% | -0.18% | -0.29% | -0.50% |
| Class E |  |  |  |  |  |  |
| Overall | -16.02% | -22.82% | -23.27% | -0.16% | -0.36% | -0.36% |
| Class D | -13.54% | -15.32% | -17.27% | -0.12% | -0.32% | -0.51% |
| Enc Time [%] | 407% |  |  | 102% |  |  |
| Dec Time [%] | 276% |  |  | 100% |  |  |

Table 3 Test 2.3.4 VTM results (All intra)

|  | All Intra Main10 – VTM configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  | Over BMS-1.0 |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -0.17% | -0.09% | -0.23% | 14.20% | 47.66% | 42.61% |
| Class A2 | -0.13% | -0.05% | -0.20% | 11.99% | 22.85% | 17.37% |
| Class B | -0.54% | -0.46% | -0.43% | 10.43% | 18.13% | 23.53% |
| Class C | -1.13% | -1.32% | -1.45% | 11.29% | 17.98% | 23.21% |
| Class E | -0.76% | 0.32% | 0.50% | 14.82% | 20.72% | 24.79% |
| Overall | -0.58% | -0.39% | -0.43% | 12.24% | 24.24% | 25.82% |
| Class D | -0.74% | -0.70% | -0.70% | 9.73% | 14.66% | 17.11% |
| Enc Time [%] | 107% |  |  | 11% |  |  |
| Dec Time [%] | 102% |  |  | 48% |  |  |
|  | Random Access Main 10 – VTM configuration |  |  |  |  |  |
|  | Over VTM-1.0 |  |  | Over BMS-1.0 |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -0.14% | -0.37% | -0.38% | 19.99% | 46.26% | 48.94% |
| Class A2 | -0.13% | -0.56% | -0.65% | 24.23% | 28.14% | 19.36% |
| Class B | -0.32% | -0.76% | -0.81% | 18.11% | 28.51% | 30.87% |
| Class C | -0.52% | -0.98% | -0.99% | 14.26% | 19.13% | 22.86% |
| Class E |  |  |  |  |  |  |
| Overall | -0.30% | -0.70% | -0.74% | 18.68% | 29.48% | 30.04% |
| Class D | -0.34% | -0.52% | -1.01% | 15.46% | 17.29% | 19.14% |
| Enc Time [%] | 102% |  |  | 28% |  |  |
| Dec Time [%] | 100% |  |  | 38% |  |  |

Table 4 Test 2.3.4 BMS results (All intra)

|  | All Intra Main10 – BMS configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  | Over BMS-1.0 |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -12.68% | -31.85% | -29.91% | -0.14% | -0.55% | -0.34% |
| Class A2 | -10.93% | -18.24% | -14.81% | -0.12% | -0.25% | -0.22% |
| Class B | -10.34% | -16.20% | -19.94% | -0.47% | -0.60% | -0.68% |
| Class C | -11.76% | -16.83% | -20.25% | -0.82% | -0.85% | -0.74% |
| Class E | -14.02% | -17.30% | -20.11% | -0.53% | -0.47% | -0.72% |
| Overall | -11.76% | -19.47% | -20.84% | -0.44% | -0.57% | -0.57% |
| Class D | -10.01% | -13.62% | -15.53% | -0.56% | -0.49% | -0.54% |
| Enc Time [%] | 971% |  |  | 101% |  |  |
| Dec Time [%] | 217% |  |  | 101% |  |  |
|  | Random Access Main 10– BMS configuration |  |  |  |  |  |
|  | Over VTM-1.0 |  |  | Over BMS-1.0 |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -16.83% | -30.99% | -32.83% | -0.08% | -0.40% | -0.28% |
| Class A2 | -19.63% | -21.97% | -16.78% | -0.10% | -0.31% | -0.33% |
| Class B | -15.74% | -23.14% | -24.44% | -0.25% | -0.60% | -0.59% |
| Class C | -13.14% | -17.09% | -19.90% | -0.27% | -0.37% | -0.69% |
| Class E |  |  |  |  |  |  |
| Overall | -16.04% | -22.86% | -23.38% | -0.19% | -0.44% | -0.50% |
| Class D | -13.63% | -15.52% | -17.12% | -0.22% | -0.55% | -0.34% |
| Enc Time [%] | 372% |  |  | 101% |  |  |
| Dec Time [%] | 264% |  |  | 100% |  |  |

References

- M. Koo, J. Heo, J. Nam, N. Park, J. Lee, J. Choi, S. Yoo, H. Jang, L. Li, J. Lim, S. Paluri, M. Salehifar, S. Kim, “Description of SDR video coding technology proposal by LG Electronics,” Document of Joint Video Experts Team (JVET), JVET-J0017, 10th Meeting: San Diego, CA, 10–20 April 2018.

- Y.-W. Chen, W.-J. Chien, H.-C. Chuang, M. Coban, J. Dong, H. E. Egilmez, N. Hu, M. Karczewicz, A. Ramasubramonian, D. Rusanovskyy, A. Said, V. Seregin, G. Van Der Auwera, K. Zhang, L. Zhang (Qualcomm), P. Bordes, Y. Chen, C. Chevance, E. François, F. Galpin, M. Kerdranvat, F. Hiron, P. de Lagrange, F. Le Léannec, K. Naser, T. Poirier, F. Racapé, G. Rath, A. Robert, F. Urban, T. Viellard (Technicolor), “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions,” Document of Joint Video Experts Team (JVET), JVET-J0021, 10th Meeting: San Diego, CA, 10–20 April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” Document of Joint Video Experts Team (JVET), JVET-J1023, 10th Meeting: San Diego, CA, 10–20 April 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).