| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0098 |
| --- | --- |

| Title: | CE 6.2.5: Layered Givens Transform (LGT) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Mehdi Salehifar Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In JVET-J0017 [1] LGE’s response to the call for proposals (CFP), Layered Givens Transform (LGT) is proposed, which consists of Givens rotations. In this contribution, LGT is applied to top-left 8x8 region that is divided into four 4x4 blocks each of which the LGT (also denoted as 4x4 LGT) is applied to. The 4x4 LGT consists of 5 Givens rotation layers (GRLs) and 6 permutations. Contrary to BMS 4x4 NSST where fixed shuffling patterns are used, the permutations in the 4x4 LGT are performed based on arrays which store shuffling information. With the help of the permutations, the number of GRLs is reduced from 8 (BMS 4x4 NSST) to 5, while increasing memory usage for the permutations. It is reported that the number of multiplication is reduced by 37.5%, and BD-rate savings are observed to be 3.12% (AI), 1.36% (RA), and 0.47% (LD) for VTM, and BD-rate difference of 0.13% (AI), 0.05% (RA), and -0.02% (LD) are observed compared to BMS 4x4 NSST on top of VTM.

Proposed method

The LGT is applied on 4×4 blocks at the top-left 8×8 region of a coded block. This consists of 5 Givens rotation layers (GRLs) and 6 permutations. Each GRL is composed of 8 Givens rotations and each permutation shuffles transform inputs or outputs of the previous stage. The overall 4×4 LGT structure is depicted in Figure 1.

Figure 1  Structure of the 4×4 LGT

In Figure 1, the elements xi (i = 0, 1, …, 15) and yj (j = 0, 1, …, 15) are the input and output vectors of the data. Each of these vectors corresponds to a 4×4 block and is rearranged by following the row-first order. Each square box with a cross mark indicates a Givens rotation. The operation of the permutation block in Figure 2 represents a shuffling process where the current elements (e.g. 16 elements) are sorted into an output array using a predetermined (or stored) order.

Figure 2  A permutation example fulfilled in the permutation blocks in Figure 1

The shuffling example in Figure 2 can also be considered as an array, [11, 7, 13, 5, 0, 8, 4, 15, 1, 6, 14, 2, 10, 3, 12, 9], which needs to be stored in memory. We denote this array Z whose ith element is Z(i) (i = 0, 1, …, 15) and, p2j and p2j+1 (j = 0, 1, … , 7) produced from jth Givens rotation in a GRL, are then routed to qZ(2j) and qZ(2j+1). The inverse 4×4 LGT is performed in an opposite direction from bottom to top in Figure 1. For each Givens rotation, -θ is applied instead of θ during the inverse 4x4 LGT process, where θ is the rotation angle.

The number of multiplications required to process one 4 ×4 block with LGT amounts to (5 GRLs) ×8 ×4 = 160. Each Givens rotation is performed with 4 multiplications. As BMS 4x4 NSST consists of 8 GRLs, the proposed 4 ×4 LGT has 37.5% less computational load than BMS 4x4 NSST.

Additional memory usage incurred by the 4×4 LGT includes information of both Givens rotations and permutations. Each Z(i) requires 4 bits, and each rotation θ is indexed with 8 bits, because θ is quantized value, (2πk/256), k = 0, 1, … , 255.

Table 1  Comparison of memory usages in 4×4 LGT and BMS 4×4 NSST

| Transform | Givens rotation | Permutation | Total |
| --- | --- | --- | --- |
| 4×4 LGT | 320 bits | 384 bits | 704 bits |
| BMS 4×4 NSST | 512 bits | 64 bits | 576 bits |

The increased memory usage of the 4×4 LGT can be regarded as the cost for reduction of computation.

Experimental results

The proposed method has been implemented based on BMS1.1 reference software and experimentally evaluated both for VTM and BMS test according to common test conditions defined in JVET-J1010 [2] and Core Experiment description in JVET-J1026 [3].

VTM-based results

Comparing the results of 2.1.1 and 2.1.2, the proposed 4x4 LGT outperforms BMS 4x4 NSST by 0.13% (AI), 0.05% (RA), and -0.02% (LD) under VTM configuration.

BMS + 4x4 LGT + disabling JEM tools

- Anchor: BMS + disabling JEM tools

BMS + NSST + disabling JEM tools

- Anchor: BMS + disabling JEM tools

BMS + 4x4 LGT + disabling JEM tools except primary transform

- Anchor: BMS + disabling JEM tools except primary transform

BMS-based results

BMS + 4x4 LGT

- Anchor: BMS

Results on Low-QP condition

By comparing the results of 2.3.1 and 2.3.2, we can figure it out that the coding performances of 4x4 LGT and BMS 4x4 NSST are almost same on Low-QP condition.

BMS + 4x4 LGT + disabling JEM tools

- Anchor: BMS + disabling JEM tools

BMS + 4x4 LGT + disabling JEM tools

- Anchor:

Conclusion

The proposed 4x4 LGT reduces the number of Givens rotation layers by inserting a permutation between two adjacent Givens rotation layers as depicted in Figure 1, with which the required multiplication count decreases by 37.5%. It was reported that the BD-rate difference of 4x4 LGT on top of VTM is -3.12% (AI), -1.36% (RA), and -0.47% (LD), which is 0.13% (AI), 0.05% (RA), and -0.02% (LD) higher than that of 4x4 NSST. The coding performances of 4x4 LGT and 4x4 NSST on top of BMS were almost same. It is recommended that 4x4 LGT should be a candidate of next secondary transform in VTM if Givens rotation based transform is considered, with which trade-off between the number of Givens rotation layers and permutation complexity could be investigated.

Reference

- M. Koo, et al., “Description of SDR video coding technology proposed by LG Electronics,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-J0017, 10th meeting, San Diego, CA, 10-20 April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

- Amir Said and Xin Zhao, “Description of Core Experiment 6: Transforms and transform signalling,” Document of Joint Video Experts Team (JVET), JVET-J1026, 10th Meeting: San Diego, CA, 10–20 April 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).