| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0100 |
| --- | --- |

| Title: | CE6-Related : Matrix multiplication based NSST with reduced memory map |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar Moonmo Koo Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd. San Diego, CA | Tel: Email: | +1-858-635-5226 mehdi.salehifar@lge.com moonmo.koo@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In JVET-J0017 [1] LGE’s response to the call for proposals (CFP), a new Non-Separable Secondary Transform (NSST) called Reduced Secondary Transform (RST) was proposed and this RST is being investigated in CE 6.2.6. In this contribution, more detailed analysis on test results of CE 6.2.6 is reported and further updated results is shown based on proposed NSST kernel mapping for memory reduction. Specifically the following four tests are investigated;

- Test A: 16x16 direct matrix multiplication for 4x4 NSST (35 (effectively 19) maps )

- Test B: Test A with memory reduction (6 maps )

- Test C: 16x16 direct matrix multiplication for 4x4 NSST + 16x64 direct multiplication for 8x8 NSST (i.e. RST 16, 35 (effectively 19) maps)

- Test D: Test C with memory reduction (6 maps)

Under VTM test condition, for test A BD-rate savings of 3.67% (AI), 1.65% (RA)) are observed.

Under VTM test condition, for test B, BD-rate savings of 3.45% (AI), 1.53% (RA)) are observed.

Under VTM test condition, for test C, BD-rate savings of 4.97% (AI), 2.53% (RA)) are observed.

Under VTM test condition, for test D, BD-rate savings of 4.32% (AI), 2.14% (RA)) are observed.

Proposed method

Test A: 16x16 direct matrix multiplication for 4x4 NSST

In current BMS SW, HyGT kernels are used for NSST. In this proposal we use 16x16 direct matrix multiplication secondary transform for 4x4 NSST. For blocks with min (W, H) = 4, one 4x4 NSST is applied to the top 4x4 corner. (This is the same 4x4 NSST we used for our CE 6.2.6)

Test B: 16x16 direct matrix multiplication for 4x4 NSST with memory reduction

This test is similar to test A, instead of using 35 (effectively 19) maps for transform kernels; we use 6 maps for memory reduction, as shown in table 1.

Table 1  Reduced memory map with 6 transform sets

Test C: 16x64 direct multiplication for 8x8 NSST (RST 16)

In this test, we use 16x64 direct matrix multiplications for 8x8 NSST. For blocks with min (W, H) = 4, up to 2 4x4 NSST is applied. For blocks with min(W,H)>=8  8x8 NSST is applied to the top 8x8 corner. This test is similar to our CE 6.2.6 test.

Test D: 16x64 direct multiplication for 8x8 NSST (RST 16) with memory reduction

This is similar test to test C, for reducing the memory complexity we use 6 maps for storing the NSST kernels, as shown in table 1.

Experimental results

The proposed method has been implemented based on BMS1.1 reference software and experimentally evaluated for VTM and tested according to common test conditions defined in JVET-J1010 [2] and Core Experiment description in JVET-J1026 [3].

- 2.1 VTM + Proposed 4x4 NSST (35 (effectively 19) maps) (Test A)

Anchor:  VTM 1.1

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.72% | -5.50% | -6.48% | 240% | 103% |
| Class A2 | -2.84% | -4.10% | -4.47% | 245% | 104% |
| Class B | -3.11% | -4.14% | -3.59% | 247% | 105% |
| Class C | -4.15% | -3.75% | -4.06% | 261% | 107% |
| Class E | -4.76% | -3.69% | -3.68% | 225% | 106% |
| Overall | -3.67% | -4.20% | -4.34% | 245% | 105% |
| Class D | -4.22% | -2.89% | -2.79% | 273% | 106% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.88% | -5.15% | -5.72% | 131% | 101% |
| Class A2 | -1.15% | -2.54% | -2.48% | 121% | 101% |
| Class B | -1.55% | -3.95% | -3.53% | 124% | 102% |
| Class C | -1.97% | -2.72% | -2.81% | 125% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.65% | -3.58% | -3.57% | 125% | 101% |
| Class D | -1.70% | -1.47% | -1.83% | 125% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- 2.2 VTM + Proposed 4x4 NSST with memory reduction (6 maps) (Test B)

Anchor: VTM 1.1

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.60% | -4.91% | -5.91% | 237% | 103% |
| Class A2 | -2.67% | -3.81% | -4.13% | 244% | 103% |
| Class B | -2.98% | -3.74% | -3.09% | 246% | 105% |
| Class C | -3.76% | -3.39% | -3.63% | 260% | 105% |
| Class E | -4.44% | -3.41% | -3.36% | 224% | 104% |
| Overall | -3.45% | -3.81% | -3.90% | 243% | 104% |
| Class D | -3.86% | -2.54% | -2.51% | 270% | 105% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.77% | -4.69% | -5.18% | 131% | 102% |
| Class A2 | -1.04% | -2.14% | -2.17% | 121% | 102% |
| Class B | -1.47% | -3.46% | -3.01% | 124% | 102% |
| Class C | -1.81% | -2.40% | -2.41% | 125% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.53% | -3.16% | -3.11% | 125% | 102% |
| Class D | -1.52% | -1.74% | -1.84% | 125% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- 2.3 VTM + Proposed 4x4 NSST + Proposed 8x8 NSST, CE6.2.6.2 (Test C)

Anchor: VTM 1.1

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.10% | -5.12% | -6.43% | 242% | 104% |
| Class A2 | -3.72% | -4.05% | -4.69% | 247% | 105% |
| Class B | -4.07% | -4.16% | -3.70% | 247% | 104% |
| Class C | -5.86% | -4.52% | -4.91% | 258% | 101% |
| Class E | -6.42% | -5.11% | -4.96% | 222% | 103% |
| Overall | -4.97% | -4.54% | -4.80% | 244% | 103% |
| Class D | -5.29% | -3.82% | -3.91% | 271% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.89% | -4.42% | -5.73% | 133% | 102% |
| Class A2 | -1.81% | -2.21% | -2.33% | 122% | 102% |
| Class B | -2.36% | -3.08% | -2.95% | 124% | 100% |
| Class C | -3.01% | -2.92% | -3.03% | 124% | 97% |
| Class E |  |  |  |  |  |
| Overall | -2.53% | -3.13% | -3.40% | 125% | 100% |
| Class D | -2.29% | -2.05% | -2.57% | 125% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- 2.4 VTM + Proposed 4x4 NSST + Proposed 8x8 NSST with memory reduction (6 maps) (Test D)

Anchor: VTM 1.1

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.43% | -4.34% | -5.34% | 245% | 106% |
| Class A2 | -3.21% | -3.43% | -3.98% | 248% | 106% |
| Class B | -3.58% | -3.48% | -2.87% | 252% | 107% |
| Class C | -5.07% | -3.77% | -4.12% | 264% | 105% |
| Class E | -5.55% | -4.28% | -4.11% | 226% | 105% |
| Overall | -4.32% | -3.81% | -3.95% | 248% | 106% |
| Class D | -4.65% | -3.23% | -3.25% | 276% | 107% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.43% | -3.79% | -4.85% | 132% | 102% |
| Class A2 | -1.42% | -1.60% | -1.84% | 121% | 102% |
| Class B | -2.01% | -2.65% | -2.48% | 124% | 101% |
| Class C | -2.62% | -2.31% | -2.60% | 125% | 100% |
| Class E |  |  |  |  |  |
| Overall | -2.14% | -2.58% | -2.85% | 125% | 101% |
| Class D | -1.98% | -1.45% | -1.72% | 125% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Memory Complexity Analysis:

For 4x4 NSST:

Per Kernel: 16*16*9 = 0.281 KB

For 8x8 NSST:

Per Kernel: 16*64*9 = 1.125 KB

Test A: Total: (17*3 + 2*2) * Per Kernel_4x4 = 15.47 KB

Test B: Total: (4*3+2*2) * Per Kernel_4x4 = 4.5 KB

Test C: Total: (17*3 + 2*2) * (Per Kernel_4x4+ Per Kernel_8x8) = 77.34 KB

Test D: Total: (4*3+2*2) * (Per Kernel_4x4+ Per Kernel_8x8) = 22.5 KB

Conclusion

A direct matrix multiplication NSST for 4x4 NSST(16x16 direct matrix multiplications) and  8x8 NSST ( 16x64 direct matrix multiplication) is introduced and investigated in this contribution. As reported above, it improves the performance significantly, meanwhile reduces the multiplication and multilayer complexity. Also results with memory reduction also reported. Therefore we recommend that the proposed 4x4 NSST and 8x8 NSST is to be included in next VTM update. In addition, we recommend that the proposed memory reduction kernel mapping is to be further studied in a CE activity.

Reference

- M. Koo, et al., “Description of SDR video coding technology proposed by LG Electronics,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-J0017, 10th meeting, San Diego, CA, 10-20 April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

- Amir Said and Xin Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling,” Document of Joint Video Experts Team (JVET), JVET-J1026, 10th Meeting: San Diego, CA, 10–20 April 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).