| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0101 |
| --- | --- |

| Title: | Vector Coding of Affine MVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Mehdi Salehifar Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131 USA | Tel: Email: | +1- 858- 635- 5300 seethal.paluri@lge.com mehdi.salehifar@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In this proposal, a new motion vector difference coding (MVD) for Affine Motion Vector Difference (AMVD) is introduced. In affine mode, two control motion vectors are used to derive affine motion vectors for each sub-block. One is motion vector from left top and the other is motion vector from right top. Hence, two MVDs are coded. The proposed method jointly exploits the similarity of the MVD between the MVD from left and MVD from right. It is reported that 0.09% BD-rate saving is observed for Random Access configuration.

# Proposed method

## Layer and index concept

Figure 1.0 below illustrates, the frequency analysis between, the horizontal-MVD components of the left (LT) and right (RT) control point motion vectors of an affine block. The center cell represents and MVD LTx = 0 and MVD RTx = 0. The center point (0,0) also corresponds to the most frequently occuring MVD combination. Further to this, blocks adjacent to the center point correspond to either an increment or decrement of the MVD value in either a single control point or in both control points. We define groups of MVD combinations that occur with similar likelihood as a Layer. Figure 1.0 illustrates four layers. As the Layers grow, so too does the magnitude of the MVDs. Additionally, within each layer, an index identifies the MVD combination being considered. It should be noted that the vertical-MVD components also exhibit the same behavior. The proposed scheme codes, the Layer and subsequently its index to indicate the combination of the LT and RT MVDs to be used.

Figure 1.0: Shows the Layer concept of MVDs used in vector coding.

## Proposed Coding Structure for Affine MVD

Figure 2.0 below shows the proposed decoder structure for the Affine MVD coding. First, a flag i.e., Layerx_GT0 is parsed and checked. If Layer Id is not greater than zero, then the decoder proceeds with the decoding of the vertical component of the MVD. If however, the Layer Id is greater than zero i.e., (Layerx_GT0 == True), then another flag is parsed to check if the Layer Id is greater than one i.e., (Layerx_GT1). If this flag is false, it indicates that the Layer Id is in fact equal to one. Subsequently, the index within the Layer one is decoded and the MVD combination is deciphered. If however, the Layer Id is greater than 1 i.e., (Layerx_GT1 == True), then the smallest Layer Id under consideration is 2. Therefore, the Layer Id is determined by parsing the remaining Layer information (i.e., Rem_Layer) using Exponential Golomb Code with order one. Here, the (Absolute Layer – 2) value is decoded. Furthermore, the index within the Layer is also parsed using Truncated Binary Code. Once, the index is parsed, the combination of MVD value is deciphered.

Figure 2.0 Shows the proposed decoder architecture.

This proposal codes the horizontal-MVDs of both the control points together and similarly codes the vertical-MVDs together.

# 2. Experimental results

The proposed method has been implemented based on BMS1.1 reference software and experimentally evaluated both for BMS according to common test conditions defined in JVET-J1010 [2]. The results were generated for 33 frames.

Table 1: Experimental results for random access test condition; anchor is BMS1.1.

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.33% | -0.12% | 100% | 99% |
| Class A2 | -0.16% | -0.20% | -0.12% | 101% | 100% |
| Class B | -0.14% | -0.31% | 0.22% | 101% | 101% |
| Class C | -0.04% | -0.02% | -0.21% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.08% | -0.03% | 101% | 101% |
| Class D | 0.09% | -0.16% | -0.07% | 101% | 102% |

# Conclusion

We observed that the proposed method provided a gain of 0.09% for the RA. We recommend that the proposed Vector Coding for Affine MVD to be further investigated for more improvements and better trade-offs between performance and complexity.

# Reference

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).